WWIV Network Docs

v2.37

Copyright (c) 1998-2000, WWIV Software Services, LLC

Copyright (c) 1995, Wayne Bell

Contents

1 HISTORY ...........................................

2 INTRODUCTION ......................................

3 REGISTRATION ......................................

4 ORGANIZATION ......................................

5 WWIV SOFTWARE .....................................

6 INSTALLATION ......................................

7 USING THE NETWORK .................................

8 NETWORK FILES .....................................

9 STARTING YOUR OWN NETWORK .........................

10 TROUBLE-SHOOTING NETWORK CONNECTIONS ..............

11 FUTURE DIRECTIONS .................................

Appendix A - Macro Language ..........................

Appendix B - Macro Examples ..........................

Appendix C - Network message types ...................

Appendix D - Identifiers Used in BBSLIST .............

Appendix E - Private Networks ........................

Appendix F - Automated subboard requests .............

Appendix G - Multi-Networking ........................

Appendix H - USING HS/LINK ...........................

Appendix I - Symbols in SUBS.XTR .....................

Appendix J - External Message Interface ..............

Appendix K - WSS Operational Policy ..................

======================================================

1.  HISTORY

======================================================

The first version of WWIVnet Docs was written by Will

Daystrom, also known as The Captain (1 @2370), who ran the

White Star Line and who copyrighted the Documentation for

WWIV v4.10 and WWIVnet Docs under White Star Line Software.

His documentation was excellent and would not have needed

to be rewritten if the WWIV BBS software and the networks

associated with it were not growing and dynamic.  However,

v4.21a of WWIV introduced the ability for a BBS to be on

more than one network, and with that ability many new

networks were started.  This documentation has been

rewritten with those changes in mind, so that the current

documentation will be useful regardless of which WWIV-based

networks a board happens to belong to.  The part of the

documentation detailing the policies and procedures of

WWIVnet has been moved to a separate document.

The second version of the WWIVnet Docs was written by

Filo in 1995, and was copyrighted by Wayne Bell and WWIV

Software Services (WSS) in order that future versions

could be built upon it without anyone's having to worry

about copyright infringements.

This version incorporates all changes up to an including

Net37 and is maintained and distributed by WSS.

======================================================

2. INTRODUCTION

======================================================

A network is a voluntary association of bulletin boards

using WWIV software and participating in a network by

calling one another to facilitate the transfer of elec-

tronic mail (email) and message bases (subs).

Through this network, a user of any of the bulletin

boards that are members may send email to a user of any

other board.  A user may also post on a message base which

may be read by the users of systems which subscribe to that

message base; thus, many of the networked subs have

international distribution.  Because this system of communi-

cation is read by others and because it has an effect on

systems other than the one on which it originates, a spirit

of cooperation must prevail.

Out of this spirit grows systems of organization and

regulation which are known as networks and which are

discussed in separate documents distributed by those

networks.  This document covers some of the aspects of the

software which should be of interest to users and/or sysops

regardless of the particular WWIV-based network(s) with

which they are affiliated.

WARNING: When you unzip the network software, you

should see "- AV" on the line next to every filename.

Furthermore, after the files have been extracted, you

should see the message:

Authentic files Verified!  # XLD658  WWIV Software Services

If you do not see the "-AV" and the above message (be

sure the number is "XLD658"), then the files you have have

been tampered with, and you should not use them.  Since

this software is used by several WWIV-based networks, the

files herein should not be extracted and included in

another network package.  If you desire to distribute

NETxx with another network software, you should include the

NETxx.ZIP file in its entirety within the other network

archive in order to preserve the AV codes.

======================================================

3. REGISTRATION

======================================================

The WWIV network software is distributed as shareware,

which means (in this case) that you can use the WWIV and

WWIV network software for up to two months before

deciding whether or not to register WWIV.  If, at the end

of the two month period, you decide to register WWIV,

please see the 'read.me' file in the WWIV*.ZIP file for

information on how to register.   If at the end of two

months you decide NOT to register WWIV (and hence not use

WWIV software for your BBS), you must stop using the WWIV

and the network software.

If/when you register WWIV (and receive a WWIV

registration number), you have also registered the network

software, and may use the bbs and network software on your

system as you wish -- as a standalone BBS, or in a network.

If you are running a BBS that is in no way based on WWIV

software, but that uses the WWIV network software, then a

similar registration procedure applies.  You can use the

network software for a two month trial period.   If you

decide to continue using the network software after the two

month trial period, then you must register the network

software at a cost of $25.  If you do not register the

network software at the end of the two month trial

period, you must stop using it.  Registration of the

network software for those systems not running a BBS

based on WWIV is done by following the instructions in the

NETREG.FRM included with each NETxx.ZIP packet.

Note that registration of the BBS or the network software

does not include LNET II packaged with the Net37 software.

LNET II is a separately licensed product of CMI Software

bundled with Net37 for your convenience.

4.  ORGANIZATION

======================================================

WWIVnet originally began in 1988 with 25 charter

members who helped Wayne Bell develop the network software

and debug it.  Since that time it has spread from a small

Los Angeles-based system of local boards to several inter-

national networks.

When WSS was purchased in 1998, the WWIVnet and the

software itself were separated from each other to allow

for a more flexible operation and administration of the

network independant of any ties to the software business

and vice versa.

WWIVnet policies are explained in separate document

provided by Snorkel 1@2100, the current Network Coordinator.

You may also want to refer to the WSS Operational Policies

included in Appendix K of this document.

5. WWIV SOFTWARE

======================================================

WWIV v4.20e and up support the automatic request for a

sub and the automatic removal provided that each board has

the network software to support the feature and provided

that the sysop has configured his board to allow auto-

requests.  Beginning with NET32, the automatic request

feature and an option for the description of the sub in the

SUBS.LST is included in BOARDEDIT.  This feature should

facilitate a board's adding or dropping subs that are

subscribed to.

The network software includes the creation of informa-

tional files on each network system which reflects subs or

messages received that have 'no place to go.'  In effect

that would mean that the receiving sysop has not created a

message base for that sub or has deleted it but is still

receiving mail for it.  The sysop should check this file

regularly and if he is receiving subs that he has not

ordered or does not want, he should notify the sending

host to please remove his system from the distribution

list.  This information and other information on network

connections may be found in your GFILES directory in files

named NETDAT0.LOG, NETDAT1.LOG and NETDAT2.LOG.  The

NETDAT0 is the newest log and NETDAT2 is the oldest log.

These logs are only kept for a three day period unless you

make other provisions to have them saved.  They may

provide useful information to you in terms of what you

are/are not receiving and in terms of problems that may

occur in your network connection.  Under v4.22 of the BBS

software the sysop can access these logs from the bulletin

board by typing a comma at the main menu.

The sysop should also occasionally review his DEAD.NET

file which will be in DATA.  Messages in this file are

those which were bound for another system but which cannot

be delivered after having arrived on your system.  Often

these are due to one of two factors.  The first case might

be due to a board's having subscribed to a sub and having

been put on the distribution list for it before that board

has been added to the bbslist.  Thus the system does not

know where to deliver it.  If that is the case, the

DEAD.NET should be left alone because once the system is

added, the network software will deliver that mail to the

new system.    The second case would occur when a board has

left the network or has been temporarily disconnected from

the network.  It may still be receiving mail because either

the sysop failed to notify the host, mail was already in

the system, or because the host sysop has failed to remove

the board from the distribution list.  In that case, the

mail may be safely deleted.  Before deleting the mail,

you should check with the board's AC to be sure that the

board is out of the network.

In addition, the sysop should write the host of the sub

whose mail is going to DEAD.NET and inform the host of

which board is no longer on the network and request that

host delete the board from the distribution list. NET32

and up also has a feature to report on undeliverable e-mail

to another system in the form of a System Short Message

(SSM).

Sysops who receive subs from other systems have the

responsibility to restrict access to the sub according to

the rules of the host.  For example, some subs may limit

access to User Number 1, to users with 255 access, or some

other requirements such as all posts must not have tag

lines.  The receiving sysop must also take steps to inform

users of the rules applying to a particular sub.  GFILES

are often a good way of doing this.

These guidelines for sysops are nothing more than

common sense and normal courtesy which reflect the desire

on the part of all to cooperate in order to make the

network work properly and efficiently.  One of the

interesting features of the network is that it is a great

leveler.  No one (except possibly a few sysops) knows the

age of the person making the post; therefore, people's

impressions of the person who posts is made entirely based

upon the language used and the thought expressed.  As a

consequence many a young user can convey the impression

that he is much older and more mature, and some older users

may convey the impression that they are irresponsible,

illiterate users.  One hopes that users will opt to convey

the impression that they are mature, responsible human

beings.

Sysops may choose to promote responsible use of the

network by asking users to make their network posts conform

to certain suggested guidelines.  For example, the Sysop

may request that users:

o    Not Use Foul language on the network

o    Not make personal attacks against others

o    Not post a lot of one-line messages on the network

o    Learn the differences between using A, W, or P to

respond to network messages.

These are merely suggestions for responsible use of the

network and are not requirements; however, some of those

suggestions are also found in the rules of the hosts of many

network subs.  Where they reflect the host rules, they are

generally considered network rules for that sub.

The following information lets you see at a glance the

type of message (ie where it originated) when reading

message bases:

[1] = messages posted by you

<2> = messages posted by someone else remotely

(3) = messages posted by someone else locally

//BOARDEDIT now manages additional information for

you, such as the host system #, whether a sub you host is

auto-requestable, and whether the sub is automatically

reported for SUBS.LST updates.  This additional info is

stored in the "SUBS.XTR" file in your data directory, but

DO NOT MODIFY THAT FILE DIRECTLY.  Use //BOARDEDIT all the

time.  If you're using net31 or earlier, you'll notice that

your nnall.net, allow.net, and subs.pub files will auto-

matically be updated for you.  With net32 or later, the

files will be deleted, as all the info is now in the

SUBS.XTR FILE.

You can now gate subboards among networks your system

is a member of.  This is done in //BOARDEDIT, simply by

entering net info for multiple networks.  In order to

support sub gating, you need to be using net32 or later.

Do not try listing different sub types in the same network,

as it probably won't work. Gating DOES work with

net-validation.  You can gate subs as either the host or

as a subscriber system, but please do not gate subs unless

you really have a good reason, and know what you're doing.

Additionally, you need the permission of the original sub

host in order to gate messages.

You can now net subboards by sub name (instead of just

sub type).  A sub name is 1-7 chars of upper-case letters

and numbers, and doesn't start with a number.  Note that in

order to use sub-by-name, the host AND all subscribers

need to be using v4.22 (or later) AND net32 (or later).

Other than that, subs-by-name works the same as subs-by-

number.  Because of changes being made by the telephone

companies (see Section 10 - Future Directions), it may soon

be necessary to run v4.22 or better in order to be able to

use the sub-by-name feature in almost all networks.  There

may even be a MANDATORY UPGRADE of both WWIV software and

NET VERSION in the near future as a result of these changes.

If you connect to the same system number in multiple

networks, you can now force a callout to either one (you

are prompted to select which one).  Future versions of the

network software will allow you to pick up all network

messages for your system on one call to the same connecting

system.

You can now (correctly) forward mail between networks.

If you're using net32 or later, if the person you forward

the email to auto-replies, the reply will correctly go back

to the original person who sent the email.  System Operators

also have the ability to Mail a received e-mail to someone

else and add a message of their own.  In this case, a reply

is sent to the sytem operator rather than to the original

author of the mail.  This feature is available in WWIV v4.24

and higher.

6.  INSTALLATION

======================================================

This section takes you step-by-step through the

installation process involved in setting up the network

executbles and other files necessary to the operation of

a WWIV-based network.

�������������������������������Ŀ

� Apply for Network Node Number �

���������������������������������

The first step is to obtain a node number.  The

appropriate method for doing this varies from network to

network and is covered in the documentation specific to the

network that you are interested in.  You may consult the

NETWORKS.LST file included with this documentation to

determine if a "contact person" has been designated for the

network you are interested in.

The node number should be entered in INIT under Option

N.  You may wish to enter information in INIT for a "Net

low time" and "Net high time".  If you do this, it means

that beginning at the low time and ending with the high

time, a user who attempts to log in will receive a message

indicating that your board is only accepting network calls

during that time period.  For example,

Net low time     : 03:00

Net high time    : 05:00

would indicate that the network time period is from 3am to

5am.  Military times are used to indicate time periods in

the network.

Also, in INIT option 1, you should insure that your

board name and telephone number are entered exactly as they

are shown in the BBSLIST.### or the BBSLIST.NET file which

are also discussed later.

�����������Ŀ

�CALLOUT.NET�

�������������

You will have a CALLOUT.NET file which should be placed

in your DATA directory, or in the case of multiple-networks-

-in the date directory for each network, and which will

show the systems that you connect with. This file is in

the following format:

@node [macro options] [transmit options] [callout

options] [password]

Each of these options is discussed in turn.  The node is

the node number of the board you are connecting with.

������Ŀ

�Macros�

��������

Macros are often used to achieve special purposes.

These purposes include (a) connecting with another board

in your area code where it is necessary to dial 1 before

dialing the board number, (b) using a telephone service

such as PcPursuit where special numbers, passwords, etc.,

must be sent, (c) connecting with another board that is

running WWIV as some form of door (i.e., WWIV is not

answering the phone--instead some other software answers).

The macro to use is designated in the CALLOUT.NET by %x

where x is an integer number.  The macro should then be

provided in the DATA directory under the name Mx.NET where

x is the same number.

����������������Ŀ

�Transmit Options�

������������������

Transmit options are basically two.  You may use the &&

parameter which means that files will be transferred each

direction when a connection takes place.  If the && is

omitted then the transfer will be uni-directional; from you

to the other board.  In such a situation, you pay to send

your files to the other board, and presumably, it will pay

to send its files to you.  This is not a very efficient

arrangement and is basically discouraged.

The other transmit option is for network compression.

If you  specify the ; parameter, then network traffic to be

transmitted to that node will be compressed, using pkzip's

compression techniques. PKzip or other compression programs

are not needed, as the compression/decompression code is

built into the network software (using the PkZip data

compression library).  You must ensure, however, that the

system for which you specify compression is using net24 or

higher; if they are using net23 or lower, all compressed

data sent to that node will be lost.

Please do not assume that compression should be used

for all your net connections.  Local connections and high

speed connections that also use V.42bis are probably not

good choices for using compression.  Also, try to avoid

using MNP5 on connections for which compressed data is

sent.  The packets that are created begin with z.   For

example,

Z5252.NET

would indicate a compressed net packet bound for node 5252.

You should check with the Sysop of the other node before

enabling compression between your system and the other.

Some experimentation may be necessary to determine which

connections benefit from compression.

���������������Ŀ

�Callout options�

�����������������

These options affect when your board will call the other

board.

/x   Where x is an integer between one and 9.

This means that the network will force a callout to that

board every x hours, even if there is no mail waiting to be

sent to that system.  This parameter is often set where

one board does not often call the other.

=   This means to call during the hours of 6 pm to 6 am

Monday thru Friday and anytime on weekends).  These para-

meters are handy for those long distance connections

utilizing certain LD calling plans.

-    The minus parameter means to call during times when

rates are  cheapest (11 pm to 7 am and anytime on

weekends).  This parameter is recommended for long distance

connections in order to minimize your phone bill.

!    This limits the number of calls per day to 1.  The

board will attempt to call out starting 20 hours after the

last successful connect.  This feature only works with WWIV

v4.12 or higher.

!x   Where x is an integer.  This limits the number of

calls per day to 20 divided by x.  The board will attempt

to call out every 20/x hours after the last connect.  This

feature only works with WWIV v4.12 or higher.

+    The plus parameter indicates that your board does

not call the indicated node; instead, that node should be

calling you. If both of you have the + parameter in

CALLOUT.NET then no transfers will take place between you.

~    The tilde (~) means that your system will never call

out to the other system, and that the other system will

never call yours to pick up mail.  The other system will

only call yours to send data to you.

^    The caret (^) is used to enable the HSLINK  protocol

between your system and another.  If present on both

systems (and the HSLINK executable is found on both

systems), the network will attempt to use HSLINK as the

protocol instead of Zmodem (or ymodem).  CAUTION: Be sure

you do NOT have the -! modifier in your HSLINK.CFG file.

NET32 now appends that modifier to the command line for

outgoing Net calls.  If the -! is present in the HSLINK.CFG

file, it may cause conflicts that could prevent optimium

performance when recieving Net calls.  See Appendix for

details on setting up HS/Link for WWIV.

*    The asterisk (*) is used to force 10 digit dialing

required by many telephone systems due to the increasing

number of telephones and area codes.

Another set of parameters may be used to designate that

the board should call between certain hours.  These

parameters are illustrated below:

(3 )5     would mean that the board should call between 3

am and 5 am.  Times are specified in 24 hour time.  Mid-

night is specified as 0.

These parameters are useful if you are having a

connection with a board that runs a NET TIME PERIOD or to

insure that local connections take place during non-busy

hours.  If none of these time delimiting parameters are

used, your board will make the connection with the other

anytime that there is mail to go out and the board is not

busy.  This is NOT recommended for long distance

connections unless you are using a WATTS line or other

system that makes long distance a fixed cost.  It may be

used for local connections if desired.

���������Ŀ

�Passwords�

�����������

You should not enter a password in your CALLOUT.NET.

The network software will generate a password between the

two systems once there is a successful connection.  This

is one way that you can tell whether or not your system

has connected with the other. If there is a password

present, there has been a connection. For more

information on passwords, see the section on Trouble

Shooting NetWork Connections.  Also note that the password

will be in quotation marks as the last item on each line

of CALLOUT.NET.  If you lose your password for some reason

such as corrupt files or hard disk failure, you will need

to contact the other sysop to have the password removed

from his CALLOUT.NET file so that a new one may be

established.

����������������Ŀ

�Network Software�

������������������

You should obtain the latest version of the Network

software and place the files in the main directory of your

BBS.  That will be the directory where the BBS.EXE file

resides.  You may determine the latest version of the

software, by asking your network contact person or other

network official to supply you with a copy of it. You

should remain alert for changes in the network version.

Currently, it is NET37; however, future versions will be

released as needed.  It is your responsibility to obtain

these versions once you are on a network.

You may obtain them from The Mountain Empire BBS

(423) 477-4015, the WSS website http://wss.wwiv.com, or from

one of the WWIV Support Boards (you will get a list of these

with the WWIV software, and the list is updated from time to

time), or from one of your networks' officials.

If you are running WWIV v4.30 or later and/or have a

FOSSIL driver insalled, you will want to delete NETWORK.EXE

and rename NETWORKF.EXE to NETWORK.EXE to take advantage of

the FOSSIL based communication routines.

�������������������Ŀ

�BBSLIST and CONNECT�

���������������������

There are basically two ways of using the network

software.  One of these ways is best suited for small

networks and the other lends itself to larger networks.

While both methods will work for small networks, the large

method must be used when a network approaches 500 systems

due to the way that some of the software functions.  A

small network will only need two of the files mentioned

below (BBSLIST.NET and CONNECT.NET) and all nodes will

have a listing in each.  Large networks will use several

BBSLIST and CONNECT files.  All of these files should be

placed in the DATA directory if your board is only on one

network.  If you are on more than one network, then the

files for a particular network should be in its separate

directory as you defined it in INIT option N.

BBSLIST.NET -  For a small network, this file can list

all nodes participating in the network.  The general format

of this file is discussed in the appendices.  A small

network does not need any of the remaining BBSLIST.x files.

In a large network, this file functions as a time/date

stamp and will normally be two bytes long.  If you did

not get a file like this with your network data files and

if things are not functioning properly, you may need to

create this file.  All that is necessary is to create a

file name of BBSLIST.NET and put a single space inside of

it and save it.

BBSLIST.0 - This contains the numbers of valid groups in

the network.  It will look like an N*.NET file, sort of.

All systems in the network will have a copy of this file.

Let us say that a particular network has groups 1,2,3,

and 60.  In that event, the BBSLIST.0 file would appear

as follows:

~754648789   A number such as the one given would appear

on the first line following a tilde.  This

number is a time/date stamp and will change

from time to time.

:A           This lets the software know that the infor-

mation for each group will be found in

separate files rather than in the

BBSLIST.NET and CONNECT.NET.

1

2

3

60           Observe that the group numbers are shown one

per line.

BBSLIST.xxx - This file will have a number in the place of

xxx.  The number will be the group number

and the file will have all of the

BBSLIST.NET entries for that group.  All

systems will have a BBSLIST.1-255 for all

groups listed in BBSLIST.0.  However, the

information in the file will pertain only

to that group.  You may note, at times,

information in your network logs regarding

bbslists ending in 5xx.  Depending upon the

type of software being used by your network

for updates, these probably indicate that a

"partial" update was received and

processed.  Normally after processing these

files (the BBSLIST.5xx and CONNECT.5xx) are

removed from the hard disk.

CONNECT.NET -- For small networks, this file will reflect

the connections of all nodes in the

network.  For a large network, this will

be a time/date stamp type file similar to

that discussed for the BBSLIST.NET.  If

things do not work right and the

CONNECT.NET file is missing, try creating

it with a space inside.

CONNECT.0 - This file will exist on all systems for a

large network and will show  connections

between systems in different groups.  For

example, if area codes 512 and 502 are part

of the same group (group 4), a connection

between boards in those groups is not between

groups, even though it is long distance, and

the connection would not be shown in the

connect.0 file (it would be shown in the

connect.4 file).  However, a connection

between 512 and 213 which are in different

groups would be shown in the connect.0 file.

Note that systems with connections listed

in the connect.0 file will almost certainly

also have connections listed in their local

(group) connection file also.  Some networks

refer to groups as zones or regions so be

alert for this difference in terminology

in the respective network policy documents.

CONNECT.xxx - This file will exist on all systems and

will show connections between systems in

that group.  This will not show connec-

tions to systems in other groups.

If your connection within the group will be calling

you, then you need only wait until you receive the files.

Thus, once this update arrives at the board which will be

calling you, that board will callout to you and you will

receive the necessary files.  One exception to this is that

some smaller networks use a different method of sending

network updates.  If you are on a small, private network,

then you should check with your network administrator

regarding what you should do (if anything) in order to

receive updates.

If you are to call the other board (and it does not call

you), then you will need to keep in touch with that sysop

so that you have an idea of when the network update comes

in.  If you obtain new network files via a download or some

process other than receiving them through the network as an

update, then you may need to force the network software to

do an analysis.  This may be done by using the following

command at the DOS level.  (Note: if you are on more than

one network, you may need to use "dot commands" as

discussed in the appendix.

NETWORK3

That will run NETWORK3.EXE which analyzes your BBSLIST,

CONNECT, and CALLOUT.NET files.  If you add a Y after the

command (ie space Y) then the network will send you a form

letter as feedback letting you know the results of the

analysis and sometimes identifying problems with your

setup.

Although it is natural for you to want to begin to

subscribe to subs and so forth, you should exercise

patience until you are 'officially' in the network.  If

you order subs before your board is official, then your

system will show up as "unknown" and the mail will not

reach you.  Since many hosts of subs like to send new

subscribers the rules regarding posting on that sub, the

fact that you are an unknown system may result in a delay

in your receiving the sub.  If you wait until you are

officially in the network, then these problems are

avoided.

If you are joining more than one network, it is

important that you establish these connections ONE AT A

TIME.  The reason for this is simply that if you have

several CALLOUT.NET files that have not established

passwords and so forth for your connections, it is

possible to have the passwords get confused (ie an

incorrect association between network and password

established).  This may be avoided if you proceed in

an orderly fashion and add one network at a time.

���������������������������������������Ŀ

�Information Needed for BBSLIST listings�

�����������������������������������������

Your Board Name -- exactly as you want it to appear in

the Network.  You may wish to peruse a current listing of

boards on the network in order to select a name that is

unique.

Your telephone  -- this should include both area code and

complete number.

Maximum Baud -- this should be the maximum baud rate that

you can support.  If you are using a high speed modem

capable of more than 2400 bps then you should indicate the

maximum throughput for your modem in modem-to-modem

exchanges.

Some networks will also want either your registration

number (if you are registered).  That information may (or

may not) be reported in brackets in the BBSLIST

information.

The information above will be included in the BBSLIST.XXX

for your group along with a group identification number.

In that listing, some networks identify area coordinators

(AC's) by a ^ next to the telephone number.  Group or Zone

Coordinators are often identified with a %.

FIDONET INSTALLATION

Beginning with Net34 and v4.24 of the WWIV BBS

software, sysops may be FidoNet compatible if they chose to

do be.  Thirdparty add-ons are required to interface with

FidoNet networks.  Installation instructions are included

with those non WSS products

PPP PROJECT INSTALLATION

In 1997, a group of sysops headed by Frank Reid developed

a freeware, open source add-on to allow for the transmission

of network packets via the Internet.  While WSS had some

input and influence in the development of this product, it

is not a WSS product nor is the software itself supported by

WSS.  There have been many hooks placed in both the network

software and the BBS to allow this add-on to be used to its

fullest capacity.  See the PPP Project documentation for

installation and operational instructions.

7.  USING THE NETWORK

======================================================

Using the network is relatively simple.  Sending

netmail, subscribing to network subs and hosting network

subs are discussed in this section.

���������������Ŀ

�Sending Netmail�

�����������������

Netmail is like email on your local system.  That is,

users on your board may send email to one another by enter-

ing either the name or user number of the person that the

mail is to be sent to.  In netmail, you must also use the

node number as part of the addressing scheme.  Suppose that

user number 5 on your system wishes to send netmail to

user 4 (Dr. Seuss) at 4000.  In that case, the netmail could

be addressed as follows:

4 @4000

-or-

Dr Seuss @4000

Please note that this is merely an example; there is no

user named DR SEUSS @4000.  If the BBS is using NET32 or

later and v4.22 or later, and that board is on multiple

networks, the software will provide a listing of choices if

the node number given is not unique (ie exists on more than

one network to which the board is a member.)

Such mail may be sent by the user in one of several

ways. The user could simply use the E option to send email

and then address it properly.  The user might use the A

response to send a private message to someone who has

posted on a national message base, or the user might use

either the A or S to respond to a message received

privately from another system.  Any of these methods will

result in netmail being sent to another system.

Mail may also be sent from one WWIV-based NetWork to

another by using the following type of addressing:

NETWORK NAME #UserNumber AT NodeNumber @XXXX where XXXX

is the gateway node number.  For example to send mail from

WWIVLink to user #1 at node number 1 on WWIVnet, the mail

would be addressed as follows:

WWIVNET #1 AT 1 @4000

That is just an example.  Any node that is running the

NET32 software can function as a gateway node provided that

it has connections in both networks.  Addressing gateway

mail by name is also permissable.  For example, to address

Wayne Bell by name in the previous example, you would use:

WWIVNET RANDOM AT 1 @4000

For those who care, Random is Wayne Bell's handle.

���������������������������Ŀ

�Subscribing to a Netted Sub�

�����������������������������

The method of subscribing to a networked message base

depends upon the version of NETxx that you are using.  To

subscribe to a message base hosted by another system (i.e.,

a netted sub) while using NET30 or earlier, there are 3

things which you must do.  First, you should send netmail

to the host of the sub requesting that she place you on

the distribution list for that sub.  It is a good idea to

name the sub and sub-type in that letter as many people

host more than one netted sub and it will prevent them from

getting confused.  Second (for versions prior to NET32),

you should enter your DATA directory and create a file.

The name of the file should be NNsub-type.NET and it

should have the host's node number in it.  Although you

can do this with an ascii text editor, it may be easiest

to do this from the DOS level with the copy con command.

To accomplish this, type in the following (note information

beginning with -- is commentary and should not be typed)

assuming that you are subscribing to  the WWIV New Sysop's

sub (sub-type 22050; hosted by 2050):

copy con NN22050.NET   --followed by an ENTER or

RETURN

2050 F6                --the F6 means press Function key

6 or you can hold control down

and press Z.

A successful result will result in the message "One

file copied" being seen on your screen.  Please be sure to

put two N's in the NNxxxx.net file.  One N is used for a

host system, so if you put a file  with one N into your

data directory, it will result in messages being doubled,

most disconcerting to the host and other systems which

subscribe to that sub.

Starting with NET29, it was possible to auto-request a

sub.  This meant that if the sysop had listed the sub in a

file called ALLOW.NET in DATA, then the subscriber/sysop

could automatically request the sub.

This method has been further refined under NET32

and v4.22 of WWIV so that the ALLOW.NET is not used and the

NNALL.NET is not used either.  SUBS.XTR contains all of the

pertinent information regarding the sub and the networks

that it is on.

The third step is to either use B (for boardedit) when

you are at  W-F-C or type //boardedit when you are logged

onto your BBS and then set up the message base.  Be sure

to indicate the sub-type number in the option for that:

Completed result for the New Sysop's Sub might look like

the following under v4.22 :

A. Name       : WWIV New Sysop's Forum

B. Filename   : NEWSYS

C. Key        : None

D. Read SL    : 60

E. Post SL    : 60

F. Anony      : No.

G. Min. Age   : 0

H. Max Msgs   : 50

I. AR         : C

J. Net Info   :

Network     Type   Host    Flags

a) WWIVNet     22050  4000

K. Storage typ: 2

L. NetValidate:  No

M. Require Ansi:  No

N. Disable Tag:  No

O. Description:  WWIV New Sysop's Forum for Help & Advice

Since the host of New Sysop's Forum permits visiting

sysops to read and post and since the sysop of this board

assigns an SL of 60 and an AR of C to visiting WWIV sysops,

he can be sure that the host's requirements are met.

Although you are not required to do so, it is a good

idea to send a short thank you or other acknowledgement to

the host to let him know when you begin to successfully

receive messages on the sub.  If the host has special rules

and regulations that you need to inform your users of, you

may do this in several ways.  You could include such

information in a form message (i.e., a message named

FORMxx.MSG where the xx may be either integers or letters)

which you send to all users.  You also might make the first

message on the message base contain the rules and then make

it a permanent message.  If you choose this form, be sure

to make such a message before you get hooked up to receive

the messages, else your message will go out on the network.

Finally, you could put a synopsis of special rules in the

GFILES area and direct your users to read that.

��������������������Ŀ

�Hosting a Netted Sub�

����������������������

As part of some networks, you will receive SUBS.LST, a

listing to be found in your network DATA directory regarding

the various networked subs that are available for you to

subscribe to.  At some point, you may wish to host a netted

sub yourself.  If you do, you should first make sure that

there is not another sub already out there serving the same

need.  If there is, then you should only host a sub on the

same topic because you think that you can do it better or

because yours will have a special slant. On the other hand,

you may have expertise in an area or information on a

subject which is not being currently addressed on your

network and for which you think that there might be a

demand.  In that case, you could decide to host such a sub.

To host a sub, you must create an Nsub-type.NET file in

DATA and in it, you should keep a list of the systems which

subscribe.  The list may be vertical or horizontal as long

as there is a space between numbers when they are placed

horizontally.  Personally, I recommend a vertical listing

from lowest to highest; that way you can easily tell when a

sub has already subscribed.

Because telephone companies are changing the tradition

method of numbering areas codes in 1995, the use of numeric

sub-types will certainly decrease.  Each network will have

its own policy with respect to whether or not numeric

subtypes can be used and if so under what circumstances.

The documentation here, will illustrate sub-by-name rather

than a numeric subtype.

Net32 and V4.22 of WWIV support a Sub-by-Name concept

which means that you may use any legal 6 letters or

characters as a SubType.  This numbering convention will

enable boards to handle more subs than was possible before

under the sub-as-integer concept.  NOTE that future changes

(see Section 10 Future Directions) may effectively

necesitate your being able to use the sub-by-name approach.

Thus if you have not upgraded your WWIV version, you should

give some thought to doing so soon.

You may want to develop a form message to send to those

sysops who subscribe to your sub.  Such a message should

remind them of the name, sub-type, and host and it should

provide any rules that you may have regarding who may

access the sub or who may read/post there.  Also you

should get in the habit of reading your mail regularly and

responding quickly to requests for the sub.  If you have

indicated (under NET32 and version 4.22) that the sub may

be auto-requested, then if you have a file called

SA......NET in GFILES, and if the dots are replaced with

either the subtype number of the first 6 letters of the

sub-by-name, then that letter will automatically be sent

to those who subscribe to the sub with the autorequest

function.

You may wish to advertise your sub.  There are some

National netted subs which have been developed just for

that purpose and you should use them if you can.

The host has the right to run his sub as he sees fit

and may establish any rules he wishes.  The only

restriction on sub topics is that they be legal.  If a host

chooses to 'throw' someone off of a sub,   the individual

has no recourse.  Of course someone may start her own sub

if she wishes.  These rules are followed on most networks,

however, you should refer to the documentation for the

particular networks you are a member of to see if that net-

work has different rules for sub hosts.

WWIV v4.12 and following introduced a feature known as

Net Validation.  This feature may be toggled on or off in

BOARDEDIT.  When it is toggled on, messages will not leave

your system until they have been validated.  A host, when

reading new messages on a sub, may press X to prevent a

message from being sent out.  After reading the messages,

she will be asked, "Net Validate these Messages?"  A Y

response will result in all messages being sent out except

those where an X was pressed.  After all messages have been

read, pressing X will cause them to be sent again.

NOTE: This should not be done for it may result in sending

out duplicate messages across the network.

8.  NETWORK FILES

======================================================

The files discussed below are the Network executable

files which should be placed in the same directory as your

BBS.EXE file.  The files which belong in your DATA

directory are discussed in the next section.

1)   NETWORK.EXE - This file is run when a BBS is calling

another board through the network.  This program handles the

modem and the network security.

2)   NETWORKF.EXE -  This is a FOSSIL based version of

NETWORK.EXE.  If you are running WWIV v4.30 or later or have

a FOSSIL driver installed, delete NETWORK.EXE and rename this

file to NETWORK.EXE

3)   NETWORK1.EXE - This program analyzes P*.NET files to

determine which are for local distribution and which are to

be sent to boards that you connect with.  The latter files

will be stored in DATA in Sxxxx.NET files where xxxx is the

node number of the receiving board, or in Zxxxx.NET files

if the packet is compressed.

4)   NETWORK2.EXE - This program analyzes local mail and

distributes it to the proper message sub or to email.

5)   NETWORK3.EXE - This program analyzes CONNECT.NET and

BBSLIST.NET. The analysis may cause the network software to

leave you messages.  The sysop should read these messages

and respond to them ASAP.  These messages are discussed in

the Appendix.

6)   LNET.EXE - This program allows the deletion of

corrupted messages prior to their being sent over the net-

work.  It can also be used to delete a message which the

Sysop does not want to go out over the network....such as

one which violates the spirit or rules of a Sub.  As a

general rule, a Sysop should not use LNET to read outgoing

messages.  One exception to this is to use LNET to read

the messages in DEAD.NET.  LNET cannot read mail in packets

which have been compressed.

DEAD.NET is a file to be found in DATA for messages

that could not be delivered because the system and/or

routing was in error.  The header for these messages in

DEAD.NET indicates the systems which it is for and the

number of days since it was written.  Sometimes messages

go there because a new board has not gotten set up yet; but

often they go there because one or more of the destination

boards have gone down.  If these messages are several weeks

old, there is probably no harm in deleting the DEAD.NET.

7)   DExxx.EXE - These programs authenticate updates

received from the network or group coordinator.  These files

are not used by all networks and are network specific.  If

you are on several networks, put these files in the network

data directory for each separate network or consult your network

documentation for installation instructions.

8)   NETINIT.C - This file needs to be used only if you

have registered WWIV and have modified the structure or

size of the userrec structure.  If you have modified it,

compile and run NETINIT and it will store information

necessary for the network in the CONFIG.DAT file.

9)   REQ.EXE - This file is provided to allow those

systems that do not run WWIV BBS software a means of auto-

requesting subs.

Additional files may be added to the network as the

development progresses or some of the existing files may be

changed and/or eliminated.

������������������������������������Ŀ

�Files in your Network Data Directory�

��������������������������������������

The files that belong in your network data directory

will vary somewhat from network to network depending upon

whether the "large" or "small" form or file organization is

being used and upon whether or not the network employs

group or zone coordinators who send out updates.  As

mentioned earlier, those executables (like DEx.EXE type

files) belong in the network data directory.

In addition, the BBSLIST.NET and possibly BBSLIST.1,

BBSLIST.2, etc., the CONNECT.NET and possibly CONNECT.1,

CONNECT.2, etc. and the CALLOUT.NET files must be present

in order for the network to function properly.  When the

network analysis is performed by NETWORK3.EXE, additional

binary data files will be created such as CONTACT.NET,

BBSDATA.*, etc.

You may also find FBACKHDR.NET (an information file

distributed by the net coordinator for the network),

SUBS.LST and perhaps SUBS.1, SUBS.2, etc which provide

information on the subs available in the network.

CATEG.NET, an information file that permits sub hosts to

categorize their subs so that they automatically show up

in SUBS.LST according to the proper category.

NETWORKS.LST, a file showing the various WWIV networks and

whom to contact for additional information on that network

as well as information on the coordinator of that list so

that persons creating a network will know whom to contact

to get their network on the listing.

"Dot Commands"

Beginning with NET32, it is possible to use "dot

commands" when it is desireable to force a network

executable to work with a specific network.  If no "dot

command" is used, the program defaults to using the network

executable with data in the DATA directory.  On the

other hand, if you are on more than one network, Option N

in INIT will create a file called NETWORKS.DAT in the DATA

directory.  Dot commands work by using data from the

networks in the order listed in the NETWORKS.DAT file which

is the same order that you see when you use option N in

INIT.  The first network listed is associated with .0; the

second with .1, etc.

For example, to force LNET to run on the DEAD.NET file

in the second network listed, you would use LNET .1.  To

force a network analysis on the third network listed, you

would use NETWORK3 Y .2.  Under NET31, it was necessary to

set environmental variables to make these commands work.

These environmental variables are no longer needed under

NET32 and should NOT be used.

9. STARTING YOUR OWN NETWORK by Jim Nunn, Former NC of IceNET

======================================================

���������������������������������Ŀ

�A BASIC COST-FACTOR WWIV NETWORK �

�����������������������������������

The following will describe a basic two node WWIV network.

The steps involved are recruiting another node, creating

the nodelist files which are common to all systems in your

network, creating unique callout.net files for each node

in your network (usually done by each sysop), adding the

network information in section N of INIT, adding the

networking software to both boards, analyzing the network

(automatically done by WWIV or can be manually done), and

making a network connection via modem to the other node.

First of all, you need to find ONE other system willing to

join your network. Once this recruitment task is complete,

you need to create two network files:

CONNECT.NET

~~~~~~~~~~~

@1  2=0.00

@2  1=0.00

BBSLIST.NET

~~~~~~~~~~~

@1   &  *111-111-1111 #2400  "@1's BBS"

@2      *222-222-2222 #2400  "@2's BBS"

Each node has both these files in their network data

directories.  To establish the network data directory, go

into INIT, section N, and add in the information as

requested in the menu there (Network Name, node number,

data directory).

Each node creates their own CALLOUT.NET file, which goes

in the network data directory, thusly:

For @1:

CALLOUT.NET

~~~~~~~~~~~

@2

For @2

CALLOUT.NET

~~~~~~~~~~~

@1

You'll both need to unzip NET33 into your main BBS

directory.  The files required are the 4 basic networking

files, NETWORK.EXE, NETWORK1.EXE, NETWORK2.EXE, and

NETWORK3.EXE.

When you start the bbs with all these files properly

installed, WWIV will automatically perform a network

analysis.  If you have done everything right, you'll

receive feedback email from your network, and at the

bottom, you'll see the message "Everything is fine".

With these basic steps done, email the other node in the

network, then sit back and watch as WWIV will automatically

callout to the other system within two minutes or so.  Upon

making the first connect, a password will be added to your

callout.net file.   You now have a completely functional

network of two systems.

OTHER CONSIDERATIONS

~~~~~~~~~~~~~~~~~~~~

The basic network above is called a 'cost factored'

network.  You'll notice network cost factors after each

node number in a connect line in the CONNECT.NET file

(e.g. 1=0.00).  These cost factors are taken into account

by the networking software to determine routing of email.

It really won't matter in a two node network just how the

costs are set since regardless of cost, you can only send

mail to one other node. However, by setting a cost factor

on one node higher than another, mail will be routed via

the node with the lower cost factor where that node has

multiple network connects.

By now, you may be wondering why there's no mention of the

more familiar bbslist.0, bbslist.1, bbslist.2, etc type

files you have seen in WWIVnet, WWIVlink, IceNET, etc.

These networks have enough systems in them that the

networking administration task has been split up into

groups, so that several network administrators are

responsible for updating the files, rather than having

one person do it all.

However, it's awkward to use the multiple bbslist/

onnect files for a small network.  WWIVnet started using

them when the network grew to around 575 nodes or so,

IceNET at about 400, and WWIVlink at about 550.  Up til

your network is in that range, you'll find it much easier

to edit only two files instead of the larger number in the

group based network.  Once your bbslist gets to around

20-25k or so, you need to begin thinking about switching to

the group approach.

STARTING YOUR OWN WWIV NETWORK

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

I am frequently asked questions about how to start and run

a network. With the multi-networking capabilities of

WWIV421a+, many of you have thought about how much fun it

would be to have that '1@1' designation by your name.  Let

me assure you, it is INDEED fun. However, when I started

IceNET, along with a few of my friends, there really were

no other major networks besides WWIVnet and WWIVlink, and

IceNET found it's niche because it is something new and

different, with a personality of it's own.

So, how DO you start a network today? It's simple. Get one

other board to join with you, and you have a two system

network, as described above. But if your serious about it,

then I think you should consider several things:

1 - What is the PURPOSE of the network I'm going

to start?

Your network should have a THEME and a PURPOSE.

2 - Your network should have a POLICY and GUIDELINES

for operation.

A policy is a way of acting, or proceding; a

course.

3 - How do I send out network updates?

To keep your network running smoothly, each system

must be using the same nodelists. As the network

grows, you will need to distribute the new files

to each system in a timely and efficient manner.

YOUR NETWORK THEME

~~~~~~~~~~~~~~~~~~

Think of this just like you would think of hosting a

message base, but in this case, you are 'hosting' and

entire network of systems. Everything that goes on in your

network doesn't necessarily have to involve the theme; but

your theme and purpose is the foundation. The theme

provides a special focus for your network, and new systems

interested in your theme will be attracted.

YOUR NETWORK POLICY

~~~~~~~~~~~~~~~~~~~

The policy you establish will be the reference source for

dealing with problems and issues as they arise in your

network. When something is wrong, you should be able to

look in the policy to get some answers about how to

proceed to resolve the problem. You don't have to re-invent

the wheel to write a good policy.  Look for the basics in

the WWIVNET guidelines, which contain sound practices which

have been demonstrated to work quite well. I believe that a

short, elegant, well thought out policy can put your

network on a firm footing right from the beginning.

Involve others in the preparation of your policy, then

establish, maintain, and enforce your policy.

A TECHNICAL MATTER - UPDATING THE NETWORK

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

As your network grows, you will need to edit the

connect.net and bbslist.net files for new systems, remove

systems that drop out, connection changes, and numerous

other things that systems within your network will want

changed.

One big question then, is how to get these edited files

distributed to all systems in the network. There are at

least three methods to do this:

1 - The email method.

2 - The message base method.

3 - Using Wayne Bell's NETUP program, or one of

several NETUP clones.

THE EMAIL METHOD

~~~~~~~~~~~~~~~~

In the beginning, with only a few systems, you can update

your network easily via email, or by just making the files

available for download from your system.  This method is

probably adequate up to about 25 systems.  If your network

grows beyond this size, you will begin to find problems:

o    Some sysops just don't get around to installing the

files

o    Unknown systems will begin to show up in email and

message bases on BBS's where the sysop has not

updated his board with the new files.

THE MESSAGE BASE METHOD

~~~~~~~~~~~~~~~~~~~~~~~

One semi-automatic method of updating the network, that

will work well for up to about 50-75 systems, is to use a

special update message base.  This message base would be

hosted by you, and you would network validate the message

base.  On all subscribing systems (everyone in the network

must subscribe), the message base would be set to ONE

message, storage type 0.

To update your network, zip up the bbslist.net and

connect.net files into one file (called, for example

NODELIST.ZIP), then uuencode that file.  Post the uuencoded

message on the message base.  All subscribing systems would

then have a batch file to run during maintenance, something

like this:

COPY C:\WWIV\MSGS\*. UPDATE.UUE

UUDECODE C:\WWIV\MSGS\UPDATE.UUE

PKUNZIP -o C:\WWIV\MSGS\NODELIST C:\WWIV\DATA

ERASE C:\WWIV\MSGS\*.

ERASE C:\WWIV\MSGS\*.UUE

ERASE C:\WWIV\MSGS\*.ZIP

As soon as WWIV recognizes the changes, network analysis

will automatically run, sending the sysop feedback as

normal.

It is critical that the message base be ONLY one message,

and that there are NO other files without extensions in

\MSGS, which is normally the case. Make sure you delete

any *. files from \MSGS just to make sure when initially

setting up this method.

I'm certain that some of you will see ways to make this

method more efficient, but the above should give you the

concept.  Of course, if you're capable, you could write

special software to accomplish the updates, but the

above method is something that anyone can easily do, it's

relatively secure, since you host the message base and

validate it, and it requires no further sysop intervention

once the message base and batch file is installed.

USING NETUP

~~~~~~~~~~

When IceNET grew to about 75 systems, I realized that

something must be done to improve the updating technique.

We had gone though the stages of using email, a message

base, and even had ICENETUD, a special program written by

Icefreezr, IceNET 3@1 (Alan Carauna) that worked with a

message base somewhat similar to the way Space Dynasty

works.

From a suggestion by DEATH KNIGHT, IceNET 1@7, I emailed

Wayne Bell, telling him 'IceNET needs your help', to which

he graciously responded to by offering me NETUP for $300.

When you buy NETUP, you get three files, the NETUP.EXE,

and two special files DE1.EXE and EN1.EXE.  As the Network

Coordinator for your network, only YOU have EN1.EXE on your

system, and all other systems have the special DE1.EXE

(this file goes into the special DATA directory you set up

in WWIV421A+ just for your network).

When you execute NETUP.EXE, you will see the following menu

on your system.  Just hit the letter, and the rest is

automatic.

Found:     22 AC's    278 systems

Using standard BBSLIST.NET and CONNECT.NET files.

B> Send BBSLIST.NET to network

C> Send CONNECT.NET to network

H> Send FBACKHDR

N> Send WWIVnews

F> Send feedback

R> Check registration info

Q> Quit

Which?

For normal updates, I just type in NETUP B C, and don't

even have to use the menu.  Recently, in order to have the

updating more uniform, I've begun running NETUP in a batch

file during maintenance, which runs every other day.  With

the rapid growth of IceNET in recent times, this frequency

of network updates is required.

If you are going to run a WWIV network, you will not find a

better resource than Wayne Bell to help you with your

software needs for updating your network. As usual with

Wayne's policy, once you've registered NETUP, he will be

there for you to answer questions and provide updates to

NETUP as they are released.

In conclusion, if you want to start a network, then do it

right.  Work with others to establish your THEME, POLICY,

and GUIDELINES.  Use the methods above to begin updating

your network. If you a lucky, and have a lot of friends to

help you like I did, one day you probably will buy NETUP,

as your network will have grown to the point it's a

necessity.

I'd like to thank all of the sysops who are in IceNET, and

in effect, allow me to be a 1@1 for a premium network. I'd

also like to thank Wayne Bell, and Filo for all their

support in so many ways.

Running a network is a lot of work and responsibility.

It's also a tremendous amount of fun. I'm having the time

of my life!

If you have any questions or there is something I can help

you with in starting your network, feel free to email me,

Jim, 1@1, IceNET :).  If you haven't yet joined IceNET, let

me know and I'll send you an application, which also

includes the IceNET Policy Statement. As a member of

IceNET, you can see how we do things by your participation

on the IceNET Sysop's Only sub, or the IceNET Guidelines

Formation Committee sub. Your users will also gain access

to a wide variety of IceNET message bases.

Happy Networking!

Jim 1@1 IceNET, August 1993

10.  TROUBLE-SHOOTING NETWORK CONNECTIONS

======================================================

Utilizing the NETWORK is really very simple.  If you

have tried everything you can think of to remedy a problem

and are unable to do so, contact one of the Sysops of a

SUPPORT Board and enlist aid.  Sometimes there are problems

with the code and/or its compatibility with different

modems; however, those type of problems can only be

addressed after all other avenues have been thoroughly

explored, and even then that may not be the solution.  For

example, many WWIV Sysops have registered the WWIV

software, obtained the source code and made extensive

modifications to the BBS.EXE.  In that event, it may be a

modification that is causing the problem rather than the

network software.

When seeking help, be prepared to provide information

on your system, your modem, the error messages you get and

so forth.  Debugging network problems is usually a process

of eliminating the various possible sources of problems one

by one.  Any information which you can provide to speed

this process makes it easier on all concerned.

A few common problems, and their solutions, are describ-

ed next.

You force callout and the cursor returns to WFC

===============================================

First, be sure that you are attempting to force the

call during any agreed upon hours.  If it is not during

the agreed upon hours, the network software will prompt

you "Are you sure?"  An affirmative response will allow the

call to proceed.  If the call does not connect, you should

double check the CALLOUT.NET, CONNECT.0, and BBSLIST.x to

be sure that no typographical errors were made.  Zeros can-

not be o's, group designators must be correct, etc.

Also, under Net20 and later you should ensure that all

files in your network dATA directory are correct as they

affect your board and the board that you connect with.  For

example, a failure to show your connection will cause an

error.

If no typographical errors were made, run network3.exe

from DOS level and force a reanalysis.  If after doing all

of these things, the board will still not call out, go to

your network data and delete BBSDATA.NET, BBSDATA.IND, and

BBSDATA.ROU as well as CONTACT.NET and rerun the NETWORK3

program which will force the reestablishment of the deleted

files.  This will often cure the problem.  If it does not,

first check your NETDAT0.LOG in GFILES to see if it

provides any clue as to the nature of the problem.  If not,

you should then contact one of the support boards and

explain your problem in full detail.   The command line

NETWORK3 Y will force local feedback to be sent to you from

the network software.  The resulting information may be

useful in determining problems in your network setup.

Your board calls out and gets a "Bad PW" message

===============================================

The "Bad Password" message will show up in your net log

which is readable from WFC by pressing N.  If that is the

case and a successful connection has never been made, then

the remote sysop should ascertain that the CALLOUT.NET is

correct.  If so, then check the CONNECT.x and BBSLIST.x.

If a successful connection has been made in the past and a

password between the two boards has been established, then

try again as line noise may be the culprit.  If the "Wrong

Password" persists over several tries, then it is possible

that a file has become corrupted.  In this event, both you

and the remote sysop need to delete the password which was

generated by the network and let the net software

reestablish the password.  If you have never successfully

connected with the other board, then the error may be due

to the other sysop's not having setup for the connection.

You should contact him and ascertain whether or not the

connection if reflected in his CONNECT.xxx file or

CALLOUT.NET file.  It may also be that a faulty connection

caused his board to create a password whereas yours did

not.

Your board calls and gets a "NO NET" message

============================================

This occurs when an unsuccessful connection occurred.

Often it is only because the other board is busy.  The

remedy is to try again.  If the board is a new board and

you know it is not busy, then both you and the other sysop

should make certain that all files are in the proper

places.  There are several Binary files created by the

network.  These are CONTACT.NET and BBSDATA.*. Sometimes

these files will become corrupted and this may be the

cause of a failure to establish a connection.  You may

delete these two files from the DATA directory and then run

NETWORK3.EXE again.  This re-analysis will recreate those

two files and may cure the problem.  This problem (no net)

may also occur when the modems have connected but are not

recognizing each other's signal. It may be that one modem

thinks the connection is at 14.4k whereas the other thinks

it is at 12k or something else.

11.  FUTURE DIRECTIONS

======================================================

In the near future, WWIV BBS software is likely to

include some of the following features:

Networked File Directories - The NFD concept is

similar to message bases.  You subscribe to a NFD

and when you or one of your users select a file from

that directory, a request is sent to the host system

for that file subject to Sysop approval.

File Transfer System - FTS is based on the FILEnet

file transfer software.  This future enhancement will

allow a network to permit the transfer of files in much

the same way as messages and email are sent today.  This

system further allows the ability to send files as

attachments to messages.

DirectMail - Based on the FidoNet crash mail concept.

Users with a sufficient SL have the option of flagging a

message as DirectMail which will be sent directly to the

destination system (if it accepts DirectMail) upon logoff.

Internal Packet Linking -  IPL is based on FLink by

Dennis Meyers. WSS has aquired the rights to FLink and

will be incorporating the abililty to link packets from

different networks over a single connection in a future

version of the software.

Appendix A - Macro Language

======================================================

A macro is not normally needed for calling another BBS

with the network software.  However, when it is needed for

reasons discussed in the body of the documentation, then it

should be used.  Use of a macro is indicated by using a %x

in the CALLOUT.NET with at least one space following the

node number and before the % sign.  The x should be an

integer.  In the network data directory, a file called

Mx.NET should be created which contains the macro.  The x

should be the same integer as that used in the CALLOUT.NET.

You may have several different macros where they are

necessary.

The network macros should have one of the COMMAND words

listed below on the left margin of the macro, followed by

additional information within quotes following the command

line.

The commands supported are listed below in alphabetical

order.  It is common to have DEBUG as the first line of the

macro.

DEBUG  -- This command should start the macro and should be

followed by an open and close quote ("").  This will cause

results to be echoed to the screen.

DIAL -- This command causes the modem to dial whatever

number that you put there.  (For example, DIAL "631-5841").

You can also use two replaceable parameters with this

command, %1 for area code and %2 for the 7 digit phone

number shown in BBSLIST.x.

FAILURE -- This command allows you to define a failure

condition.  Typically the failure is "BUSY".

PARITY -- This command allows you to set the parity at

something other than 8N1.

SEND -- This command causes whatever is in quotation marks

to be sent via the modem.  The information in quotes must

end  with a left brace ({) which in the macro language

indicates a carriage return.  You can also use the send

command to dial the phone if yout put everything that must

be provided to the modem (for example,

SEND "ATDT631-5841{" would cause the modem to dial the

number indicated.

SETBAUD -- This command allows you to set the baud rate for

the call.

SPEED --  This command allows you to set the comport speed

for the call.

SUCCESS -- This command allows you to define a success

string.

TIMEOUT -- This command will cause the modem to pause for

the number of seconds indicated or until a WAITFOR

condition has been met, whichever comes first.

WAIT -- This command is similar to the TIMEOUT command

except that the modem or macro will wait the number of

seconds indicated regardless of any following WAITFOR

condition.

WAITFOR -- Anything in quotes following this command will

be the condition that the macro waits for before

continuing.

Additional Information on Macros is in Appendix K which is

chapter 24 in this version of the net docs.

Appendix B - Macro Examples

======================================================

In conjunction with DIAL or SEND, you may use %1 for

the area code and %2 for the 7 digit telephone number.  If

you use DIAL, you do not need to use the ATDT command; but

with SEND you need it.

Using the simple script language contained above, you

can develop elaborate scripts.  In our area, we have

written scripts that logged the network onto QBBS and

RBBS boards and selected Door programs which were WWIV

setups which in turn ran the network.

If you have trouble developing the scripts to use for a

particular application, you should be able to get help from

any Support Board.  You will need a text file taken from a

screen capture of what you are logging into.  Then the

support board should be able to help you develop the

appropriate script for you to use.

Dial Without Dialing 1 or area code

DEBUG ""

DIAL "%2"

Dial With a 1 but not area code

DEBUG ""

DIAL "1,%2"

Dial With a 1 and Area Code

DEBUG ""

DIAL "1,%1,%2"

Make a Connection, Wait a few Seconds, Send 22 for Special

Access and then the phone number (without area code)

DEBUG ""

DIAL "555-1212"            (see comments below)

WAIT "5"

SEND "22{"

WAIT "5"

Send "%2{"

That macro would dial a special access number (do not use

the one shown as that is the long distance information

service).  The macro then waits 5 seconds and sends a

special access code (22) and waits another 5 seconds before

sending the phone number (last 7 digits listed in CONNECT.*)

These scripts can be very sophisticated and employ the VERBS

shown in the other section dealing with Macros.  If you

have any problems using these, please feel free to ask a

support board for help.  Remember that Macros should begin

with M, be followed by a number and be placed in the network

data directory for the network that you are calling.  The

CALLOUT.NET file is then modified to show the macro to be

used when calling that particular board.  Consider the

following entry for CALLOUT.NET:

@4001 %1 &&

The %1 informs the software to use M1.NET as the macro.

Appendix C - Network message types

======================================================

The network recognizes various types of major and

minor type messages, and these are often reflected during

the analysis which takes place after a network message is

received.  The information which follows briefly explains

each of these message types.

These types may be expanded in the future.

main type 1 - net info

(all type 1 msgs must be use method==1)

0 - feedback to sysop

1 - bbslist.net

2 - connect.net

3 - subs.lst

4 - wwivnews.net

5 - fbackhdr.net

6 - Additional wwivnews.net text

7 - categ.net

8 - networks.lst

9 - Files sent from NC

main type 2 - email

(no method requirements)

minor type not used

main type 3 - post (from host to subscriber.

numeric sub types only)

(no method requirements)

minor type is sub type

main type 4 - Network Files (future)

main type 5 - pre-post (from subscriber to host.

numeric sub types only)

(no method requirements)

minor type is sub type

main type 6 - external msgs

(no method requirements)

minor type is external message type

main type 7 - email by name

(no method requirements)

minor type not used

(name of destination user at start of msg text)

main type 8 - net edit msgs

0  - A partial update to the BBSLIST information.

1  - A request for BBSLIST information to be changed.

2  - A partial update to the connection information.

3  - A request for connection information to change.

4  - An update to NETEDIT's registration record.

5  - A transmittal of the installation message.

6  - A request for to transmit a registration record.

7  - A response to request for a registration record.

8  - A remote request for a net analysis ("/A").

9  - An ASCII text response to a remote analysis.

10 - Network Editor E-mail and/or automatic feedback.

11 - A message reporting an error condition.

12 - A request for installation and ver information.

13 - A request for a remote node's ALIASES.NET file.

14 - A request for a node's aliases.

15 - A response to the alias request.

main type 9 - subs.lst

0 - subs.lst

1 - subs.1

2 - subs.2

...

main type 10 - UNUSED

main type 11 - bbslist.*

(method must be 1 or ((minor_type % 256)+256))

0..255   - bbslist.<minor-type>

- full bbslist update NC-net

257..511 - bbslist.a<minor-less-256-in-hex>

- full bbslist update GC-NC

513..767 - bbslist.<minor-type>

- partial bbslist up NC-net

main type 12 - connect.*

(method must be 1 or ((minor_type % 256)+256))

0..255   - bbslist.<minor-type>

- full connect update NC-net

257..511 - bbslist.a<minor-less-256-in-hex>

- full connect update GC-NC

main type 13 - unused

main type 14 - group info (from GC)

(method must be 257..511)

0 - email to sysop

main type 15 - ssm

(no method requirement)

minor type unused

main type 16 - sub add request

(no method requirement)

0 - sub by name, sub name first part of text

other - minor-type is sub-type

main type 17 - sub drop request

(no method requirement)

0 - sub by name, sub name first part of text

other - sub type is minor type

main type 18 - sub add response

0 - sub by name, sub name is first part of text

other - sub type is minor type

first byte of text (after sub name, if any) is status

of request

main type 19 - sub drop response

0 - sub by name, sub name is first part of text

other - sub type is minor type

first byte of text (after sub name, if any) is status

of request

1 - not host

2 - not there (can't drop you)

main type 26 - post by name

(no method requirements)

minor type unused

sub type is first part of msg text

This is from subscriber to host, and from host to net

main type 27 - new external

(no method requirements)

minor type is external type

(see Appendix J)

main type 28 - game packs

minor type assigned by WSS

Used for transmitting game data

NOTE: all major type 1, 11, 12, and 14 messages are

appropriately source-verified.

Information for this appendix supplied by Wayne Bell

Appendix D - Identifiers Used in BBSLIST

======================================================

Effective in Net37, the BBSlist identifiers have

been changed.  The previous modem type identifiers have

not been used since net34 was released.  To allow for a

more seamless operation with third party add-ons and to

provide future expanability some of the identifiers have

been changed.  It is IMPERATIVE that network coordinators

make the required changes in their respective network

files as soon as is possible to prevent network problems.

The following are the identifiers in use as of Net37:

$ - System is PPP capable (See PPP Project Documentation)

\ - System runs Fido frontend

| - System is a telnet node

< - System refuses links           (future expansion)

> - System uses FTS/BLT system     (future expansion)

! - System accepts direct connects (future expansion)

/ - System is unregistered

? - System accepts faxes

_ - System is a dead end node

+ - System is network server

= - Unused

& - Network Coordinator

% - Group Coordinator

^ - Area Coordinator

~ - Subs Coordinator

The identifiers above may be stacked together.  For

example, "$\|" would indicate the system participate in the PPP Project,

has a Fido Front End and is telnet capable.

The identifier for a dead-end node should not be used

except in emergencies.  It is a means to force the network

to treat a node that would NOT normally be a dead end node

as if it were one in order to facilitate temporary

re-routing of messages.

Appendix E - Private Networks

======================================================

The network software may be used for any legitimate

private network as long as the user understands that the

author is under no obligation to provide the software which

distributes network updates and as long as the functioning

of the private network does not interfere in any way with

the functioning of other WWIV-based networks.  WSS

assumes no responsibility for insuring the operation of

any private networks utilizing the software.

Further, the WWIV support board Sysops are under no

obligation to explain how to set up and/or operate a

private network.  Of course, even for private networks, in

order to continue using the WWIV network software past the

two month trial period, you must have either registered

WWIV, or (for BBS software that is in no way based on WWIV

software) registered the WWIVnet software.  The NetUp package

developed and distributed by WSS and used by some networks

for updating their network files may be purchased for $100

by contacting 1@50 on most WWIV based networks or

wss@wwiv.com on the Internet.

Appendix F - Automated subboard requests

======================================================

Net29 and above support automated subboard subscriptions.

In order for this to work, BOTH systems (the host and the

subscriber) must be running Net29 or later.  The program

'REQ.EXE' can be used to subscribe or drop subboards.

There are some files associated with automated

subscription under Net29, Net30, and Net 31:

ALLOW.NET (in the DATA directory) - lists subboard types

that are under automated control.  If you host sub type

1701, and you want systems to be able to automatically

subscribe to it, you would have the number 1701 in the

ALLOW.NET file.  If a sub type is not listed in the file,

or the file does not exist, then sysops will not be able to

automatically subscribe to the subboard.

DISALLOW.NET (in the DATA directory) - lists system

numbers that are NOT allowed to automatically subscribe

/drop subboards.  If you want to keep a certain system

from subscribing to any of your subs, place their system

number in the DISALLOW.NET file.

SA*.NET (in GFILES directory) - This is a text file that is

sent, as part of a pseudo-email, to a sysop when they are

added to a sub.  If you host sub 1701, then the file

'SA1701.NET' would be appended as part of a piece of mail

to the sysop that is subscribed to the sub.  It can give

any rules of the sub, etc.

The DISALLOW.NET file may still be used with NET32 and

higher.

SR*.NET (in GFILES directory) - If a system is not

allowed to automatically subscribe to a sub (if the sub

isn't listed in the ALLOW.NET, for example), then this

piece of mail is appended to a message informing the

sysop that he cannot be automatically added to the sub.

The file should list why it isn't under automated control,

and if it would be worth the effort to email the sysop

and ask for it.  This option also works with NET32 and

higher.

The REQ.EXE program is very simple to use.  You need to

know only if you want to add or drop the subboard, the sub

type, and the host.  You then put all this info on a

command-line, such as:

REQ A 1701 1

To REQuest an Add to type 1701 hosted by @1.  To drop the

sub, you'd say:

REQ D 1701 1

You should get email back from the system when you are

automatically added/dropped from a subboard.  You will get

no mail back, and nothing will happen, if the remote system

isn't running net29 or later.  If you request to be added

to a subboard that you don't have configured into your

system (in //boardedit), then when the response comes back

indicating you were added, the network will automatically

request that you be dropped from the subboard (since there

is nowhere for the  messages to go), and you will NOT get

an indication in feedback.

Appendix G - Multi-Networking

======================================================

Beginning with NET31 and v4.21a of WWIV it is possible

to network among WWIV-based networks rather easily.  The

methodology for doing this is discussed more thoroughly in

the documentation for WWIV v4.22.  Key things to remember

about multi-networking:

a)  Each network should be in its own data directory.

b)  Each network should be setup in INIT option N

c)  All files unique to a network should be put in

that network's DATA directory.

d)  Network directories should be DOS legal names.

Appendix H - USING HS/LINK

======================================================

HS/Link WILL work properly on most WWIV systems using its

DEFAULT settings (NO HSLINK.CFG file).  Put HSLINK.EXE in

your main BBS directory. Add a caret (^) to the desired

lines in CALLOUT.NET and have the systems you connect do

the same (it must be present at BOTH ends). Once this is

down, if you have 103k free when the system attempts a NET

callout and HST protocol is NOT in use, it will use

HS/Link for the NET transfer.

This probably will NOT work if you are using PC Pursuit.

(see below)

For more information on using and optimizing HS/Link for

WWIV, download and read WWIVHSLK.ZIP which can be found on

most Source Distribution Systems, and many others. It may

also appear under the filename of HSLKWWIV.ZIP

CURRENT HS/LINK VERSION :

Be sure to use the LATEST release of HS/Link. While the

current version is always compatible with older versions,

you will not get the benefit of the latest enhancements and

fixes if you are using an old version.

LOG FILE

Starting with v1.13D0, HS/Link will create a logfile if

the environment variable HSERR is defined. To create a

logfile called HSLOG.TXT on your C drive in your Gfiles

directory, just add this line to your autoexec.bat file.

SET HSERR=C:\GFILES\HSLOG.TXT  HS/Link will keep appending

to this file, so you will want to provide some means of

erasing, renaming, and or zipping it in your nightly event.

=== CAUTIONS if using a CONFIG file ===

DOWNLOAD DIRECTORY (-U) :

This option controls the destination directory for

incoming files. By default, HS/Link will put incoming files

into the "current" directory.  This is where WWIV is

expecting to find them. WWIV changes to the 'TEMP' direct-

ory before receiving files. The BBS will move the files

to where they belong, so the -U OPTION SHOULD NOT BE USED

for the BBS.

FORCE REMOTE TO USE LOCAL OPTIONS (-!)

This option causes the remote (called) end to use some

of the options specified by the calling end. This does NOT

affect any of the options having to do with security, such

as the upload path, or the overwrite option. It does affect

block size (-s), xon/xoff (-hx), and windows (-w). NET32

will append the -! option to the HS/Link command line for

outgoing calls, thus forcing the remote to use these

settings for NET transfers. Since it is not present on the

command line for incoming NET calls, the calling system

will be able to use the settings that are best for his

phone lines. It SHOULD NEVER BE PRESENT IN THE CFG FILE THE

BBS WILL BE USING!

MINIMUM BLOCKS (-nm) :

Removes block numbers from each block, thus saving a

few bytes. No improvement in performance has been measured.

It's effect is similar to that of Ymodem-G, and the

results can be catastrophic if an error does creep in. This

option SHOULD NEVER BE USED!

SLOW HANDSHAKE (-hs) :

Sends Xoff or lowers RTS during disk I/O. This causes

the computer to signal the modem not to send any data

during disk I/O. It is available for systems with slow disk

access. It may help if you get frequent CRC errors or COM

overruns on clean lines. Be sure NOT TO DISABLE Xon/Xoff

HANDSHAKING IF THIS OPTION IS USED.

Information for this appendix supplied by Lance Halle

Appendix I - Symbols in SUBS.XTR

======================================================

You are reminded that you should not modify these

symbols directly.  Always use BOARDEDIT when making changes

to this file.

Meaning of symbols

A typical entry in SUBS.XTR might look like this:

!8

@Novice & Veteran Sysops Helping Each other with WWIV

#0

$WWIVnet 22050 3 0

!x  =   The number of the Sub on the board (ie the sub

index in BOARDEDIT.

@   =   The long description for the sub (for auto-subs-

info)

#0  =   Flags (currently not used but for future expansion)

$NETWORK  SubType Flags  Host Category

Flags are 1 for auto-addrop, 2 for auto-info (3 for both).

Host is 0 if you are the host (you host it), otherwise the

host system #.

Category will be an option that will permit the sysop to

self-classify a sub according to pre-defined categories,

thus facilitating the maintenance of network wide SUBS.LST

type files.  The categories will be listed in CATEG.NET

which will be found in the network data directory.

Appendix J - External Message Interface

======================================================

Net33 (and later versions) will include a method whereby

messages for external programs can be transferred through

the network.  This does not describe how to read network

data files, or how to write net packets, only how to

integrate a program that does those things with the

network.  (see the WWIV source code, or WWIVnet technical

docs, for info on that.)

All received message packets that are destined for the

local system are stored into "LOCAL.NET" in your network

data directory.  The network2.exe program is then run to

process the messages.  If any messages are of the "new

external" type (main_type_new_extern, 0x001b), then they

may be saved for processing by an external net program.

Each external network program requires a program (.exe

or .com, NOT .bat), which will take as parameters a

filename of a network file (p*.net format), and a network

number identifier (.net_num).  Each external network

program can take as input a contiguous range of

minor_type's.

External network programs are described in the

'EPROGS.NET' which (optionally) exists in the network data

directory.  The eprogs.net file is a text file, and has a

line describing one external net program per line.  Each

line has three pieces of information: the program name

(1-8 chars), the starting minor_type, and the stopping

minor_type.  So, for example, if an external net program

'netprog.exe' is to handle main_type 0x1b, minor types

100-103, then the line in the eprogs.net file for that

processor would be:

netprog 100 103

(If the program handles only one minor_type, then the

starting and stopping minor_types should both be the same,

and equal to the minor_type to be processed.)

As local.net is processed, any main_type_new_extern

messages encountered cause network2 to search the

eprogs.net file to see if any external net programs are

set up to handle that minor_type.  If none are found, the

net message is thrown away.  If there are two or more

programs listed to process that minor_type, only the first

receives the message.  Net messages accepted for an

external net program are stored to a temporary file while

local.net has been processed.  After local.net has been

completely processed, and deleted, network2 runs the

external net programs, in order, passing them the

filename of the temporary net file for that program, and

an indication of which network is being processed.  After

the program exits, the temporary net file is deleted.

Note that it IS allowable for an external net program

to create a p*.net file for outgoing messages, OR to put

net messages to the local.net file which will be

immediately re-processed by the BBS.

Please do not blindly grab minor_types.  If you need a

lot of separate message types, the message type can be

stored in the text part of the message and a single

minor_type used instead.

There are also local.net pre-processors allowed in

net32.  The preprocessors are listed in the 'epreproc.net'

file in the network data directory, one per line.  BEFORE

network2 processes ANY of local.net, each preprocessor is

run, in order, passing each the ".net_num" parameter to

indicate which network is being processed.  The pre-

processor may then scan through local.net, and mark as

deleted (main_type=65535) any message processed by that

preprocessor.  The preprocessor may even append additional

messages to the local.net file, if that is desired.

Appendix K - WSS Operational Policies

======================================================

WWIV Software Services LLC

PO Box 4468

Johnson City, TN 37602

Reference:  WSS Operational Policy as of: 2 January 2000

1. General:  WWIV Software Services (WSS) dictates policy for the

software under its control only and does not attempt to influence

the operation of any network in any manner.  Network policies are

dictated by the Network Coordinator (NC) or administrative body

of the individual networks.  WSS does however, reserve the right

to interpret our policy for the individual user and/or network

administrator(s).  Any question as to applicability or

interpretation of these policies should be brought to the

attention of WSS BEFORE the user or network official take any

action.  Decisions or interpretations rendered by WSS on any

issue are final.  WSS reserves the right to revise, rescind,

expand or alter the terms and conditions of this policy at any

time and may do so without notice.  Revised policy will be

grandfathered in most cases to allow no degradation of

eligibility, status, or position of the individuals it affects.

2. Distribution, Software Registration and License: WWIV BBS and

and NetXX programs are distributed under the shareware concept.

The user has 60 (sixty) days to evaluate suitability of the

software to his or her needs. Not later than the sixtieth day,

the user must register the software using one of the methods

contained in paragraph 3 below, or discontinue use of the

software and remove all operational copies from his or her

computer(s).  The user may download evaluation versions of the

next release to try the software again and the 60-day trial

starts over.  Reinstallation of the same version, for any reason,

after the initial 60-day trial DOES NOT, constitute a new trial

period and is a violation of the shareware license agreement and

is illegal.  When purchasing a registration, the user is not

purchasing the software itself but is purchasing a license to use

the software.  Registration does not imply nor entail ownership

of the software.  The user's license may be revoked by WSS at

any time should he or she violate any provision of this document

or the specifications contained in the license agreements

contained in the distribution archives.

3. Registration Options:  Registration entitles the user to

free upgrades to all releases under the major revision

registered.  For example, if a user registered version 4.10, s/he

is entitled to free upgrades to all 4.xx releases.  Upgrade fees

for major revisions may or may not be charged and if applicable,

will be published well in advance of major releases.  There are

currently two methods of registration are available:

a. Normal Mail in Registration: Registration is $80.00 (US)

for the DOS platform of the BBS Software and $25.00 (US)

for the network software alone for use with other BBS

systems. Pricing for other platforms will be published

as they are released.

b. Registration Payment Plan: WSS offers a payment plan

for the purchase of BBS registrations.  The payment plan

for 4.30 consists of four payments of $25.00 (US).  Each

payment extends the users trial period by sixty days.

ALL PAYMENTS ARE NON REFUNDABLE.  If a user fails to

make scheduled payments on time or to complete the

payment plan within 240 days (8 months), the account

will be in default, the payment plan will be terminated

and no refund will be issued.  The registration number

and passcode are not issued nor is the user registered

until all payments have been made.  WSS will notify any

network administration of payments received based on the

requests of the user.

4. Instance Upgrades: The unregistered shareware version allows

the user to run a single instance of the BBS software.

Registered versions allow two instances.  At the time of

registration or at a later date, the user may purchase upgrades

to allow the operation of more instances.  The upgrade pricing

is as follows:

�������������������������������������������������Ŀ

�Upgrade From  �   2  �   4  �   8  �  16  �  32  �

�������������������������������������������������Ĵ

�Upgrade to 4  �  $20 �      �      �      �      �

�������������������������������������������������Ĵ

�Upgrade to 8  �  $45 �  $25 �      �      �      �

�������������������������������������������������Ĵ

�Upgrade to 16 �  $80 �  $60 �  $35 �      �      �

�������������������������������������������������Ĵ

�Upgrade to 32 � $120 � $100 �  $75 �  $45 �      �

�������������������������������������������������Ĵ

�Unlimited     � $220 � $200 � $175 � $145 � $100 �

���������������������������������������������������

5. Transfer of Registrations: REGISTRATIONS MAY ONLY BE

TRANSFERRED BY WWIV SOFTWARE SERVICES.  Transfer requests must

include Old Owner and New Owner Transfer forms; a new

registration form completed by the new owner and a check or

money order for $25.00 (US).  It is up to the parties involved

as to who pays the transfer fee.  Any sales price beyond the

transfer fee are the concern of the parties involved and are not

the responsibility or concern of WSS.  The necessary forms for

transfers may be obtained by e-mailing WSS.  Transfers will not

be processed until all completed forms and fees are received.

All entitlements to source code and SDS Access for the Old Owner

are terminated upon the proper execution of the transfer.

6. Registration Exemptions:  Subject to the conditions below,

registration exemptions are authorized to be granted by the

Network Coordinators of:

FILEnet     GLOBALnet   IceNet

TerraNet    WWIVLink    WWIVnet

The NC of these networks may grant an exemption for systems

acting in the capacity of network servers.  NC's may place

additional requirements on the system; however, the following

minimum criteria must be met in order to grant the exemption:

a. The requesting sysop must have purchased a valid

registration for at least one copy of WWIV and must be

running a public BBS on the network in question using

that registration number.

b. The primary purpose of the exempt system must be the

movement of network traffic and not operate as a public

dialup system.  User accounts on the server will be limited

to number one accounts of connecting systems generated by

the server operator.

c. The server operator will provide a dialup telephone

number or Internet address (telnet) for the system to allow

the NC or WSS to log on to the system.  This is to verify

that the system is in fact a server system and maintains

compliance with both WSS and network policy.

d. The requesting system must operate for the good of the

network as a whole in the judgement of the network

administration.

Network Coordinators desiring authorization to grant exemptions

should e-mail WSS directly.

7. Source Code Availability: Currently, registration of the BBS

software entitles the user to the source code to the BBS.  If,

at any time in the future, it becomes impossible or too difficult

to control the distribution of source code, this policy may be

rescinded.  FOR SECURITY REASONS, THE SOURCE CODE TO INIT.EXE,

NETWORK SOFTWARE OR NETUP IS NOT AVAILABLE TO ANYONE.  Source

code to the current version of WWIV may be obtained on disk

directly by sending a completed registration form and $10.00 (US)

to WSS.  Registered users may also download the current source

code from an authorized Source Distribution System (SDS).  A

list of dialup SDS Systems is available upon request from WSS.

SDS systems are authorized by WSS to distribute source code

without charge to registered users after verification of their

registration information with WSS.  In order to download the

source code from a SDS, the registered user must mail WSS FROM

the SDS system using the Registration Verification System

installed on all SDS sites.  (Type //SDS from the mail menu) and

provide the following information:

a.  Registration Number

b.  Registration Passcode (if issued)

c.  Real Name

d.  Address at the time of registration (if no passcode

has been issued)

Source code access is also available via the Internet on our

Internet SDS at http://sds.wwiv.com/.  You will be required to

provide a through d above on this site during the application

process.  Processing of these applications is normally

accomplished within three business days.

8. Source Code Modification Publication: Registered users may

publish modifications on sub-boards designed for that purpose on

individual networks.  It is the responsibility of the host of

these subs to ensure that only registered users are permitted to

subscribe to subs where modifications are posted. Source code

modifications that contain more than 50 lines of original BBS

code require a release authorization for publication from WSS.

E-mail the mod in its entirety to WSS.  Releases or Denial of

Release will be issued within five business days.

9. Modification Access Restrictions: Registered sysops will

control access to all BBS modifications available for download,

viewing, reading or capture on their system.  The registration

information for ALL individuals desiring access to modifications

in any form must be verified with WSS before granting access.

Email WSS for the required modification to send access requests.

WSS will only accept validation requests generated by this

authorized modification.

10. Source Code License Agreement: Unauthorized distribution of

WWIV BBS Source Code or portions thereof is a crime.  Registered

and/or unregistered individuals found in violation of the Source

Code License Agreement contained in the source code archive will

be prosecuted to the fullest extent of both civil and criminal

laws.  All registered users should review of the Source Code

License Agreement to ensure they fully understand and are in

compliance with it.  Registered users in violation of this

agreement will have theirregistration and license permanently

revoked in addition to any legal action taken.

11. WSS Source Distribution Systems (SDS):  WSS has several

systems specially authorized to distribute source code to

registered sysops.  The number of systems is limited and only

the most qualified systems are selected for this function.

Meeting all the qualifications of the criteria listed below is no

guarantee of selection.

a. You MUST be the sysop (#1) of your system.

b. You MUST have been a registered sysop for at least 2 years.

c. You MUST have been running your BBS continuously for the

last 2 years.

d. Your BBS MUST be a full time system, open to anyone who

wishes to call.   No part-time boards, closed boards, or

boards that require a new user password to sign on.

e. You MUST be 21 years of age or older and provide a

photocopy of an official photo ID to WSS with your

application.

f. You MUST have been a member of at least one major network

for a minimum of one year. (preferably WWIVnet, WWIVLink

IceNet, TerraNet or FILEnet)

g. You MUST provide a list of at least 20 registered WWIV

Sysops who want to be able to download the WWIV Source

Code from you, listing their real names and registration

numbers.

h. Upon acceptance, you must be willing to sign a

distribution agreement with WSS that outlines your legal

responsibilities and obligations as a Source Distribution

System.

12. WSS Support Board Network:  WSS provides a network of highly

skilled, knowledgeable sysops to the BBS community.  Sysops

selected for this status are time tested and have been recognized

for the amount and quality of support they offer.  Below is a

description of the types of support offered and selection

criteria for each:

a. The Support Coordinator and active Support Boards select

new Support Boards (SB).  To become an official Support

Board, the sysop, as a minimum, must meet the requirements

of 11a through 11d above.  In addition, the following

conditions must be met:

1. The sysop must maintain a library of all official

WSS releases, and a reasonably stocked library of WWIV

support files, readily accessible to the public, without

restrictions or ratios.

2. The system must offer Auto Sysop Validation (ASV)

and/or Guest Sysop Account (GSA)

3. You MUST have been a member of at least one major

network for a minimum of one year. (preferably WWIVnet,

WWIVlink, IceNet, TerraNet or FILEnet)

4. The sysop must be thoroughly knowledgeable concerning

BBS and network operations and will be required to

provide prompt, courteous replies to requests for

assistance as well as extensive assistance to new sysops

who may request help.

b. WSS and the active Core Sysops select Core Support Boards

(CSB).  To be considered for Core Status, the sysop must have

a minimum of 2 years of exemplary service as a Support Board.

There is no application process as with the SB system.  Core

Support Boards are selected solely on tenure and merit of

service.

Support Board status of either type may be removed at any

time by either WSS or the Support Coordinator if the sysop fails

to meet minimum requirements or when individual conduct is

contrary to the spirit and intent of the support network or

relects negatively on WSS or WWIV as a whole.