cFos ISDN FOSSIL appendage, Revision 1.2

The functions are to be called with  AH=0x90  and  the  function

number in AL. See the FOSSIL doc for more information on  FOSSIL

programming and user applications.

Please note that the appendage functions are only active if  you

use a 'cFos/Pro' or 'cFos/Plus' version.

========================================================================

ISDN appendage, appendage code=0x90:

------- 00 - check for ISDN appendage ----------------------------------

INPUT:

AX = 0x9000

OUTPUT:

AX = 0x1969

BX = maximum function number supported

CX = 'cF'

DX = 'os'

NOTE:

if AX != 0x1969, this FOSSIL doesn't feature any

ISDN appendage.

------- 01 - reset ISDN driver/hardware --------------------------------

This function terminates all connections on all ports and resets

the ISDN driver/hardware. If 'cFos' was loaded  with  -jr,  this

function does nothing.

INPUT:

AX = 0x9001

OUTPUT:

AX = 0x0000 on success

------- 02 - get ISDN connection info ----------------------------------

INPUT:

AX = 0x9002

DX = port number

CX = number of bytes to get from appendage

ES:BX = ptr to ISDN info block in application space

OUTPUT:

AX = 0000, success

CX = number of bytes transfered into application buffer

ISDN info block:

WORD    offset   into   additional    infomation

elements in  bytes,  starting  from  the

beginning of the block

FAR PTR incoming call: incoming caller-id

outgoing call: dialed number

far ptr to ASCIZ string, segment first

BYTE    number of b-channels currently in use by

this port

DWORD   connection characteristics

bit 0 = 1   b2-protocol supports control

flow

bit 1 = 1   b2-protocol  supports  error

correction

bit 2 = 0   incoming call

1   outgoing call

all other bits are reserved  for  future

use

WORD    current CPS rate

the  ISDN  info  block  can  contain  additional

infomation  elements  reflecting  national  ISDN

parameters, e.g. LLC/MSN/Subaddress/EAZ/...

Currently defined:

BYTE    type 1 = 1.TR.6

BYTE    requested EAZ

BYTE    Service Indicator

BYTE    Additional Service Indicator

------- 03 - get charging info -----------------------------------------

INPUT:

AX = 0x9003

DX = port

BX = no of b-channel of this port

(0=main channel, 1=first aux channel...)

ffff = sum of all charges on all channels,

independant of carrier status

OUTPUT:

AX = 0000, success

0001, no valid b-channel

BX = num. of charges used for current connection

CX = num. of seconds remaining in current charging  unit

(valid after  2nd  charging  unit,  otherwise  link

duration in seconds)

Note: If no connection is active this  function  can  be

called with BX = 0 (as well as BX = 0xffff).  Then

it returns CX = 0  and  BX  the  total  number  of

charging  unit  of  the  last  call  (main  +  aux

channels)

------- 04 - add/remove b-channel --------------------------------------

INPUT:

AX = 0x9004

DX = port

BX = 1 -> add, 2 -> remove

OUTPUT:

AX = 0000, success

0001, no active connection

0002, connect/disconnect already in progress

After a  call  to  this  function,  'cFos'  initiates  a

connect/disconnect sequence. During  this  sequence  all

calls  to  this  function  result  in  error  0002.  The

completion can be determined by monitoring the number  B

channels in use.

Note: Dynamic CCB will be switched to  static  CCB  when

this function is called.

------- 05 - get ISDN time/date ----------------------------------------

INPUT:

AX = 0x9005

BX = 0000

OUTPUT:

AX = 0000, success

CX:BX = difference between ISDN time and local  time  in

seconds. CX = most significant word.

======= End of Text ====================================================

#