X00 Utility (XU)

Copyright 1994,  by Raymond L. Gwinn

12469 Cavalier Drive

Woodbridge, Virginia 22192

All Rights Reserved

i

INTRODUCTION

XU  is  a   utility  program  intended  to  support   the  Serial

Input/Output (SIO) device driver X00.

XU  combines  several individual  utilities that  were previously

distributed  with X00.  XU  gives the user  the ability to change

most of the X00 command line  options that were specified in  the

CONFIG.SYS file,  or when  X00 was  installed as  a TSR.   Future

versions of XU will provide additional functions such as port/IRQ

re-mapping.

Use this program at your  own risk.  No warranties  or guarantees

are given or implied.

You  may  enter as  many  commands as  you  wish in  a  single XU

execution.   The  only limitation  is the  length of  DOS command

lines.

All references in this manual to the 16550A apply to the 16550AF,

16550AFN,  16C551 aka  16551,  and 16C552  aka  16552.   See  the

appendix  in X00USER.DOC  for a  discussion on  the  various chip

types.

ii

TABLE OF CONTENTS

INTRODUCTION  . . . . . . . . . . . . . . . . . . . . . . . .   i

BOOT  . . . . . . . . . . . . . . . . . . . . . . . . . . . .   1

CAPTURE . . . . . . . . . . . . . . . . . . . . . . . . . . .   2

FIFO  . . . . . . . . . . . . . . . . . . . . . . . . . . . .   3

DTR . . . . . . . . . . . . . . . . . . . . . . . . . . . . .   4

DV  . . . . . . . . . . . . . . . . . . . . . . . . . . . . .   5

LOCK  . . . . . . . . . . . . . . . . . . . . . . . . . . . .   6

Valid baud rates . . . . . . . . . . . . . . . . . . . .   6

Valid parity . . . . . . . . . . . . . . . . . . . . . .   6

data bits per character  . . . . . . . . . . . . . . . .   6

stop bits  . . . . . . . . . . . . . . . . . . . . . . .   7

NASTY . . . . . . . . . . . . . . . . . . . . . . . . . . . .   8

PORT  . . . . . . . . . . . . . . . . . . . . . . . . . . . .   9

SET . . . . . . . . . . . . . . . . . . . . . . . . . . . .    10

STATUS or S . . . . . . . . . . . . . . . . . . . . . . . .    11

USEBIOS . . . . . . . . . . . . . . . . . . . . . . . . . .    12

WATCHCD . . . . . . . . . . . . . . . . . . . . . . . . . .    13

1

BOOT

This function should cause a warm boot.  The BOOT functions works

in  most environments, including  DESQview on most  systems.  The

BOOT function will not work on some 386 systems running protected

mode software.

An example of the usage is:

XU BOOT

One appropriate place  to use this  function is  in a batch  file

that  is controlling  an  automated system  like  a  BBS.   If  a

condition is detected that is considered by the user to be severe

enough, the BOOT command can be issued to restart the system.

2

CAPTURE

X00 is a  FOSSIL serial I/O device driver.   A FOSSIL is normally

entered (called)  using an 80x86  INT 14h.   This means  that the

vector  (pointer)  for interrupt  14h must  point  to X00.   Some

programs will alter the  interrupt 14h vector (pointer).   If the

INT 14h vector is changed, X00 may not function properly.

Application  programmers please note that the  INT 14h vector can

be  avoided  by using  the HLLAPI  routines  included in  the X00

distribution file.

The  CAPTURE function is provided to instruct X00 to take control

of, or to release the INT 14h vector.

Examples of usage are:

XU CAPTURE:ON

XU CAPTURE:OFF

XU CAPTURE

CAPTURE:ON  means  that  X00  will take/re-take  the  vector  for

interrupt  14h.   That  is,  X00  will make  the  INT 14h  vector

(pointer) point to the X00 function dispatcher.  The value of the

old INT  14h vector  is saved.   X00  does not save  the INT  14h

vector if it already points to X00.

CAPTURE:OFF  means that  X00 is  to release  the INT  14h vector.

When X00 is initialized,  or when a CAPTURE:ON is issued, the INT

14h  vector  is saved.   The  CAPTURE:OFF  function tells  X00 to

restore the  last INT  14h vector  that it saved.   It  should be

noted  that the  last  INT  14h  vector that  X00  saved  is  not

necessary the correct vector.  An explanation is beyond the scope

of  this  document.    CAPTURE:OFF  will  also  disable the  BIOS

emulator.   CAPTURE:OFF  does  not deactivate  any FOSSIL  active

ports.

If  you  are having  problems with  a  FOSSIL aware  program, try

CAPTURE:ON  just  prior to  executing the  program.   If  you are

having problems  with a communications program that is not FOSSIL

aware, try CAPTURE:OFF just prior to executing the program.

This function takes effect immediately upon execution.

3

FIFO

This function is  provided to give the  user control of FIFOs  in

the  SIO  chip.   At  this  time the  only  FIFOed  chip that  is

supported is the 16550A  and later versions.  Other  FIFOed chips

are being worked on and will be supported in the future.

Some early PS/2  systems had 16550 (non A) chips  installed.  X00

will identify the non A versions (and tell you that you have one)

but it will  not use the FIFOs.   The FIFOs of the  16550 (non A)

are not reliable.

If  the SIO  chip is  not a  FIFOed device,  this command  has no

effect.

Examples of usage are:

XU FIFO:15

XU FIFO:8

XU FIFO:OFF

X00  defaults to  8 for a  16550A.   If an invalid  FIFO value is

specified, the default value will be substituted.

When  a number  parameter is  given, it  specifies the  number of

characters that can be loaded  into the transmit FIFO of  the SIO

chip.  The receive FIFO is always set at the maximum.

The  FIFO:OFF  command disables  both  the  receive and  transmit

FIFOs.

I have seen problems with the 16550A when the  FIFO is set to the

maximum of 16.   The problems go away with the FIFO  value set to

15.  If you are seeing an occasional error during file transfers,

try lowering the FIFO value.

This  command will  not take  effect for  ports that  are already

FOSSIL  active.  Once the port is reactivated, the specified FIFO

parameters will take effect.

4

DTR

This function is provided to turn on  and or off DTR for a  given

port.

Examples of usage are:

XU DTR:0:ON

XU DTR:0:OFF

XU DTR:0:OFF DTR:0:ON

In the third  example note that DTR is first  turned off and then

back  on in the  same execution of  XU.  Some  modems may require

that  DTR  be  off  for  a  given  period  of  time  before being

recognized as  a valid off.  If this  is the case for your modem,

turning DTR off, then on in the same execution may not work.

5

DV

This  function  is provided  to  support  software running  under

DESQview.  Normally DESQview will provide a  full time slice to a

program whether it is doing anything productive or not.  However,

DESQview provides a method,  called PAUSE, that allows  a program

to end its current time slice.

Examples of usage are:

XU DV:ON

XU DV:OFF

When the DV option is enabled (DV:ON) X00 will make a guess as to

whether  the program is doing anything  productive.  For example,

if the program is simply waiting for the next character and there

is no character  available, then  time can be  made available  to

another program running under DESQview.   DV:OFF will disable the

releasing of time to other programs.

As stated  above, X00  tries  to guess  if the  program is  doing

something productive.  In some cases for some programs, the guess

is  going to be wrong.  That  is, X00 will incorrectly assume the

program is, in effect, idle when it  is not.  In such cases, your

application program may  slow down considerably.  The  only thing

you can do to correct this is to issue a DV:OFF command.

As time goes on, I believe X00's guesses will get better.  So, if

you  have problems with DV:ON, try it  again with new releases of

X00.

This function takes effect immediately upon execution.

6

LOCK

Some modems allow  or require a fixed baud rate from the computer

to the modem.  This function is intended to support those modems.

Note  that  LOCK does  not set  the  specified parameters  in the

hardware  when   issued.    See  the  following  SET  command  to

immediately set the operational characteristics  of a port.  This

function locks  the operational  characteristics only for  FOSSIL

aware programs  and programs using  the BIOS emulator.   Programs

that  perform direct I/O to  the serial I/O  registers can change

the operational characteristics.

Examples of usage are:

XU LOCK:0:19200

XU LOCK:0:19200:8N1

XU LOCK:0:OFF

XU LOCK:0:7E1

XU LOCK:0:OFF:7S1

The command  LOCK:0:19200 will cause  the computer to  modem baud

rate (for logical port 0, usually COM1) to be 19200  baud even if

the application program requests that the baud rate be changed to

another value.  RTS/CTS  hardware handshaking is implied (forced)

when a locked baud rate is specified.

Valid baud rates for  locking are: 75, 110, 134.5, 150, 300, 600,

1200,  1800, 2000,  2400, 3600, 4800,  7200, 9600,  19200, 28800,

38400, 57600, and 115200.

Valid  parity etc settings can  be made up  of any combination of

the following first second and third characters.

First     Second    Third

5         N         1

6         E         2

7         M

8         O

S

The first number specifies the number of data bits per character.

7

The second character specifies the parity as follows:

N = None

E = Even

M = Mark (always 1)

O = Odd

S = Space (always 0)

The third character specifies  the number of  stop bits, 1 or  2.

If 5 data bits are specified, 2 really means 1.5 stop bits.

The  command LOCK:0:OFF turns the  locking off for  both baud and

parity for port 0 off.  Request from  the application program for

baud rate changes will be honored.

If the indicated port (0 in the above examples) is FOSSIL active,

the locking or  unlocking will not take effect until  the port is

reactivated.

Note that a single OFF parameter ie XU LOCK:0:OFF will reset  all

locked parameters on the specified port.  If you wish to turn off

only one of the locked parameters, use a command similar to this:

XU LOCK:0:19200:OFF

8

NASTY

This  function is not very nice to  other programs.  It will also

slow down processing when it is  in effect.  NASTY should only be

turned on if you can not get your program to work any other  way.

There is  also a good chance  that this option will  lock up your

system randomly.

This option will cause  X00 to alter memory of programs that have

taken interrupt vectors  away from X00.  This may  allow the user

to achieve desired results in a hostile program environment.

Examples of usage are:

XU NASTY:ON

XU NASTY:OFF

Again, use this function only if it is absolutely necessary.  The

potential for subtle and major problems is not worth it if you do

not need the NASTY feature.

This function takes effect immediately upon execution.

9

PORT

This function  allows the user  to make a  port FOSSIL  active or

inactive.    This  function  is provided  to  (attempt  to)  make

programs that are not FOSSIL  aware use the FOSSIL.   Thus, these

programs gain the advantages of interrupt driven serial I/O.

Examples of usage are:

XU PORT:0:ON

XU PORT:0:OFF

The command PORT:0:ON  will make  the indicated port  (0 in  this

case)  FOSSIL active.  This means that interrupts will be enabled

for the port and I/O will be buffered.

The command PORT:0:OFF will restore  the indicated port to normal

operation.   The  OFF command  is also  useful for  poorly tested

programs  that unintentionally  leaves the  FOSSIL hot  when they

terminate.

Several of  the alpha testers of  XU seemed compelled to  issue a

PORT:0:ON  command prior to running a  FOSSIL aware program, like

BINKLEY or OPUS, with mixed results.  There is no need to turn on

the  port  for  FOSSIL aware  programs.   In  fact  it  may cause

problems if you do.

You should  not use XU to  activate or deactivate a  port that is

currently being used by a program.  For example, do not use XU in

one  DESQview window  to activate  or deactivate  a port  that is

being  used by a program in another window.  Random problems will

result if you do.

Remember,  this function is intended to get programs that are NOT

FOSSIL  aware to  use  the FOSSIL.    You must  not activate  and

deactivate ports for FOSSIL aware programs.

This function takes effect immediately upon execution.

10

SET

The syntax for  the SET command is identical  to the LOCK command

above.   This command is  use to immediately  set the operational

characteristics of a serial I/O port.  This command does not lock

the settings.  An application program using INT 14h is allowed to

change the operational characteristics.

Examples of usage are:

XU SET:0:19200

XU SET:1:1200:8N2

XU SET:0:115200

11

STATUS or S

This command will display the current operational characteristics

of X00 and the serial I/O ports that X00 knows about.

12

USEBIOS

This function is  provided to maintain  compatibility with  older

versions of X00.

Out of self  defense, I found it necessary to  include a BIOS INT

14h emulator  in X00.   The emulator processes  INT 14h  calls to

serial  I/O  ports that  are not  FOSSIL  active.   This function

allows the user to enable and disable the emulator.

Examples of usage are:

XU USEBIOS:ON

XU USEBIOS:OFF

The USEBIOS:ON command  tells X00 to pass  calls to the BIOS  INT

14h processor.  That is, it disables the X00 BIOS emulator.

The  USEBIOS:OFF command tells X00  to use the  BIOS emulator for

INT 14h calls to FOSSIL inactive  ports.  That is, it enables the

X00 BIOS emulator.

Many  DOOR  type  programs would  not  work  with  X00 until  the

emulator was added.  By default, X00 uses the emulator.

If a  program normally uses  BIOS INT 14h  (and it will  not work

with a FOSSIL),  you will probably see improved  performance when

the  BIOS emulator  is  used.   Especially if  you have  a 16550A

installed.

The  emulator provides X00 with  a fixed frame  of reference with

respect to the serial I/O ports.   The emulator should be enabled

(default or USEBIOS:OFF) for the vast majority of programs.

This function takes effect immediately upon execution.

13

WATCHCD

This  function is  provided to  enable and  disable a  monitor of

carrier detect.

A description for  the need of this function  is beyond the scope

of this document.  The reason is obvious to those who need it.

Examples of usage are:

XU WATCHCD:0:ON

XU WATCHCD:0:OFF

The WATCHCD:0:ON command enables the monitor.  If carrier for the

specified  port (0 in the  above examples) goes  away, the system

will be booted.

The WATCHCD:0:OFF command disables the monitor.

This function takes effect immediately upon execution.

14

INDEX

16550 (non A) . . . . . . . . . . . . . . . . . . . . . . . . . 3

16550A  . . . . . . . . . . . . . . . . . . . . . . . .  i, 3, 12

16550AF . . . . . . . . . . . . . . . . . . . . . . . . . . . . i

16550AFN  . . . . . . . . . . . . . . . . . . . . . . . . . . . i

16C551 aka 16551  . . . . . . . . . . . . . . . . . . . . . . . i

16C552 aka 16552  . . . . . . . . . . . . . . . . . . . . . . . i

BOOT  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 1

CAPTURE . . . . . . . . . . . . . . . . . . . . . . . . . . . . 2

Data bits per character . . . . . . . . . . . . . . . . . . . . 6

DTR . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 4

DV  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 5

FIFO  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 3

FOSSIL  . . . . . . . . . . . . . . . . . . . . 2, 3, 6, 7, 9, 12

INTRODUCTION  . . . . . . . . . . . . . . . . . . . . . . . . . i

LOCK  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 6

data bits per character  . . . . . . . . . . . . . . . . . 6

parity . . . . . . . . . . . . . . . . . . . . . . . .  6, 7

stop bits  . . . . . . . . . . . . . . . . . . . . . . . . 7

Valid baud rates . . . . . . . . . . . . . . . . . . . . . 6

NASTY . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 8

Parity  . . . . . . . . . . . . . . . . . . . . . . . . . .  6, 7

PORT  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 9

RTS/CTS hardware handshaking  . . . . . . . . . . . . . . . . . 6

SET . . . . . . . . . . . . . . . . . . . . . . . . . . . . .  10

STATUS or S . . . . . . . . . . . . . . . . . . . . . . . . .  11

Stop bits . . . . . . . . . . . . . . . . . . . . . . . . . . . 7

TABLE OF CONTENTS . . . . . . . . . . . . . . . . . . . . . .  ii

USEBIOS . . . . . . . . . . . . . . . . . . . . . . . . . . .  12

WATCHCD . . . . . . . . . . . . . . . . . . . . . . . . . . .  13