Technical Product Summary

Premiere/PCI

Baby-AT Motherboard &

Expandable Desktop Platform

Baby-AT Motherboard

BP5D60AT

Expandable Desktop Platform

SP5D60DT8F

OmniRACK Rackmountable Platform

ORP605P8

Version 1.0

October, 1993

Order Number PRELIMINARY

Intel Corporation makes no warranty for the use of its products and assumes no responsibility for any errors which may appear in this document nor does it make a commient to update the information contained herein.

Intel Corporation retains the right to make changes to these specifications at any time, without notice.

Contact your local sales office to obtain the latest specifications before placing your order.

The following are trademarks of Intel Corporation and may only be used to identify Intel products:

AboveÔ

ActionMediaâ

DVIâ

EtherExpressÔ

ExCAÔ

FlashFileÔ

i287Ô

i386Ô

i387Ô

i486Ô

i487Ô

i750â

i860ä

i960â

ICEä

iCOMPä

Indeoä

Intel287Ô

Intel386Ô

Intel387Ô

Intel486Ô

Intel487Ô

Intel Insideâ

iPSCâ

LANDeskÔ

MCSâ

NetPortÔ

OverDriveÔ

OverDriveä Ready

ParagonÔ

PentiumÔ

RapdiCADä

SatisFAXtionâ

StorageExpressä

TokenExpressÔ

The Indeo, OverDrive and Pentium logos are trademarks and the Intel and Intel Inside logos are registered trademarks of Intel Corporation.

Intel Corporation and Intel’s FASTPATH are not affiliated with Kinetics, a division of Excelan, Inc. or its FASTPATH trademark or products.

MDS is an ordering code only and is not used as a product name or trademark. MDS is a trademark or registered trademark of its respective owner.

Additional copies of Intel literature may be obtained from:

Intel Corporation

Literature Sales

P.O. Box 7641

Mt. Prospect, IL 60056-7641

TRADEMARK ACKNOWLEDGMENTS

Centronics is a trademark of Centronics Data Computer Corp.

IBM, MDA, MCGA, EGA, VGA, Personal System/2, PS/2, PC/XT and PC/AT are trademarks of International Business Machines.

PolySwitch is a registered trademark of Raychem Corporation.

All other trademarks are the property of their respective owners.

ã INTEL CORPORATION, 1993

Premiere/PCI Baby-AT Motherboard & Expandable Desktop

Technical Product Summary

Table of Contents

µIntroduction	4

Board Level Features	5

CPU

Performance Upgrades

Second Level Cache

System Memory

Expansion Slots

Peripheral Component Interconnect (PCI) Chip Set

SMC 37C665 Super I/O Controller

Keyboard Interface

Dallas DS12887 Real Time Clock, CMOS RAM and Battery

System BIOS

Connectors

Power Consumption

System Level Features	12

Chassis

Peripheral Bays

Fan

Expansion Slots

Front Panel

Back Panel

Power Supply

Floppy Drive

Speaker

Security

Chassis Color

Premiere/PCI OmniRACK Rackmount Chassis	14

Performance Benchmarks	15

Optional Accessories	15

Appendices

A	-	User-Installable Upgrades	16

B	-	Jumpers	17

C	-	AMIBIOS Setup	18

D	-	AMIBIOS Error messages and Beep Codes	21

E	-	BIOS Upgrades and Recovery	23

F	-	Memory Map	23

G	-	I/O Map	24

H	-	Interrupts & DMA Channels	24

I	-	Connectors	25

J	-	Environmental Standards	30

K	-	Reliability Data	30

L	-	Customer Support	31

# Introduction

The Premiere/PCI Expandable Desktop is a state-of-the-art platform delivering the latest Pentiumä microprocessor and I/O technology on an industry-standard Baby-AT form factor motherboard. It is ideal for demanding desktop applications requiring the maximum in I/O expansion flexibility.

A Pentium processor is complemented by a 256 KB second level write-through cache to provide workstation- level computing performance. A Pentium OverDriveä socket provides access to future performance enhancements, and SIMM sockets support up to 128 MB of system memory.

The Premiere/PCI Baby-AT motherboard offers outstanding I/O capabilities. Three PCI local bus slots provide a high bandwidth data path for data-intensive functions such as graphics and disk I/O. An integrated SMC 37C665 Super I/O component provides floppy and IDE drive interfaces, as well as two FIFO serial ports and an Enhanced Capabilities Parallel port. Five ISA slots complete the I/O mix.

The Premiere/PCI Baby-AT Motherboard provides the foundation for cost effective, high performance, highly expandable platforms which deliver the latest in CPU and I/O technology.

## Standard Baby-AT Form Factor

The Premiere/PCI Baby-AT motherboard conforms to the industry standard Baby-AT form factor. It meets the specifications for maximum board size, board mounting locations, expansion slot placement, and keyboard connector location. In addition to complying to the standard, the Premiere/PCI Baby-AT provides three slots which will accommodate PCI expansion cards.

### µ §

Figure 1. Premiere/PCI Baby-AT Motherboard dimensions.

# Board Level Features

## CPU

The Premiere/PCI Baby-AT Motherboard is designed to operate with a 60 MHz Pentium microprocessor. This processor is compatible with the 8086, 80286, i386 and i486 CPUs. It supports both read and write burst mode bus cycles, and includes an on-chip 16 KB cache which is split into 8 KB code and 8 KB data caches employing a write-back policy. The Pentium processor also integrates an advanced numeric coprocessor which significantly increases the speed of floating point operations, while maintaining compatibility with i486äDX math coprocessors and complying with ANSI/IEEE standard 754-1985.

The motherboard contains two mounting holes which allow a dedicated fan to be located directly in front of the CPU socket. A 12 volt connector suitable for a CPU fan is located close to the mounting holes.

## Performance Upgrade

A 273-pin Type 4 Zero Insertion Force socket provides users with a performance upgrade path using Pentium OverDriveä processors. The OverDrive component being developed for use in this socket will provide performance beyond that delivered by the Pentium microprocessor.

## Second Level Cache

The Pentium processor's internal cache is complemented with a 256 KB direct mapped, write-thru second level cache implemented with eight 32Kx8, 15 ns SRAM devices. Tag and control logic is contained in the 82434LX PCMC core chip.

## System Memory

The Premiere/PCI motherboard provides four 72-pin SIMM sites for memory expansion. The sockets support 256K x 36 (1 MB), 512K x 36 (2 MB), 1M x 36 (4 MB), 2M x 36 (8 MB), 4M x 36 (16 MB), and 8M x 36 (32 MB) SIMM modules. Minimum memory size is 2 MB and maximum memory size, using four 8M x 36 SIMM modules, is 128 MB. Memory timing requires 70 ns fast page devices. Parity generation/checking is provided for each byte. The parity generation/checking circuitry can be disabled to allow the use of non-parity 32 bit SIMMs.

The four sockets are arranged as Bank A and Bank B, with each bank consisting of two sockets and providing a 64-bit wide data path and 8 parity bits. Both SIMMs in a bank must be of the same memory size and type, although Banks A and B may have different types of memory installed. Bank A only, Bank B only, or both banks may be populated. There are no jumper settings required for the memory size, which is automatically detected by the system BIOS. Tin lead SIMMs should be used when adding DRAM.

The memory array is controlled by the Intel 82434TX PCMC and data buffering is provided by two Intel 82433LX Local Bus Extension devices.

## Expansion Slots

Up to seven expansion slots may be populated on the Premiere/PCI Baby-AT motherboard. Expansion cards are oriented perpendicular to the motherboard.

There are five ISA bus expansion connectors and three PCI expansion connectors. One slot is shared by connectors that will accommodate either an ISA or a PCI expansion card, but not both at the same time (accounting for the disparity between the number of slots and connectors). All three PCI expansion slots accept PCI master cards to fully support the PCI specification.

The motherboard design limits the length of add-in cards in three slots. The height of the CPU heat sink limits the use of the two PCI connectors at J10C1 and J10C2 to less than full-length devices, and a full-length card may not be installed in the ISA connector at J10F1 if a SIMM is installed in the vertical SIMM socket (J3F1).

### PCI 3.3 Volt Capabilities

To maintain strict compliance with the PCI specification, the motherboard provides a connector which can be used to route 3.3 volt power to the PCI slots. The connector may be used with a separate 3.3 volt power supply or with a custom designed voltage converter.

### IDE SUPPORT

PCI slot 3 (J10C2, the PCI slot closest to the 3.3V connector) makes use of some of the reserved pins on the PCI interface to support a PCI IDE paddle card for improved disk performance. The use of the reserved pins will not affect the operation of any PCI add-in cards as of this publication date. This paddle card is discussed in more detail in the Optional Accessories section. A detailed pinout for PCI slot 3 appears in Appendix I.

## PERIPHERAL COMPONENT INTERCONNECT (PCI) Chip Set

The Intel Mercury chip set consists of one 82434LX PCI/Cache/Memory Controller (PCMC), two 82433LX Local Bus eXtension (LBX) devices, and one 82378IB System I/O (SIO) bridge chip. The Mercury chip set provides the following functions:

· 	CPU reset control

· 	CPU L1 cache control

· 	CPU burst mode control

· 	CPU interface control

· 	Integrated second level write-through cache

controller with tag comparator

·	Fast Page-mode DRAM controller

· 	Burst memory read/write control logic

· 	Data bus conversion to PCI

· 	Parity generation/detection to memory

· 	AT-BUS direction control

· 	Chip select for keyboard controller and RTC

· 	Speaker control

· 	NMI logic

· 	Floating-point coprocessor interface

· 	Keyboard reset, gate A20 emulation logic

· 	DMA controller

· 	Interrupt controller

· 	Counters/Timers

### 82434LX PCI/Cache/Memory Controller (PCMC)

The 82434LX provides all control signals necessary to drive a second level cache and the DRAM array, including multiplexed address signals. It also controls system access to memory and generates snoop controls to maintain cache coherency.

### 82433LX Local Bus eXtension (LBX)

There are two 82433LX components which provide data bus buffering and dual port buffering to the memory array. Controlled by the 82434LX, the 82433LX devices add one load each to the PCI bus and perform all the necessary byte and word swapping. The devices include memory and I/O write buffers.

### 82378IB System I/O (SIO)

The 82378IB integrates seven 32-bit DMA channels, five 16-bit timer/counters, two eight-channel interrupt controllers, NMI logic, ISA refresh address generation, and PCI/ISA bus arbitration circuitry together onto the same device.

Detailed information on the chip set is available in the Intel 82430 PCISet data sheet.

## SMC 37C665 Super I/O Controller

Control for the integrated serial ports, parallel port, floppy drive and IDE hard drive interface is incorporated into a single component, the SMC FDC37C665. This component provides:

·	Two NS16C552-compatible UARTs with send/receive 16 byte FIFO

·	Multi-mode bi-directional parallel port

- Standard mode; IBM and Centronics compatible

- Enhanced Parallel Port (EPP) with BIOS/Driver support

- High Speed mode; Enhanced Capabilities Port (ECP) compatible

·	Industry standard floppy controller with 16 byte data FIFO (2.88 MB floppy support)

·	IDE hard disk decode and chip select

Header pins located near the back of the board allow cabling to use these interfaces. The serial ports can be enabled as COM1 and COM2, or disabled. The parallel port can be configured via the Setup program as LPT1 or disabled, and can be set as bi-directional or output only when enabled. The IDE interface supports Compatible, Type B, and Type F DMA.

## Keyboard Interface

An Intel 8742 surface mount microcontroller contains the Phoenix Technologies-compatible keyboard/mouse controller code. An AT-style keyboard connector is located on the back panel side of the motherboard. The 5V line on this connector is protected with a PolySwitch* circuit which acts much like a self-healing fuse, re-establishing the connection after an over-current condition is removed. While this device eliminates the possibility of having to replace a fuse, care should be taken to turn off the system power before installing or removing a keyboard.

The 8742 microcontroller code supports Power-On/Reset (POR) password protection. The POR password is set via the Setup program. The keyboard controller also provides for the following "hot key" sequences:

·	CTRL-ALT-DEL: System software reset. This sequence performs a software reset of the system by jumping to the beginning of the BIOS code and running the POST operation.

·	CTRL-ALT+ and CTRL-ALT-: Turbo mode selection. CTRL-ALT- sets the system for de-turbo mode (emulation of an 8 MHz 80286 CPU) and CTRL-ALT+ sets the system for turbo mode (normal operation at 60 MHz). Changing the Turbo mode may be prohibited by an operating system or application software.

## Dallas DS12887 Real Time Clock, CMOS RAM and Battery

The Real Time Clock (RTC) is implemented using a Dallas DS12887 device. The DS12887 is accurate to within 13 minutes/year and requires no external support (the battery and oscillator are integrated into the device). The component is socketed and can be replaced if the internal battery loses its charge (the internal battery has an estimated lifetime of ten years).

The RTC can be set via the BIOS SETUP Program. CMOS memory supports the standard 128-byte battery-backed RAM, fourteen bytes for clock and control registers, and 114 bytes of general purpose non-volatile CMOS RAM. All CMOS RAM is reserved for BIOS use. The CMOS RAM can be set to specific values or cleared to the system default values using the BIOS SETUP program. Also, the CMOS RAM values can be cleared to the system defaults by using a hardware jumper. Appendix B lists jumper configurations.

## System BIOS

The Premiere/PCI Baby-AT motherboard uses an American Megatrends Incorporated (AMI) ROM BIOS, which is stored in Flash EEPROM and easily upgraded using a floppy disk-based program. BIOS upgrades will be available for download from the Intel bulletin board system. In addition to the AMIBIOS, the Flash EEPROM also contains the Setup utility, Power-On Self Tests (POST), update recovery code, and the PCI auto-configuration utility. This motherboard supports system BIOS shadowing, allowing the BIOS to execute from 32-bit on-board write-protected DRAM.

The BIOS displays a sign-on message during POST identifying the type of BIOS and a five-digit revision code. First production units will display a revision code of 1.00.01.AF1 As BIOS updates occur, the revision number will increase to 1.00.02.AF1, and so on.

Information on BIOS functions can be found in the IBM PS/2 and Personal Computer BIOS Technical Reference published by IBM, and the ISA and EISA Hi-Flex AMIBIOS Technical Reference published by AMI. Both manuals are available at most technical bookstores.

### FLASH Implementationtc  \l 3 "Flash Implementation"§

The Intel 28F001BXT 1 Mb FLASH component is organized as 128K x 8 (128 KB). The Flash device is divided into five areas, as described in Table 1.

| System Address |  | FLASH Memory Area |
| --- | --- | --- |
| F0000H | FFFFFH | 64 KB Main BIOS |
| EE000H | EFFFFH | 8 KB Boot Block (Not FLASH erasable) |
| ED000H | EDFFFH | 4 KB OEM LOGO Area |
| E8000H | ECFFFH | 20 KB System BIOS Reserved |
| E0000H | E7FFFH | 32 KB Not Used |

Table 1. Flash Memory Organization

The FLASH device resides in system memory in two 64 KB segments starting at E0000H, and can be mapped two different ways, depending on the mode of operation. In Normal Mode, address line A16 is inverted, setting the E000H and F000H segments so that the BIOS is organized as shown in the system address column above. Recovery mode removes the inversion on address line A16, swapping the E000H and F000H segments so that the 8 KB boot block resides at FE000H where the CPU expects the bootstrap loader to exist. This mode is only necessary in the unlikely event that a BIOS upgrade procedure is interrupted, causing the BIOS area to be left in an unusable state. For information on recovering the BIOS in the event of a catastrophic failure, refer to Appendix E.

### BIOS Upgradestc  \l 3 "Upgrade Utility"§

Flash memory makes distributing BIOS upgrades easy. A new version of the BIOS can be installed from a diskette. BIOS upgrades will be available for download from the bulletin board system.

The disk-based Flash upgrade utility (FMUP.EXE) has three options for BIOS upgrades:

· 	The Flash BIOS can be updated from a file on a disk;

· 	The current BIOS code can be copied from the Flash EEPROM to a disk file as a backup in the event that an upgrade cannot be successfully completed; or

· 	The BIOS in the Flash device can be compared with a disk file to ensure the system has the correct BIOS version.

The upgrade utility ensures the upgrade BIOS matches the target system to prevent accidentally installing a BIOS for a different type of system. Security to prevent unauthorized changes to the BIOS is provided via a write-protect jumper. The default setting is to allow BIOS upgrades. A recovery jumper is provided to allow recovery in the unlikely event of an unsuccessful BIOS upgrade. The jumper forces the ROM decode to access an 8 KB block of write protected recovery code in the Flash device.

### Setup Utility

The ROM-based Setup utility allows the configuration to be modified without opening the system for most basic changes. The Premiere/PCI Baby-AT motherboard also incorporates many new capabilities into the Flash EEPROM, including:

·	Auto configuration of IDE hard disks (manual selection of the drive type is no longer required).

·	Support for two IDE disk interfaces (primary and secondary) providing access to 4 IDE devices (when a second IDE controller is added on the PCI or ISA bus).

·	An option allowing the user to assign a block of addresses below the 1 MB boundary as non-shadowed, non-cached. This area is primarily used for expansion card ROM which causes timing problems when shadowed and cached.

·	ISA interrupts IRQ9, IRQ10 and IRQ11 may be assigned to add-in card ISA adapters, thereby informing the PCI configuration utility which interrupts not to use.

The Setup utility is accessible only during the Power-On Self Test (POST) by pressing the <DEL> or <F1> key after the POST memory test has begun and before boot begins. A prompt may be enabled that informs users to press the <F1> key to access Setup. A motherboard jumper can be set to prevent user access to Setup for security purposes. Setup options are detailed in Appendix C.

### PCI Auto-configuration Capability

The PCI auto-configuration utility operates in conjunction with the system Setup utility to allow the insertion and removal of PCI cards to the system without user intervention. When the system is turned on after adding a PCI add-in card, the BIOS automatically configures interrupts, DMA channels, I/O space, and other parameters. The user does not have to configure jumpers or worry about potential resource conflicts. Because PCI cards use the same interrupt resources as ISA cards, the user must specify the interrupts used by ISA add-in cards in the Setup utility. The PCI Auto-Configuration function complies with version 2.0G of the PCI BIOS specification.

### Flash LOGO Area

The Premiere/PCI Baby-AT board supports a 4 KB programmable FLASH user area located at ED000-EDFFF. An OEM may use this area to display a custom logo. The BIOS accesses the user area just after completing POST. The area may be updated by running the FMUP.EXE utility, which expects the update files to have a .USR extension.

### Security features

#### BIOS Password

A BIOS password feature provides security during the boot process. A password can be entered using the Setup utility and must be re-entered prior to disk boot each time the system is reset. To enable, disable, or change the password, refer to the Setup program options in Appendix C.

If the password is forgotten, it can be cleared by turning off the system and setting the "password clear" jumper to the clear position.

#### Setup Enable Jumper

A baseboard jumper controls access to the BIOS Setup utility. By setting jumper to the disable position, the user is prevented from accessing the Setup utility at any time.

## Connectors

### Front panel connections

Figure 2: Front Panel Connectors

The external speaker provides error beep code information during the Power-On Self Test if the system cannot use the video interface. See Appendix D - AMIBIOS Error Messages and Beep Codes for more information about error beep codes.

### Back panel connections

The Premiere/PCI Baby-AT includes an AT-style keyboard connector integrated on the back panel side of the motherboard.

### I/O connections

The motherboard contains stake pin header connections for cabling the serial, parallel, floppy, and IDE interfaces. Figure 3 shows the locations of these connectors, orientation of pin 1 on each, the numbering of each connector, and for IDE and Floppy, the missing key pin.

µ §

Figure 3. I/O Connections

## Power Consumption

Table 2 lists the current used by system resources in a configuration which includes 8 MB of DRAM. Table 3 lists the typical power consumed by the same configuration. This information is preliminary and is provided only as a guide for approximating total system power usage with additional resources added.

### Current

| DC Voltage | Max. Continuous Current | Peak Current 15 Seconds | Minimum Current Load |
| --- | --- | --- | --- |
| +5V | 18.5A | 18.5 | 2.5A |
| -5V | 0.9A | 0.9A | 0A |
| +12V | 4.6A | 9.5A | 0.5A |
| -12V | 0.5A | 0.5A | 0A |

Table 2. Premiere/PCI   Baby-AT Current Requirements (Preliminary)

### Watts

| Resource | Typical Power |
| --- | --- |
| Premiere/PCI Baby-AT baseboard, Pentium microprocessor, 8 MB (4 MB in each of two banks), 256 KB cache | 50 Watts |
| Teac 3½" Floppy drive | 1.7 Watts |

Table 3. Current Use by System Resources (Preliminary)

# System Level Features

## Chassis

The Premiere/PCI Expandable Desktop easily provides the expandability required for most traditional PC applications - five peripheral bays, seven expansion slots, an ample 200 Watt power supply and an additional fan, located at the front left side of the chassis, to help keep the system cool. The user can access all five peripheral bays from the front of the chassis. System security is provided by a mechanical keyboard lock and a BIOS password feature. The system also meets stringent environmental requirements.

## Peripheral Bays

Five expansion bays are available for peripherals and other add-in devices. These include three 5¼" half-height bays and two 3½" one-inch bays. All of the drive bays are accessible from the front panel.

Figure 4. Premiere/PCI Expandable Desktop Front Panel

## FAN

The Premiere/PCI Expandable Desktop has two fans to keep the system cool. One fan within the power supply provides 28 cfm of air movement. The second fan, located behind the card guide, provides 26 cfm across the add-in cards and the CPU. The second fan receives 12 Vdc directly from the baseboard at stake pin location J1A1.

## Expansion slots

The Premiere/PCI Expandable Desktop has seven expansion card slots, as shown in Figure 5. Several of the slots are intended for use with less than full-length cards. One ISA slot is limited in length by the 3½" peripheral bay carrier, one ISA card slot is limited in length by the left-most SIMM socket, two ISA card slots are full length, two PCI slots are limited in length by the CPU heatsink, and one is a full length shared slot, accepting either an ISA or a PCI expansion card.

## Front Panel

The front panel consists of a power switch, a hardware reset switch, a power-on LED, a hard disk access LED and a keyboard lock. All five peripheral bays also are accessed from the front panel.

## Back panel

The back panel consists of access panels for the expansion slots (one location is unused), two DB9 RS-232 serial ports, a bi-directional parallel port, an AT-style keyboard connector, a 115/230 voltage switch, a power supply input, an auxiliary power output and the power supply fan.

µ §

Figure 5. Premiere/PCI Expandable Desktop Back Panel.

## Power Supply

The Premiere/PCI Expandable Desktop integrates a 200 watt switchable power supply for all onboard resources, add-in cards, and peripherals. The Astec SA201-3440 supports operation at 100-120 VAC (5 Amps AC) or 200-240 VAC (3 Amps AC). The input voltage is selected using a switch on the back panel.

### AC Power input Specifications

| Input frequency 50/60 Hz |  |
| --- | --- |
| AC Voltage | Current |
| 90-135 | 5.0 A |
| 180-265 | 3.0 A |

Table 4. Power Supply Input Specifications.

### DC output SPECIFICATIONS

| DC Voltage | Max. Continuous Current | Peak Current 15 Seconds | Minimum Current Load |
| --- | --- | --- | --- |
| +5V | 22.0A | - | 3.0A |
| -5V | 0.5A | - | 0A |
| +12V | 8.0A | 9.0 A | 0A |
| -12V | 0.5A | - | 0A |

Table 5. Power Supply DC Output Specifications.

## Floppy Drive

The Premiere/PCI Expandable Desktop integrates a 3½" Teac Floppy drive Model FD-235HF into the right 3½" bay. This is the same proven floppy disk drive that has been used on many previous Intel systems.

## Speaker

The standard system ships with an external speaker installed. The speaker provides error beep code information during POST if the system cannot use the video interface. See Appendix D or the Premiere/PCI Expandable Desktop product guide (Order # 616545-001) for detailed beep and error code information.

## Security

### SYSTEM Key lock

When engaged, the system key lock prevents keyboard input. Each system is provided with a set of two identical keys.

## Chassis color

The chassis color is beige. The bottom and back of the chassis are not painted.

# Premiere/PCI OmniRACK Chassis

The Premiere/PCI Baby-AT motherboard also is available in Intel's OmniRACK rackmountable chassis. OmniRACK can be used in an industry-standard REA 19-inch equipment rack or as an embedded microcomputer. Premiere/PCI OmniRACK offers a 230 watt power supply with temperature-controlled internal cooling fans. The system contains four peripheral bays (two 5¼" x 1.6", one 3½" x 1.6", and one 3½" x 1") housed in a removable drive cage. For more information on the OmniRack chassis, see the OmniRACK Technical Product Summary, available from Intel Literature or the bulletin board system.

The order code for the Premiere/PCI OmniRACK product is ORP605P8.

µ §

Figure 6. OmniRACK Chassis with access door closed

# Performance Benchmarks

Performance benchmarks for the Premiere/PCI Expandable Desktop will be posted on FaxBack and the bulletin board system and will be updated periodically. This will allow incorporation of new benchmarks as they emerge in the industry.

# Optional Accessories

## PCI IDE paddle card (pciide)

An IDE paddle card accessory provides a PCI local bus connection for IDE devices. This paddle card provides improved IDE performance over the IDE interface integrated onto the motherboard. When installed, the paddle card is configured to be the primary IDE interface. The on-board IDE interface can still be used as a secondary IDE channel, thereby providing support for up to four IDE devices. The paddle card is supported only in PCI slot 3 (J10C2. the PCI slot closest to the 3.3V connector), which uses some of the reserved pins on the PCI interface. The use of the reserved pins will not affect the operation of any PCI add-in cards as of this publication date. A detailed pinout for PCI slot 3 appears in Appendix I - Connectors.

## PCI Video Card (pcis3928)

A PCI Video card accessory takes advantage of the high performance of the PCI bus. 1MB of VRAM allows resolutions up to 1280x1024, and colors up to 16.7 million. The card is based on the S3 928 video controller to provide industry leading Windows acceleration, and screen refresh rates up to 100 Hz to meet ergonomic considerations. Drivers are available for the most common operating environments, including Windows, OS/2, Windows NT, SCO, and many CAD applications.

# Appendix A - User-Installable Upgrades

## SYSTEM MEMORY

Table A-1 shows the possible memory combinations. The Premiere/PCI Baby-AT motherboard supports both parity and non-parity SIMMs, but they cannot be mixed within the same memory bank. SIMM requirements are 70ns, Fast Page Mode, with tin-lead connectors.

| SIMM 1,2 (Bank 0) SIMM Type (Amount) | SIMM 3,4 (Bank 1) SIMM Type (Amount) | Total System Memory |
| --- | --- | --- |
| 256K X 36 (1 MB) | Empty | 2 MB |
| 256K X 36 (1 MB) | 256K X 36 (1 MB) | 4 MB |
| 256K X 36 (1 MB) | 512K X 36 (2 MB) | 6 MB |
| 256K X 36 (1 MB) | 1M X 36 (4 MB) | 10 MB |
| 256K X 36 (1 MB) | 2M X 36 (8 MB) | 18 MB |
| 256K X 36 (1 MB) | 4M X 36 (16 MB) | 34 MB |
| 256K X 36 (1 MB) | 8M X 36 (32 MB) | 66 MB |
| 512K X 36 (2 MB) | Empty | 4 MB |
| 512K X 36 (2 MB) | 256K X 36 (1 MB) | 6 MB |
| 512K X 36 (2 MB) | 512K X 36 (2 MB) | 8 MB |
| 512K X 36 (2 MB) | 1M X 36 (4 MB) | 12 MB |
| 512K X 36 (2 MB) | 2M X 36 (8 MB) | 20 MB |
| 512K X 36 (2 MB) | 4M X 36 (16 MB) | 36 MB |
| 512K X 36 (2 MB) | 8M X 36 (32 MB) | 68 MB |
| 1M X 36 (4 MB) | Empty | 8 MB |
| 1M X 36 (4 MB) | 256K X 36 (1 MB) | 10 MB |
| 1M X 36 (4 MB) | 512K X 36 (2 MB) | 12 MB |
| 1M X 36 (4 MB) | 1M X 36 (4 MB) | 16 MB |
| 1M X 36 (4 MB) | 2M X 36 (8 MB) | 24 MB |
| 1M X 36 (4 MB) | 4M X 36 (16 MB) | 40 MB |
| 1M X 36 (4 MB) | 8M X 36 (32 MB) | 72 MB |
| 2M X 36 (8 MB) | Empty | 16 MB |
| 2M X 36 (8 MB) | 256K X 36 (1 MB) | 18 MB |
| 2M X 36 (8 MB) | 512K X 36 (2 MB) | 20 MB |
| 2M X 36 (8 MB) | 1M X 36 (4 MB) | 24 MB |
| 2M X 36 (8 MB) | 2M X 36 (8 MB) | 32 MB |
| 2M X 36 (8 MB) | 4M X 36 (16 MB) | 48 MB |
| 2M X 36 (8 MB) | 8M X 36 (32 MB) | 80 MB |
| 4M X 36 (16 MB) | Empty | 32 MB |
| 4M X 36 (16 MB) | 256K X 36 (1 MB) | 34 MB |
| 4M X 36 (16 MB) | 512K X 36 (2 MB) | 36 MB |
| 4M X 36 (16 MB) | 1M X 36 (4 MB) | 40 MB |
| 4M X 36 (16 MB) | 2M X 36 (8 MB) | 48 MB |
| 4M X 36 (16 MB) | 4M X 36 (16 MB) | 64 MB |
| 4M X 36 (16 MB) | 8M X 36 (32 MB) | 96 MB |
| 8M X 36 (32 MB) | Empty | 64 MB |
| 8M X 36 (32 MB) | 256K X 36 (1 MB) | 66 MB |
| 8M X 36 (32 MB) | 512K X 36 (2 MB) | 68 MB |
| 8M X 36 (32 MB) | 1M X 36 (4 MB) | 72 MB |
| 8M X 36 (32 MB) | 2M X 36 (8 MB) | 80 MB |
| 8M X 36 (32 MB) | 4M X 36 (16 MB) | 96 MB |
| 8M X 36 (32 MB) | 8M X 36 (32 MB) | 128 MB |

Table A-1. Possible SIMM Memory Combinations

# Appendix B - Jumpers

µ §

Figure B-1. Jumper locations and settings (*denotes default setting)

## 66/60

This jumper is set at 60 MHz by the factory and may not be changed by the user. If changed from the factory setting, improper operation may result.

## FLASH Recovery (RCVR flash)

Allows recovery if the system FLASH update process results in corrupted EPROM.

## Clear CMOS (NRM/clr)

Allows CMOS settings to be reset to default values.

## Setup ENABLE (Setup)

Allows access to CMOS Setup Utility to be disabled.

## Flash Write Protect (flash WP)

Allows reprogramming of FLASH EPROM to be disabled.

## Color/Mono (mono/clr)

Chooses Monochrome or Color video mode at boot.

## Password clear (ena/dis pswd)

Allows system password to be cleared.

# Appendix C - AMIBIOS Setup

## Entering the setup program

Press and hold the <DEL> or <F1> key during the power-up sequence. Figure C-1 shows the first screen.

STANDARD CMOS SETUP

ADVANCED CMOS SETUP

PERIPHERAL MANAGEMENT SETUP

CONFIGURE WITH BIOS DEFAULTS

CONFIGURE WITH POWER-ON DEFAULTS

CHANGE PASSWORD

WRITE TO CMOS AND EXIT

DO NOT WRITE TO CMOS AND EXIT

Figure C-1. Opening Screen on Entering Setup Program

## Overview of selections

### Standard CMOS setup

Allows the user to modify basic options for the PC, such as time, date, floppies, hard drives and keyboard.

### Advanced CMOS setup

Allows the user to modify more advanced features, including enabling or disabling parity checking, cache, numlock, shadowing, IDE DMA transfer modes, ISA interrupts, and ISA linear frame buffer.

### Peripheral management setup

Allows the user to modify the peripheral options associated with the SMC37C665 I/O controller, including serial and parallel port addresses.

### Configure with BIOS defaults

Reloads CMOS with default values from the ROM table.

### Configure with power-on defaults

Reloads CMOS with power-up values.

### Change password

Allows the user to specify a password that will be needed to enter the Setup program or boot DOS. Once the password is specified, it can be changed, deleted or disabled using this utility. To delete or disable the password function, simply enter a carriage return instead of a new password. If the password is forgotten, a jumper on the board can be moved to clear the password (see Appendix B - Jumpers).

### Write to CMOS and exit

Saves the configuration and any changes made to CMOS.

### Do not write to CMOS and exit

Allows the user to exit the setup utility without saving any configuration changes to CMOS.

## STANDARD CMOS SETUP

| DATE TIME | Specify the current date Specify the current time |
| --- | --- |
| Floppy Drive A: Floppy Drive B: | If present, select the size and type If present, select the size and type |
| Drive C: Type Drive D: Type Drive E: Type Drive F: Type | There are three choices for configuring an IDE hard drive: · Auto Configure; the BIOS will automatically sense the hard drive in the system and configure it appropriately · User Type 1; allows the user to explicitly configure the hard drive in the event the hard drive in the system does not support the identify drive command needed to support auto-configuration · User Type 2; Same as user type 1 Support for multiple IDE drives. Same options as Drive C: Support for multiple IDE drives. Same options as Drive C: Support for multiple IDE drives. Same options as Drive C: |
| Drive C: Time-out Drive D: Time-out Drive E: Time-out Drive F: Time-out | Time-out specification for the IDE auto-configuration. If the drive does not auto-configure in this amount of time, there will be an error message. Default is 5 seconds. Choices are 0, 5, 15, and 31 seconds. Same as the Drive C: Time-out description Same as the Drive C: Time-out description Same as the Drive C: Time-out description |
| Keyboard | Installed or Not Installed. Default is Installed. |
| Base Memory Extended Memory | Reports the amount of base memory. There are no options. Reports the amount of extended memory. There are no options. |

## Advanced CMOS setup

| Typematic Rate Programming | Enabled or Disabled. Default is disabled. When enabled, the following two typematic options are valid. |
| --- | --- |
| Typematic Rate Delay (msec.) | 250, 500, 750 or 1000 millisecond delay. Default is 500. Determines how long it takes for the auto-repeat function to start when a person is holding down a key on the keyboard. |
| Typematic Rate (Chars/sec) | 6, 30, 24, 20, 15, 12, 10 or 8 characters/second. Default is 15. Determines the speed at which characters repeat when you hold down a key on the keyboard. The higher the number, the faster the repeat. |
| Hit <F1> Message Display | Enabled or Disabled. Default is disabled. If enabled, the message "Hit <F1> to Enter Setup" appears on the screen during the power up sequence. |
| System Boot Up Num Lock | On or Off. Default is on, reflecting the state of the "Num Lock" keyboard feature when the system boots. |
| System Boot Up Sequence | Drive A: before C:, Drive C: before A:, or Drive C: Only. Default is to check Drive A: before C: for an operating system, which allows the user to boot from a floppy if necessary. |
| System Boot Up CPU Speed | High or Low. Default is High. If high is selected, boot up will occur at full speed. If low is selected, the board is put into de-turbo mode, which results in slower operation. |
| Cache Memory | Enabled or Disabled. Default is enabled. Both the L1 and L2 cache are enabled or disabled by this selection. |
| Network Password Checking | Enabled or Disabled. Default is disabled. If enabled, the user is prompted for a password when trying to connect to a network. |
| ISA Linear Frame Buffer | Disabled, 1 MB, 2 MB, or 4 MB. Default is disabled. The linear frame buffer is primarily for ISA video cards. If enabled, improvement may be noted in performance of ISA video cards using the linear frame buffer. It also provides a means to access memory located on the ISA bus immediately below 16MB. |

## Advanced CMOS setup (Continued)

| Set Linear Frame Address to | When you enable the ISA linear frame buffer, this entry is displayed automatically. When you disable the linear frame buffer, this entry becomes N/A. |
| --- | --- |
| Disable Shadow Memory Size | Enabled or Disabled. Default is disabled. This selection is tied to the next selection "Disable Shadow Memory Base". If the shadow memory base is set to either C0000H or D0000H, then you can select a shadow memory size of either 16, 32, 48, or 64 KBytes. |
| Disable Shadow Memory Base | This selection allows the user to alter where the non-shadowed memory is located. The choices are either in the C0000H or D0000H range. There are several options within each range. For example, within the C0000H range you can choose C0000H, C4000H, C8000H, or CC000H. This setting effects the previous menu selection. Once you have made your selection in this menu your choices in the previous menu "Disable Shadow Memory Size" may be altered. The maximum size of the shadow memory is 64KB. You can only achieve 64kB if you specify the Shadow Memory Base to be at C0000H or D0000H. As you select the other shadow memory base options, you will notice that the shadow memory size begins to get smaller. |
| Base Memory Size | 640/512. Default is 640KB. Provides a means to disable on-bd DRAM to access memory locations from 80000-9FFFFH on the ISA bus. |
| IDE DMA Transfer Mode | Default is disabled. When enabled, the user can choose from 3 different types of IDE DMA transfers: Standard (compatible), Type B, or Type F. Type F is the fastest DMA transfer mode. |
| IDE Multiple Sector Mode | Default is disabled and this selection is tied to the previous selection (IDE DMA Transfer Mode). When enabled, choices are 1, 2, 4, 8, 16, 32, or 64 sectors per burst (S/B). This setting programs the IDE DMA cycles to transfer multiple sectors per burst. A setting of 64 S/B assures the maximum burst size supported by the drive that is installed in the system. |
| Enhanced ISA Timing | Enabled or Disabled. Default is disabled. When enabled, the ISA bus will run at 10 MHz. This results in faster system performance if the ISA agents in the system can tolerate 10 MHz (instead of the standard 8 MHz). |
| ISA IRQ 9 | Free or Used. If an ISA agent in the system uses this interrupt, it must be specified here as used. The PCI auto-configuration code checks this entry and may attempt to use the interrupt is it is free. |
| ISA IRQ 10 | Free or Used. If an ISA agent in the system uses this interrupt, it must be specified here as used. The PCI auto-configuration code checks this entry and may attempt to use the interrupt is it is free. |
| ISA IRQ 11 | Free or Used. If an ISA agent in the system uses this interrupt, it must be specified here as used. The PCI auto-configuration code checks this entry and may attempt to use the interrupt is it is free. |

## Peripheral management setup

| Programming option | Auto or Manual. Default is Auto, meaning that the peripherals controlled by the SMC665 I/O component are automatically configured during power up. If Auto is selected, the following menu options have no effect. If Manual is selected, settings must be entered for all of the following peripherals. |
| --- | --- |
| On-Board Floppy Drive | Enabled or disabled. |
| On-Board IDE drive | Enabled or disabled. |
| First Serial Port Address | Disabled, 2E8H, 3E8H, 2F8H, or 3F8H. The address is automatically set in auto programming mode, although this entry will show "disabled" |
| Second Serial Port Address | Disabled, 2E8H, 3E8H, 2F8H, or 3F8H. The address is automatically set in auto programming mode, although this entry will show "disabled" |
| Parallel Port Address | Disabled, 278H, 378H, or 3BCH. The address is automatically set in auto programming mode, although this entry will show "disabled" |
| IRQ active State | High or low. Default is High. |
| Parallel Port Mode | Normal or Extended. Default is normal. Extended enables the port for bi-directional operation. |

# Appendix D - AMIBIOS Error messages and Beep Codes

Errors can occur during POST (Power On Self Test) which is performed every time the system is powered on. Fatal errors, which prevent the system to continue the boot process, are communicated through a series of audible beeps. Other errors are displayed in the following format:

ERROR Message Line 1

ERROR Message Line 2

For most displayed error messages, there is only one message. If a second message appears, it is "RUN SETUP". If this message occurs, press <F1> to run AMIBIOS Setup.

## BEEP CODES

| Beeps | Error Message | Description |
| --- | --- | --- |
| 1 | Refresh Failure | The memory refresh circuitry on the motherboard is faulty. |
| 2 | Parity Error | Parity error in the first 64 KB of memory. |
| 3 | Base 64 KB Memory Failure | Memory failure in the first 64 KB. |
| 4 | Timer Not Operational | Memory failure in the first 64 KB of memory, or Timer 1 on the motherboard is not functioning. |
| 5 | Processor Error | The CPU on the motherboard generated an error. |
| 6 | 8042 - Gate A20 Failure | The keyboard controller (8042) may be bad. The BIOS cannot switch to protected mode. |
| 7 | Processor Exception Interrupt Error | The CPU generated an exception interrupt. |
| 8 | Display Memory Read/Write Error | The system video adapter is either missing or its memory is faulty. This is not a fatal error. |
| 9 | ROM Checksum Error | ROM checksum value does not match the value encoded in BIOS. |
| 10 | CMOS Shutdown Register Rd/Wrt Error | The shutdown register for CMOS RAM failed. |
| 11 | Cache Error / External Cache Bad | The external cache is faulty. |

## error messages

| Error Message | Explanation |
| --- | --- |
| 8042 Gate - A20 Error | Gate A20 on the keyboard controller (8042) is not working. Replace the 8042. |
| Address Line Short! | Error in the address decoding circuitry on the motherboard. |
| Cache Memory Bad, Do Not Enable Cache! | Cache memory is defective. Replace it. |
| CH-2 Timer Error | Most AT systems include two timers. There is an error in timer 2. |
| CMOS Battery State Low | CMOS RAM is powered by a battery. The battery power is low. Replace the battery. |
| CMOS Checksum Failure | After CMOS RAM values are saved, a checksum value is generated for error checking. The previous value is different from the current value. Run AMIBIOS Setup. |
| CMOS System Options Not Set | The values stored in CMOS RAM are either corrupt or nonexistent. Run Setup. |
| CMOS Display Type Mismatch | The video type in CMOS RAM does not match the type detected by the BIOS. Run AMIBIOS Setup. |
| CMOS Memory Size Mismatch | The amount of memory on the motherboard is different than the amount in CMOS RAM. Run AMIBIOS Setup. |

## ERROR MESSAGES (cont.)

| CMOS Time and Date Not Set | Run Standard CMOS Setup to set the date and time in CMOS RAM. |
| --- | --- |
| Diskette Boot Failure | The boot disk in floppy drive A: is corrupt. It cannot be used to boot the system. Use another boot disk and follow the screen instructions. |
| Display Switch Not Proper | Some systems require a video switch on the motherboard be set to either color or monochrome. Turn the system off, set the switch, then power on. |
| DMA Error | Error in the DMA controller. |
| DMA #1 Error | Error in the first DMA channel. |
| DMA #2 Error | Error in the second DMA channel. |
| FDD Controller Failure | The BIOS cannot communicate with the floppy disk drive controller. Check all appropriate connections after the system is powered down. |
| HDD Controller Failure | The BIOS cannot communicate with the hard disk drive controller. Check all appropriate connections after the system is powered down. |
| INTR #1 Error | Interrupt channel 1 failed POST. |
| INTR #2 Error | Interrupt channel 2 failed POST. |
| Invalid Boot Diskette | The BIOS can read the disk in floppy drive A:, but cannot boot the system. Use another boot disk. |
| Keyboard Is Locked...Unlock It | The keyboard lock on the system is engaged. The system must be unlocked to continue. |
| Keyboard Error | There is a timing problem with the keyboard. Set the Keyboard option in Standard CMOS Setup to Not Installed to skip the keyboard POST routines. |
| KB/Interface Error | There is an error in the keyboard connector. |
| Off Board Parity Error | Parity error in memory installed in an expansion slot. The format is: OFF BOARD PARITY ERROR ADDR (HEX) = (XXXX) XXXX is the hex address where the error occurred. |
| On Board Parity Error | Parity error in motherboard memory. The format is: OFF BOARD PARITY ERROR ADDR (HEX) = (XXXX) XXXX is the hex address where the error occurred. |
| Parity Error ???? | Parity error in system memory at an unknown address. |

## ISA NMI Messages

| ISA NMI Message | Explanation |
| --- | --- |
| Memory Parity Error at xxxxx | Memory failed. If the memory location can be determined, it is displayed as xxxxx. If not, the message is Memory Parity Error ????. |
| I/O Card Parity Error at xxxxx | An expansion card failed. If the address can be determined, it is displayed as xxxxx. If not, the message is I/O Card Parity Error ????. |
| DMA Bus Time-out | A device has driven the bus signal for more than 7.8 microseconds. |

# Appendix E - BIOS Upgrades and Recovery

The Premiere/PCI Baby-AT motherboard incorporates the AMIBIOS in a Flash memory component. Flash BIOS allows easy upgrades without the need to replace an EPROM. The upgrade utility fits on a floppy diskette and provides the capability to save, verify, and update the system BIOS. The upgrade utility can be run from a hard drive or a network drive, but no memory managers can be installed during upgrades.

The latest upgrade utility and BIOS code are available in the BIOS section of the iPAN bulletin board.

## Using the FLASH upgrade utility

If the utility is obtained from the bulletin board, UNZIP the archive and copy the files to a bootable MS-DOS 3.3, 4.01, 5.0, or 6.0 diskette. Reboot the system with the upgrade diskette in the bootable floppy drive and follow the directions in the easy to use menu-driven program.

## Recovery mode

In the unlikely event that a FLASH upgrade is interrupted catastrophically, it is possible the BIOS may be left in an unusable state. Recovering from this condition requires the following steps (be sure a power supply and speaker have been attached to the board, and a floppy drive is connected as drive A:):

1.	Change Flash Recovery jumper (RCVR FLASH) to the recovery mode position.

2.	Install the bootable upgrade diskette into drive A:

3.	Reboot the system.

4.	Because of the small amount of code available in the non-erasable boot block area, no video is available to direct the procedure. The procedure can be monitored by listening to the speaker and looking at the floppy drive LED. When the system beeps and the floppy drive LED is lit, the system is copying the recovery code into the FLASH device. As soon as the drive LED goes off, the recovery is complete.

5.	Turn the system off.

6.	Change the "FLASH RECOVERY" jumper back to the default position.

7.	Leave the upgrade floppy in drive A: and turn the system on.

8.	Continue with the original upgrade.

# Appendix F- Memory Map

| Address Range (Decimal) | Address Range (hex) | Size | Description |
| --- | --- | --- | --- |
| 1024K-131072K | 100000-8000000 | 130048K | Extended Memory |
| 960K-1023K | F0000-FFFFF | 64K | AMI System BIOS |
| 952K-959K | EE000-EFFFF | 8K | FLASH Boot Block (Available as HIMEM) |
| 948K-951K | ED000-EDFFF | 4K | OEM LOGO (available as HIMEM if no user info is here) |
| 928K-947K | E8000-ECFFF | 20K | BIOS RESERVED |
| 640K-927K | A0000-E7FFF | 288K | Available HI DOS Memory (open to the ISA & PCI bus) |
| 639K | 9FC00-9FFFF | 1K | Extended BIOS Data (moveable by QEMM, 386MAX) |
| 512K-638K | 80000-9FBFF | 127K | Extended conventional |
| 0K-511K | 00000-7FFFF | 512K | Conventional |

Table F-1. Premiere/PCI   Baby-AT motherboard Memory Map

# Appendix G - I/O Map

| Address (hex) | Size(Dec) | Description |
| --- | --- | --- |
| 0000 - 000F | 16 bytes | SIO - DMA 1 |
| 0020 - 0021 | 2 bytes | SIO - Interrupt Controller 1 |
| 0040 - 0043 | 4 bytes | SIO - Timer 1 |
| 0048 - 004B | 4 bytes | SIO - Timer 2 |
| 0060 | 1 byte | Keyboard Controller Data Byte |
| 0061 | 1 byte | SIO - NMI, speaker control |
| 0064 | 1 byte | Kbd Controller, CMD/STAT Byte |
| 0070, bit 7 | 1 bit | SIO - Enable NMI |
| 0070, bits 6:0 | 7 bits | SIO - Real Time Clock, Address |
| 0071 | 1 byte | SIO - Real Time Clock, Data |
| 0073 | 1 byte | Reserved - Brd. Config. |
| 0075 | 1 byte | Reserved - Brd. Config.,RD only |
| 0078 | 1 byte | SIO - BIOS Timer |
| 0080 - 008F | 16 bytes | SIO - DMA Page Register |
| 00A0 - 00A1 | 2 bytes | SIO - Interrupt Controller 2 |
| 00C0 - 00DE | 31 bytes | SIO - DMA 2 |
| 00F0 | 1 bytes | Reset Numeric Error |
| 0170 - 0177 | 8 bytes | Secondary IDE Channel |

| Address (hex) | Size(Dec) | Description |
| --- | --- | --- |
| 01F0 - 01F7 | 8 bytes | Primary IDE Channel |
| 0278 - 027B | 4 bytes | Parallel Port 2 |
| 02F8 - 02FF | 8 bytes | On-Board Serial Port 2 |
| 0376 | 1 byte | Secondary IDE Chan Cmd Port |
| 0377 | 1 byte | Secondary IDE Chan Stat Port |
| 0378 - 037F | 8 bytes | Parallel Port 1 |
| 03BC - 03BF | 4 bytes | Parallel Port x |
| 03E8 - 03EF | 8 bytes | Serial Port 3 |
| 03F0 - 03F5 | 6 bytes | Floppy Channel 1 |
| 03F6 | 1 bytes | Primary IDE Channel Cmnd Port |
| 03F7 (Write) | 1 byte | Floppy Channel 1 Command |
| 03F7, bit 7 | 1 bit | Floppy Disk Change Channel 1 |
| 03F7, bits 6:0 | 7 bits | Primary IDE Channel Status Port |
| 03F8 - 03FF | 8 bytes | On-Board Serial Port 1 |
| 0CF8 | 1 byte | PCI Configuration Space Enable |
| 0CF9 | 1 byte | Deturbo Mode Enable |
| C000 - C0FF | 256 byte | 82434LX Config. Registers * |
| C200 - C2FF | 256 byte | 82378IB Config. Registers * |

Table G-1. Premiere/PCI  Baby-AT I/O Address Map

* Only accessible after PCI configuration space is enabled.

# Appendix H - Interrupts & DMA Channels

| IRQ | System Resource |
| --- | --- |
| NMI | Parity Error |
| 0 | Reserved, Interval Timer |
| 1 | Reserved, Keyboard buffer full |
| 2 | Reserved, Cascade interrupt from slave PIC |
| 3 | Serial Port 2 |
| 4 | Serial Port 1 |
| 5 | Parallel Port 2 |
| 6 | Floppy |
| 7 | Parallel Port 1 |
| 8 | Real Time Clock |
| 9 | User available |
| 10 | User available |
| 11 | User available |
| 12 | PS/2 Mouse Port, User available on AT style bds |
| 13 | Reserved, Math coprocessor |
| 14 | Primary IDE if enabled, else available to user |
| 15 | Secondary IDE if enabled, else available to user |

Table H-1. Premiere/PCI  Baby-AT Interrupts

| DMA | Data Width | System Resource |
| --- | --- | --- |
| 0 | 8- or 16-bits | Open |
| 1 | 8- or 16-bits | Open - Normally used for LAN |
| 2 | 8- or 16-bits | Floppy |
| 3 | 8- or 16-bits | Parallel Port |
| 4 |  | Reserved - Cascade channel |
| 5 | 16-bits | Open |
| 6 | 16-bits | Open |
| 7 | 16-bits | IDE |

Table H-2. Premiere/PCI  Baby-AT DMA Map

# Appendix I - Connectors

## AT Style Keyboard Port

| Pin | Signal Name |
| --- | --- |
| 1 | Clock |
| 2 | Data |
| 3 | No Connect |
| 4 | Ground |
| 5 | Vcc (fused) |

## Primary Power

| Pin | Name | Function |
| --- | --- | --- |
| 1 | PWRGD | Power Good |
| 2 | +5 V | + 5 volts Vcc |
| 3 | +12 V | + 12 volts |
| 4 | -12 V | - 12 volts |
| 5 | GND | Ground |
| 6 | GND | Ground |
| 7 | GND | Ground |
| 8 | GND | Ground |
| 9 | -5 V | -5 volts |
| 10 | +5 V | + 5 volts Vcc |
| 11 | +5 V | + 5 volts Vcc |
| 12 | +5 V | + 5 volts Vcc |

## auxiliary (3.3V) Power

| Pin | Name | Function |
| --- | --- | --- |
| 1 | GND | Ground |
| 2 | GND | Ground |
| 3 | GND | Ground |
| 4 | +3.3 V | + 3.3 volts |
| 5 | +3.3V | + 3.3 volts |
| 6 | +3.3 V | + 3.3 volts |

## Speaker Connector

| Pin | Signal Name |
| --- | --- |
| 1 | SPKR_DAT |
| 2 | Key |
| 3 | No Connect |
| 4 | +5V Vcc |

## Reset Connector

| Pin | Signal Name |
| --- | --- |
| 1 | RESET |
| 2 | Ground |

## Key lock/Power LED

| Pin | Signal Name |
| --- | --- |
| 1 | LED_PWR |
| 2 | Key |
| 3 | Ground |
| 4 | KEY LOCK |
| 5 | Ground |

## Hard Drive LED (Disk)

| Pin | Signal Name |
| --- | --- |
| 1 | PULL_UP_330 |
| 2 | HD ACTIVE- |
| 3 | Key |
| 4 | PULL_UP_330 |

## Turbo LED

| Pin | Signal Name |
| --- | --- |
| 1 | PULL_UP_330 |
| 2 | LED_TURBO- |

## Turbo Switch

| Pin | Signal Name |
| --- | --- |
| 1 | TURBO |
| 2 | Ground |
| 3 | No Connect |

## Auxiliary 12V Fan Power

| Pin | Signal Name |
| --- | --- |
| 1 | Ground |
| 2 | +12 V (fused) |
| 3 | Ground |

## Serial Ports

| Pin | Signal Name |
| --- | --- |
| 1 | DCD |
| 2 | DSR |
| 3 | Serial In - (SIN) |
| 4 | RTS |
| 5 | Serial Out - (SOUT) |
| 6 | CTS |
| 7 | DTR |
| 8 | RI |
| 9 | GND |
| 10 | N.C. |

## IDE Connector

| Signal Name | Pin | Pin | Signal Name |
| --- | --- | --- | --- |
| Reset IDE | 1 | 2 | Ground |
| Host Data 7 | 3 | 4 | Host Data 8 |
| Host Data 6 | 5 | 6 | Host Data 9 |
| Host Data 5 | 7 | 8 | Host Data 10 |
| Host Data 4 | 9 | 10 | Host Data 11 |
| Host Data 3 | 11 | 12 | Host Data 12 |
| Host Data 2 | 13 | 14 | Host Data 13 |
| Host Data 1 | 15 | 16 | Host Data 14 |
| Host Data 0 | 17 | 18 | Host Data 15 |
| Ground | 19 | 20 | Key |
| DRQ3 | 21 | 22 | Ground |
| I/O Write- | 23 | 24 | Ground |
| I/O Read- | 25 | 26 | Ground |
| IOCHRDY | 27 | 28 | BALE |
| DACK3- | 29 | 30 | Ground |
| IRQ14 | 31 | 32 | IOCS16- |
| Addr 1 | 33 | 34 | Ground |
| Addr 0 | 35 | 36 | Addr 2 |
| Chip Select 0- | 37 | 38 | Chip Select 1- |
| Activity | 39 | 40 | Ground |

## Floppy Connector

| Signal Name | Pin | Pin | Signal Name |
| --- | --- | --- | --- |
| Ground | 1 | 2 | FDHDIN |
| Ground | 3 | 4 | Reserved |
| Key | 5 | 6 | FDEDIN |
| Ground | 7 | 8 | Index- |
| Ground | 9 | 10 | Motor Enable A- |
| Ground | 11 | 12 | Drive Select B- |
| Ground | 13 | 14 | Drive Select A- |
| Ground | 15 | 16 | Motor Enable B- |
| Ground | 17 | 18 | DIR- |
| Ground | 19 | 20 | STEP- |
| Ground | 21 | 22 | Write Data- |
| Ground | 23 | 24 | Write Gate- |
| Ground | 25 | 26 | Track 00- |
| Ground | 27 | 28 | Write Protect- |
| Ground | 29 | 30 | Read Data- |
| Ground | 31 | 32 | Side 1 Select- |
| Ground | 33 | 34 | Diskette Change- |

## Parallel Port Connector

| Signal Name | Pin | Pin | Signal Name |
| --- | --- | --- | --- |
| STROBE- | 1 | 2 | AUTO FEED- |
| Data Bit 0 | 3 | 4 | ERROR- |
| Data Bit 1 | 5 | 6 | INIT- |
| Data Bit 2 | 7 | 8 | SLCT IN- |
| Data Bit 3 | 9 | 10 | Ground |
| Data Bit 4 | 11 | 12 | Ground |
| Data Bit 5 | 13 | 14 | Ground |
| Data Bit 6 | 15 | 16 | Ground |
| Data Bit 7 | 17 | 18 | Ground |
| ACJ- | 19 | 20 | Ground |
| BUSY | 21 | 22 | Ground |
| PE (Paper End) | 23 | 24 | Ground |
| SLCT | 25 | 26 | N.C. |

## ISA Connectors

| Signal Name | Pin | Pin | Signal Name |
| --- | --- | --- | --- |
| GND | B1 | A1 | IOCHK- |
| RSTDRV | B2 | A2 | SD7 |
| Vcc | B3 | A3 | SD6 |
| IRQ9 | B4 | A4 | SD5 |
| -5V | B5 | A5 | SD4 |
| DRQ2 | B6 | A6 | SD3 |
| -12V | B7 | A7 | SD2 |
| 0WS- | B8 | A8 | SD1 |
| +12V | B9 | A9 | SD0 |
| GND | B10 | A10 | IOCHRDY |
| SMEMW- | B11 | A11 | AEN |
| SMEMR- | B12 | A12 | SA19 |
| IOW- | B13 | A13 | SA18 |
| IOR- | B14 | A14 | SA17 |
| DACK3- | B15 | A15 | SA16 |
| DRQ3 | B16 | A16 | SA15 |
| DACK1- | B17 | A17 | SA14 |
| DRQ1 | B18 | A18 | SA13 |
| REFRESH- | B19 | A19 | SA12 |
| SYSCLK | B20 | A20 | SA11 |
| IRQ7 | B21 | A21 | SA10 |
| IRQ6 | B22 | A22 | SA9 |
| IRQ5 | B23 | A23 | SA8 |
| IRQ4 | B24 | A24 | SA7 |
| IRQ3 | B25 | A25 | SA6 |
| DACK2- | B26 | A26 | SA5 |
| TC | B27 | A27 | SA4 |
| BALE | B28 | A28 | SA3 |
| Vcc | B29 | A29 | SA2 |
| OSC | B30 | A30 | SA1 |
| GND | B31 | A31 | SA0 |
|  | KEY | KEY |  |
| MEMCS16- | D1 | C1 | SBHE- |
| IOCS16- | D2 | C2 | LA23 |
| IRQ10 | D3 | C3 | LA22 |
| IRQ11 | D4 | C4 | LA21 |
| IRQ12 | D5 | C5 | LA20 |
| IRQ15 | D6 | C6 | LA19 |
| IRQ14 | D7 | C7 | LA18 |
| DACK0- | D8 | C8 | LA17 |
| DRQ0 | D9 | C9 | MEMR- |
| DACK5- | D10 | C10 | MEMW- |
| DRQ5 | D11 | C11 | SD8 |
| DACK6- | D12 | C12 | SD9 |
| DRQ6 | D13 | C13 | SD10 |
| DACK7- | D14 | C14 | SD11 |
| DRQ7 | D15 | C15 | SD12 |
| Vcc | D16 | C16 | SD13 |
| Master- | D17 | C17 | SD14 |
| GND | D18 | C18 | SD15 |

## PCI Connectors 1 & 2 (J10b1 & J10c1)

| Signal Name | Pin | Pin | Signal Name |  | Signal Name | Pin | Pin | Signal Name |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GND | A1 | B1 | -12V |  | AD16 | A32 | B32 | AD17 |
| +12V | A2 | B2 | No Connect |  | 3.3V | A33 | B33 | CBE2- |
| No Connect | A3 | B3 | GND |  | FRAME- | A34 | B34 | GND |
| No Connect | A4 | B4 | No Connect |  | GND | A35 | B35 | IRDY- |
| Vcc | A5 | B5 | Vcc |  | TRDY- | A36 | B36 | 3.3V |
| PCIINT3- | A6 | B6 | Vcc |  | GND | A37 | B37 | DEVSEL- |
| PCIINT1- | A7 | B7 | PCIINT2- |  | STOP- | A38 | B38 | GND |
| Vcc | A8 | B8 | PCIINT4- |  | 3.3V | A39 | B39 | PLOCK- |
| Reserved | A9 | B9 | No Connect |  | SDONE | A40 | B40 | PERR- |
| Vcc | A10 | B10 | Reserved |  | SBO- | A41 | B41 | 3.3V |
| Reserved | A11 | B11 | No Connect |  | GND | A42 | B42 | SERR- |
| GND | A12 | B12 | GND |  | PAR | A43 | B43 | 3.3V |
| GND | A13 | B13 | GND |  | AD15 | A44 | B44 | CBE1- |
| Reserved | A14 | B14 | Reserved |  | 3.3V | A45 | B45 | AD14 |
| SPCIRST- | A15 | B15 | GND |  | AD13 | A46 | B46 | GND |
| Vcc | A16 | B16 | PCLKE |  | AD11 | A47 | B47 | AD12 |
| AGNT- | A17 | B17 | GND |  | GND | A48 | B48 | AD10 |
| GND | A18 | B18 | REQA- |  | AD9 | A49 | B49 | GND |
| Reserved | A19 | B19 | Vcc |  | KEY | A50 | B50 | KEY |
| AD30 | A20 | B20 | AD31 |  | KEY | A51 | B51 | KEY |
| 3.3V | A21 | B21 | AD29 |  | CBEO- | A52 | B52 | AD8 |
| AD28 | A22 | B22 | GND |  | 3.3V | A53 | B53 | AD7 |
| AD26 | A23 | B23 | AD27 |  | AD6 | A54 | B54 | 3.3V |
| GND | A24 | B24 | AD25 |  | AD4 | A55 | B55 | AD5 |
| AD24 | A25 | B25 | 3.3V |  | GND | A56 | B56 | AD3 |
| AD22 (IDSEL) | A26 | B26 | CBE3- |  | AD2 | A57 | B57 | GND |
| 3.3V | A27 | B27 | AD23 |  | AD0 | A58 | B58 | AD1 |
| AD22 | A28 | B28 | GND |  | Vcc | A59 | B59 | Vcc |
| AD20 | A29 | B29 | AD21 |  | SREQ64- | A60 | B60 | SACK64- |
| GND | A30 | B30 | AD19 |  | Vcc | A61 | B61 | Vcc |
| AD18 | A31 | B31 | 3.3V |  | Vcc | A62 | B62 | Vcc |

## PCI Connector 3 (J10c2, SUPPORTS IDE)

| Signal Name | Pin | Pin | Signal Name |  | Signal Name | Pin | Pin | Signal Name |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GND | A1 | B1 | -12V |  | AD16 | A32 | B32 | AD17 |
| +12V | A2 | B2 | STCK (for IDE) |  | 3.3V | A33 | B33 | CBE2- |
| No Connect | A3 | B3 | GND |  | FRAME- | A34 | B34 | GND |
| STDI | A4 | B4 | STD0 (for IDE) |  | GND | A35 | B35 | IRDY- |
| Vcc | A5 | B5 | Vcc |  | TRDY- | A36 | B36 | 3.3V |
| PCIINT3- | A6 | B6 | Vcc |  | GND | A37 | B37 | DEVSEL- |
| PCIINT1- | A7 | B7 | PCIINT2- |  | STOP- | A38 | B38 | GND |
| Vcc | A8 | B8 | PCIINT4- |  | 3.3V | A39 | B39 | PLOCK- |
| SRSVD1 (for IDE) | A9 | B9 | No Connect |  | SDONE | A40 | B40 | PERR- |
| Vcc | A10 | B10 | SRSVD2 (for IDE) |  | SBO- | A41 | B41 | 3.3V |
| SRSVD3 (for IDE) | A11 | B11 | No Connect |  | GND | A42 | B42 | SERR- |
| GND | A12 | B12 | GND |  | PAR | A43 | B43 | 3.3V |
| GND | A13 | B13 | GND |  | AD15 | A44 | B44 | CBE1- |
| SRSVD4 (for IDE) | A14 | B14 | SRSVD5 (for IDE) |  | 3.3V | A45 | B45 | AD14 |
| SPCIRST- | A15 | B15 | GND |  | AD13 | A46 | B46 | GND |
| Vcc | A16 | B16 | PCLKE |  | AD11 | A47 | B47 | AD12 |
| AGNT- | A17 | B17 | GND |  | GND | A48 | B48 | AD10 |
| GND | A18 | B18 | REQA- |  | AD9 | A49 | B49 | GND |
| SRSVD6 (for IDE) | A19 | B19 | Vcc |  | KEY | A50 | B50 | KEY |
| AD30 | A20 | B20 | AD31 |  | KEY | A51 | B51 | KEY |
| 3.3V | A21 | B21 | AD29 |  | CBEO- | A52 | B52 | AD8 |
| AD28 | A22 | B22 | GND |  | 3.3V | A53 | B53 | AD7 |
| AD26 | A23 | B23 | AD27 |  | AD6 | A54 | B54 | 3.3V |
| GND | A24 | B24 | AD25 |  | AD4 | A55 | B55 | AD5 |
| AD24 | A25 | B25 | 3.3V |  | GND | A56 | B56 | AD3 |
| AD22 (IDSEL) | A26 | B26 | CBE3- |  | AD2 | A57 | B57 | GND |
| 3.3V | A27 | B27 | AD23 |  | AD0 | A58 | B58 | AD1 |
| AD22 | A28 | B28 | GND |  | Vcc | A59 | B59 | Vcc |
| AD20 | A29 | B29 | AD21 |  | SREQ64- | A60 | B60 | SACK64- |
| GND | A30 | B30 | AD19 |  | Vcc | A61 | B61 | Vcc |
| AD18 | A31 | B31 | 3.3V |  | Vcc | A62 | B62 | Vcc |

# Appendix J - Environmental Standards

| Parameter | Condition | Specification |
| --- | --- | --- |
| Temperature |  |  |
|  | Non-Operating | -40oC to +70oC |
|  | Operating | +0oC to +55oC |
| Humidity |  |  |
|  | Non-Operating | 92% Relative Humidity max. @ 36oC |
|  | Operating | 80% Relative Humidity max. @ 36°C |
| Altitude |  |  |
|  | Non-Operating | 50,000 feet (15,240 meters) |
|  | Operating | 10,000 feet (3048 meters) |
| ESD |  |  |
|  | 1.0kV | No Errors |
|  | 2.5kV | No Errors |
|  | 5.0kV | 5% Soft Errors, 0% Hard Errors, No physical damage |
|  | 7.5kV | 10% Soft Errors, 0% Hard Errors, No physical damage |
|  | 10.0kV | 25% Soft Errors, 5% Hard Errors, No physical damage |
|  | 12.5kV | 50% Soft, 10% Hard, No physical damage |
|  | 15.0kV | 100% Soft, 25% Hard, No physical damage |
|  | 25.0kV | 100% Soft, 100% Hard, No physical damage |
| Shock | Non-Operating | 30.0G, 11ms, 1/2 sine |

Table J-1. Environmental Standards

# Appendix K - Reliability Data

The Mean-Time-Between-Failures (MTBF) data is calculated from predicted data @ 55C.

Premiere/PCI   Baby-AT baseboard			TBD (Testing in progress)

# Appendix L - Customer Support

The Premiere/PCI Baby-AT is backed by Intel's industry-leading support groups in the OEM Products and Services Division (OPSD), including IntelTechDirectä, which includes these major services:

## iNTEL APPLICATIONS SUPPORT BBS (iPAN)

Intel Product Assistance Network Bulletin board with product information, demo software and more.

- Available worldwide through direct-dial

U.S	916-356-3600

Europe	44-793-496340

- Modem speeds up to 14.4k baud with standard software

- FLASH BIOS upgrade files

- Modem set at no parity, 8 data bits, 1 stop bit.

## iPUB

Intel Product Update Bulletin.

- Monthly Product updates available 24 hours a day from iPAN

- Official notification of engineering changes and technical data

- Easy information retrieval using Windows Help file format

- Intel platform system, board, and BIOS revision histories

- Hardware and software compatibility notes

- Documentation updates, spare parts and order information

## iPALS

Intel Phone Action Line Support provides a direct link to highly qualified and well trained technical personnel.

- Toll-free access to Intel support engineers for problem resolution

- Responses within 24 hours Monday-Friday

- Expert assistance geared to the special needs of OEMs and VARs

## FaxBack

- Product descriptions and technical data sent to any fax machine from a touch-tone phone

- Information on End-of-Life products

- Available worldwide:	U.S. Toll Free	1-800-628-2283 (1-916-356-3105)

Europe		44-793-496646