OPSD BIOS ENGINEERING

Intel Corporation, 5200 NE Elam Young Parkway
Hillsboro, OR  97214-6497

DATE:	August 18,1994

TO:	

FROM:	BIOS Engineering

SUBJECT:	BIOS 19  (AM0)

About This Release

o	BIOS 0.01.19.AM0 (AM0)

Features/Errata Fixed In This Release

o	Fixes 256M and greater caching anomoly.

o	Altered EISA function return codes to accomodate changes in installed memory size without invalidating all EISA data structures.

o	Only Slot 0 function 0 returns 1 for choice 25, earlier BIOSs returned 1 for all slots and functions. (We are using S1/F1/C25 as a BIOS/ECU communications bit).

o	DPT with IDE failure, root cause was fixed in the planar BIOS IDE code.

o	Error code and screen management was re-worked to assure persistence of some error codes on the screen.

o	Parity RAM failures are isolated and reported to the SIMM level.

o	Base memory is cleared on warm boot.

o	Fixed error loging that was broken in BIOS 18.

o	Assure DX is zero before boot attempt.

Features Included In This Release

o	Changed to more performance oriented defaults in setup.

o	Bypasses the “keyboard lock boot pause” when Network password feature is enabled.

o	All Adaptec initialization code (with the exception of H/W disable) was removed from the BIOS. BIOS relies in the SCSI BIOS to correctly accomplish

Known Errata With This Release

o	None.

Features Still To Be Added / Comments About This Release

o	None.