**********************************************

*	 README  For SAMSUNG CD-ROM Drives       *

*                  May   1996                *

**********************************************

(c) Copyright SAMSUNG Electronics , 1996

====================================

Tips on using MSCDEX.EXE switch

====================================

1. [ /M:10 ]

It is the added switch in AUTOEXEC.BAT when running CDSETUP.EXE.

( Not specified in the user's manual)

===========================================

Tips on using SSCDROM.SYS switches

( Not specified in the user's manual)

===========================================

1. [ /S:xxx,yy,z ]

Specifies the IDE port that is connected to the drive, and the interrupt request.

Where

* xxx is the IDE port address

xxx = 1f0 : Primary (Channel 1)

170 : Secondary(Channel 2)

1e8 : Channel 3

168 : Channel 4

* yy  is the Interrupt channel.

yy  = 14 : Primary (Channel 1)

15 : Secondary (Channel 2)

11 : Channel 3

10 : Channel 4

* z is the Master/Slave selection.

z =  0 : Master

1 : Slave

Note: If /s:xxx,yy,zz is valid,the device driver will only test that

specific IDE port, and skip the auto-detection. If the /s switch is

not used or if any parameter is invalid, the device driver will still use

auto-detection algorithm.

For example, when the CD-ROM drive is connected to the secondary port using

interrupt 15 as slave, the syntax in the CONFIG.SYS file should be as follows:

Device = C:\SAMSUNG\SSCDROM.SYS /D:SSCD000 /S:170,15,1 /V

2. [ /V ]

Displays CD-ROM Drive Model Name.