IDECHECK v1.1 - Fixed Disk Subsystem Integrity and Stress Test

==============================================================

IDECHECK performs a harddisk subsystem test by writing and reading large

amounts of data in several threads at a time, thus stressing the IDE

controller to its limits. There are some known problems with PCI boards

and IDE timing (see APAR PJ15378)**. These problems cause data-corruption,

and are caused by hardware-timing problems on the main board. A replacement

IBM1S506.ADD is available.

IDECHECK will detect these problems, and might detect other (yet unknown)

problems with your new hardware.

Invocation:

===========

IDECHECK [/w:nnn] [/r:nnn] [/t:nn] [/c]

Where:

nn  is an integer between 1 and 99

nnn is an integer between 1 and 999

/w:nnn is the number of write-operations that should be performed per

thread. (Default is 50).

/r:nnn is the number of read-operations per write. (Default is 100).

/t:nn  is the number of concurrent R/W threads that should be started.

(Default is 10).

/c     causes IDECHECK to continue after an integrity error was found.

(Default is to exit).

Each thread performs a write operation, followed by a number of read-operations

on a randomly chosen fixed disk. The data read is compared against the data

which was initially written to disk.

Each thread will create a data file of 0.5 Megabytes in size, which will

automatically be deleted if no errors are encountered.

Beware that the amount of data transferred from/to disk is:

0.5 Mb x NumThreads x (NumReads x NumWrites + NumWrites),

so that the defaults will cause a total data-transfer of:

0.5 Mb x 10 x (50 x 100 + 50) = 25250 Mb, or approx. 25 Gb !!

As you can imagine this test is pretty thorough, but also takes a fair

amount of time, and can best be ran overnight on the hardware to be tested.

You can of course override the values with the command-line parameters for

a faster (and less thorough) test, or increase them to *really* hurt the HW.

**Known PCI/VLB board IDE problems:

===================================

Currently follwing HW problems are known:

- PC-Tech chipset RZ1000

- Intel Pentium PCI main board(s)

Error Description:

When doing I/O on IDE hard-drives data becomes corrupted. No errors appear,

and VERIFY ON does NOT solve this! Only way to see this behaviour is by

issuing a COMPARE command after the copy (or by running IDECHECK!).

Symptoms:

CPS error in HPFS, corruption, random TRAPs/hangs.

Cause:

These chipsets fill a buffer with data from disk, but trigger interrupt

too early. OS/2's IRQ handler will empty the buffer BEFORE all data is

actually there... Result is last 2 bytes of buffer drop, and data is

copied incorrectly (2 bytes missing in target file, size still matches).

VERIFY ON will assume the incorrect buffer is correct, and will verify

okay after the write.

Comments, Suggestions and Bugs...

=================================

Please direct all of the above to:

Jeroen van den Horn

IBM Personal Software Products

The Netherlands

Internet: horne@vnet.ibm.com

IEA:      NLIBMGFP@IBMMAIL

VNET:     NL34086 at EAMSVM1

Or contact somebody from IBM, who will most certainly be able to hunt me down.