The following program is provided as is without any warranty of any

kind, either expressed or implied. The entire risk as to the quality and

performance of the program is with you the user. In no event will MAXTOR

CORPORATION be liable to you for any damages, including any loss of profit

or savings, arising out of the use or inability to use this software.

Maxtor Corporation makes no representations or warranties with respect to

the contents hereof and specifically disclaims any implied warranties of

merchantability or fitness for any particular purpose.

GREENDRIVE 2.2 For Maxtor 7000AT Series Drives:

By:  Sean E. Dykstra

Greendrive utilizes the IDLE command in the ATA specification.

This allows the user to specify a given timeout value in seconds

to spin down the drive.  The timeout value can be any number

between 20 and 1279.  Any other number will be recognized as a 0

by the program.

PLEASE NOTE:  While this does save power, it also will cause

additional wear and tear on  the drive (spinning the drive up and down).

The user may also specify to send the command to the Master, the Slave,

or both.  The proper protocol is as follows:

greendrv [timeout value] [m/s] [m/s]

Below are several examples utilizing this program:

Master only, spin down drive after 100 seconds:

greendrv 100 m

Slave only, spin down after 30 seconds:

greendrv 30 s

Master and Slave, spin both down after 200 seconds:

greendrv 200 m s

DISABLE low power mode on Master and Slave:

greendrv 0 m s

DISABLE low power mode on Slave only

greendrv 0 s

NOTE:  Please verify with your hard drive manufacturer that your hard drive

properly supports low power mode IDLE (E3h).

Usage: "GREENDRV XXX" where XXX is the number of seconds from 20 to 1279

(21 minutes 19 seconds maximum).

Entering GREEDRV with no XXX values will spin down the drive but disable

the the "spindown after delay" mode.

Put this program in your AUTOEXEC.BAT file so the drive will be programmed

at startup otherwise the drive will revert to no standby mode after a reset.

Rev C  3/15/94  Dykstra/Anderson