Here is a short explaination of how to update an mhs system to

the new driver files:

The update procedure would be:

1) halt your mhs and exit it.

2) (on a network) re-login as a supervisor equivalent (necessary to

change the mhs configuration and place the files in the directory)

3) copy the new LOG and DEF files to the MHS\SYS directory

4) attach you new modem

5) run MHS with 2 returns (to get to the main menu)

select directory manager

select set up this host

6) go to host set up

set modem type to the correct driver name HSTD-HUB

set Default speed for modem to 19200

7) exit back to the directory manager

exit back to the main menu

8) go to transport server setup

set Speed sensing enabled to NO.

change the modem type to the correct driver name (same as in step 6)

change the modem speed to 19200

9) Make sure that all host entries which are "hosts which this host calls"

have a baud rate of 19200 set in their host defination at this host.

10) exit mhs, logout, and log mhs back in (for non network just restart MHS).

NOTE: If you select 600 baud (instead of 19200) the ACTUAL baud rate used

is 38.4k.  THIS IS NOT THE RECOMENDED SETTING. If you want to use

this baud rate it will require 286/12 computers AT LEAST.

Try it if you want to.

SPECIAL DEBUGING NOTES:

if you use the SPECIAL 600 baud entry (to get 38.4k) make sure you select

600 baud in steps 6, 8, and 9.

If you can't get the modem to respond, try this:

1) power the modem OFF.

2) set switch 10 DOWN.

3) power the modem ON.

4) run MHS BUT DO NOT ALLOW IT TO MAKE ANY OUT GOING CALLS.

i.e.  make sure you have no outgoing mail, and that there are no FORCED

calls pending.  Then start the connectivity manager.  Let it get

to WAITING FOR CALLS, then hit ESC and exit out of mhs.

5) DO NOT TURN THE MODEM OFF.

6) set switch 10 UP.

7) turn the modem off now.

8) turn the modem on.

9) test the modem with mhs to make sure it can now talk to the modem.