U-1496 SERIES UNIVERSAL MODEM User's Manual

===========================================

Document No.: 81053001

DOC NO. = Rev. 2.1

ZyXEL COMMUNICATIONS CORPORATION

--------------------------------

NOTICE: ZyXEL COMMUNICATIONS CORPORATION does not assume any liability

arising out of the application or use of any products, or software described

herein, neither does it convey any license under its patent rights nor the

patent rights of others.

ZyXEL COMMUNICATIONS CORPORATION further reserves the right to make changes

in any products described herein without notice. This document is subject to

change without notice.

PUBLISHED BY:

ZyXEL COMMUNICATIONS CORPORATION

2F., 58 Park Avenue II

Science-Based Industrial Park

Hsinchu, Taiwan 30077 R.O.C.

1992 by ZyXEL Communications Corporation

All rights reserved. No part of the contents of this book may be reproduced

or transmitted in any form or by any means without the written permission of

the publisher.

ACKNOWLEDGEMENTS:

-----------------

U-Modem, ZyFAX are trademarks of ZyXEL Communications Corporation.

Smartmodem is a registered trademark of Hayes Microcomputer Products, Inc.

MNP is a registered trademark of Microcom, Inc.

IBM PC, XT, AT are trademarks of International Business Machine Corporation.

Touch-tone is a trademark of American Telephone and Telegraph Corporation.

WINDOWS is a trademark of Microsoft Corporation.

FCC PART 15 INFORMATION

=======================

This device complies with Part 15 of the FCC rules, operation is subject to

the following two conditions:

(1) This device may not cause harmful interference.

(2) This device must accept any interference receivedincluding interference

that may cause undesired operations.

For U-1496 & U-1496B:

---------------------

NOTE:

This equipment has been tested and found to comply with the limits for a

CLASS A Digital Device pursuant to Part 15 of the FCC rules. These limits

are designed to provide reasonable protection against harmful interference

when the equipment is operated in a commercial environment. This equipment

generates, uses, and can radiate radio frequency energy and if not installed

and used in accordance with the instruction manual, may cause harmful

interference to radio communications. Operation of this equipment in a

residential area is likely to cause harmful interference in which case the

user will be required to correct the interference at his own expense.

Changes or modifications not expressly approved by the party responsible for

compliance could void the user's authority to operate the equipment. The

shielded RS-232 cable are required to be used in order to ensure compliance

with FCC Part 15, and it is the responsibility of the user to provide and

use shielded RS-232 cable.

For U-1496E:

------------

NOTE:

This equipment has been tested and found to comply with the limits for a

CLASS B digital device pursuant to Part 15 of the FCC Rules. These limits

are designed to provide reasonable protection against harmful interference

in a residential installation. This equipment generates, uses and can

radiate radio frequency energy and if not installed and used in accordance

with the instructions, may cause harmful interference to radio

communications. However, there is no guarantee that interference will not

occur in a particular installation.

If this equipment does cause harmful interference to radio/television

reception, which can be determined by turning the equipment off and on, the

user is encouraged to try to correct the interference by one or more of the

following measures:

- Reorient or relocate the receiving antenna.

- Increase the separation between the equipment and the receiver.

- Connect the equipment into an outlet on a circuit different from that to

which the receiver is connected.

- Consult the dealer or an experienced radio/TV technician for help.

Changes or modifications not expressly approved by the party responsible for

compliance could void the user's authority to operate the equipment.

NOTE:

U-1496E FCC ID# I88U1496E

FCC REQUIREMENTS

================

This equipment complies with Part 68 of the FCC Rules. On the base unit of

this equipment is a label that contains, among other information, the FCC

Registration Number and the Ringer Equivalence Number (REN) for this

equipment. IF REQUESTED, THIS INFORMATION MUST BE GIVEN TO THE TELEPHONE

COMPANY.

The REN is useful to determine the quantity of the devices you may connect

to your telephone line and still have all of those devices ring when your

telephone number is called. In most, but not all areas, the sum of the REN's

of all devices connected to one line should not exceed five (5.0). To be

certain of the number of devices you may connect to your line, as determined

by the REN, you should contact your local telephone company to determine the

maximum REN for your calling area.

If your equipment causes harm to the telephone network, the telephone

company may discontinue your service temporarily. If possible, they will

notify you in advance. But if advance notice isn't practical, you will be

notified as soon as possible. You will be informed of your right to file a

complaint with the FCC.

Your telephone company may make changes in its facilities, equipment,

operations or procedures that could affect the proper functioning of your

equipment. If they do, you will be notified in advance to give you an

opportunity to maintain uninterrupted telephone service. If you experience

trouble with this telephone equipment, please contact the Address and Phone

number listed in the warranty card for information on obtaining service or

repairs.

The telephone company may ask that you disconnect this equipment from the

network until the problem has been corrected or until you are sure that the

equipment is not malfunctioning.

This equipment may not be used on coin service provided by the telephone

company. Connection to party lines is subject to state tariffs.

TELEPHONE COMPANY REQUIREMENTS

==============================

It is not necessary to notify your telephone company before installing the

modem but your telephone company may request the following informations:

- Telephone number to which the modem is connected.

- Manufacturer and Model Number:

ZyXEL Communications Corporation

Model Name               FCC number          REN#

U-1496                   1ROTAI-18563-MD-E   1.1B

U-1496 B                 1ROTAI-18518-MD-E   0.8B

U-1496 E                 1ROTAI-65020-MD-E   0.5B

Information for Canadian Users

The U-1496 series modems are certified by the Canadian Department of

communications (DOC) on a family approval basis. Listed below are the

certificate number, certification number, and load number of each model.

Model Name       Certificate No.  Certification No.  Load No.

U-1496           9165             1817 4783 AB       11

U-1496E          9166             1817 4783 AB       11

U-1496B          9167             1817 4783 AB       11

U-1496R          9168             1817 4783 AB       11

The Canadian Department of Communications (DOC) label identifies certified

equipment. This certification means that the equipment meets certain

telecommunications network protective, operational and safety requirements.

The department does not guarantee the equipment will operate to a user's

satisfaction.

The Load Number (LN) assigned to each terminal device denotes the percentage

of the total load to be connected to the telephone loop used by the device

without overloading. The termination on a loop may consist of any

combination of devices, subject only to the requirement that the total of

the Load Numbers of all the devices not exceed 100.

Limited Warranty

================

ZyXEL Communications Corporation warrants to the original customer or other

end user purchaser that this product is free from defects in materials or

workmanship for a period of two (2) years from the date of purchase. During

the warranty period, and upon proof of purchase, should the product have

indications of failure due to faulty workmanship and/or materials, ZyXEL

will, at its option, repair or replace the defective products or components

without charge for either parts or labor, and to whatever extent it shall

deem necessary to restore the product or component to proper operating

condition. Any replacement will be either a new or remanufactured

functionally equivalent product of equal value, and will be solely at the

option of ZyXEL.

This warranty shall not apply if the product is modified, misused, tampered

with, or subjected to abnormal working conditions.

NOTE:  REPAIR OR REPLACEMENT AS PROVIDED UNDER THIS WARRANTY IS THE

EXCLUSIVE REMEDY OF THE PURCHASER. THIS WARRANTY IS IN LIEU OF ALL OTHER

WARRANTIES, EXPRESS OR IMPLIED, INCLUDING ANY IMPLIED WARRANTY OF

MERCHANTABILITY OR FITNESS FOR A PARTICULAR USE OR PURPOSE, AND ZYXEL

COMMUNICATIONS CORPORATION SHALL IN NO EVENT BE HELD LIABLE FOR INDIRECT

OR CONSEQUENTIAL DAMAGES OF ANY KIND OR CHARACTER TO THE PURCHASER.

To avail of the services of this warranty, please contact ZyXEL SERVICE

CENTER  (refer to separate WARRANTY CARD)for your Return Authorization

Number (RAN).

Products must be returned Postage Prepaid. It is recommended that the unit

be insured when shipped. Any returned products without proof of purchase or

out-dated warranty will be repaired or replaced (at the option of ZyXEL) and

the customer will be billed for parts and labor.

All repaired or replaced products will be shipped by ZyXEL to the

corresponding return address, Postage Paid (US and territories only).

If the customer desires some other return destination beyond the USA

borders, the customer shall bear the cost of the return shipment.

This warranty gives you specific legal rights, and you may also have other

rights which varies from state to state.

Contents                              (I)

========

CHAPTER 1                INTRODUCTION                            1-1

THE U-1496  SERIES  UNIVERSAL  MODEMS   1-1

HOW TO  USE  THIS  MANUAL               1-1

COMPATIBILITY                           1-2

THE U-1496 SERIES STANDARD FEATURES     1-3

BEFORE  YOU  GET STARTED                1-4

HOW  TO BECOME  A  REGISTERED  OWNER    1-4

WHAT  YOU  NEED TO  HAVE                1-4

COMMUNICATION SOFTWARE                  1-5

CONNECTING TO YOUR PHONE                1-5

DIAL-UP OR LEASED LINE                  1-5

CHAPTER 2                MODEM AND FAX BASICS                    2-1

WHAT IS  MODEM                          2-1

WHAT IS DTE AND DCE                     2-1

WHAT IS RS-232C OR  EIA-232D            2-1

WHAT IS SERIAL PORT                     2-1

SERIAL RS-232C CABLE                    2-1

SYNCHRONOUS AND  ASYNCHRONOUS

COMMUNICATION                           2-2

WHAT IS UART                            2-2

MODEM STANDARDS AND SPEEDS              2-2

TYPE OF TELEPHONE  LINE                 2-3

INTELLIGENT MODEM                       2-3

AT COMMAND  SET                         2-3

V.25bis COMMAND  SET                    2-3

ERROR CORRECTION                        2-4

DATA  COMPRESSION                       2-4

MNP  PROTOCOLS                          2-4

V.42bis AND V.42                        2-4

XMODEM,  YMODEM, AND ZMODEM             2-4

FAX AND FACSIMILE                       2-4

FAX CARD                                2-5

MODEM AS A FAX                          2-5

EIA CLASS 2  FAX COMMAND                2-5

AUTODETECTION OF FAX OR DATA CALL       2-5

CALLER ID                               2-5

DISTINCTIVE RING                        2-6

DIGITIZED VOICE                         2-6

CELLULAR MODEM                          2-6

CHAPTER 3               MODEM INSTALLATION                      3-1

U-1496 FRONT PANEL                      3-1

U-1496 FRONT PANEL INDICATORS           3-1

LED Indicators                       3-1

U-1496 FRONT  PANEL SWITCHES            3-1

U-1496  REAR PANEL MARKINGS             3-2

MODEM  CONNECTION                       3-2

POWERING  UP                            3-2

U-1496E  FRONT  PANEL                   3-3

U-1496E  FRONT PANEL INDICATORS         3-3

LED Indicators                       3-3

U-1496E  FRONT PANEL SWITCHES           3-3

U-1496E  REAR PANEL MARKINGS            3-3

(II)

CHAPTER 4               QUICK START                             4-1

LCD PANEL                               4-1

PANEL OPERATION                         4-1

MENU TREE                               4-2

ARROW ON THE SCREEN                     4-2

DIALING                                 4-2

DIAL MEMORY                             4-2

STORE NUMBER                            4-3

DIAL NUMBER                             4-3

MANUAL DIAL AND DATA/VOICE SWITCHING    4-3

REPEAT LAST DIAL                        4-4

AUTO ANSWER                             4-4

DIALING MESSAGE                         4-4

PANEL LOCK                              4-4

CHAPTER 5               MODEM PARAMETER SETTINGS                5-1

PARAMETER MENU                          5-1

PARAMETER SELECTION                     5-1

REGISTER'S CONTENT MODIFICATION

(STATUS REGISTER)                       5-2

TERMINAL OPTIONS                        5-2

DATA FORMAT                             5-2

CHARACTER LENGTH                        5-2

COMMAND SET                             5-2

DTE RATE                                5-3

DTR OPTIONS                             5-3

DCD OPTIONS                             5-3

RTS OPTIONS                             5-3

DSR OPTIONS                             5-3

MODEM OPTIONS                           5-4

LINK OPTIONS                            5-4

QUALITY ACTION                          5-4

DEFAULT DIAL                            5-5

DIAL BACKUP                             5-5

GUARD TONE                              5-5

RDL REQUEST                             5-5

LLINE TX POWER                          5-5

PHONE JACK                              5-5

MAKE/BREAK RATIO                        5-5

SECONDARY CHANNEL (Optional feature)    5-6

SYNC CLOCK                              5-6

AUTO HANDSHAKE                          5-6

LINE TYPE                               5-6

ERROR CONTROL                           5-6

FLOW CONTROL                            5-6

NEGOTIA(TION) FALLBACK                  5-7

BREAK HANDLING                          5-7

AUDIO  OPTIONS                          5-7

SPEAKER CONTROL                         5-7

SPEAKER VOLUME                          5-7

RING VOLUME                             5-7

(III)

CHAPTER 6               PROFILES                                6-1

AT&V0                                   6-1

AT&V1                                   6-2

AT&V2                                   6-2

AT&V3                                   6-3

AT&V4                                   6-4

AT&V5                                   6-4

RESET FROM PROFILE                      6-5

SAVE TO PROFILE                         6-5

RESET  PROFILES                         6-5

CHAPTER 7               STATUS REGISTERS                        7-1

READ AND SET S-REGISTERS                7-1

S-REGISTER  DESCRIPTIONS                7-1

CHAPTER 8               AT COMMAND SET SUMMARY                  8-1

BASIC AT COMMAND SET                    8-1

EXTENDED AT  COMMAND SET                8-3

ENHANCED AT COMMAND SET                 8-5

CHAPTER 9               ERROR CONTROL AND DATA COMPRESSION      9-1

ERROR  CONTROL                          9-1

DATA COMPRESSION                        9-2

Run-length Encoding                  9-2

Adaptive Frequency Encoding          9-2

String Coding                        9-2

NEGOTIATION FAIL FALL BACK              9-3

ADAPTIVE  BLOCK SIZE                    9-3

AVERAGING  THROUGHPUT                   9-3

HINT FOR HIGH-SPEED OPERATION           9-4

FLOW  CONTROL                           9-4

Hardware CTS/RTS Flow Control        9-4

Software XON/XOFF Flow Control       9-4

CHAPTER 10              SYNCHRONOUS OPERATION                  10-1

CLOCK OPTIONS                          10-1

RTS OPTIONS                            10-1

MODE OPTIONS                           10-1

DIAL FROM SYNCHRONOUS MODE             10-1

AUTO-ANSWER FROM SYNCHRONOUS MODE      10-2

MANUAL  ANSWER FROM SYNCHRONOUS  MODE  10-2

CHANGE FROM SYNCHRONOUS MODE TO

ASYNCHRONOUS MODE                      10-2

CHAPTER 11              LEASED  LINE  OPERATION                11-1

LEASED LINE CONFIGURATION              11-1

LINE TYPE                              11-1

POWER LEVEL                            11-1

HANDSHAKE MODE                         11-1

MANUAL CONNECTION                      11-2

AUTO-HANDSHAKE                         11-2

LEASED LINE DIAL-BACKUP                11-2

DIAL BACKUP                            11-2

ABORT FROM  LEASED LINE                11-3

DISCONNECT LEASED LINE                 11-3

(IV)

CHAPTER 12              SPECIAL FUNCTIONS                      12-1

SECURITY FUNCTION                      12-1

REMOTE CONFIGURATION                   12-2

CALLER  NUMBER  DELIVERY  (CND)        12-3

DISTINCTIVE RING                       12-4

V.25bis COMMAND SET                    12-5

CHAPTER 13              FAX OPERATION                          13-1

THE CCITT T.30 FAX PROTOCOL            13-1

THE FAX COMMAND SETS                   13-1

The Class 2 Command Set             13-1

Extended FAX AT Commands            13-2

Mode Setting                        13-2

Parameter Setting                   13-3

STATUS REPORT RESULT CODES             13-3

FLOW CONTROL                           13-4

PARALLEL RECEIVE WITH THE FAX MACHINE  13-4

RECEIVE FAX IN THE BBS SYSTEM          13-5

CHAPTER 14              ADVANCED VOICE CAPABILITY              14-1

VOICE DATA COMPRESSION                 14-1

VOICE  AT  COMMANDS                    14-1

COMMAND SYNTAX                         14-2

RESPONSE SYNTAX                        14-2

FLOW  CONTROL                          14-2

VOICE PARAMETER COMMANDS               14-2

VOICE ACTION  COMMANDS                 14-4

CHAPTER 15              DIAGNOSTICS                            15-1

POWER-ON SELF-TEST                     15-1

ANALOG LOOPBACK TEST (AT&T1)           15-2

ANALOG LOOPBACK WITH SELF-TEST (AT&T8) 15-2

LOCAL DIGITAL LOOPBACK TEST (AT&T3)    15-2

REMOTE DIGITAL LOOPBACK TEST (AT&T6)   15-3

REMOTE DIGITAL LOOPBACK WITH

SELF-TEST (AT&T7)                      15-3

LINE CONDITION STATUS DISPLAY          15-3

Signal to Noise Ratio (S/N)         15-4

Receiving Power Level (RX)          15-4

Phase Jitter (PJ)                   15-4

Frequency Offset (FO)               15-4

Retrain Granted (RG)                15-4

Retrain Requested (RT)              15-4

Round Trip Echo Delay (ED)          15-4

Carrier Loss Counter (CL)           15-4

LINK STATUS REPORT  (ATI2)             15-5

THROUGHPUT DISPLAY                     15-6

RE-TRANSMISSION INDICATOR              15-6

DIALING INDICATOR                      15-6

HANDSHAKING AND RETRAIN INDICATOR      15-6

EYE PATTERN GENERATOR                  15-7

RESETTING THE MODEM                    15-7

FIRMWARE UPDATE                        15-7

(V)

CHAPTER 16              U-1496B PC-INTERNAL CARD INSTALLATION  16-1

THE U-1496B MODEM                      16-1

COMMUNICATION PORT SETUP               16-1

CHAPTER 17              HINTS, TIPS, QUESTIONS, AND ANSWERS    17-1

LOW THROUGHPUT OR  LOSE DATA IN

FILE RECEIVING                         17-1

USE V.25bis COMMAND SET                17-1

WANT TO DISABLE DATA COMPRESSION       17-1

USE WITH WINFAX OR WINFAX PRO SOFTWARE 17-1

USE WITH BITFAX  SOFTWARE              17-1

USE WITH VIDEOTEXT SERVICE

(BTX, MINITEL, ETC.)                   17-1

USE WITH PROCOM PLUS  SOFTWARE         17-1

USE WITH TELIX SOFTWARE                17-1

USE WITH PCANYWHERE SOFTWARE           17-1

USE WITH CARBON COPY SOFTWARE          17-2

USE WITH CLOSE-UP SOFTWARE             17-2

USE WITH QMODEM SOFTWARE               17-2

NEED FAX SOFTWARE FOR UNIX?            17-2

TIPS FOR USING MODEM IN UNIX SYSTEM    17-2

TIPS FOR LEASED LINE USAGE             17-2

APPENDIX A              DTE/DCE EIA-232D INTERFACE SIGNALS      A-1

APPENDIX B              PHONE JACK PIN ASSIGNMENTS              B-1

APPENDIX C              V.25bis  COMMAND SET                    C-1

CHAPTER 1                                                           (1 - 1)

INTRODUCTION

============

THE U-1496 SERIES UNIVERSAL MODEMS

There are four models in the U-1496 series. They are:

MODEL = U-1496 External stand alone model with 20 x 2 LCD display and

menu control. Includes 2/4-wire leased line support.

MODEL = U-1496E External stand alone model with LED status indicators.

2-wire application only.

MODEL = U-1496B PC internal card version.

MODEL = U-1496R Rack mount modem card version. Up to 16 U-1496Rs can be

housed in a RS-1600 rack with central panel control and 20 x 4 LCD

display.

There are also models with plus (+) suffix, such as U-1496E+, and

U-1496B+. These plus versions have a faster processor and more memory

space that they can have higher speed upgradability, advanced voice

capability, and more enhanced feature update. The basic feature, function,

and operation of the plus versions are the same as the normal verison. We

do not specifically mention the plus version in this manual unless there

are functional or operational differences. All new U-1496 and U-1496R are

enhanced models.

HOW TO USE THIS MANUAL

======================

This manual describes the use of four models in the U-1496 series, the

U-1496, U-1496E, U-1496B, and U-1496R. Basically, this manual describes

the operation of the U-1496, the model with the most functions. If

something does not apply to a certain model, just skip that description.

The panel operation of the U-1496 is described. The U-1496 has an LCD

display menu control panel. The rack system has a central menu control

panel similar to that of U-1496. The other models can consult the

equivalent AT command of each menu operation description because they use

the AT command for the same function.

Chapter 2 describes the basics of modem operation and related matters.

Novice user can get a quick understanding of modem operation, features,

and usage.  U-1496B has an installation guide in Chapter 16. The COM port

and interrupt number must first be properly chosen before the modem/FAX

card can be used.  The RS-1600 rack system comes with a supplemental

User's Guide describing the use and installation of the rack system.

COMPATIBILITY                           (1 - 2)

=============

The ZyXEL U-1496 series modems are high performance universal modems

capable of transmission speed up to 14.4 Kbps on a 2-wire full-duplex

mode. Universal compatibility covers a broad range of CCITT and BELL

standards, and provides data compression and error free transmission.

Various operation modes that can be achieved are as follows:

Bit rate	   Baud rate	 Modulation	 Carrier

(+/-0.01%)   (+/-0.01%)                     Freq

bps          baud                         Hz

V.33            14,400        2400          128-TCM         1800

V.33            12,000        2400           64-TCM         1800

V.32bis         14,400        2400          128-TCM         1800

V.32bis         12,000	      2400           64-TCM         1800

V.32bis	         7,200        2400           16-TCM         1800

V.32             9,600        2400           32-TCM         1800

V.32 uncoded     9,600        2400           16-QAM         1800

V.32             4,800        2400           4-DPSK         1800

V.29             9,600        2400           16-QAM         1700

V.29             7,200        2400            8-QAM         1700

V.29             4,800        2400           4-DPSK         1700

V.27ter          4,800        1600            8-PSK         1800

V.27ter          2,400        1200           4-DPSK         1800

V.23           1200/75     1200/75              FSK

V.23            600/75      600/75              FSK

V.22bis          2,400         600           16-QAM         1200 Orig.

2400 Ans.

V.22             1,200         600           4-DPSK         1200 Orig.

(BELL 212A)                                                 2400 Ans.

V.21               300         300             FSK

BELL 103           300         300             FSK

G3 Fax          implemented according to T.30, V.17, V.29 and V.27ter.

1800 Hz guard tone for V.22bis/V.22 answer mode, 6 dB below data signal

level.

Note:

V.33, V.29, and V.27ter are only available in models U-1496 and

U-1496R for 4-wire leased line operation. The other operation modes

are common to all models.

THE U-1496 SERIES STANDARD FEATURES                (1 - 3)

===================================

Synchronous/Asynchronous operations for external stand alone model and

rack mount model.

Asynchronous operations for PC internal card model.

MNP class 4/5 error correction/data compression.

V.42/V.42bis error correction/data compression.

Extended AT command set.

V.25bis async/sync command set.

Operate on 2-wire dial-up line or 2/4-wire leased line. (4-wire leased

line on U-1496 and U-1496R only)

Auto-dial/answer and manual originate/answer.

Tone/Pulse dialing.

Dial tone, busy, and ringback detection.

Programmable speaker volume control.

Non-volatile memory storage.

Remote configuration.

Security call back.

Caller ID detection.

Distinctive ring detection.

Diagnostics available for:           Modem full self test

Analog loopback (w/ self test)

Remote digital loopback (w/ self test)

Digital loopback

XON/XOFF or CTS/RTS flow control.

Line status monitoring available for: Signal-to-noise ratio

(U-1496 and U-1496R)                  Receive signal level

Frequency offset

Phase jitter

Retrain granted

Retrain requested

Round trip echo delay

Carrier loss counter

20 X 2 LCD display and directional keypads. (U-1496, 20 x 4 on rack)

G3 Fax send and receive capability with speed up to 14400 bps.

Automatic detection of data or fax call.

Digitized voice capability with speech compression.

BEFORE YOU GET STARTED                       (1 - 4)

======================

Before you proceed further, please check to see if you have all the

necessary materials from the shipping package and make sure that nothing

is missing. The complete package should include:

one (1) U-1496 series universal modem

one (1) power adapter pack (stand alone model only)

one (1) RJ11 telephone cable

one (1) JM8 leased line cable (U-1496 only)

one (1) U-1496 series user's manual

one (1) software floppy disk

one (1) warranty card

one (1) registration card

Contact your dealer or the store where you bought the modem immediately if

anything is missing.

After you unpack the modem, please check for shipping damage. If you find

any damage, contact the shipping agency immediately.

Please retain the shipping carton and

cushioning materials for future storage or shipping needs.

Please direct any additional questions about damaged or missing materials

to your dealer or distributor or to factory with address listed on your

warranty card.

HOW TO BECOME A REGISTERED OWNER

================================

Simply fill in the registration card and mail this to the address found on

the reverse side of the card. Registered owner will get future product and

update informations. You don't have to register in order to get warranty

service.  A dated invoice as proof of purchase along with the warranty

card are sufficient.

WHAT YOU NEED TO HAVE

=====================

You need the following equipments to be able to operate your modem:

Computer/Terminal with communication software.

RS-232 serial cable (not supplied with modem).

Note: Internal PC card does not need this.

A telephone line from your telephone company (dial-up or leased

line).

COMMUNICATION SOFTWARE                     (1 - 5)

======================

Communication software controls the modem connected to the computer and

directs data to and from the modem. For most cases in the PC environment,

the modem is used in an asynchronous mode and the software controls the

modem by the AT command set.

ZyXEL modems support the extended AT command set and are compatible with

most communication software. However, there are some unique features in

the ZyXEL modem and there are some unique commands to control them.

Some communication software needs a "modem data file" to tell it what

commands and features the modem supports.  The accompanied floppy disk

includes some sample data files for some popular communication softwares.

CONNECTING TO YOUR PHONE

========================

If you would like to use your phone for talking and modem connection,

remove your phone cable from the RJ11 junction box and plug one end of the

RJ11 cable (supplied with unit) to the phone junction, then plug the other

end of the cable to the rear panel of the modem labeled "DIAL-UP LINE" or

"WALL". Finish by plugging in your phone cable to the rear panel of the

modem labeled "PHONE". Please refer your connection to the illustration In

Fig. 3-3.

DIAL-UP OR LEASED LINE

======================

The U-1496E and U-1496B can connect to 2-wire dial-up or leased line only.

The U-1496 and U-1496R offer you a choice of connecting up to 2-wire

dial-up line (more commonly known as public lines to which telephone users

have access) and 2/4-wire leased line.

To use the U-1496 on a leased line, you must order and install a USOC JM8

jack. You must also specify 4-wire operation without TEK leads for the

wiring option.

When setting up a pair of modems for leased line point to point operation,

one must be configured for originate mode and the other for answer mode.

Both RJ11C and JM8 cables are supplied for your convenience.

CHAPTER 2                                                           (2 - 1)

MODEM AND FAX BASICS

====================

This chapter is intended for those who know nothing or do not know much

about what is modem and fax and how they are used. This chapter also

explains some of the terminologies used in modem and fax.

Knowledgeable persons can just skip this chapter.

WHAT IS MODEM

-------------

Modem is a compound word of MOdulator and DEModulator, it is used for

computer communication. Refer to Fig. 2.1 for modem application.

Modem translates computer data to analog signal (modulation) that can

travel through the telephone network and reaches the other modem. The

remote modem translates the analog signal received back to data

(demodulation) and sends to the receiving end computer. Modem is the

"phone set" used in computer talk.

WHAT IS DTE AND DCE

-------------------

DTE and DCE are the terminologies used in data communication. DTE stands

for Data Terminal Equipment and DCE stands for Data Communication

Equipment. Refer to Fig. 2.1, the computer or terminal is the DTE and the

modem is the DCE.

WHAT IS RS-232C OR EIA-232D

---------------------------

RS-232C is the Recommended Standard by Electrical Industry Association

(EIA) defining the serial communication interface between a DTE and a

DCE. The most commonly used revision is the "C" revision.

For "D" revision, it changes the prefix to EIA. Except for a few added

but not commonly used signals, there is no practical difference between

the "C" and "D" revisions. RS-232C is equivalent to CCITT V.24 and V.28.

WHAT IS SERIAL PORT

-------------------

Serial port is the serial data connector on the DTE or DCE with electrical

and mechanical characteristics according to RS-232C. Since some signals

are going from the DTE port to DCE port and some signals are going in the

other direction, the signal pin is a transmitter on one port and receiver

on the other. The DTE serial port is different from the DCE serial port in

terms of signals on the connector pins. There are also mechanical

differences in terms of male or female connector.

SERIAL RS-232C CABLE

--------------------

A serial RS-232C cable is used to connect a DTE port to a DCE port. A

normal RS-232C connector has 25 pins and a normal RS-232C cable has 25

wires. Many signals in the RS-232C are not used in common applications

and a 9-wire RS-232C cable is sufficient in most applications.

The PC-AT's serial port has only 9 connector pins eliminating the

unnecessary pins.

SYNCHRONOUS AND ASYNCHRONOUS COMMUNICATION                          (2 - 2)

------------------------------------------

There are two kinds of serial data communication. One is called

synchronous and the other is called asynchronous. In synchronous

communication, data are transmitted and received bit by bit and they are

timed by an accompanied clock signal. In asynchronous communication, data

are sent character by character (or octet by octet), the idle time between

characters is variable. No clock signal is sent, character timing is

recovered from data itself.

PC's COM1 and COM2 are asynchronous serial ports. Most of PC's and Unix

system's serial data communication is asynchronous. The serial data

communication on IBM mainframe or mini is synchronous.

WHAT IS UART

------------

UART (Universal Asynchronous Receiver Transmitter) is the device used in

DTE or DCE for asynchronous data receiving and transmitting. The normal

UART device used in PC is of the type NS16450. For high-speed serial data

receiving (38400 bps and up), the PC may not serve the UART fast enough

and data may get lost. In this case, a UART with data buffer is needed

such as of the type NS16550A.

MODEM STANDARDS AND SPEEDS

--------------------------

The CCITT (International Telegraph and Telephone Consultative Committee)

is the international standard making body for telecommunication. They make

"recommendations". The recommendations they made for modem application

have a "V" prefix and are called V-series recommendations.

The commonly used CCITT standards for 2-wire dial-up line are:

V.32bis       14400/12000/7200 bps

V.32                 9600/4800 bps

V.22bis                   2400 bps

V.22                      1200 bps

V.21                       300 bps

V.23                   1200/75 bps

Note: "bis" is the French word for second.

In USA, Bell System used to make the de facto standard such as Bell 212A

for 1200 bps modem and Bell 103 for 300 bps modem. Everybody is following

CCITT standard now for newer and higher speed modems.

ZyXEL U-1496 series modems support all the above mentioned modem standards

and are compatible with existing modems.

TYPE OF TELEPHONE LINE                                              (2 - 3)

----------------------

The commonly used phone service is a 2-wire dial-up line. There are only

two wires connecting modem to phone company's central office. The same two

wires are used for DC current feeding, ringing, dialing, on/off-hook

monitoring, and signal transmission for both direction. This is the type

of phone service most people get in their home or office. Since the same

two wires are used for both transmitting and receiving signals, echo of

the transmitting signal will also be received and it is the modem

receiver's task to remove the echo before demodulation.

A leased-line phone service is also available from phone company in

4-wire form. Here the transmitting and receiving signal each uses a

separate 2-wire pair and thus no echo problem will result. Normally no DC

current feeding, ringing, or dialing function is provided on leased line.

There is also the case of 2-wire leased line which is a permanent 2-wire

connection.

INTELLIGENT MODEM

-----------------

In old days, modem's function, settings, and operation are simple and

everything is controlled by manual switches, either internal or external,

or by wire-strapping settings. No computer control is provided.

Intelligent modem has everything of its function, including dialing and

answering, controllable by the computer or terminal it is connected to

through the same RS-232 serial interface used for data connection.

Intelligent modem operates in one of two states: the command state and

the data state.

In command state, the modem interprets data received from serial interface

as "commands" and sends back action result as response. In data state, the

modem modulates the data received from the serial interface and sends the

demodulated data to the serial interface as received data.

The user needs to know whether the modem is in the command state or data

state and how to switch it.

AT COMMAND SET

--------------

AT command set is the industry de facto standard used to control the

modem in command state. It got its name by that every command line has

a prefix "AT".

AT command was first used in Hayes Smartmodem and hence adopted by other

modem manufacturers and communication software writers. The original set

of AT commands such as dialing, answering, etc., is used by everyone else

and is standardized.

The AT command set is extended by each modem manufacturer for the control

of the ever-increasing modem functions and capabilities. This "extended AT

command set" is not standardized whatsoever.

AT command is an asynchronous command used only on asynchronous serial

data interface.

ZyXEL has defined a set of extended AT commands to support many of its

unique functions and features.

V.25bis COMMAND SET

-------------------

V.25bis is a set of auto-dialing commands defined by CCITT. V.25bis

supports both synchronous and asynchronous data interface and finds most

of its use in Europe and IBM mainframe or mini environment which uses

synchronous communication.

ERROR CORRECTION                                                    (2 - 4)

----------------

In synchronous data communication, data is checked and corrected in the

host by the so called "link layer protocol" to ensure data integrity.

Normally, no data checking is provided in the host for asynchronous data

communication.

Nowadays intelligent modem incorporates error correction protocol inside

the modem for asynchronous data communication and it is transparent to the

host. The modem packs the asynchronous data characters into blocks, the

data blocks are error checked at the remote end and the erroneous block is

retransmitted.

The commonly used error correction protocols are MNP and V.42 protocols.

DATA COMPRESSION

----------------

Intelligent modem uses redundancy removing methods to reduce the number of

data bits actually sent for asynchronous character transmission. The full

character data is recovered at the other end, thus it increases the total

throughput of the modem data transmission.

The commonly used data compression protocols by modem are MNP class 5 and

V.42bis.

Data compression works on redundancy removing and its efficiency is both

algorithm and data dependent.

MNP PROTOCOLS

-------------

MNP (Microcom Network Protocol) is a set of protocols first introduced by

Microcom, Inc., and later used by many modem manufacturers. It consists of

many classes, class 1 to 4 are for error correction and class 5 is for

data compression. MNP class 1 and 2 are obsolete, normally class 4 is used

for error correction and it uses class 3 internally.

MNP class 5 is a data compression protocol with a maximum efficiency of 2

to 1. MNP-5 is used with MNP-4 for error correction.

V.42bis AND V.42

----------------

V.42bis and V.42 are data compression and error correction standards set

by CCITT. V.42bis has a better data compression efficiency than MNP-5 and

is a more advanced data compression scheme.

V.42bis is used with V.42 for error correction.

XMODEM, YMODEM, AND ZMODEM

--------------------------

These are file transfer protocols used by the host (e.g., communication

program in the PC). It does error checking and ensures data integrity of

the file transferred.

Some variations of these protocols also appear, like Xmodem-1k, Ymodem-G,

etc. Whenever possible we recommend the use of Zmodem.

FAX AND FACSIMILE

-----------------

Fax is the abbreviation for facsimile. There are four major parts in a

normal facsimile machine, namely scanner, encoding and decoding device,

modem, and printer. A sent page is first scanned, the bit-mapped data are

encoded with data compression and then transmitted across the phone line

by an internal modem module. The remote facsimile receives the data with

its internal modem, the data are decoded to bit-mapped image data and

then printed on paper, normally with a thermal printer on thermal

sensitive paper.

FAX CARD                                                            (2 - 5)

--------

A fax card is a PC internal plug-in card. It has the modem function of a

facsimile machine. A PC loaded with fax software can send or receive fax

through a fax card. The PC is used as the input and output parts of a

facsimile machine and the fax card is used as the transmit/receive part

and may also do the encoding/decoding function depending on the design.

The PC controls and communicates with the fax card through a set of

proprietary hardware and software interfaces which are different for each

fax card manufacturer.

MODEM AS A FAX

--------------

Normal modem can also be designed to have the fax transmitting and

receiving function as does the fax card. Since the modem's interface with

the computer is the standard serial RS-232 interface. This interface

shall be used for both modem and fax operations. Since the data throughput

of a serial RS-232 interface is limited, only compressed fax image data

can be carried through this serial interface, fax image coding and

decoding must be done in the computer.

Modem/Fax is also called faxmodem, it can be either an external stand

alone unit or an internal PC plug-in card. External stand alone unit can

be connected to any computer with a standard RS-232 serial port. This

computer needs not to be a PC.

EIA CLASS 2 FAX COMMAND

-----------------------

EIA class 2 fax command is a set of AT fax commands defined by EIA/TIA

(Telecommunication Industry Association) for controlling faxmodem from

computer through serial RS-232 interface. Faxmodem and fax software

supporting this standard will work together. The problem is that there

are several revisions of this standard. Implementations conforming to

different revisions may not work together.

AUTODETECTION OF FAX OR DATA CALL

---------------------------------

Fax and data modem have different handshaking signal sequences specified

by the CCITT at the beginning of a call. It is possible to automatically

detect whether an incoming call is from a modem or a fax device by testing

and recognizing its initial handshaking sequence. A modem with this

capability can make a computer process both data and fax call on the same

phone line. An example is that a BBS (Bulletin Board System) can also

receive fax.

ZyXEL modem can automatically detect data and fax call. ZyXEL also has

fax receiving and printing program to make it possible a BBS also receives

fax and prints it out in background.

CALLER ID

---------

Normally called Caller Number Delivery (CND), this is a service feature

available from phone companies in some parts of USA and in some other

countries. With this service, the phone company sends calling party

information along with ring signal to the called party. The major part

of this information is the telephone number of the calling party or

caller ID.

ZyXEL modems can detect and report caller ID.

DISTINCTIVE RING                                                    (2 - 6)

----------------

This is a service feature available from some phone companies in some

area of US and other countries. You can have several phone numbers on the

same subscriber phone line. The phone company sends a different ring

signal for each phone number assigned to the same line. You can have your

phone equipments answer only to a certain type of ring corresponding to a

certain phone number.

ZyXEL modems can be set to answer on one of or a combination of four

types of ring signal. There is a problem that each country may have a

different specification for different ring types.

DIGITIZED VOICE

---------------

Modem with digitized voice capability can digitize the incoming voice

message and the computer can store it as a file. The modem can also

playback a recorded digital voice message either locally or to the line

as an announcement.

Digital voice message needs speech compression to reduce the storage

requirement. This speech compression is done in real time by the modem

and it does the decompression in playback. A simple voice capability

using ADPCM algorithm can reduce speech data rate to 1/2 or 1/3. We call

it advanced voice capability when it can reduce speech data rate even

further with davanced algorithm like CELP (Code Excited Linear

Prediction).

CELLULAR MODEM

--------------

Cellular modem is a data modem that can be used on the analog cellular

mobile phone system. It has an increasing importance for people on the

move with the popularity of notebook computer.

There are two kinds of phenomenon in cellular mobile phone system that are

particularly bad for modem data communication. One is called cell hand-off

that a cellular phone is instructed to change radio link from one cell

site to the other when travelling. The radio link will be temporarily

interrupted and most modems will retrain or even hang up. When a cellular

phone is instructed to change transmitting power it will cause similar

problem. The other particularly annoying phenomenon is multipath fading

that a moving cellular phone will experience periodical drop-out of radio

signal. This causes data error rate high and throughput low.

Communication protocol like MNP10 does not do anything on improving

physical layer data communication robustness and can not really help in

cellular data communication.

ZyXEL developed a cellular modem using proprietary data pump that conquers

the cell hand-off and multipath problems. Link layer protocol with

selective reject greatly improves data file transfer throughput.

CHAPTER 3                                                           (3 - 1)

MODEM INSTALLATION

==================

This chapter describes the panel function and installation guide for model

U-1496 and U-1496E. The U-1496R comes with it's own installation guide.

Installation guide for U-1496B is in Chapter 16.

The shielded RS-232 cable is required to be used to ensure compliance

with FCC Part 15, and it is the responsibility of the user to provide and

use shielded RS-232 cable.

Make sure that your installation site is clean and well ventilated. The

ventilation slot of your ZyXEL modem located on the sides and bottom

should not be covered and should allow free movement of air.

U-1496 FRONT PANEL

------------------

Fig. 3-1 shows the front panel of the U-1496. There are 6 LED indicators,

a 20 x 2 LCD display, and four key switches.

U-1496 FRONT PANEL INDICATORS

-----------------------------

LED Indicators

PWR     Power ON indicator, lights up when U-1496 is turned ON.

OH      Off-Hook indicator, lights up when U-1496 is in data mode

or off-hook, and goes out when U-1496 is in talk mode or

on-hook.

AA      Auto-Answer indicator, lights up when U-1496 is in the Auto

Answer Mode; flashes when modem rings. In error control

mode, it flashes when retransmitting.

DTR     Data Terminal Ready indicator, lights up when your DTE or

computer indicates that it is ready to begin communication.

TXD     Transmit indicator, ON when your DTE/computer transmits data

on the serial port.

RXD     Receive indicator, ON when your DTE/computer receives data

on the serial port.

U-1496 FRONT PANEL SWITCHES

---------------------------

The four arrow keys are for the panel operation manipulation. They are

for intuitive menu tree operation and status display. Refer to Chapter 4

for the detail of menu key operations.

U-1496 REAR PANEL MARKINGS                                          (3 - 2)

--------------------------

Fig. 3-2 shows the U-1496 rear panel. The following explains the

connectors and switch on the rear panel.

POWER         Power switch, turns the modem ON or OFF.

POWER JACK    Input terminal for power adapter.

LEASED LINE   Leased-line JM8 terminal jack, for connection to a

2/4-wire leased line.

DIAL-UP LINE  Dial-up line RJ11 terminal jack, for connection to a

2-wire dial-up line.

PHONE         Telephone line RJ11 terminal jack, for connection to

a telephone set.

EIA-232D      Serial port DB25S connector, for connection to the

serial port of a DTE (computer/terminal). Pin

assignments are listed in Appendix A for your

reference.

The signal-pin assignments on RJ11 and JM8 phone jacks

are listed in Appendix B.

MODEM CONNECTION

----------------

When you connect your U-1496 to the power line, make sure you only use

the ZyXEL power adapter that is supplied when you purchase this unit. Use

of other adapter may not allow your modem to operate and could result in

serious damage to the unit. This adapter is rated for direct connection

to an AC power outlet and has a 2-meter long cord. Connect the U-1496 as

shown in Fig. 3-3. Connect U-1496E in the same way except that there is

no Leased Line jack. 2-wire leased line can be connected to the DIAL-UP

LINE jack.

If leased line is not used, simply leave the connection out. There are no

DIP switches or configuration settings that you need to worry about. The

U-1496 comes factory pre-set and allows you to fully configure the modem

from the LCD display after you turn the power on. User configurations are

also conveniently stored in user selectable non-volatile memories and can

be recalled as often as needed.

POWERING UP

-----------

Once the U-1496's power switch is turned ON, the U-1496 will perform a

series of diagnostic tests that will last for a while with message shown

on the LCD panel. For a more detailed description of these diagnostic

tests, please refer to Chapter 15 Diagnostics.

After performing the diagnostic tests, your U-1496 will display the

current modem settings on the front LCD panel as shown in Fig. 3-4.

Your U-1496 modem is now ready for use.

U-1496E FRONT PANEL

-------------------

Fig. 3-5 shows the front panel of the U-1496E. There are 12 LED indicators

and two switches.

U-1496E FRONT PANEL INDICATORS                                      (3 - 3)

------------------------------

LED Indicators

HS     High Speed indicator, lights up when modem is in V.32 or

V.32bis mode. Dialing indicator, flashes one second on and

one second off when dialing. Retrain indicator, flashes half

second on and half second off when modem is in retrain.

AA     Auto-Answer indicator, lights up when modem is in the Auto

Answer mode; flashes when modem rings.

CD     Carrier Detect indicator, lights up when a valid carrier is

present on the line.

OH     Off-Hook indicator, lights up when modem is in data mode or

off-hook, and goes out when modem is in voice mode or on-hook.

DTR    Data Terminal Ready indicator, lights up when the DTE or

computer indicates that it is ready for communication.

DSR    Data Set Ready indicator, lights up when modem is ready for

communication.

CTS    Clear To Send indicator, lights up when modem can accept data

for transmission.

TXD    Transmit Data indicator, lights up when the DTE/ computer is

transmitting data on the serial port.

RXD    Receive Data indicator, lights up when the DTE/computer is

receiving data on the serial port.

EC     Error Control indicator, lights up when modem is operated in

an error control mode like MNP-4 or V.42. Data compression

mode (V.42bis or MNP 5) implies error control mode.

Retransmission indicator, lights up when retransmitting.

SQ     Signal Quality indicator, lights up when signal quality is

good; flashes when it is on margin.

TST    Test indicator, lights up when modem is in a test mode.

U-1496E FRONT PANEL SWITCHES

----------------------------

VOICE/DATA   A toggle switch that toggles the modem on-line

(off-hook, DATA mode) or off-line (on-hook, VOICE mode,

the telephone set is connected to the line).

If S35 bit 4 is set, pressing this switch will make

modem go off-hook and dial the default number.

ANS/ORG      A locked switch, it determines the modem is in

originate mode or answer mode when the modem goes

off-hook by toggling the VOICE/DATA switch.

Locked up position means answer mode and locked down

position means originate mode.

Note:

This switch determines on-line mode only when manual off-hook. ATD or

ATA command will automatically determine the mode.

U-1496E REAR PANEL MARKINGS

---------------------------

Fig. 3-6 shows the rear panel of U-1496E. The rear panel markings are the

same as those of U-1496 in Fig. 3-2 except that U-1496E does not have the

leased line jack.

Refer to U-1496 rear panel description and modem connection for U-1496E's

rear panel explanation.

CHAPTER 4                                                           (4 - 1)

QUICK START

===========

This chapter tells you how to dial and answer on a modem. If you use this

modem with a PC, most probably you don't need to worry about modem

settings, the factory default maybe just right for you. Connect the modem

according to Chapter 3, install the communication software on the PC, make

the modem dial as instructed in this chapter, and you are all set.

This chapter also familiarizes you with the panel operation of U-1496. The

U-1496 modem comes with a menu driven 20 x 2 LCD display. All functions of

the modem are displayed and selectable from this menu.

This chapter will also familiarize the user with the menu tree structure.

U-1496E and U-1496B modem users also can consult the U-1496's panel menu

operations, but use the equivalent AT command listed in this chapter

instead.

LCD PANEL

---------

The U-1496's commands, options, and parameters can be set, changed, and

viewed from the front panel or viewed and changed through AT commands by

the computer or terminal connected to the modem. We will discuss panel

operations in this chapter.

Once powered up, the U-1496 goes into (1) Idle Screen (if not leased line

selected), it then waits for commands from a PC/terminal or any key pad

operation from the front panel.

Refer to Fig. 3-4, there are four ON SCREEN BUTTONS:

D    Dialing directory.

R    Redial last number.

O    Originate mode on-line.

A    Answer mode on-line.

Also, the status and current settings are displayed:

Link Option        V.32bis

Link Speed         14400 bps

Error Control      V.42 (implied by V.42bis)

Data Compression   V.42bis

Line Type          Dial-up line

Data Format        Asynchronous

DTE Speed          57600 bps

PANEL OPERATION

---------------

Four arrow keys manipulate the menu tree:

Left/Right #arrow keys

used for horizontal item selection and options selection.

Up arrow key

also called the MENU key, is used for vertical menu selection

and is used to access the previous menu. The up arrow key also

serves as a quit command when you want to stop the current

action.

Down arrow key

also called the ENTER key, is solely used to register the chosen

selection and is very similar to the CARRIAGE RETURN key or

ENTER key on a standard keyboard.

MENU TREE                                                           (4 - 2)

---------

Fig. 4-1 shows the menu tree structure. there are majorly two menu trees,

one is CONFIGURATION menu, the other is DIAGNOSTIC menu. Fig. 4-2 shows

the submenus under CONFIGURATION menu. In Fig. 4-1, screen (1) is the idle

screen and screen (1)' is the data state screen.

In data state, screen (1)'' and (1)''' can be accessed by pressing left or

right arrow keys. A dialing menu can be accessed from idle screen. Flip

between idle screen and menu screen is by pressing the MENU or up arrow

key.

ARROW ON THE SCREEN

-------------------

In some menu screens, you will see double left arrow (<<) or double

right arrow (>>). These mean there are more items on the left or right.

Use the left or right arrow key to select them. If you can only see

a double right arrow, this indicates that the current item is the first

item of the menu. By pressing the left arrow key, the screen will wrap

over to the last item. On the other hand, if you only find a double left

arrow displayed, this indicates that your current item is the last item

in that menu. Similarly, pressing right arrow key at this point will cause

the screen to wrap around to the first item.

DIALING

-------

From IDLE SCREEN (1) of Fig. 4-1 you can use the right or left arrow key

to scroll through D, R, O, and A. Move the blinking cursor to "D", press

ENTER, and you will go into Dialing Directory.

Valid digits/characters for dialing are:

0 1 2 3 4 5 6 7 8 9 * #

,                        Pause

;                        Return to COMMAND STATE

@                        5 second silent wait

!                        Hook flash

P                        Pulse dialing

R                        Reverse dialing

T                        Tone dialing

W                        Wait for dial tone

-                        Digit separator

<bs>                     backspace

DIAL MEMORY

-----------

The first menu of the Dialing Directory is DIAL MEMORY. Pressing the ENTER

key will show the following screen:

Equivalent AT commands:

AT&Z?     View all stored phone numbers.

ATDSn     n=0-9, dial stored number.

The first stored number shown on the menu is the default dial number and

can be selected from DEFAULT DIAL menu. By pressing the left or right

arrow key, you can view up to ten different numbers stored in memory.

Pressing the ENTER key will dial the selected number. The first line of

the LCD can display up to 16 characters, however up to 25 characters can

be entered. Character 17 through 25 are displayed on the second line of

the LCD display.

STORE NUMBER                                                        (4 - 3)

------------

Again, by pressing the right arrow key from DIAL MEMORY will move the menu

to STORE NUMBER. Pressing the ENTER key at this point will produce the

following screen display:

Equivalent AT commands:

AT&Z?           View all stored phone numbers.

AT&Zn=1234567   Store phone number to memory location 0-9.

Just like DIAL MEMORY, ten different numbers can be viewed and selected.

If you want to add a number to memory, select an unused memory location

and press ENTER. The blinking cursor will change into an underscore and

will move to the next right position of the colon(:). SELECT MEMORY will

change to SELECT DIGIT. Pressing the left and right arrow keys to scroll

through the valid digits and characters for dialing. Pressing ENTER will

confirm the selected digit and cause the cursor to move to the next

position.

Repeat the above procedure to finish entering the number, then press ENTER

again to save the entry in the EEPROM. The stored numbers will remain in

memory until you revise or delete it. Pressing the up arrow key in the

process  will abort and exit the current number entry and nothing will be

changed or saved.

DIAL NUMBER

-----------

The same procedure is used in entering number as in STORE NUMBER

operation. After the number entry is completed, press the ENTER key to

dial the number.

Equivalent AT command:

ATD1234567     Dial phone number.

MANUAL DIAL AND DATA/VOICE SWITCHING

------------------------------------

Use of a telephone set to make a call, in which an answer tone comes back,

is normally called a Manual Dial. Once the other side answers, access the

`O' (Originate) by using the left or right arrow key so that the blinking

cursor settles on the `O'. Then press the ENTER key to start the modem

handshaking sequence.

Equivalent AT command:

ATD      Originate on-line.

When dialing to a person, use the telephone to make the call. When someone

on the other side answers, decide who will be the Originate modem and who

will be the Answer modem. Once an agreement is reached, use the left or

right arrow key to select the `O' or `A' for Originate and Answer. Then

press the ENTER key to start the modem handshaking sequence.

Equivalent AT commands:

ATD       Originate on-line.

ATA       Answer on-line.

(4 - 4)

When you want to switch form modem to voice, simply lift up the handsets

of both sides and press the ENTER key while the blinking cursor is on the

`H' or Hang Up. The LCD display will show "DISCONNECTING ?" to ask for a

confirmation. Press ENTER again and the modem will change from data to

voice communication. When both sides wish to return to data communication,

one side should press `O' while the other side press `A' (as explained

previously), this operation will reinitiate the modem handshaking and

enable data communication.

For U-1496E, use the VOICE/DATA toggle switch for data/voice switching and

use the ANS/ORG switch to select originate mode or answer mode on-line.

Equivalent AT commands:

+++      Escape sequence code; causes a modem to return to command

mode. Enter escape sequence in data state and wait for the

modem to respond.

ATH      On-hook; disconnect the modem.

ATD      Originate on-line.

ATA      Answer on-line.

REPEAT LAST DIAL

----------------

Accessing `R' (Repeat last dial) by pressing the left or right arrow key

and the ENTER key. Before a connection is made, any key pressed from the

front panel or terminal keyboard will abort the redial operation.

Equivalent AT commands:

ATDL      Repeat last dial.

A/        Repeat last command once.

A>        Repeat last command once, retry the last call up to 9

(depending on the country) times.

AUTO ANSWER

-----------

Move to the STATUS REGISTER menu, and change S0 value to the number of

rings on which the modem will automatically answer. A value of zero

disables auto answer.

Equivalent AT commands:

ATS0=0    Disable auto-answer.

ATS0=n    Auto answer on n rings.

DIALING MESSAGE

---------------

Dialing from the terminal or front panel will cause the following screen

to be displayed on the LCD.

Dialed phone number will be displayed on the second line of the 20 X 2 LCD

screen. If the number exceeds 20 digits, only the first 20 numbers/

characters will be displayed. If the phone jack is not plugged in or not

attached, the LCD screen will display the following:

then the modem will go back to the IDLE STATUS screen. If the line is

busy, after 3-5 seconds, the LCD will display:

(4 - 5)

then goes back to IDLE STATUS screen. If any key interrupt occurs (from

the terminal or panel) and breaks in before connection is made, the

screen will display:

then goes back to IDLE STATUS screen. If the time interval depicted in S7

register expires, and the line is still not connected, the LCD screen will

show:

then goes back to IDLE STATUS screen. If the first handshaking signal has

been detected, the LCD screen will display:

If a connection has been made, the LCD will display the actual status

connected. These are data state screens.

The ON-LINE STATUS screen 1 shown above means:

Link Option            V.32bis

Line Speed             14400 bps

Error Control          V.42 (implied by V.42bis)

Data Compression       V.42bis

Line Type              Dial-up

Data Format            Asynchronous

Instant Transmission   25820 bps (BLANK if synchronous mode was

Throughput             connected)

Instant Receiving      21324 bps (Link speed displayed if

Throughput             synchronous mode was connected)

The ON-LINE STATUS screen 2 shows:

Signal to Noise ratio                  26.7 dB

Receiving Signal level                 -25.1 dBm

Phase Jitter in #received signal       0.4 degree

Frequency Offset in received signal    - 0.6 Hz

The ON-LINE STATUS screen 3 shows:

Retrain Granted                        0

ReTrain requested                      0

Round trip Echo Delay                  128 T (1/2400 sec)

Carrier Loss counts                    0

By pressing the left or right arrow key in any one of the ON-LINE STATUS

screens will cause the LCD to scroll through the three screens. We will

discuss the ON-LINE STATUS screen 2 and screen 3 in more detail in Chapter

15 Diagnostics.

Pressing ENTER key in any ON-LINE STATUS screen will cause the LCD to

display the following:

Equivalent AT commands:

+++     Escape sequence code, cause the modem to return to command

mode. Enter escape sequence and wait for the modem to

respond.

ATH     Go on-hook; disconnect the modem.

(4 - 6)

Pressing up or selecting `NO' will switch the screen back to the ON-LINE

STATUS screen 1 and modem remains on-line. Pressing ENTER will hang up the

line and cause the screen to revert to the IDLE STATUS screen, awaiting

for your next command.

Pressing up in either screen will change your current environment to

CONFIGURATION menu. This will be discussed in the succeeding chapter.

PANEL LOCK

----------

The panel keyboard can be locked to prevent inadvertent change of modem

settings. This is done by setting the S-Register S28 bit 1 value to 1

from the panel, or by issuing AT*L1 command from the terminal. Issuing the

AT*L0 command will return the key operation to normal. Alternatively,

pressing the left and right arrow keys simultaneously will toggle the panel

between locked and unlocked state. In locked panel, the ENTER key will lose

effect in changing or saving settings; panel command execution is also

disabled. However, viewing in panel is not affected.

CHAPTER 5                                                           (5 - 1)

MODEM PARAMETER SETTINGS

========================

This chapter explains how to view and set various major modem parameters

by means of U-1496 CONFIGURATION  menu tree. The U-1496E and U-1496B

users can also consult this menu tree for available parameter options, but

use the equivalent AT command to set the parameter.

Here we define the various levels of the CONFIGURATION menu tree as:

CONFIGURATION menu

submenus

parameter menus

parameter selection

where CONFIGURATION menu is the root of this menu tree. Some submenus,

like STATUS REGISTER, SAVE TO, and RESET, have a direct parameter

selection under them and do not have the parameter menu level.

Under the CONFIGURATION menu, there are seven submenus: TERMINAL OPTIONS,

MODEM OPTIONS, ERROR CONTROL, AUDIO OPTIONS, STATUS REGISTER, SAVE TO,

and RESET (Fig.4-1).

Every submenu screen defines certain parameters of the U-1496 operation.

Submenu selections are made by using the left and right arrow panel keys.

Pressing the ENTER key will access the parameter menu and pressing the up

arrow key will return you to the CONFIGURATION menu.

PARAMETER MENU

--------------

The parameter menu selections are made by using left and right arrow key

to move from one to the other. Pressing up arrow key will return you to

the submenu screen. Press ENTER to go into Parameter Selection. You will

notice that a blinking cursor will appear at the left side of the

"=" sign.

PARAMETER SELECTION

-------------------

Press left or right arrow key to view and scroll the parameters available.

Only the parameter in current effect will have the "=" sign appearing

before their respective selections, otherwise a ":" sign will appear,

meaning that this current parameter is a selectable option. By pressing up

to quit from the viewing, the screen will return to parameter menu and the

parameter will remain unchanged. Press ENTER to cause the selected

parameter to become active.

To save the current parameter settings to non-volatile EEPROM, access

SAVE TO submenu in the CONFIGURATION menu and select from the various

profiles you wish to save the changes to, and the parameter settings will

be permanent in that profile. You can always call in these settings by

resetting from that profile. Access RESET and choose "=FACTORY DEFAULT"

and the active parameter settings will revert to original factory

defaults.

Also, S-Register's content can be individually modified in STATUS REGISTER

submenu.

REGISTER'S CONTENT MODIFICATION (STATUS REGISTER)                   (5 - 2)

-------------------------------------------------

When you access the STATUS REGISTER submenu in the CONFIGURATION menu, the

blinking cursor will be changed into an underscore `_' and the content

will be blanked out. Press the left or right arrow key to scroll through

a series of hex digits, press ENTER when the desired digit appears.

Remember that both the two digits must be entered. Press up will quit from

your entry procedure, the blinking cursor will reappear and the content

will remain unchanged.

For the rest of this chapter, we will explain the parameter selections of

each parameter menu and of each configuration submenu. The large boldface

title of each section is the name of each configuration submenu under

which each boldface subtitle is the name of each parameter menu. Under

each parameter menu, there are a list of parameter items. These are the

selectable parameter items in this parameter menu. On the right of each

parameter item is the equivalent AT command to select or set this

parameter. After that is a brief explanation of this  parameter.

A "+" sign in front of a parameter item means that it is the default

parameter setting.

TERMINAL OPTIONS

----------------

DATA FORMAT

+ ASYNC     AT&M0      Operate as an asynchronous modem, with data

buffering, may work with data compression

and/or error control.

SYNC DATA   AT&M1      Modem accepts asynchronous command, but

changes to synchronous data mode and

exchanges data synchronously with remote

modem.

DIRECT      AT&M2      Direct asynchronous mode without data

buffering.

SYNC        AT&M3      Modem accepts synchronous command (V.25bis)

and exchanges data synchronously with a

remote modem. You can also dial from the

front panel or use a telephone set to make a

connection.

CHARACTER LENGTH

+ 10        AT*C0      10-bit characters.

11        AT*C1      11-bit characters.

9        AT*C2       9-bit characters.

8        AT*C3       8-bit characters.

The character length includes start bit, data

bits, parity, and stop bit(s).

COMMAND SET

+ AT        AT*I0      Modem accepts asynchronous AT commands.

V.25bis     AT*I1      Modem accepts asynchronous or bit-oriented

synchronous V.25bis commands.

Note:

Only asynchronous V.25bis available on U-1496B.

DTE RATE                                                            (5 - 3)

+ FIXED     AT&B1      Modem serial port speed is always fixed no

matter what link rate (modem speed) is

connected.

FOLLOWS     AT&B0      Modem serial port speed is set in accordance

LINK RATE             with the link rate.

DTR OPTIONS

IGNORED     AT&D0      Modem assumes DTR always ON, ignores the

changes.

108.1       AT&D1      An OFF-ON DTR transition causes the modem to

dial the number specified under the DEFAULT

DIAL of the MODEM OPTIONS submenu. An ON-OFF

transition disconnects the modem from the

line.

+ DTR/108.2 AT&D2      Data Terminal Ready, an ON-OFF transition of

DTR will disconnect modem from line.

108.2 + RST AT&D3      Same as &D2 but DTR OFF also causes reset

from profile 0.

DCD OPTIONS

+ TRACKS     AT&C1     DCD ON when carrier is detected.

CARRIER

ALWAYS ON    AT&C0     Modem forces DCD always ON.

RTS OPTIONS

+ IGNORED    AT&R1     Modem assumes RTS always ON, ignores the

changes.

CTS TRACKS   AT&R0     Delay before CTS responds to RTS's change.

RTS                 The delay is set in S-Register 26.

Note:

This setting is ignored in asynchronous mode. RTS is always used as

hardware flow control in this case.

DSR OPTIONS

+ ALWAYS ON   AT&S0      Modem forces DSR always ON.

DATA SET      AT&S1      According to CCITT. DSR ON during handshake.

READY

MODEM OPTIONS                                                       (5 - 4)

-------------

LINK OPTIONS

+ MULTIAUTO  AT&N0      Autohandshaking with

V.32bis     14400/12000/7200

V.32         9600T/9600/4800

V.22bis            2400/1200

V.22                    1200

BELL 212A               1200

G3 Fax     14400/12000/9600/

7200/4800/2400

(V.17, V.29, V.27ter)

V.33   14400  AT&N1     V.33       14400/12000

V.33   12000  AT&N2     V.33       12000

V.32    9600T AT&N3     V.32       9600T/9600/7200T/4800

V.32    9600  AT&N4     V.32       9600(non-trellis coded)

/7200T/4800

V.32    4800  AT&N5     V.32       4800

V.29    9600  AT&N6     V.29       9600

V.29    7200  AT&N7     V.29       7200

V.29    4800  AT&N8     V.29       4800

V.27t   4800  AT&N9     V.27ter    4800

V.27t   2400  AT&N10    V.27ter    2400

V.26b   2400* AT&N11    V.26bis    2400

V.23    1200  AT&N12    V.23       1200/75

V.23     600  AT&N13    V.23       600/75

V.22b   2400  AT&N14    V.22bis    2400

V.22    1200  AT&N15    V.22       1200

V.21     300  AT&N16    V.21       300

V.32b  14400  AT&N17    V.32bis    14400/12000/9600/7200/4800

V.32b  12000  AT&N18    V.32bis    12000/9600/7200/4800

V.32b   7200  AT&N19    V.32bis    7200/4800

B212A   1200  AT&N24    BELL 212A  1200

B103     300  AT&N25    BELL 103   300

V17G3 FAX     AT&N32    ZyXEL Fax  14400/12000/9600/#7200/4800

ZyX 14400 @   AT&N34    ZyXEL      14400

ZyX 16800 @   AT&N35    ZyXEL      16800

ZyX 19200 @   AT&N36    ZyXEL      19200

Notes:

* are not available at this moment.

@ are not available in this model.

QUALITY ACTION

--------------

AUTO RETRAIN    AT*Q1    When poor signal quality is detected,

automatically retrain.

+ ADAPTIVE RATE   AT*Q2    When poor signal quality is detected,

automatically fall-back to next lower

speed. When signal quality is improved,

automatically fall-forward to next higher

speed.

DISCONNECT      AT*Q3    When poor signal quality is detected, modem

disconnects itself.

NO RESPONSE     AT*Q0    No response to poor signal quality.

DEFAULT DIAL                                                        (5 - 5)

PH0-PH9         AT*D0-9  The first phone number displayed in  DIAL

MEMORY menu (default is PH0). If DTR option

108.1 is active, DTR OFF-ON transition will

cause modem to dial the number stored in

the memory address selected (memory address

0 to 9).

DIAL BACKUP

+ DISABLED         AT*B0   Leased line dial backup disabled.

PH0-PH8          AT*B1-9 When Leased line is impossible to continue

operation, the originate mode modem will

automatically dial the number stored in the

memory address selected. The answer mode

modem also needs to select a non-zero

memory address to enable answering at

dial-up line during dial backup.

GUARD TONE

(for V.22 and V.22bis answer mode operation)

+ NONE              AT&G0  No guard tone provided.

RESERVED          AT&G1  Reserved.

1800 HZ           AT&G2  1800 Hz guard tone provided.

RDL REQUEST

GRANT             AT&T4  Accepts remote digital loopback test

request from remote modem. Must be set

when remote configuration is needed.

+ DENY              AT&T5  Ignores remote digital loopback test

request from remote modem.

LLINE TX POWER

0 to -15 dBm      AT*P0-15  Select transmit power from 0 dBm to

-15 dBm. Default is -9 dBm. Effective

in leased line operation only. Select

in the range of -12 to -27 dBm when S35

bit 3 is set.

PHONE JACK

+ SINGLE RJ11       AT&J0     RJ11 permissive telephone jack for

dial-up line.

MULTIPLE RJ12/13  AT&J1     Modem supports A/A1 multi-extension

telephone.

MAKE/BREAK RATIO

+ 39% / 61%         AT&P0     For use in US/Canada and some other

countries.

33% / 67%         AT&P1     For use in UK and some other countries.

Controls the ratio of Make (Off-hook) and Break

(On-hook) interval for pulse dialing.

SECONDARY CHANNEL (Optional feature)                                (5 - 6)

+ DISABLED          AT*S0     Secondary channel disabled.

ENABLED           AT*S1     Secondary channel enabled. 2400 bps or

75-300 bps.

SYNC CLOCK

+ INTERNAL          AT&X0     Modem provides the transmit clock.

EXTERNAL          AT&X1     DTE provides the transmit clock.

SLAVE             AT&X2     Modem's receiver clock is used as the

transmit clock.

AUTO HANDSHAKE

+ ORIGINATE          AT*M0    When operating over a leased line, modem

will handshake in originate mode.

ANSWER             AT*M1    When operating over a leased line, modem

will handshake in answer mode.

Note:

If you need two modems to automatically connect on leased line when

poweris turned ON, one modem must be setto originate mode, and the

other set to answer mode. And these settings should be saved in

power-on default profile.

LINE TYPE

+ DIAL-UP            AT&L0    Modem is connected to dial-up telephone

line.

2W LEASED          AT&L1    Modem is connected to 2-wire leased

line, phone jack should be plugged into

the assigned `LEASED LINE' receptacle

on the rear of the modem.

4W LEASED          AT&L2    Modem is connected to 4-wire leased

line. Plug the line in the same location

as that for the 2W LEASED line.

Note:

For wiring information on the leased line, please refer to

Appendix B. U-1496E and U-1496B can only connect 2-wire leased line

to their dial-up jack.

ERROR CONTROL

Control Level           Error Control           Data Compression

NONE         AT&K0     Disabled                  Disabled

MNP4(MNP3)   AT&K1     MNP4                      Disabled

MNP4+MNP5    AT&K2     MNP4                      MNP5

V.42(MNP4)   AT&K3     V.42 or MNP4              Disabled

+ V.42(b)+     AT&K4     V.42 or MNP4              V.42bis based

MNP4(5)                                          on V.42 or MNP5

based on MNP4

FLOW CONTROL

DISABLED           AT&H0     DTE flow control disabled.

+ CTS/RTS            AT&H3     Hardware flow control CTS/RTS enabled.

LOCAL XON/XOFF     AT&H4     Local software flow control XON/XOFF

enabled.

PASS XON/XOFF      AT&H5     Reserved.

NEGOTIA(TION) FALLBACK                                              (5 - 7)

+ STAYS ON-LINE      AT*E0     If error control negotiation fails,

keep the no error control connection.

DISCONNECT         AT*E1     If not error control linked, disconnect

the line.

BREAK HANDLING

DESTRUCTIVE        AT&Y0     Destructive, expedited. Send break,

throw away data in buffer.

+ EXPEDITED          AT&Y1     Non-destructive, expedited. Send break

first, ahead of data in buffer.

IN SEQUENCE        AT&Y2     Non-destructive, non-expedited. Send

break in sequence with data.

Note:

Only expedited break is implemented in transmission but all types of

break are accepted in receiving.

AUDIO OPTIONS

-------------

SPEAKER CONTROL

ALWAYS OFF         ATM0       Speaker always OFF. Ring is

independently controlled by RING

VOLUME.

+ ON UNTIL CONNECT   ATM1       Speaker ON until carrier detected.

ALWAYS ON          ATM2       Speaker always ON.

DIALED TO CONNECT  ATM3       Speaker ON after final digit dialed

out and speaker OFF when carrier

detected. Tone dialing will not be

heard.

SPEAKER VOLUME

0-7                 ATL0-7     Default is 4. The higher the value,

the higher the volume. Speaker

volume is set to maximum when in

call progress monitoring if not

ATM0.

RING VOLUME

0-7                ATN0-7     Default is 5. The higher the value,

the higher the volume.

CHAPTER 6                                                      (6 - 1)

PROFILES

========

You can save the entire modem configuration in the non-volatile memory

EEPROM, that you can later call it up as the default settings, or you can

use it as the power-on default.

The U-1496 series modems let you save up to 4 sets of configurations, which

are called Profiles.

Use the "SAVE TO" configuration submenu to save the current configuration

to a profile or use the &Wn command to save to profile n.

AT&Wn

There are 5 sets of pre-programmed profiles that come with each U-1496

series modem.

One is the factory default that is stored in the ROM and is not user

alterable.

The other four are stored in the non-volatile memory that are pre-programmed to suit the most common applications. You can modify these profiles and save them for your own particular applications.

You can view each set of profile settings by typing AT&Vn (n=0-5) from the

terminal. Following are the commands and their responses. The value shown

for each register is a decimal number.

AT&V0

Current Settings.................

B0   E1   L4   M1   N5   Q0   V1   X5

&B1  &C1  &D2  &G0  &H3  &J0  &K4  &L0  &M0  &N0  &P0  &R1

&S0  &X0  &Y1

*B0  *C0  *D0  *E0  *F0  *G0  *I0  *L0  *M0  *P9  *Q2  *S0

S00=000  S01=000  S02=043  S03=013  S04=010

S05=008  S06=003  S07=060  S08=002  S09=006

S10=007  S11=070  S12=000  S13=000  S14=002

S15=130  S16=000  S17=018  S18=000  S19=000

S20=002  S21=178  S22=000  S23=105  S24=138

S25=000  S26=000  S27=156  S28=068  S29=000

S30=000  S31=017  S32=019  S33=000  S34=030

S35=000  S36=000  S37=000  S38=000  S39=000

S40=000  S41=000  S42=000  S43=000  S44=000

S45=000  S46=000  S47=000  S48=000  S49=000

OK

Current settings are the active settings which will take effect on the

modem's functions until power is OFF.

You can temporarily change the setting and use it but without saving it.

AT&V1                                                               (6 - 2)

Profile 0 Settings.................

B0   E1   L4   M1   N5   Q0   V1   X5

&B1  &C1  &D2  &G0  &H3  &J0  &K4  &L0  &M0  &N0  &P0

&R1  &S0  &X0  &Y1

*B0  *C0  *D0  *E0  *F0  *G0  *I0  *L0  *M0  *P9  *Q2  *S0

S00=000  S01=000  S02=043  S03=013  S04=010

S05=008  S06=003  S07=060  S08=002  S09=006

S10=007  S11=070  S12=000  S13=000  S14=002

S15=130  S16=000  S17=018  S18=000  S19=000

S20=002  S21=178  S22=000  S23=105  S24=138

S25=000  S26=000  S27=156  S28=068  S29=000

S30=000  S31=017  S32=019  S33=000  S34=030

S35=000  S36=000  S37=000  S38=000  S39=000

S40=000  S41=000  S42=000  S43=000  S44=000

S45=000  S46=000  S47=000  S48=000  S49=000

OK

Profile 0 satisfies most asynchronous applications. ZyXEL preset this

profile to be the same as the factory default.

A summary of this profile is:

DTE speed           38400 bps

Link Option         MULTI-AUTO

Data Format         ASYNC

Command Set         AT

DTR                 108.2 Data Terminal Ready

RTS                 Ignored

DCD                 Tracks Carrier

DSR                 Always On

Error Control       V.42 + V.42bis

Flow Control        CTS/RTS

Quality Action      Adaptive Rate

Line Type           Dial-up

AT&V2

Profile 1 Settings.................

B0   E1   L4   M1   N5   Q0   V1   X5

&B1  &C1  &D0  &G0  &H3  &J0  &K4  &L1  &M0  &N17  &P0

&R1  &S0  &X0  &Y1

*B0  *C0  *D0  *E0  *F0  *G0  *I0  *L0  *M0  *P9  *Q1  *S0

S00=000  S01=000  S02=043  S03=013  S04=010

S05=008  S06=003  S07=060  S08=002  S09=006

S10=007  S11=070  S12=000  S13=000  S14=004

S15=130  S16=000  S17=018  S18=000  S19=017

S20=002  S21=050  S22=000  S23=105  S24=138

S25=000  S26=000  S27=092  S28=068  S29=000

S30=000  S31=017  S32=019  S33=000  S34=030

S35=002  S36=000  S37=000  S38=000  S39=000

S40=000  S41=000  S42=000  S43=000  S44=000

S45=000  S46=000  S47=000  S48=000  S49=000

OK

(6 - 3)

Profile 1 is set for the needs of asynchronous 2-wire leased line

operation. For U-1496B the default setting for this profile is the same

as profile 0.

A summary of this profile is:

DTE speed           38400 bps

Link Option         V.32bis 14400

Data Format         ASYNC

Command Set         AT

DTR                 Ignored

RTS                 Ignored

DCD                 Tracks Carrier

DSR                 Always On

Error Control       V.42 + V.42bis

Flow Control        CTS/ RTS

Quality Action      Auto Retrain

Line Type           2W Leased Line

Terminal Keyboard   Disabled

Aborting

Auto Handshake      Originate

AT&V3

Profile 2 Settings.................

B0   E1   L4   M1   N5   Q0   V1   X5

&B1  &C1  &D2  &G0  &H0  &J0  &K0  &L0  &M3  &N0  &P0

&R0  &S1  &X0  &Y1

*B0  *C0  *D0  *E0  *F0  *G0  *I0  *L0  *M0  *P9  *Q2  *S0

S00=000  S01=000  S02=043  S03=013  S04=010

S05=008  S06=003  S07=060  S08=002  S09=006

S10=007  S11=070  S12=000  S13=000  S14=192

S15=130  S16=000  S17=018  S18=000  S19=000

S20=002  S21=154  S22=000  S23=105  S24=138

S25=000  S26=000  S27=128  S28=068  S29=000

S30=000  S31=017  S32=019  S33=000  S34=030

S35=000  S36=000  S37=000  S38=000  S39=000

S40=000  S41=000  S42=000  S43=000  S44=000

S45=000  S46=000  S47=000  S48=000  S49=000

OK

Profile 2 is set for the needs of synchronous dial-up line operation. For

U-1496B the default setting of this profile is the same as profile 0 since

there is no synchronous mode.

Link Option         V.32bis 14400

Data Format         SYNC

DTR                 108.2, Data Terminal Ready

RTS                 CTS Tracks RTS

DCD                 Tracks Carrier

DSR                 CCITT

Quality Action      Adaptive Rate

Sync Clock          Internal

Line Type           Dial-up

AT&V4                                                               (6 - 4)

Profile 3 Settings.................

B0   E0   L4   M1   N5   Q0   V1   X5

&B1  &C1  &D0  &G0  &H0  &J0  &K0  &L2  &M3  &N17  &P0

&R0  &S0  &X0  &Y1

*B0  *C0  *D0  *E0  *F0  *G0  *I0  *L0  *M0  *P9  *Q1  *S0

S00=000  S01=000  S02=043  S03=013  S04=010

S05=008  S06=003  S07=060  S08=002  S09=006

S10=007  S11=070  S12=000  S13=000  S14=200

S15=130  S16=000  S17=018  S18=000  S19=017

S20=002  S21=018  S22=000  S23=105  S24=138

S25=000  S26=000  S27=064  S28=068  S29=000

S30=000  S31=017  S32=019  S33=000  S34=030

S35=000  S36=000  S37=000  S38=000  S39=000

S40=000  S41=000  S42=000  S43=000  S44=000

S45=000  S46=000  S47=000  S48=000  S49=000

OK

Profile 3 is set for the needs of synchronous 4-wire leased line operation.

For U-1496E and U-1496B, the default setting of this profile is the same as

profile 0 since they don't have 4-wire mode.

Link Option         V.32bis 14400

Data Format         SYNC

DTR                 Ignored

RTS                 CTS Tracks RTS

DCD                 Tracks Carrier

DSR                 Always On

Quality Action      Auto Retrain

Sync Clock          Internal

Line Type           4W Leased Line

Auto Handshake      Originate

AT&V5

Factory Settings.................

B0   E1   L4   M1   N5   Q0   V1   X5

&B1  &C1  &D2  &G0  &H3  &J0  &K4  &L0  &M0  &N0  &P0

&R1  &S0  &X0  &Y1

*B0  *C0  *D0  *E0  *F0  *G0  *I0  *L0  *M0  *P9  *Q2  *S0

S00=000  S01=000  S02=043  S03=013  S04=010

S05=008  S06=003  S07=060  S08=002  S09=006

S10=007  S11=070  S12=000  S13=000  S14=002

S15=130  S16=000  S17=018  S18=000  S19=000

S20=002  S21=178  S22=000  S23=105  S24=138

S25=000  S26=000  S27=156  S28=068  S29=000

S30=000  S31=017  S32=019  S33=000  S34=030

S35=000  S36=000  S37=000  S38=000  S39=000

S40=000  S41=000  S42=000  S43=000  S44=000

S45=000  S46=000  S47=000  S48=000  S49=000

OK

(6 - 5)

Factory default setting is stored in ROM, ZyXEL presets User Profile 0

the same as the factory default.

RESET FROM PROFILE

------------------

You can reset the modem and load a user profile as the active settings or

you can designate a user profile as the power-on default profile. This can

be done from the panel control of U-1496 or by issuing commands from your

terminal.

On RESET submenu screen, the profile that follows  the equal sign "=" means

that the profile is the profile that will be loaded upon power-up. This

setting also affect the SAVE TO submenu default profile, see next section.

PROFILE 0        ATZ0  Reset modem and load user profile 0.

PROFILE 1        ATZ1  Reset modem and load user profile 1.

PROFILE 2        ATZ2  Reset modem and load user profile 2.

PROFILE 3        ATZ3  Reset modem and load user profile 3.

FACTORY DEFAULT  ATZ4  Reset modem and load the factory default profile.

SAVE TO PROFILE

---------------

You can save your current settings to one of the four user profiles. If

you want to modify one of the profiles, recall it as the active settings,

then use the panel menu or AT command to do modifications, save it by using

the SAVE TO submenu or AT command. For example, save to the profile 2, you

can issue AT&W2 from the terminal or select SAVE TO submenu from the LCD

panel, then press the ENTER key on profile 2. On SAVE TO submenu screen,

the profile accompanying the equal "=" sign is the profile selected in the

RESET submenu, i.e., the power-on default profile, unless the power-on

profile is set to factory default, in which case profile 0 is the SAVE TO

default.

PROFILE 0   AT&W0   Save current settings to user profile 0 (EEPROM).

PROFILE 1   AT&W1   Save current settings to user profile 1 (EEPROM).

PROFILE 2   AT&W2   Save current settings to user profile 2 (EEPROM).

PROFILE 3   AT&W3   Save current settings to user profile 3 (EEPROM).

RESET PROFILES

--------------

After you have changed the user profile settings, if you wish, you can

reset all the four user profiles to factory preset values. This is done

by holding the ENTER key down while you power-on the modem. Remember that

by doing this, all of your user profile settings are lost.

On U-1496E, holding the VOICE/DATA switch down while powering on the modem

will reset the modem. On U-1496B, shorting the jumper TS2 and then turning

on the power will reset the modem.

Note the reset stated here is a total reset to factory state. If you get

somewhere and can not get it back, this is the way to get to a known state.

Modem is also doing continuous self testing after this reset, see Chapter

15 for detail. Turn off power to stop testing.

CHAPTER 7                                                           (7 - 1)

STATUS  REGISTERS

=================

S-Registers (Status Registers) contain values that determine and reflect

how your modem operates and executes commands. You can read their values

and change them the same as  you could from terminal commands or through

the modem's panel controls.

Every user profile corresponds to a set of S-Register values, but when we

mention S-Registers, we are referring to the ones corresponding to the

active profile. If you want to read or change the values in the profile

that is currently inactive, you will have to recall the profile to make it

active (see Chapter 6).

The U-1496 series modems have 50 S-Registers from S0 to S49. S0 to S11 are

standard AT S-Registers; and S13 to S49 are mostly bit-map configured.

Changes in the bit-map configuration can also have the same effect as

issuing commands. But typing in the commands are recommended.

READ  AND  SET  S-REGISTERS

---------------------------

See Register's Content Modification section in Chapter 5 for reading and

setting S-Registers using U-1496's LCD panel. When using AT command, use

"ATSr ?" to display value stored in S-Register "r"; use

"ATSr = n" to set S-Register `r' to value `n'.

The above two commands have "n" as a decimal value. For bit-mapped

registers, you need to do a conversion to or from the binary number to

find out the bits. For example, if you want to set S38 bit 3 to 1 for a

specific application, do an ATS38 = ? to find the original value, convert

it to binary, set the bit 3 to "1" (do a logic OR with hex "08"; if reset

the bit, do a logic AND with hex "F7"), convert it to decimal number again

and do an ATS38 = n to set the register.

S-REGISTER DESCRIPTIONS

-----------------------

Following are the descriptions for each S-Register. In most bit mapped

S-Registers, the default bit value is 0 which is the normal situation and

only the non-default situation is described. Some reserved bits are for

factory use and user should not change them.

S0         Default= 0;         Sets the number of rings on which the

modem will answer automatically. S0=0

disables autoanswer. The range of rings

is 0 to 255. (Country specific)

S1         Default= 0;         Counts the incoming ring and stores it

to this register. User can read but

should not change this value.

S2         Default=43;         S2 holds the ASCII decimal value for

Escape Sequence character. Default

character is "+". A value of 128-255

disables the escape code.

S3         Default=13;         Holds the ASCII decimal value of Carriage

Return character.

S4         Default=10;         Holds the ASCII decimal value of Line

Feed character.

(7 - 2)

S5         Default=8;          Holds the ASCII decimal value of

Backspace character. A value of 128-255

disables the Backspace key's delete

function.

S6         Default=3;          Sets the number of seconds the modem will

wait before dialing if X0 or X1 is

selected. If X2, X3, X4, X5, X6, or X7

is selected, modem will dial as soon as

it detects a dial tone. This S-Register

also sets the time-out interval for `W'

dial modifier to wait for the dial tone.

Range 1 to 255 seconds.

S7         Default=60;         Sets the number of seconds the modem

waits for a carrier. If the modem does

not detect a carrier within this time,

it hangs up and returns the NO CARRIER

result code.

S8         Default=2;          Sets the duration of delay generated by

comma (,) dial modifier.

S9         Default=6;          Sets the duration, in tenths of a second,

that the remote modem's carrier signal

must be present before recognition. When

operating in non-FSK or half-duplex

operation, the modem ignores the setting

of this register.

S10        Default=7;          Sets the duration, in tenths of a second,

that the modem waits after loss of carrier

before hanging up. This guard time allows

the modem to distinguish between a line

hit, or another cause that momentarily

breaks the connection, and a true

disconnection (hanging up) by the remote

modem.

S11        Default=70;         Sets the duration and spacing in

millisecond, in touch-tone dialing.

S12                            Reserved.

S13                            Reserved.

S14                       Bit mapped register

&Mn        (7,6)=0;    Asynchronous, buffered.

=1;    Async command, sync data.

=2;    Direct asynchronous, no buffer.

=3;    Synchronous.

&Xn        (5,4)=0;    Internal clock.

=1;    External clock.

=2;    Remote clock.

&Ln        (3,2)=0;    Dial-up line.

=1;    2W leased line.

=2;    4W leased line.

&T4          (1)=0;    Grant remote digital loopback test request.

&T5             =1;    Deny remote digital loopback test request.

*Mn          (0)=0;    Auto-handshake on originate mode in leased

line operation.

=1;    Auto-handshake on answer mode in leased line

operation.

(7 - 3)

S15                       Bit mapped register

Zn       (7,6,5)=0-4;  Profile as active settings after power ON.

*Cn        (4,3)=0;    10-bit character length.

=1;    11-bit character length.

=2;    9-bit character length.

=3;    8-bit character length.

Note:

Character length includes start bit, data bits,

parity, and stop bit(s).

(2)=0;    1 stop bit.

=1;    2 stop bits.

(1,0)=0;    Even parity.

=1;    Odd parity.

=2;    No parity.

S16                       Test status register

=0;    No test in progress. (Default)

=1;    Analog loopback test in progress.

=2;    Reserved.

=3;    Local digital loopback test in progress.

=6;    Remote digital loopback test in progress.

=7;    Remote digital loopback with self-test in

progress.

=8;    Analog loopback with self-test in progress.

S17                       Bit mapped register

*In          (6)=0;    AT command set.

=1;    V.25bis command set.

*Pn    (4,3,2,1)=0-15; Leased line transmission power level.

*Sn          (0)=0;    Disable secondary channel.

=1;    Enable secondary channel.

S18                       Reserved.

(7 - 4)

S19                       Modem connection mode

&Nn             =0;    Multi-auto, auto negotiate highest possible

link rate.

V.32bis 14400/12000/7200,

V.32 9600T/9600/7200T/4800,

V.22bis 2400/1200,

V.22 1200,

BELL 212A 1200,

V.17FAX 14400/12000/9600/7200,

V.29FAX 9600/7200,

V.27terFAX 4800/2400

=1;    V.33 14400/12000.

=2;    V.33 12000.

=3;    V.32 9600T/9600/7200T/4800.

=4;    V.32 9600/7200T/4800.

=5;    V.32 4800.

=6;    V.29 9600.

=7;    V.29 7200.

=8;    V.29 4800.

=9;    V.27ter 4800.

=10;   V.27ter 2400.

=11;   V.26bis 2400.

=12;   V.23 1200/75.

=13;   V.23 600/75.

=14;   V.22bis 2400/1200.

=15;   V.22 1200.

=16;   V.21 300.

=17;   V.32bis 14400/12000/9600/7200/4800.

=18;   V.32bis 12000/9600/7200/4800.

=19;   V.32bis 7200/4800.

=24;   Bell 212A 1200.

=25;   Bell 103 300.

=32;   V.17FAX 14400/12000/9600/7200,

V.29FAX 9600/7200,

V.27terFAX 4800/2400.

=34;   Reserved.

=35;   Reserved for 16800.

=36;   Reserved for 19200.

Note: Not all modem modes are available, check the

compatibility table for available modes.

S20                       DTE speed (Auto detected from AT command)

=0;    76.8 Kbps.

=1;    57.6 Kbps.

=2;    38.4 Kbps.

=3;    19.2 Kbps.

=4;    16.8 Kbps.

=5;    14.4 Kbps.

=6;    12.0 Kbps.

=7;     9.6 Kbps.

=8;     7.2 Kbps.

=9;     4.8 Kbps.

=10;    3.6 Kbps.

=11;    2.4 Kbps.

=12;    1.8 Kbps.

=13;    1.2 Kbps.

=14;     600 bps.

=15;     300 bps.

(7 - 5)

S21                        Bit mapped register

&Dn        (7,6)=0;    Modem ignores DTR signal, assumes DTR always

ON.

=1;    108.1, DTR OFF-ON transition causes dial of

the default number. DTR OFF causes hang-up.

=2;    108.2, Data Terminal Ready, DTR OFF causes

the modem to hang up and return to command

state.

&Rn          (5)=0;    CTS follows RTS.

=1;    Ignore RTS (CTS always ON).

&Cn          (4)=0;    CD always ON.

=1;    CD tracks presence of data carrier.

&Sn          (3)=0;    Modem forces DSR always ON.

=1;    According to CCITT.

Mn         (2,1)=0;    Speaker always OFF.

=1;    Speaker ON until carrier is detected.

=2;    Speaker always ON.

=3;    Speaker ON after last digit is dialed out

until carrier detected.

*En          (0)=0;    Maintain no error control connection when

error control handshake fails.

=1;    Drop connection when error control handshake

fails.

S22                       Reserved.

S23                       Bit mapped register

Qn           (7)=0;    Modem returns result code.

=1;    Modem does not return result code.

Vn           (6)=0;    Display result code in numeric format.

=1;    Display result code in verbose format.

Xn       (5,4,3)=0;    Basic result code (0-4).

=1;    Result code (0-5, 10-21).

=2;    Result code (0-6, 10-21).

=3;    Result code (0-5, 7-21).

=4;    Result code (0-21).

=5;    Error control result code enabled.

=6;    Error control result code enabled.

=7;    Error control result code enabled.

&Pn          (2)=0;    Pulse dial make/break ratio=39%/61%.

=1;    Pulse dial make/break ratio=33%/67%.

T/P          (1)=0;    Tone dial.

=1;    Pulse dial.

En           (0)=0;    Command echo disabled.

=1;    Command echo enabled.

S24                      Bit mapped register

Ln       (7,6,5)=0-7;  Speaker volume control.

Nn       (3,2,1)=0-7;  Ring volume control.

S25                      Reserved.

S26                      RTS/CTS Delay.

Default=0;    Sets thedelay, in 10 millisecond units,

between RTS and modem's CTS response in

synchronous mode.

(7 - 6)

S27                      Bit mapped register

*Qn        (7,6)=0;    No response to poor signal quality.

=1;    Initiate retrain to poor signal quality.

=2;    Adaptive rate (auto fall-back and

fall-forward) when signal quality is

changing.

=3;    Disconnect when signal quality is poor.

&Hn      (5,4,3)=0;    Flow control disabled.

=1;    Reserved.

=2;    Reserved.

=3;    Hardware CTS/RTS flow control.

=4;    Software XON/XOFF flow control.

=5;    Reserved.

&Kn      (2,1,0)=0;    No error control.

=1;    MNP4 (includes MNP3).

=2;    MNP4 + MNP5.

=3;    V.42 +MNP4.

=4;    V.42 + V.42bis (compatible with &K2).

S28                       Bit mapped register

Bn           (7)=0;    Select V.22 for 1200 bps communication.

=1;    Select Bell 212A for 1200 bps communication.

&Bn          (6)=0;    DTE/DCE rate follows link rate.

=1;    DTE/DCE rate fixed at DTE settings, range

from 300 to 76.8 Kbps.

&Gn        (5,4)=0;    No guard tone.

=1;    Reserved.

=2;    1800 Hz guard tone.

Break type

&Yn        (3,2)=0;    Destructive, expedited.

=1;    Non-destructive, expedited.

=2;    Non-destructive, non-expedited.

*Ln          (1)=0;    Panel key normal.

=1;    Panel key locked.

&Jn          (0)=0;    Single phone line, RJ11 phone jack.

=1;    Multiple phone/modem lines, RJ12/RJ13 phone

jack.

S29                       Default dial phone number pointer.

*Dn             =n;    Set default dial phone number pointer at

location n (n=0-9) in the EEPROM.

S30                       Dial-backup phone number pointer.

*Bn             =0;    Disable dial backup.

=n;    Enable dial backup and set dial backup phone

number pointer at location n-1 (n=1-9) in

the EEPROM.

Note:

When the leased line has become unoperational, modem will

automatically switch to dial line and the originate mode

modem will dial the number according to the pointer saved

here if dial backup was enabled. The answer mode modem will

try the leased line and wait for ring if dial backup is

enabled (a non-zero number is selected).

(7 - 7)

S31           Default=17;   Holds the ASCII decimal value of the XON

character.

S32           Default=19;   Holds the ASCII decimal value of the XOFF

character.

S33                         Reserved.

S34           Default=30;   Dial-backup restoral lookback timer, in

minutes.

Note: In dial backup operation, in addition to switch to

the dial line, the modem will automatically

"lookback" whether the leased line is again

operational. This timer will disconnect current dial

line and attempt to connect on the original leased

line. If it is still unavailable, the dial line will

continue to be used.

S35                       Bit mapped register

(4)=1;    Effective only on the U-1496E and U-1496R.

When Data/Voice switch is pressed, modem

will dial the default number.

(3)=1;    Extend the leased line transmission power

range from -12dBm to -27dBm.

(2)=1;    V.26bis alternative A.

(1)=1;    Disable aborting from terminal during modem

handshaking.

(0)=1;*   4800 trellis code for V.32 4800.

Note: Available only on special request, otherwise this feature

is reserved.

S36                       Bit mapped register/Security function control

*Gn      (7,6,5)=0;    Disable security function

=1;    Enable level 1* security, with password

check.

=2;    Enable level 1 security, with password check

and call back.

=3;    Enable level 2* security, with password

check.

=4;    Enable level 2 security, with password check

and call back.

=5;    Enable level 2 security, with password check

and call back, remote site enter the call

back number.

Note: In security level 1, the remote site must be a U-1496 series

modem. In security level 2, the remote site can be any other

modem.

Remote configuration control

(1)=0;    Write from local profile to remote profile.

=1;    Read from remote profile to local profile.

(0)=0;    Deny remote request for configuration.

=1;    Grant remote request for configuration.

(7 - 8)

S37                       Bit mapped register

(7,6,5,4)=0-5;  Local profile number.

(3,2,1,0)=0-5;  Remote profile number.

Note: When using U-1496 panel for remote configuration, set S36

bit 1 and S37 first, then activate it in DIAGNOSTIC menu.

S38                       Bit mapped register

(7,6)       Reserved.

(5)=1;    Disable MNP5 negotiation regardless of error

control setting.

(4)=0;    Enable FAX answering in MULTI-AUTO mode.

=1;    Disable FAX answering in MULTI-AUTO mode.

Hang up if FAX.

(3)=0;    CD on after CONNECT message.

=1;    CD on before CONNECT message.

(2,1)       Reserved.

(0)=1;    Enable repeat dialing the default number if

not connected.

S39                       Bit mapped register

(7,6,5)       Reserved.

(4,3)=1;    BitFax software compatible.

(2)=1;    Reverse Answer; enter originate mode on

incoming RING.

(1,0)       Reserved.

S40                       Bit mapped register

(7)       Reserved.

(6)=1;    Enable type 4 ring detection.

(5)=1;    Enable type 3 ring detection.

(4)=1;    Enable type 2 ring detection.

(3)=1;    Enable type 1 ring detection.

(2)=1;    Enable caller ID detection.

(1)=1;    ATQ2, modem does not return result code in

answering.  No RING or CONNECT message.

(0)       Reserved.

Note: Bits (6,5,4,3) enable ring detection on a combination of

four ring types. All 0's enable ring detection on any ring

longer than 100ms.

S41                      Bit mapped register

(7)       Reserved.

(6)=1;    Do not answer on the first ring.

(5)=1;    DSR follows DCD.

(4)=1;    In X2-X7 setting, modem waits for S6 seconds

before dialing and ignores dial tone

detection.

(3)=1;    Enable CCITT signals 140 and 141 on EIA-232D

interface.

(2)=1;    Extend the retrain time-out timer to 5

minutes.

(1)=0;    Flag idle in V.25bis HDLC.

=1;    Mark idle in V.25bis HDLC.

(0)       Reserved.

(7 - 9)

S42                      Bit mapped register

(7,6,5,4,3,2)       Reserved.

(1)=1;    Enable throughput averaging.

(0)=1;    Enable 2400 bps secondary channel.

Note:

Secondary channel is available only on special model.

S43 - S49                   Reserved.

CHAPTER 8                                                           (8 - 1)

AT  COMMAND  SET  SUMMARY

=========================

An AT command is a command issued by the computer/terminal to the modem

through the RS232 serial port in asynchronous data format when the modem

is in command state. An "AT" command prefix (ATtention) precedes each

command line, except in the case of "A/", "A>", and "+++". The "A/"

re-executes the last command once. `A>' re-executes the last command once,

and repeats the last call up to 9 times until aborted by physically

pressing any key on the keyboard or front panel or until a successful

connection with a remote modem has been made. These commands are not

preceded by AT nor followed by characters. Up to 40 commands can be entered

in a single command line with a single AT prefix.

"+++" is the escape sequence code, entered in data state and the modem will

return to command state.

BASIC  AT  COMMAND  SET

-----------------------

Following is a list of the basic AT command set. Each command should be

entered following an AT prefix. A "+" mark placed before an AT command in

the list indicates that it is a default setting.

A          Answer mode on-line.

+ B0         Select CCITT V.22 standard for 1200 bps communication

(Default).

B1         Select Bell 212A standard for 1200 bps communication.

D          Enter originate mode, dial number that follows, attempt to go

on-line.

Digits and modifiers that can be used with the D command:

0-9,#,*     Digits for dialing.

P           Pulse dial.

T           Tone dial.

W           Wait for second dial tone.

R           Originate a call in answer mode. Entered as the

last digit.

,           Pause for a time designated by S8.

;           Return to command state after dialing.

@           Wait for a 5-second silence before proceeding,

otherwise return `NO ANSWER'.

!           Hook flash (call transfer).

DL          Dial the last-dialed number.

DSn         Dial number stored in EEPROM at location n(0-9).

E0                     Disable character echo in command state.

+ E1                     Enable character echo in command state (Default).

H0                     Go on-hook (hang-up).

H1                     Go off-hook.

I0                     Display product information.

I1                     Display results of ROM checksum.

I2                     Display link diagnostic report.

L0-7                   Speaker volume control. The higher the value, the

higher the volume (Default is L4).

(8 - 2)

M0                     Speaker always OFF.

+ M1                     Speaker ON until carrier detected (Default).

M2                     Speaker always ON.

M3                     Speaker ON after last digit is dialed out and OFF

when carrier is detected. Tone dialing is not

heard.

N0-7                   Ring volume control. When a call comes in,

speaker will ring to indicate an incoming call.

N0 will disable the ring function (Default is

N5).

O                      Return to on-line state.

P                      Pulse dial.

+ Q0                     Modem returns result code (Default).

Q1                     Modem does not return result code.

Q2                     Modem returns result code but quiet after

answering on a RING.

Sr=n                   Set S-Register "r" to value "n", n must be a

decimal number between 0-255.

Sr=?                   Display value stored in S-Register "r".

+ T                      Tone dial (Default).

V0                     Display result code in numeric form.

+ V1                     Display result code in verbose form (Default).

Xn                     Result code and call progress monitor options

(Default is X5).

Result codes.       X0     X1     X2     X3     X4     X5     X6     X7

0   OK              0      0      0      0      0      0      0      0

1   CONNECT         0      0      0      0      0      +      +      +

2   RING            0      0      0      0      0      0      0      0

3   NO CARRIER      0      0      0      0      0      0      0      0

4   ERROR           0      0      0      0      0      0      0      0

5   CONNECT 1200           0      0      0      0      +      +      +

6   NO DIAL TONE                  0             0      0      0      0

7   BUSY                                 0      0      0      0      0

8   NO ANSWER                            0      0      0      0      0

9   RINGING                              0      0      0      0      0

10  CONNECT 2400           0      0      0      0      +      +      +

11  CONNECT 4800           0      0      0      0      +      +      +

12  CONNECT 9600           0      0      0      0      +      +      +

14  CONNECT 19200          0      0      0      0      +      -      -

15  CONNECT 7200           0      0      0      0      +      +      +

16  CONNECT 12000          0      0      0      0      +      +      +

17  CONNECT 14400          0      0      0      0      +      +      +

18  CONNECT 16800          0      0      0      0      +      -      -

19  CONNECT 38400          0      0      0      0      +      -      -

20  CONNECT 57600          0      0      0      0      +      -      -

21  CONNECT 76800          0      0      0      0      +      -      -

Notes (+): If error control result codes enabled (X5, X6, or X7),

the result message will be formatted as:

CONNECT DTE speed/Link speed/*Error control level - - - X5

CONNECT Link speed/ARQ  - - - - - - - - - - - - - - - - X6

CONNECT Link speed/ARQ/*Error control level - - - - - - X7

where ARQ means Automatic retransmission ReQuest type of

error control is enabled.

*: Data compression included.

(8 - 3)

Examples:

For X5 ---       CONNECT 38400/V.32 9600T/MNP5

CONNECT 57600/V.32b 14400/V.42b

Error control level is from NONE to

V.42b.

For X6 ---       CONNECT 9600/ARQ

CONNECT 14400/ARQ

Error control level is either ARQ or not

given at all.

For X7 ---       CONNECT 9600/ARQ/MNP5

CONNECT 14400/ARQ/V.42b

Z0                     Reset modem and load user profile 0.

Z1                     Reset modem and load user profile 1.

Z2                     Reset modem and load user profile 2.

Z3                     Reset modem and load user profile 3.

Z4                     Reset modem and load factory settings.

Note: Factory default (Z4) is the default power-on profile when

the modem is shipped out of factory.

+++                    Escape sequence code, entered in data state,

wait for modem to return to command state.

$                      Basic command summary HELP.

&$                     Extended command summary HELP.

*$                     Enhanced command summary HELP.

EXTENDED  AT  COMMAND  SET

--------------------------

&B0                    DTE/DCE rate follows link rate. (If the

communication software package has auto baud

rate detection, please turn it ON).

+ &B1                    DTE/DCE rate fixed at DTE setting, range from

300 to 76.8Kbps (Default).

&C0                    Assume that the carrier is always present (force

CD always ON).

+ &C1                    CD tracks presence of carrier (Default).

&D0                    Ignore DTR signal, assume DTR always ON.

&D1                    108.1, DTR OFF-ON transition causes dial of the

default number.

+ &D2                    108.2, Data Terminal Ready, DTR OFF causes the

modem to hang up and return to command state

(Default).

&D3                    Same as &D2 but DTR OFF also causes the loading

and reset from profile 0.

&F                     Load factory settings to RAM as active

configuration.

+ &G0                    No guard tone (within US, Canada) (Default).

&G1                    Reserved.

&G2                    1800 Hz guard tone.

&H0                    Flow control disabled.

&H1                    Reserved.

&H2                    Reserved.

+ &H3                    Hardware CTS/RTS flow control (Default).

&H4                    Software XON/XOFF flow control.

&H5                    Reserved.

+ &J0                    Single phone line RJ11 phone jack (Default).

&J1                    Multiple phone/modem lines RJ12/RJ13 phone jack.

(8 - 4)

&K0                    No error control.

&K1                    MNP4 (includes MNP3).

&K2                    MNP4 + MNP5.

&K3                    V.42, compatible with &K1.

+ &K4                    V.42 + V.42bis, compatible with &K2 (Default).

+ &L0                    Dial-up line (Default).

&L1                    2-wire leased line.

&L2                    4-wire leased line.

+ &M0                    Asynchronous mode, buffered (Default).

&M1                    Asynchronous command, synchronous data.

&M2                    Direct asynchronous mode, no buffer.

&M3                    Synchronous mode.

+ &N0                    Multi-Auto, auto negotiate highest possible link

rate:

V.32bis 14400/12000/7200,

V.32 9600T/9600/7200T/4800,

V.22bis 2400/1200, V.22 1200, Bell 212A 1200,

V.17 FAX 14400/12000/9600/7200,

V.29 FAX 9600/7200, and V.27ter FAX 4800/2400

(Default).

&N1                    V.33 14400/12000.

&N2                    V.33 12000.

&N3                    V.32 9600T/9600/7200T/4800.

&N4                    V.32 9600/7200T/4800.

&N5                    V.32 4800.

&N6                    V.29 9600.

&N7                    V.29 7200.

&N8                    V.29 4800.

&N9                    V.27ter 4800.

&N10                   V.27ter 2400.

&N11                   V.26bis 2400.

&N12                   V.23 1200/75.

&N13                   V.23 600/75.

&N14                   V.22bis 2400/1200.

&N15                   V.22 1200.

&N16                   V.21 300.

&N17                   V.32bis 14400/12000/9600/7200/4800.

&N18                   V.32bis 12000/9600/7200/4800.

&N19                   V.32bis 7200/4800.

&N24                   Bell 212A 1200.

&N25                   Bell 103 300.

&N32                   V.17 FAX 14400/12000/9600/7200,

V.29 FAX 9600/7200,

V.27ter FAX 4800/2400.

&N34                   Reserved.

&N35                   Reserved for 16800.

&N36                   Reserved for 19200.

Note: All speeds over 9600 bps are trellis coded except V.29;

V.32bis 7200 is also trellis coded. Not all modes available

in all models. Check the compatibility table.

+ &P0                    Pulse dial make/break ratio= 39%/61% (Default).

&P1                    Pulse dial make/break ratio= 33%/67%.

(8 - 5)

&R0                    CTS tracks RTS, response delay is set in

S-Register 26.

+ &R1                    Modem assumes RTS always ON, ignores the changes

(Default).

+ &S0                    Modem forces DSR always ON (Default).

&S1                    DSR according to CCITT.

&T0                    Terminate test in progress.

&T1                    Initiate Analog LoopBack (ALB) test.

&T2                    Reserved.

&T3                    Initiate Local Digital Loopback (LDL) test.

&T4                    Grant Remote Digital Loopback request from remote

modem.

+ &T5                    Deny Remote Digital Loopback request from remote

modem (Default).

&T6                    Initiate Remote Digital Loopback (RDL) test.

&T7                    Initiate Remote Digital Loopback with self test

(RDL + ST).

&T8                    Initiate Analog LoopBack with self test (ALB+ST).

&V0                    View current active settings.

&V1                    View user profile 0 settings.

&V2                    View user profile 1 settings.

&V3                    View user profile 2 settings.

&V4                    View user profile 3 settings.

&V5                    View factory default settings.

&W0                    Save current settings to user profile 0.

&W1                    Save current settings to user profile 1.

&W2                    Save current settings to user profile 2.

&W3                    Save current settings to user profile 3.

+ &X0                    Modem provides synchronous transmit clock signal

(Internal clock) (Default).

&X1                    Terminal provides synchronous transmit clock

signal (External clock).

&X2                    Receiving carrier provides synchronous transmit

clock signal (Remote or Slave clock).

&Y0                    Destructive break, expedited.

+ &Y1                    Non-destructive break, expedited (Default).

&Y2                    Non-destructive break, non-expedited.

&Zn=s                  Store the dial string (s) to EEPROM at location n

(n=0-9).

&Z?                    Display all the phone numbers stored in EEPROM.

ENHANCED  AT  COMMAND  SET

--------------------------

+ *B0                    Disable dial-backup (Default).

*Bn                    Enable dial-backup and set dial-backup pointer at

location n-1.

+ *C0                    10-bit character length (Default).

*C1                    11-bit character length.

*C2                    9-bit character length.

*C3                    8-bit character length.

*Dn                    Set default dial pointer at location n.

+ *E0                    If error control negotiation fails, keep the no

error control connection (Default).

*E1                    If error control handshake fails, disconnect the

call (hang up).

(8 - 6)

+ *F0                    Deny remote configuration (Default).

*F1                    Accept remote configuration (and Remote Digital

Loopback must be granted---AT&T4).

+ *G0                    Disable security function (Default).

*G1                    Enable type 1 security, with password check.

*G2                    Enable type 1 security, with password check and

call back.

*G3                    Enable type 2 security, with password check.

*G4                    Enable type 2 security, with password check and

call back.

*G5                    Enable type 2 security, with password check and

call back, remote site enter the call back

number.

Notes:

1. The command *Gn needs supervisory password checking.

2. In security type 1, the remote site must be a ZyXEL modem.

3. In security type 2, the remote site can be any other modems.

4. Modem can store 10 sets ( 0-9 ) of telephone numbers. If call

back security disabled, modem will search the password table

to check the remote modem's password. If match, modem will

keep the connection, otherwise modem will hangup. If call back

security enabled, modem will complete password checking, if no

match, modem will just disconnect the line, otherwise modem

will disconnect the line, find the corresponding phone number

and call back immediately. Remote modem should set to auto

answer the call and response.

*HS                    Modify supervisory password.

Note: The default supervisory password is "ZyXEL".

*Hn                    Modify user password (n=0-9).

Note: These commands *Hn and *HS need supervisory password checking.

Any character can be entered in the password (0-255). Maximum

password length is eight.

+ *I0                    AT command set (Default).

*I1                    V.25bis command set.

Note: Enter RST command in V.25bis mode will revert modem to

asynchronous AT command mode.

+ *L0                    Panel key normal (Default).

*L1                    Panel key locked.

+ *M0                    Leased line auto-handshake on originate mode

(Default).

*M1                    Leased line auto-handshake on answer mode.

*P0-15                 Set leased line transmission power level, range

from 0 dBm to -15dBm (Default = -9dBm). Range

from -12dBm to -27dBm if S35 bit 3 is set.

*Q0                    No response to poor signal quality.

*Q1                    Retrain action taken if signal quality is poor.

+ *Q2                    Adaptive rate; auto fallback or fall forward

(Default).

*Q3                    Disconnect for poor signal quality.

(8 - 7)

*Rab                   Read remote configuration to local profile.

a=0-3, local user profile.

b=0-3, remote user profile.

4, remote active configuration.

5, remote factory default.

Note: The command *Rab only works when modem is on-line, no

matter what kind of connection.

+ *S0                    Secondary channel disabled (Default).

*S1                    Secondary channel enabled.

*T                     Display last caller ID message.

*V                     View password table.

Note: The command *V needs supervisory password checking.

*Wab                   Write local configuration profile to remote user

profile and reset remote modem from that

profile.

a=0-3, local user profile.

4, local active configuration.

5, local factory default settings.

b=0-3, remote user profile.

Note: The command *Wab only works when modem is on-line, no

matter what kind of connection, async or sync.

CHAPTER 9                                                           (9 - 1)

ERROR CONTROL AND DATA COMPRESSION

==================================

ERROR CONTROL

--------------

Error Control keeps the modem data link error free by detecting and

retransmitting the data in error. ZyXEL modems support both MNP and V.42

error control protocols. MNP protocol is an industry de facto standard

developed and licensed by Microcom, Inc. ZyXEL modems support the level 4

and 3 error control protocols, commonly denoted as MNP4 and MNP3. V.42 is a

newly developed standard by CCITT -- the international telecommunication

standard making body. V.42 supports both LAPM -- Link Access Procedure

for Modem, and MNP4. V.42 handshake will try LAPM connection first, if not

successful it will try MNP4.

The error control (MNP4, LAPM) methods used on modem to modem connections

are based on some techniques utilized by both modems. These techniques are

explained below.

1. Asynchronous to Synchronous Conversion

Error control function can be applied only when the DTE (terminal or

computer) to DCE (modem) interface is asynchronous. If the interface

is synchronous, then the hosts on both sides will do the error control

function. Modem converts asynchronous characters to synchronous data

stream. Error control protocol (LAPM or MNP4) removes the start and

stop "framing" bits in the asynchronous format and forms the data

stream into blocks. Every block is framed by a start flag (01111110)

and an end flag (01111110). The maximum data block size used in the

U-1496 series is 256 bytes. The maximum number of outstanding blocks

without acknowledgements is 8. These values are adjustable according

to the modem on the other side.

2. CRC (Cyclical Redundancy Check) Error Detection

At the end of every data block, a 16-bit number CRC which is

calculated through a polynomial function is sent. The receiving modem

receives the block, calculates its own CRC through the same polynomial

function, then compares the number. If it is the same as the received

CRC, everything is alright. If not, an error(s) has occurred somewhere

in the block. The modem checks every block received for error(s).

3. Automatic Re-transmission Request (ARQ) Error Correction

Once a data block is received error free, the receiving modem will

acknowledge this block immediately. The sending modem receives the

acknowledgement and will check if any block(s) is not acknowledged.

For example, block 10-16 have been sent, and an acknowledgement for

block number 14 has been received. If no further acknowledgement has

been received until timeout or another acknowledgement of block 14 has

been received, it means that an error(s) has occurred in block 15. The

sending modem will automatically retransmit from block 15.

The error control protocol practically can detect and recover almost

100% of errors to make an error-free transmission.

DATA  COMPRESSION                                                   (9 - 2)

-----------------

Data compression works by representing the original data information in

less bits and transmitting the reduced data bits through the data link.

The receiver recovers the original information by reversing the

"representing" process. The process of representing original data in less

bits is called redundancy removing. Its effectiveness is both algorithm and

data dependent. Random data file is not compressable. Data file with high

degree of predictability, like ASCII English text file or database file, is

suitable for data compression.

In the modem, the data compression is activated between asynchronous to

synchronous conversion in an attempt to reduce the number of bits actually

sent. The receiving modem applies these techniques in reverse to recover

the actual data from the compressed data stream.

U-1496 series modems support both V.42bis and MNP5 data compression

protocols. Data compression needs an error-free data link in order to work,

otherwise the corrupted compressed data stream will ruin the decompression

process. MNP5 is used with MNP4 error control and V.42bis is used with V.42

error control.

MNP5 data compression utilizes the run-length encoding and adaptive

frequency encoding techniques. V.42bis uses a string coding algorithm.

The compression efficiency of V.42bis is generally somewhat higher than that

of MNP5. In some cases it can be 50% to 100% higher and in some cases it is

just a little bit higher. In general, it is about 50% better in efficiency.

Run-length Encoding

Run-length encoding is applied in an attempt to avoid sending long

sequences of repeated characters (data). When three or more repeated

characters appear in succession, only the first three tokens (representing

the compressed format of that character) and a repetition count will be

sent.

Adaptive Frequency Encoding

Adaptive frequency encoding is applied after removing repeated characters

(data). In adaptive frequency encoding, a compression token is substituted

in the data stream for the actually occurring character in an attempt to

send fewer than 8 bits for each character. The token is generated from a

dynamic tabulation of character appearing frequency. Total token number is

256, only the first 32 tokens are fewer than 8 bits, so random data will

take no advantage from this technique.

String Coding

Instead of sending each data character individually, a token for a character

string is sent. The modem adaptively builds a dictionary of string token

according to data appearing. U-1496 series modems support a dictionary size

up to 2K string tokens. The input data characters are combined and checked

for matched string in the dictionary. The token is sent for the longest

matched string. Compressibility is high if there are some regularities of

character pattern in the data.

For the U-1496 series of modems, the error control and data compression

option can be enabled from the front panel or terminal.

(9 - 3)

CONTROL LEVEL                ERROR CONTROL             DATA COMPRESSION

None              AT&K0      Disabled                  Disabled

MNP4(MNP3)        AT&K1      MNP4                      Disabled

MNP4+MNP5         AT&K2      MNP4                      MNP5

V.42(MNP4)        AT&K3      V.42 or MNP4              Disabled.

V.42(b)+MNP4(5)   AT&K4      V.42 or MNP4              V.42bis or MNP5

For some applications, like BBS, transmitting already compressed file,

MNP5 data compression process actually produces more redundant data and

slows down transmission. Set S38 bit 5 will disable MNP5 protocol

negotiation regardless of error control setting.

NEGOTIATION  FAIL  FALL-BACK

----------------------------

There are two options you can choose if the error control negotiation fails

at modem initial connection handshaking:

STAYS ON-LINE     AT*E0     If error control negotiation fails, keep

the no error control connection.

DISCONNECT        AT*E1     If not error control linked, disconnect the

line.

No matter what control level is enabled, DTE speed can always be set

different from the modem's speed except in the case where you set "FOLLOW

LINK RATE". For maximum throughput, it is better for you to set AT&B1

(default) "FIXED AT DTE RATE", the DTE speed will be independent from what

speed the modem is linked to. The DTE speed options range from 300 bps to

76800 bps. What DTE speed is optimized for your application depends on how

quick your computer and communication software can access the serial port.

U-1496 series modems use some controls and commands to make the modem

increase its performance.

ADAPTIVE BLOCK  SIZE

---------------------

Depending on the line condition, ZyXEL U-1496 series modems will

automatically adjust the block size to minimize the error rate and

re-transmission time.

AVERAGING THROUGHPUT

---------------------

The most reliable DTE speed for most PC is 19200 bps. Over this speed, the

slow PC has the probability to lose characters. Even when most

PC software's protocol has error correction capability, it will still take

time to recover the errors. The U-1496 averages the throughput before

sending characters to DTE. This will decrease the probability for the PC to

lose characters due to "burst" high-speed character strings.

Since the newer PC is much faster and there is an increasing use of 16550A

UART serial port chip, the PC's speed is less a problem now. The default

setting for throughput averaging is disabled. This will reduce data delay

and increase throughput in Unix application. You can enable throughput

averaging by set S42 bit 1 to 1.

HINT  FOR  HIGH-SPEED  OPERATION                                    (9 - 4)

--------------------------------

If you set DTE speed above 38400 bps, check your PC and communication

software first, see if they can handle character strings at this speed. The

new fast 286 and 386/486 PCs can handle higher speed serial data. PC with

buffered serial port, such as those using 16550 type UART chip, is very

helpful for operating at DTE speed of 57600 bps.

If you run communication software under WINDOWS, its speed will be low

because WINDOWS takes a lot of computation power out of the CPU. In this

case, set the DTE speed at 19200 bps or lower.

The communication software tends to lose incoming serial data when it is

doing block disk data transfer, because the hard disk interrupt priority

level is higher than that of the serial port interrupt in the PC. The

communication software can do a hardware flow control during disk data

transfer to prevent this.

PC with monochrome non-bit-mapped display also helps, because it will spend

less time in updating the screen. Choice of an efficient assembly coded

communication software is also very important. When using communication

software, use a simple protocol without error control (modem has done this

job) such as YModem-G and ZModem. PC software with complicated protocol

handling will slow down its speed in handling serial data.

FLOW CONTROL

-------------

This feature refers to stop and restart of the flow of data into and out of

the modem's transmit and receive data buffers. Flow control is necessary so

that a device (computer or modem) does not receive more data than it can

handle. The U-1496 series modems provide two kinds of flow control methods.

Hardware CTS/RTS Flow Control

It is a two-directional flow control. When the modem's transmit buffer is

about to be full, the modem will drop the CTS to signal DTE that it can not

take any more data. Turn ON the CTS to notify DTE that it can keep sending

data to the modem. In some software's application, when the receive buffer

of the software is about to be full, it will drop RTS to signal the modem

to stop sending data to DTE. Turn ON the RTS and the modem will restart to

send data to DTE. In asynchronous full-duplex application, the U-1496 always

responds RTS signal as a flow control signal. The U-1496 defaults

automatically to this hardware flow control setting and it is a better

choice.

Software XON/XOFF Flow Control

It is a two-directional flow control. XON and XOFF character defaults are

decimal 17 and 19, these can be changed by modifying S-Registers 31 and 32.

Both modems and DTE will treat XOFF as a signal to stop transmitting data,

and will treat XON as a signal to restart sending data. Modems will not send

these characters received from the local DTE to the remote modem.

CHAPTER 10                                                          (10 - 1)

SYNCHRONOUS OPERATION

=====================

This chapter gets you started using the U-1496 series modem for synchronous

operation. Use the U-1496 as a synchronous modem when it is connected to a

synchronous computer or terminal. Be sure that the remote modem and system

are set to synchronous also. Synchronous operation applies to all the

non-FSK modes that the modem is supplied with.

The U-1496, U-1496E, and U-1496R external and rackmountable models support

synchronous mode operation, but the U-1496B has a built-in PC asynchronous

serial port and can only be used in asynchronous mode only.

Before you start to send and receive data synchronously, some initial

settings should first be made.

CLOCK OPTIONS

-------------

Synchronous data must be transmitted and received with a common timing

clock. This timing clock is used to transmit data from DTE to modem and the

modem modulates the data according to this clock. The receiving modem

recovers the clock and data from the carrier and sends the data to the

receiving DTE according to this clock. Generally, there are three types of

transmitting synchronous clock sources you can choose from. Internal clock

is the one that suits most applications and is the default. Slave clock is

used when in remote digital loopback mode.

+ INTERNAL        AT&X0   Modem provides the clock and sends it to DTE.

EXTERNAL        AT&X1   DTE provides the clock and sends it to modem.

SLAVE           AT&X2   Modem recovered receiver clock is used as the

transmit clock.

RTS  OPTIONS

------------

There are two RTS options, the choice depends on application and host/

erminal requirement. This setting is ignored in asynchronous mode where

RTS is used as a hardware flow control signal.

+ IGNORED         AT&R1   Modem ignores RTS, assumes RTS always ON.

CTS TRACKS RTS  AT&R0   Delay before CTS responds to RTS's change. The

delay is set in S-Register 26.

MODE  OPTIONS

-------------

For synchronous data communication, there are two options you can choose to

define how the modem will operate in the command state.

SYNC DATA       AT&M1   Modem accepts asynchronous command in command

state, but exchanges data synchronously in data

state.

SYNC            AT&M3   Modem accepts synchronous command (V.25bis) and

exchanges data synchronously with a remote

modem.

DIAL  FROM  SYNCHRONOUS  MODE

-----------------------------

1) Dial from terminal by asynchronous command

Set &M1 and use asynchronous AT command to dial the number. After the

modem is connected, the modem will enter into synchronous operation.

(10 - 2)

2) Dial by the computer using V.25bis commands

Some communication software on the synchronous computer will make

dialing using V.25bis synchronous commands. In this case set &M3 and *I1

and modem will accept synchronous V.25bis dialing command and make a

synchronous connection.

3) Using DTR to dial a stored number in synchronous mode

Store the telephone number from the front panel STORE NUMBER menu or by

asynchronous command &Zn=. Use DEFAULT DIAL parameter menu or *Dn command

to select the default dial pointer. Set &D1 and either &M1 or &M3, then

raise DTR from your terminal or communication software. The modem will

dial the default stored number. After the modem is connected, the modem

will enter into synchronous operation.

On the U-1496E, with S35 bit 4 set, pressing DATA/VOICE switch will also

cause dialing of the default number.

4) Dial from panel

This is available only with U-1496 or U-1496R's LCD panel. Set modem to

synchronous mode, then select DIAL MEMORY or DIAL NUMBER to dial out the

number you want. After the modem is connected, the modem will enter into

synchronous operation.

5) Manual dial

Set modem to synchronous mode (&M1 or &M3), then dial a number from a

telephone set. When you hear the answer tone, type ATD (if set &M1) or

press "O" on the IDLE SCREEN menu from panel. On the U-1496E, press the

VOICE/DATA switch with the ANS/ORG switch set to ORG. The modem will go

into synchronous operation after connection.

AUTO-ANSWER  FROM  SYNCHRONOUS  MODE

------------------------------------

Set the modem to synchronous mode. Just as in the asynchronous setup, set

S-Register S0=the number of rings to auto-answer. This can be set from the

terminal (if set &M1) using asynchronous AT command or select STATUS

REGISTER menu to change the S0 value.

MANUAL ANSWER  FROM  SYNCHRONOUS MODE

---------------------------------------

Type "ATA" from the terminal (if set &M1) or press "A" on the IDLE SCREEN

menu from the front panel. On the U-1496E, press the VOICE/DATA switch with

the ANS/ORG switch set to ANS.

CHANGE  FROM  SYNCHRONOUS  MODE  TO  ASYNCHRONOUS  MODE

-------------------------------------------------------

Set the modem to asynchronous mode from panel or just reset from an

asynchronous profile. On the U-1496E, press the DATA/VOICE button while

turning on the power will reset all profiles to their factory presets. The

active setting is factory default which is asynchronous.

If modem is in &M1 mode, use AT command AT&M0 to change back to asynchronous

mode.

If modem is in V.25bis command mode, enter RST command will change modem to

asynchronous AT command mode.

CHAPTER 11                                                          (11 - 1)

LEASED LINE  OPERATION

=======================

A leased line is a permanent telephone connection between two fixed points.

It can be dedicated copper wires or a leased telephone circuit from the

telephone company. The U-1496 and U-1496R support 4-wire and 2-wire leased

lines. 4-wire leased line uses a pair of wires to transmit data and the

second pair to receive. 2-wire leased line transmits and receives on the

same pair of wires.

The U-1496 and U-1496R have a separate jack/connector for leased line

connection. U-1496E and U-1496B have only a phone jack for dial-up line

connection, you can only connect to the 2-wire leased line on the same jack.

LEASED LINE CONFIGURATION

-------------------------

For the U-1496, the default line type is dial-up line (DL). It includes a

separate leased line jack. The leased line plug must be plugged into the

jack labelled "LEASED LINE", located at the rear of the modem, for leased

line operation. You can set the U-1496 for leased line operation by typing

commands from the terminal or selecting it from the front panel. Leased line

phone jack configuration is shown in Appendix B.

The U-1496R has both dial-up and leased line terminal connectors at the rear

panel of the rack it is housed in.

LINE TYPE

---------

2W LEASED      AT&L1     Modem connected to 2-wire leased line, the line

plug should be plugged into the jack assigned

"LEASED LINE" on the rear panel.

4W LEASED      AT&L2     Modem connected to 4-wire leased line. Plug the

line in the "LEASED LINE" jack.

For U-1496E and U-1496B, only 2-wire leased line can be used and the same

phone jack is used for both dial-up and leased line connections. Dial

backup feature is not available in this case.

POWER LEVEL

-----------

The leased line mode transmit power level of the U-1496 and U-1496R can be

adjusted from 0 dBm to -27 dBm in 1 dBm increment.

LLINE TX POWER    AT*P0-15    The default is -9 dBm. Adjustable range

is 0 to -15dBm. Effective in leased line

operation only. If bit 3 of S35 is set,

this range will change to -12 to -27 dBm.

HANDSHAKE MODE

--------------

In a typical dial-up connection, the originating modem dials the number and

waits for the answering modem's carrier signal. The answering modem can

answer the call after a specified number of rings or be commanded to answer

the call immediately.

In a leased line connection, the communication circuit between two modems

is always connected. Dialing and waiting for rings do not happen for this

situation. If these two modems wants to establish a data link, one must be

designated the originator and the other the answerer. You can do this

setting either manually or automatically.

MANUAL CONNECTION                                                   (11 - 2)

-----------------

Set the modem to leased line mode. Type asynchronous command "ATD" to the

originating modem and "ATA" to the answering modem or press "O" on the IDLE

SCREEN menu of the originating modem and press "A" on the IDLE SCREEN of

the answering modem. On U-1496E, press the VOICE/DATA switch to make modem

go off-hook and on-line. Use the ANS/ORG switch to select originate or

answer mode.

AUTO-HANDSHAKE

--------------

If you want the handshaking to automatically occur upon power-up, you have

to save the leased line configuration to the power-up profile. How to

designate a profile to be a power-up profile can be referred to in

Chapter 6: Profiles. Please remember to set the handshake mode before you

save the configuration. Be sure that one modem is set to originate mode and

the other answer mode.

+ ORIGINATE   AT*M0      When operating over a leased line, modem will

handshake in originate mode.

ANSWER      AT*M1      When operating over a leased line, modem will

handshake in answer mode.

The U-1496, U-1496R and U-1496E support both asynchronous and synchronous

leased line operation. If the signal to noise ratio of your leased line is

guaranteed above 26 dB, V.32bis 14400 is a better choice. V.32bis 14400 is

also compatible with V.32bis 12000/7200 and V.32 9600T/9600/4800.

LEASED  LINE  DIAL-BACKUP

-------------------------

When leased line connection fails, some actions may be taken by the U-1496

or U-1496R.

1. Retry to connect on the leased line indefinitely if dial back-up option

is disabled.

2. If dial backup is enabled, the originating modem attempts to dial a

pre-stored telephone number to backup the connection; the answering

modem will retry to connect on to the leased line and monitor the

dial-up line for rings. Only U-1496 and U-1496R have the dial backup

function and capability. They have separate dial-up and leased line

phone jacks/connectors.

DIAL BACKUP

-----------

Dial backup option can be set by issuing an asynchronous AT command from

the terminal or using the front panel control.

+ DISABLED       AT*B0     Leased line dial backup disabled.

PH0-PH8        AT*B1-9   When leased line is impossible to continue

operation. The originate mode modem will

automatically dial the number stored in the

memory address selected. The answer mode modem

will monitor the dial-up line for rings.

(11 - 3)

The originating modem will try to dial the number up to nine times or until

a 3-minute timer timeout happens. If the dial backup attempt fails, the

originating modem will go back to try the leased line connection, if this

fails, it will try the dial-up line again.

It will try these two alternatives indefinitely. If the dial backup attempt

succeeds, the value of S-Register S34 determines the interval of dial

backup connection before going back to check the leased line.

S34=0;         Keep using dial backup line indefinitely.

S34=1-255;     Retry leased line after `n' minutes (Default is 30). If

the leased line is still not recovered, the modem will

dial backup again.

ABORT  FROM LEASED  LINE

-------------------------

If the leased line has not been connected, pressing any key on the front

panel will abort the attempt to connect on leased line. If dial backup is

enabled, backup action will be taken. Pressing any key again will abort the

backup action, and will revert back to IDLE SCREEN.

DISCONNECT  LEASED  LINE

------------------------

Disconnect (manually) the leased line will force the U-1496 to switch to

IDLE STATUS  screen. Either manual on-line or, if you have set leased line

as power-on default, turn OFF and ON the power will make the modem go to

leased line again.

On the U-1496E, press the VOICE/DATA switch will toggle the modem on/off

the connected leased line.

CHAPTER 12                                                          (12 - 1)

SPECIAL FUNCTIONS

=================

This chapter describes the functions and uses of some special features of

the U-1496 series modems.

SECURITY  FUNCTION

------------------

The U-1496 series modems provide a security function that when enabled,

unauthorized user can not dial in and make a connection. Two types of

security function are provided, type 1 security is used when the remote

modem is also a U-1496 series modem; type 2 security function is used when

the remote modem is any other brand modem. In type 1 connection, the

dial-in (remote) modem will send in its supervisory password for checking

at initial connection handshaking and the local modem will check this

password against its preprogrammed acceptable password list. In type 2

connection, the remote terminal will be prompted to enter the password at

initial connection and the local modem will do the password checking.

Two levels of security are provided. In level 1 security, the local modem

will maintain the connection if the password checking is OK, otherwise the

line is disconnected. In level 2 security, if the password is found in

local modem's preprogrammed acceptable list, it will disconnect the line,

dial back at the phone number corresponding to the dial-in password. The

line is simply disconnected if the password does not match.

There can be 10 user passwords, the corresponding 10 dial back numbers are

common to the modem's 10 stored phone numbers.

Security function is only accessable through AT command in terminal mode.

Any access attempt will be prompted to enter the supervisory password, the

attempt will be rejected if the entered password is not correct. The default

supervisory password is <169>ZyXEL<170> when modem is shipped out from the

factory. This supervisory password is also the password sent for automatic

password checking in type 1 connection. To modify the supervisory password,

use

AT*HS

You will be asked for original password and then asked to enter new

password. For example:

Password: (Enter supervisory password)

*****

Password: (Enter new supervisory password)

*****

OK

The command AT*Hn will modify the nth user password. You will be prompted

to enter supervisory password first and then be prompted to enter the nth

user password.

The command AT*V will list the 10 user passwords and the supervisory

password on the screen for viewing. Again, you will be prompted to enter

supervisory password first.

(12 - 2)

The following commands will enable different type and level of security

functions:

*G0            Disable security function

*G1            Enable type 1 and level 1 security, with password check.

*G2            Enable type 1 and level 2 security, with password check

and call back.

*G3            Enable type 2 and level 1 security, with password check.

*G4            Enable type 2 and level 2 security, with password check

and call back.

*G5            Enable type 2 and level 2 security, with password check

and call back, remote site enter the call back number.

For type 2 security, the remote site will be prompted to enter the user

password. A maximum of 3 tries in 40 seconds is allowed. If a correct

password is not entered within this limit the line will be disconnected.

If the remote site is to enter the call back number it will be prompted

to do so.

REMOTE  CONFIGURATION

---------------------

The U-1496 series modems provide the remote configuration capability that a

local modem can reconfigurate the remote modem and reconnect at the new

configuration. Remote configuration is provided at a profile to profile

batch mode. The line connection does not need to be always maintained

during remote configuration modification.

When on line, the remote modem's configuration or one of its profiles can

be read into one of local modem's user profiles. This profile can be

modified locally and the line can be disconnected during this time. Local

profile modification is done by loading this profile as active settings and

then modifying and saving the active setlings back to profile. Reading a

remote profile "b" into a local profile "a" is done by the command

AT*Rab

where

a=0-3     which is the local user profile number and

b=0-3     which is the remote user profile number or

b=4       for remote current configuration or

b=5       for remote factory default.

Make the modems on-line again, the local modem can download (write) its

profile to the remote modem's profile and have the remote modem reset from

the new profile. This is done with the command

AT*Wab

where

a=0-3     which is the local user profile number or

a=4       for local active configuration or

a=5       for local factory default and

b=0-3     which is the remote user profile number.

Remote profile read and write commands - *Rab and *Wab, only work on

on-line condition. It does not matter what speed and what mode the

connection is. The remote modem must be set to accept remote configuration

(12 - 3)

by AT*F1. AT*F0 command will have the modem deny remote configuration

request. Because the modem uses remote digital loopback mode to request

remote configuration, remote digital loopback request must also be granted

(AT&T4) on the remote modem to accept remote configuration request.

Remote configuration can also be activated from U-1496 or U-1496R's LCD

panel. The remote modem is set to grant remote configuration by setting S36

bit 0 to 1. For the local modem, set S36 bit 1 to 1 for read operation or 0

for write operation. Set S37 bit 0-3 to the remote profile number and bit

4-7 to the local profile number. After the S-Register settings are done,

activate the remote configuration operation by going to DIAGNOSTIC menu and

activating the REMOTE CONFIGURATION submenu.

Batch mode remote configuration is convenient when you can preconfigurate

the remote modem in one of the local modem's user profiles and you can

configurate the remote modem in one action. It is particularly useful when

there are many remote modems and a set of standard configurations is

available that you can store them into user profiles. You just need to

configurate the remote modem into one of the standard configurations by

activating the remote configuration once.

CALLER NUMBER DELIVERY (CND)

------------------------------

Caller number delivery (CND), or commonly called caller ID, is a kind of

new phone service that may be offered by your local phone company. Check

your phone company for availability. You must subscribe it and usually pay

additional monthly service charge in order to have this service.

With CND service, the phone company central office will send the coded

caller information to the called station. This information is sent once

between the first and second ring. The U-1496 series modems are equipped

with the capability to decode this caller information and present it to the

connected computer/terminal as part of the call progress ring message. The

modem will also report the caller ID information if asked by command AT*T.

There are two kinds of caller information message format sent by the phone

company. One is the single message format which includes date, time, and

caller ID; the other is the multiple message format which also includes the

caller name as registered with the phone company.

S-Register S40 bit 2 is used to enable (1) or disable (0) the caller ID

detection function. The default is disable, enable it only when you have

this service and want to enable its detection. Note that the caller ID

message may confuse some communication software not expecting it.

In single message format, the modem will present ring message to the

terminal as

RING

MM-DD hh:mm Caller_ID

RING

where MM is the two-digit month message, DD is the two-digit date message,

hh is the hour and mm is the minute of the time, and Caller_ID is the phone

number of the caller.

Following is an example:

RING

04-28 12:30  7135551414

RING

(12 - 4)

In multiple message format, if the caller's number and name are available,

the ring message will be

RING

TIME: MM-DD hh:mm

CALLER NUMBER: Caller_ID

CALLER NAME: Caller_Name

RING

Following is an example:

RING

TIME: 04-28 12:30

CALLER NUMBER: 7135551414

CALLER NAME: Michael Smith

RING

If the caller number and name are not available, the ring message will be

RING

TIME: 04-28 12:30

REASON FOR NO NUMBER: OUT_OF_AREA

REASON FOR NO NAME: PRIVACY

RING

The AT*T command will have the modem send the last CND message.

DISTINCTIVE  RING

-----------------

This is a phone service that may be offered by your phone company, check

your phone company for availability. With this service, you can have

several phone numbers assigned to the same phone line. The phone company

will send a different type of ring signal for each phone number being

called. The subscriber can distinguish which number is called by

distinguishing which type of ring is received.

A simple use of this feature is that you can have three numbers on the same

line and you can list the three numbers for voice, data, and fax,

respectively. You can then have your fax machine only answer to the ring

corresponding to the fax number and have your modem only answer to the ring

corresponding to the data number. The voice call will not be answered by

either fax machine or data modem and it will only be answered when someone

pick up the phone. Or you can have the answering machine only answer to the

voice ring. A more complicated use is that you can have one number for more

uses, e.g., one number for both data and fax.

Ring signal is a repeated on and off of ring. Different type of ring

usually corresponds to a different composition of the `ON' part of the ring.

The U-1496 series modems can distinguish up to four types of ring signal and

can be commanded to answer or not answer to any one of these four types of

ring signal. Following is a list of these four types of ring signal. These

are the ring types used in the USA. The difference between the ring types is

on the 2-second ON part of the ring signal. It comprises a long, double

short, or triple short ring.

Type 1:    1.2S or 2S on; 4S off.

Type 2:    0.8S on, 0.4S off, 0.8S on; 4S off.

Type 3:    0.4S on, 0.2S off, 0.4S on, 0.2 S off, 0.8S on; 4S off.

Type 4:    0.3S on, 0.2S off, 1 S on, 0.2 S off, 0.3S on; 4S off.

(12 - 5)

S-Register S40 bit 3-6 are used for distinctive ring control. Each bit

controls the answering of a particular ring type. Set the bit to "1" to

enable answering, set it to "0" to reject the ring. Note that the ring may

still be heard and a ring message sent to the terminal even if it is not

counted as an accepted ring by the modem. The control relationships between

bits in S40 and different ring types are:

Bit 3 - Ring type 1

Bit 4 - Ring type 2

Bit 5 - Ring type 3

Bit 6 - Ring type 4

If all these bits are "0", any ring of duration longer than 100 ms will be

accepted. Use this default if you do not have distinctive ring service.

Other countries may have different specifications for different types of

ring. The manufacturer may append other set of ring type specifications for

user to choose to suit other countries' need. See manual amendment or

firmware release note in the software disk accompanying this modem for

updated detail.

V.25bis  COMMAND  SET

---------------------

V.25bis is a dialing command set standard defined by CCITT. It supports both

asynchronous and synchronous dialing. People use AT command dialing in most

asynchronous cases because it is much widely used and it supports lots more

other functions than dialing. However, AT command only supports asynchronous

dialing and V.25bis is used in many synchronous dialing situations.

Synchronous V25bis dialing is used in many IBM mainframe and mini

environments where synchronous communication is used. It is supported by the

communication software on the computer.

The U-1496 series modems support both asynchronous and synchronous V.25bis

dialing. However, the internal card supports only asynchronous mode because

it is asynchronous only with its built-it asynchronous serial port. The

modem supports the bit-oriented HDLC (High-level Data Link Control)

synchronous protocol which is used in most synchronous communication links.

Appendix C lists the supported V.25bis commands/indications and their

formats.

Use AT*I1 to enable V.25bis commands. For synchronous application, normally

the modem is permanently used for one application. Save the desired

settings in the power-on profile and the modem will be initialized to

synchronous mode (&M3) with V.25bis command enabled (*I1) when power on.

V.25bis command set is also selectable through U-1496's LCD panel menu. A

special command RST is provided to get the modem back to asynchronous AT

command mode from V.25bis mode. You can enter this RST command in either

synchronous or asynchronous V.25bis mode.

CHAPTER 13                                                          (13 - 1)

FAX OPERATION

=============

The U-1496 series universal modem supports Group 3 send and receive

facsimile function. You must connect the modem to a computer, usually the

PC, in order to use this fax function. The computer is the input/output

device for the fax function. The interface between the modem and the

computer is the normal RS-232 serial connection. It uses the same interface

for both data and fax applications. In fax operation, the modem does the

function of protocol handshaking and image data transfer, and the computer

does the job of image data conversion, compression, decompression,

retrieving, and storing.

In the following sections, we will describe the CCITT T.30 fax protocol, the

fax commands, status report result codes, flow control protocol, and fax

applications. The instruction on using the accompanied modem/fax utility

program is contained in the document file in the software disk, please refer

to the updated document disk file for software usage instruction.

THE  CCITT  T.30  FAX  PROTOCOL

-------------------------------

The CCITT T.30 fax protocol describes the G3 fax handshake signals and

procedures. The U-1496 series modem takes full control of this protocol,

initiates and terminates fax calls, manages the communication session, and

transports the image data. The modem shields the computer fax software from

T.30 protocol handling.

Along with T.30, the U-1496 series modem provides the highest fax speed up

to 14400 bps using the new fax transmission standard. When connecting to an

existing normal fax device, the modem provides a highest speed of 9600 bps

and will automatically step down to 7200, 4800, and 2400 bps if the line

quality is not good enough. This modem supports the new higher speed and

better performance V.17 fax standard. When connecting to a similar machine,

14400 bps transmission speed can be achieved. It will step down to 12000,

9600, or 7200 bps if the line quality is not good enough.

THE FAX COMMAND SETS

--------------------

The U-1496 series universal modem supports two command sets for the fax

function. The EIA PN 2388 Class 2 command set and the Extended Fax AT

command set. We will briefly describe the supported Class 2 commands here,

please refer to EIA document for detail explanation. All the following

sections are related to Extended FAX AT commands.

The Class 2 Command Set

-----------------------

The following Class 2 commands are supported and implemented per EIA PN

2388 (8/20/90).

AT+FAA         Auto answer mode.

AT+FBOR        Phase C data bit order.

AT+FCLASS      Service class identification and control.

AT+FMFR        Identify manufacturer.

AT+FDCC        Establish DCE capability.

AT+FDIS        Current session negotiating position.

AT+FDCS        Current session parameters (Read only).

AT+FLID        Local FAX station ID string for TSI and CSI.

(13 - 2)

AT+FCR         Capability to receive.

AT+FDR         Begin or continue phase C data reception.

AT+FDT         Begin or continue phase C data transmission.

AT+FET         End the page or document.

All other +F commands are not supported but the modem will response OK. In

many cases it is "don't care". See PN 2388 for command details.

Extended FAX AT Commands

------------------------

These commands are unique to the U-1496 series modems. The computer controls

the modem through a set of extended fax AT commands and the modem responds

with a set of status report result codes. During data state, compressed fax

image data is flowing between the modem and computer. The default serial

connection speed is 19200 bps and it is higher than the fax link rate.

CTS/RTS hardware flow control is used to regulate the data flow.

The modem accepts the extended fax AT commands to set modem mode and fax

parameters. Besides the extended fax AT commands, the modem accepts all the

other AT commands described in Chapter 8. For instance, you can use "ATD" to

make a fax call, or "ATA" to answer an incoming fax call. When using the

extended Fax AT commands, you need to send the command "AT+FCLASS=6" first.

Following are the fax related AT commands:

Mode Setting

#F             Set the modem into "V17G3 FAX" mode, same function as the

extended AT command "AT&N32".

#B0            Set fax receiving mode. The received fax data are sent to

DTE continuously following the connect message.

#B1            Set fax receiving mode, the connect messages are

separated from the received fax data. The modem sends

"CONNECT FAX" and "ZyXEL" first, then it waits for the

DC2 character (hex18) to send the fax data. When the

receivess a DC2 from the DTE, it starts to send the fax

connection parameters

"/SnnnnVnTnRnLnCnPxxxxxxxxxxxxxxxxxxx<<CR>><<LF>>"

and then the received fax data.

In this mode, the modem will wait for DC2 at the

beginning of every page. The DTE software should detect

the page separator "RTC" and then sends the DC2 to

receive the next page fax data. This mode is used with

BBS receiving fax.

#B2            This mode is for the polling feature of the modem. In

this mode, the modem will send a polling signal to the

remote fax device to ask for receiving a fax from the

remote fax device. The received fax data will be sent to

the DTE continuously following the connect message.

Note:

#B2 is not implemented yet at this writing.

Parameter Setting                                                   (13 - 3)

#V0            Set to normal vertical resolution.

#V1            Set to high vertical resolution.

#T0            Set to one dimensional coding scheme.

#T1            Set to two dimensional coding scheme.

#R0            Set recording width, 1728 picture elements along a scan

line length of 215mm.

#R1            Set recording width, 2048 picture elements along a scan

line length of 255mm.

#R2            Set recording width, 2432 picture elements along a scan

line length of 303mm.

#L0            Set maximum recording length, A4 (297mm ).

#L1            Set maximum recording length, B4 ( 364mm ).

#L2            Set maximum recording length, unlimited.

#C0            Set minimum scan line time capability at the receiver,

20 ms at 3.85 line/mm, T(7.7)=T(3.85).

#C1            Set minimum scan line time capability at the receiver,

5 ms at 3.85 line/mm, T(7.7)=T(3.85).

#C2            Set minimum scan line time capability at the receiver,

10 ms at 3.85 line/mm, T(7.7)=T(3.85).

#C3            Set minimum scan line time capability at the receiver,

20 ms at 3.85 line/mm, T(7.7)=1/2 T(3.85).

#C4            Set minimum scan line time capability at the receiver,

40 ms at 3.85 line/mm, T(7.7)=T(3.85).

#C5            Set minimum scan line time capability at the receiver,

40 ms at 3.85 line/mm, T(7.7)=1/2 T(3.85).

#C6            Set minimum scan line time capability at the receiver,

10 ms at 3.85 line/mm, T(7.7)=1/2 T(3.85).

#C7            Set minimum scan line time capability at the receiver,

0 ms at 3.85 line/mm, T(7.7)=T(3.85).

#P             Set the local phone number, the phone number following

the character "P" can be up to 25 characters. The modem

will exchange this phone number with remote fax machine

during initial handshaking.

STATUS  REPORT  RESULT  CODES

-----------------------------

When the modem is set in the fax mode, each "ATD" or "ATA" command will make

the modem try to establish a fax connection and the modem will send a status

report result code back to the DTE (computer).

Following are the possible result codes:

NO DIAL TONE       Try to dial but no dial tone is detected.

NO CARRIER         Handshake fail or no carrier detected or timeout.

BUSY               The other party's phone line is busy.

NO ANSWER          Expected quiet answer is not detected before timeout.

CONNECT FAX/SnnnnVnTnRnLnCnPxxxxxxxxxxxxxxxxxxxx

This report informs DTE the connecting status which includes the connection

speed and the fax parameters after handshaking with the remote facsimile.

Snnnn              Fax connection speed; nnnn is a 4-digit number

representing the connection speed. nnnn=1440, 1200,

9600, 7200, etc., where 1440 and 1200 stand for 14400

and 12000.

Vn                 Vertical resolution; n=0 or 1.

(13 - 4)

Tn                 Coding scheme; n=0 or 1.

Rn                 Recording width; n=0, 1, or 2.

Ln                 Recording length; n=0, 1, or 2.

Cn                 Scan line time; n=1 to 7.

Pxxxx              Remote fax number.

After each fax disconnection, the following result code is sent back to the

DTE (computer):

DISCONNECTnPxxxxxxxxxxxxxxxxxxxx

This result code informs DTE the disconnecting status.

DISCONNECT0         Disconnect with remote confirmation.

DISCONNECT1         Disconnect without remote confirmaton.

Pxxxx               Remote fax number.

The basic AT commands "ATV0" and "ATQ1" do not affect the above CONNECT and

DISCONNECT status report result codes, the modem will always return the same

status form as above.

FLOW  CONTROL

-------------

In extended FAX AT command mode, the modem always uses hardware (CTS/RTS)

flow control. The following flow control signaling is used during sending a

fax:

CTS is used by the modem to flow control the DTE. When the modem turns

CTS off, the buffer inside the modem is full and can not accept any more

data. The computer should send data only when CTS is ON.

RTS is used by the computer to signal the modem that the fax message is

finished. As soon as RTS off is detected, the modem starts the post

message handshaking to make sure that the remote facsimile has received

the fax message successfully, and then hangs up the phone. After hanging

up, the modem sends a status report result code to the DTE.

If you want to send a multi-page fax, you need only add the RTC signal

between the fax message of two pages. The modem detects the RTC signal

automatically, handshakes multi-page procedure with remote facsimile and

sends the next page message.

The following flow control signaling is used during receiving a fax:

CTS is not used when receiving fax.

RTS is used to inform the modem that the computer can not accept data

at current time. The modem will not pass any received data to DTE if

RTS is turned off.

When finishing receiving the fax message, the modem will turn off CD and

then send a status report result code to the DTE.

PARALLEL  RECEIVE  WITH  THE FAX MACHINE

------------------------------------------

You can connect the modem and a fax machine on the same line and have the

modem answer data call and the fax machine answer fax call, respectively.

The modem will distinguish a data call from a fax call and make the switch.

Following is the setup procedure.                                   (13 - 5)

1. Plug the phone cable from the wall jack into the modem's Line Jack.

2. Plug the phone cable from the fax machine into the modem's Phone Jack.

3. Power on the DTE terminal and set S38 bit 4 of the modem to 1 by sending

the command "ATS38=16" to the modem. This will disable the modem from

receiving fax.

4. Set the modem autoanswer ring number to be the same as your fax machine.

For example, if you have set your fax machine to answer an incoming call

after 2 rings, then you need to set the modem with the same ring number

by sending the command "ATS0=2" to the modem.

Now when an incoming call comes, both the modem and fax machine will go

off-hook after the same number of rings but the line is connected to the

modem and cut-off from the fax machine initially. The modem will try to

handshake on the incoming call. If it is a data call, the handshake will be

successful and a connection made. The fax machine being cut off from the

line will fail the handshake and go on-hook after time-out. If the modem

finds that it is a fax call during handshake, it will disconnect since S38

bit 4 is set and the fax machine will get the line. The fax machine will

handshake successfully in this case.

RECEIVE  FAX  IN  THE  BBS  SYSTEM

----------------------------------

ZyXEL modem has the unique feature of automatic detection of data and fax

call and can make the BBS receive fax on the same phone line. To make your

BBS receive the incoming FAX, you need to do the following setup change in

your BBS:

1. Add the string "#B1+FCLASS=6" to the init string. Be sure that the

"+FCLASS=6" command is be the last command.

2. Change one of the connect message in the list to "CONNECT FAX".

3. Set the external mail string to "ZyXEL" and give it an error level.

4. In your BBS batch file, if the error level matches the external mail,

execute "rcvfax 2 /p:comport /w:workpath /b:backprint".

This setting will enable the BBS to receive a fax, store and print it out

automatically. Omit the /b: option if you don't want it to be printed out.

This capability actually makes your BBS also a plain paper fax machine.

The following is a sample setting in the FrontDoor system:

Add to the init String : X7#B1+FCLASS=6

Change connect message as:

Set external mail as:

The following is a sample setting in the "binkley.cfg" file for Binkley

systems.

Init AT&FX7S0=1#B1+FCLASS=6

ExtrnMail ZyXEL errorlevel

CHAPTER 14                                                          (14 - 1)

ADVANCED  VOICE  CAPABILITY

===========================

Voice capability means the modem can digitize the incoming voice message

for the computer to store and forward. It also means the modem can playback

the recorded digitized voice either off-line for local message listening or

on-line for message announcement.

VOICE  DATA  COMPRESSION

------------------------

The main issue in digitized voice is the amount of storage required. A good

phone quality voice digitization will produce about 64 Kbits of data for

each second of voice. The hard disk will quickly be filled up by digitized

voice for this digitization rate. Speech compression is needed to reduce the

digitization data rate. A relatively simple ADPCM (Adaptive Delta Pulse Code

Modulation) algorithm can reduce the speech data rate to half and maintain

about the same voice quality. This algorithm can also be used to reduce the

speech data rate to 1/3 or 1/4 of the original rate but with voice quality

degradation. Further reducing the speech data rate and maintaining good

voice quality require sophisticated and complicated signal processing

algorithm. It also requires a lot of digital signal processing computation

power. We call this sophisticated speech data compression capability

advanced voice capability.

The U-1496 series modems support three voice digitization schemes. Two

schemes use the ADPCM algorithm and one uses the advanced CELP (Code Excited

Linear Prediction) algorithm to achieve near-phone-quality voice at 9.6 kbps

speech data rate. A summary of these three schemes are:

Digitization         Speech Compression             Data Rate

Scheme                  Algorithm

CELP         Code Excited Linear Prediction	      9600 bps

2ADPCM       ADPCM, 2 bits/sample                 19200 bps

3ADPCM       ADPCM, 3 bits/sample                 28800 bps

where the sampling rate used is 9600 samples per second.

The ADPCM voice modes are supported on all U-1496 series modems. However,

the advanced CELP scheme is supported only on the plus version and the

U-1496 LCD model.

Silence detection is implemented to eliminate voice data coding in silence

period.

The modem also supports simultaneous DTMF (Dual Tone Multiple Frequency)

tone detection that a user can use the keypad of a tone-dialing push-button

telephone set to control, instruct, or answer the modem.

VOICE  AT  COMMANDS

-------------------

The U-1496 series modems support a set of voice AT commands. These commands

are basically consistent with the IVP (Integrated Voice Protocol) group

proposal. The command implementation is not final at this writing. It is

certain that it will be continuously updated at the initial stage. Please

refer to the manual amendment or firmware release note in the software disk

accompanying this modem for updated detail.

(14 - 2)

The following is a summary of the supported voice AT commands:

COMMAND  SYNTAX

---------------

AT is used as the command line prefix, the voice command takes one of the

following forms:

+V<CM>?                     read current setting

+V<CM>=?                    read permissable setting

+V<CM>=<<single value>>     set single-valued parameter

+V<CM>=(value string)       set compound parameter

where <CM> represents a two-letter command syntax, a value string consists

of values separated by comma and enclosed in parenthesis. The first two

command forms are for read action, the last two are for write action. A

command may have both action types or just read or write type only.

RESPONSE  SYNTAX

----------------

For each command line received, the modem issues response to each command in

the command line followed by a final response.

Each command response is of the form:

<CR><LF>

<value> or <value range>     response of "?" or "=?" command

<CR><LF>

The final response is

<CR><LF>

OK or ERROR   Command line response

<CR><LF>

The final response is "OK" if all the commands in the command line have

been successfully executed, otherwise it is "ERROR".

FLOW  CONTROL

-------------

Software XON/XOFF flow control is used.

VOICE  PARAMETER  COMMANDS

--------------------------

+VCL         Voice Service Class

Read/Write  voice mode.

Value = 1 for voice mode, 0 for non-voice mode. While not in voice mode,

the modem will reject all voice commands except the +VCL command.

+ VLS        Line Select

Read/Write  line/device select.

Value = 00h-FFh, a 2-digit hexadecimal number, equivalent to an 8-bit

binary number. Each bit in the number indicates a line/device connection to

the voice channel, only three bits are defined now. Bit 2 is not

implemented in current model.

(14 -3)

Bit number     Line/Device connected to the voice channel

Bit 1          Dial-up line. (Line Jack)

Bit 2          Local phone set. (Phone Jack)

Bit 4          Modem speaker. (for play only)

If both bit 1 and bit 2 are 0s, the dial-up line is connected to the local

phone set.

+VTH           Silence Threshold

Read/Write silence threshold level.

Value = (0  =<  n  <=  31), silence detection threshold level is n dB below

the factory maximum.

+VSI           Silence Interval

Read/Write silence interval threshold.

Value = (0  =<  n  <=  255), silence is detected for continuous low singnal

level longer than n x 0.1 seconds.

+VSM           Sampling Mode

Read/Write sampling mode.

Value = 96CELP, 2ADPCM, or 3ADPCM. Assign or read the sampling mode used.

+VPR           Port Rate

Read/Write DTE link serial port rate.

Value = n, n=0     auto-baud up to 57600 bps.

n>0                Fix rate at n x 2400 bps.

+VMK           Event Mask

Read/Write a 16-bit event mask.

Value = 0000h - FFFFH, a 4-digit hexadecimal number, equivalent to a 16-bit

binary number. Each bit in the number indicates an event detection. "1"

means the event detection is supported or enabled, "0" means otherwise.

Four event detections are currently supported as summarized below:

Bit            Event                   Report Code

Bit 0          DTMF signal detected	   <dle>t

Bit 1          T.30 CNG detected       <dle>N

Bit 3          Silence detect<E>       <dle>S

Bit 6          Remote hang-up	   <dle>H

where t = 0-9 or A-F represents the detected DTMF tone. The report code is

inserted in data if in data state or put between a leading and a trailing

<CR><LF> if in command state.

+VAS           Auto-Answer/Select

Read/Write modem auto-answer/select mode.

Value = 0-4, it determines how the modem will answer an incoming call.

Value 3 is not implemented yet.

(14 - 4)

Value           Mode

0             Auto-answer Data mode only

1             Auto-answer/select Fax/Data mode only

2             Auto-answer/select Voice/Fax  mode

3             Auto-answer/select Voice/Fax/Data mode

4             Auto-answer Voice mode only

+VAT            Activity Timer

Read/Write  activity timer.

Value = (0  = <  n  <=  255), default is 60. Absence of activity across the

DTE/DCE interface for a period longer than n seconds will cause the modem

to revert to its default state.

VOICE  ACTION  COMMANDS

-----------------------

+VIP            Initialize Parameter

Write  initialization parameter.

Value = 0 or 1. Modem initializes to the factory-default state for value=0.

Modem keeps its voice mode parameters but initializes to a defined state if

value = 1. Modem will terminate an existing +VPY or +VRD command when

executing +VIP command.

+VPY             Play

Write  play command.

Should be the last command in a command line, the remainder, if any, will

be ignored. Start playing voice message to the selected device (+VLS),

voice data will follow after receiving response and data flow control

message. Playing is terminated when +VIP is received or voice data

underrun. Playing is suspended by a Halt command.

+VRD             Record

Write record command.

Should be the last command in a command line, the remainder, if any, will

be ignored. Start recording voice signal from the selected device (+VLS).

Voice data transfer may begin after receiving flow control message from DTE.

Recording is terminated by a +VIP command. A Halt command will suspend the

recording.

<dle>p      Halt PLAY

<E><dle>r	Halt RECORD

Write  an in-data Halt command.

Receiving a Halt command causes modem to suspend the currently active +VPY

or +VRD operation and enter into the command mode.

(14 - 5)

+VRS        Resume

Write  a resume command.

Receiving a +VRS command causes modem to re-start the lately suspended +VPY

or +VRD operation. Should be the last command in a command line, the

remainder in the command line, if any, will be ignored.

VST          Status

Read event status.

The DTE may obtain the 16-bit event status (as defined in +VMK) at any time

by using +VST? command. The 1-state of a bit in the event status means that

the event, whether it is selected or not, has been detected since last

reporting. The event mask is cleared after sending the status response.

+VXF         Transition to Data/Fax

Write  transition to Data/Fax command.

This command causes modem to transfer control to Data/Fax mode.

CHAPTER 15                                                          (15 - 1)

DIAGNOSTICS

===========

The U-1496 series modems provide several diagnostic capabilities:

Power-Up Self-Test

Analog Loopback Test

Analog Loopback with Self-Test

Local Digital Loopback Test

Remote Digital Loopback Test

Remote Digital Loopback with Self-Test

Line Condition Status Display

Eye Pattern Display

Re-transmission Indicator

Throughput Display

Link Status Report

Modem Reset

All these capabilities can apply to asynchronous or synchronous, error

controlled or non-error controlled, data compression enabled or disabled

data mode. You can use these capabilities to verify the line condition and

the modem's functioning and performance and to locate the source of a

communication problem.

POWER-ON  SELF-TEST

-------------------

Each time the modem is powered-up or receives the reset command from panel,

the modem will test the ROM code checksum, system RAM memory, DSP code

checksum, DSP RAM memory, EEPROM, digital circuits, and analog circuit

calibrations.

On the U-1496 panel, the LCD will display the results of the power-on

self-test:

SYSTEM TESTING....            Self-test start indicator, if no error

occurs, this message will continue up to

the end of the test.

ROM TEST FAIL....             ROM code checksum error.

RAM TEST FAIL....             System RAM fail.

XROM CHKSUM ERROR....         EEPROM checksum error, program will

download the factory default settings to

EEPROM and re-initialize the self-test.

This is not a real error.

DSPA RAM FAIL....             The testing of DSP A RAM fails.

DSPB RAM FAIL....             The testing of DSP B RAM fails.

V0 CALIBRATION FAIL...        Analog circuit calibration error.

VR CALIBRATION FAIL....       Analog circuit calibration error.

FR CALIBRATION FAIL....       Analog circuit calibration error.

DSPA COMM FAIL....            Communications between controller and

DSP A fail.

DSPB COMM FAIL....            Communications between controller and

DSP B fail.

(15 - 2)

Reset command from panel will restart the power-on self-test.

If error message is displayed on the LCD screen, turn off the modem, wait

for more than 10 seconds, turn the power on again. If the modem continues

to display an error message, call ZyXEL Service Center for assistance, or

follow return procedure to return the modem to ZyXEL. If the normal IDLE

SCREEN shows up, the original failure was due to a temporary power-on reset

problem and will not affect modem operation.

The U-1496E has only LED indicators. The TST LED will be ON during power-on

self-test, and OFF after the test if the test is OK. The SQ LED will flash

if the test fails, the number of flashes indicates the error number

corresponding to the above error message.

Analog Loopback Test, Local Digital Loopback Test, Remote Digital Loopback

Test, Analog Loopback Test with Self-Test, and Remote Digital Loopback Test

with Self-Test all can be selected from the terminal or LCD front panel

under "DIAGNOSTIC" menu. Use front panel or AT&T0 command to terminate the

test.

ANALOG  LOOPBACK  TEST  (AT&T1)

-------------------------------

This test can check almost every part of the modem and RS-232 cable except

the telephone line outgoing interface. During Analog Loopback testing, data

from terminal or computer are sent through RS-232 cable into the modem's

transmitter and are modulated to analog signal then loopback to the

receiver, demodulated to digital form and through the RS-232 cable back to

the terminal or computer's screen. You can tell if anything is wrong from

the screen. The screen should show whatever data you have sent to the modem.

Also the Control Level field of the LCD screen will be indicated as `ALB'.

The U-1496E will have the TST LED ON during test.

This test can only be initiated when modem is off-line.

ANALOG  LOOPBACK  WITH  SELF-TEST  (AT&T8)

------------------------------------------

This test generates data from modem itself instead of input data from RS-232

cable. The data will go through the same path as Analog Loopback Test does.

In asynchronous mode, the pattern is printable ASCII characters, you can

see the result on screen. In synchronous mode, the pattern is scrambled

binary 1 and the Throughput Meter will change to a Bit Error Rate Meter.

The left reader displays accumulated bit errors while the right reader

displays accumulated bits sent. The U-1496E TST LED flashes for any bit

error. The Control Level field of the U-1496 LCD screen will be indicated as

`ALBST', which stands for ALB with self-test.

This test can only be initiated when modem is Off-line. The following Bit

Error Rate Meter example shows 11120 Kbits sent, 22 bits in error. When the

number exceeds 99999 Kbits, it will reset itself back to zero.

LOCAL  DIGITAL  LOOPBACK  TEST  (AT&T3)

---------------------------------------

This test will loopback the digital form data demodulated from receiver to

the input of the transmitter. During testing, data received from remote

modem will all be returned to the remote modem. This test is applicable

when the remote modem does not provide V.54 Remote Digital Loopback

capability. Also the Control Level field of the LCD screen will be indicated

as `LDL'. The U-1496E TST LED indicator will be ON.

This test can be initiated by the local modem when modems are on-line.

REMOTE  DIGITAL  LOOPBACK  TEST  (AT&T6)                            (15 - 3)

----------------------------------------

This test will request the remote modem to do a digital loopback. During

testing, the local modem will send a remote digital loopback request to the

remote modem according to V.54. If the remote modem supports V.54 and is

programmed to grant this kind of request, it will resend all received data

back, and the local terminal or computer will receive all data it send out.

This test is applicable when the remote modem and local modem both provide

V.54 Remote Digital Loopback capability. The Control Level field of the LCD

screen for initiated modem will be indicated as `RLB', and for accepted

modem will be indicated as `LDL'. The U-1496E will have the TST LED

indicator ON during test.

This test can be initiated by either the remote or local modem when modems

are on-line.

REMOTE  DIGITAL  LOOPBACK  WITH  SELF-TEST  (AT&T7)

---------------------------------------------------

This test generates data from the modem itself instead of input data from

RS-232 cable. The data will go through the same path as the Remote Digital

Loopback Test does. In asynchronous mode, the pattern is printable ASCII

characters, you can see the result on the screen. In synchronous mode, the

pattern is scrambled binary 1 and the Throughput Meter of the initiated

modem will change to a Bit Error Rate Meter. The left reader displays

accumulated bit errors and the right reader displays accumulated bits sent.

On U-1496E, the TST LED flashes for any bit error. This test is applicable

when the remote modem and local modem both provide V.54 Remote Digital

Loopback capability. The Control Level field of the LCD screen for initiated

modem will be indicated as `RLBST', and for accepted modem will be indicated

as `LDL'.

This test can be initiated by either the remote or local modem when modems

are on-line.

LINE  CONDITION  STATUS  DISPLAY

--------------------------------

The LCD screen display of this section only applies to the U-1496 and

U-1496R models.

Regarding data being transmitted on telephone line circuits, there are

a lot of factors that can affect the data to be correctly received by

the remote end. The U-1496 can measure four major line impairments.

From the impairment readings, you can understand the current line

condition. The U-1496 also logs down line events over a long period

that you know what has been the line condition. We will describe these

readings and logs as follows:

Note:

The readings displayed are just for reference and not to be

taken for measurements.

Signal to Noise Ratio (S/N)                                         (15 - 4)

This reading is expressed in 0.1 dB resolution. The higher the speed, the

higher the S/N ratio is required. The modem measures the S/N ratio by

measuring the distance between the demodulated signal point and the ideal

signal point. The modem measured S/N ratio is generally about 2 dB higher

than that is actually on the line because the modem rejects some of the out

of band noise. For V.22/22bis, the difference can be as high as 8 dB because

modem only use part of the 3KHz bandwidth and rejects more than half of the

voice band noise.

Receiving Power Level (RX)

This reading is expressed in 0.1 dBm resolution. The modem measured

receiving signal power is generally within 1-2 dBm of the actual value with

a DC offset. The receiver sensitivity specification for U-1496 is -43 dBm.

Strong signal power could cause signal saturation in the channel and degrade

the data validity. To avoid this situation, sometimes it is required to

decrease the transmitting power of the remote modem. For increased line

operability, the U-1496's transmitting power level on leased line operation

could be adjusted from 0 dBm to -27 dBm, default value is -9 dBm.

Phase Jitter (PJ)

This reading is expressed in peak-to-peak degrees in the 20 to 300 Hz

bandwidth in 0.1 degree resolution.

Frequency Offset (FO)

Frequency Offset is expressed in 0.1 Hz resolution. It is the difference

between the transmitted frequency and the received frequency.

In ON-LINE SCREEN 3 there are four counters for recording the action or

reaction that modem takes when line condition is changed.

Retrain Granted (RG)

It is the count for remote modem's retrain request.

Retrain Requested (RT)

It is the count for U-1496's request for remote modem to retrain when signal

quality is poor.

Round Trip Echo Delay (ED)

It is measured in T (1/2400 sec.). Will be re-measured in every retrain

action.

Carrier Loss Counter (CL)

When operated over leased line, it is the count for the carrier's temporary

drop-out if there is any.

LINK  STATUS  REPORT  (ATI2)                                        (15 - 5)

----------------------------

During a connection, the modem will record information about link

operations. You can use AT command (ATI2) to see the summary, as in the

following sample:

ZyXEL U-MODEM LINK STATUS REPORT

Chars Sent          2420           Chars Received      2420

Octets Sent          950           Octets Received      950

Blocks Sent           13           Blocks Received       13

Blocks Resent          0           Max Outstanding        8

Max Block Size       256           Retrain Requested      0

Link Duration          1           Retrain Granted        0

T401 Timeouts          0           T402 Timeouts          0

FCS Errors             0           Round Trip Delay     110

Xmitter Underrun       0           Receiver Overrun       0

Last Speed/Protocol                Connect  14400/ARQ/V42b

Disconnect Reason                  Local Hang Up

This Link Status Report is very helpful in finding out the link condition

and what is wrong with the link if there is any. Following are

the explanation for each item and terminology used in this report.

Chars                       Data characters transmitted between DTE

(computer/terminal) and modem.

Octets                      Data bytes transmitted between the modems,

handled by error control protocol, may be

compressed data. Data compression efficiency

may be seen from the ratio of chars count to

octets count.

Block                       Framed octets transmitted between error control

protocols of the two modems.

Chars Sent                  Characters sent from DTE to modem for

transmission.

Chars Received              Characters received by modem from line and sent

to DTE.

Octets Sent                 Data bytes sent to remote modem.

Octets Received             Data bytes received from remote modem.

Blocks Sent                 Blocks sent to remote modem.

Blocks Received             Blocks received from remote modem.

Blocks Resent               Number of blocks resent because of error. If

there are too many resents, it may be a bad line

or protocol incompatibility.

Max Outstanding             Maximum number of blocks sent or received

without acknowledgement.

Max Block Size              Maximum number of octets in a block. Actual

block size can be smaller.

Retrains Requested          Number of times the local modem has requested

retrain. Each request is a bad receiving

indication.

Retrains Granted            Number of times retrain requested by remote

modem were granted.

Link Duration               Connection time in minutes.

(15 - 6)

T401 Timeouts and T402      They are normally zero. They are for

Timeouts                 manufacturer checking link layer operation.

T401 and T402 are timers used in MNP4 or V.42.

FCS Errors                  Frame Check Sum Errors or number of block

errors. Many FCS errors mean a bad line.

Round Trip Delay            Round trip delay between modems in T (1/2400 sec)

units. Normally should be in the range of

100-200. A satellite link will introduce a delay

of about 1300 T.

Xmitter Underrun            Number of times modem fails to provide data octet

in time for transmission.

Receiver Overrun            Number of times modem fails to take data octet

in time from receiver.

Last speed/protocol         The speed and protocol of the link before

hang-up. It may be different from the initial

speed because of retrain and speed fallback or

fallforward. Speed change is a clue of changing

line condition.

Disconnect Reason           Possible reasons are:

Local Hang up

Remote Hang up

Carrier Lost

On Line

Resent Expiration

Protocol Error

Break Timeout

DTR Dropped

Carrier Lost 1 (No handshaking response)

Carrier Lost 2 (Remote hang up, busy tone heard)

THROUGHPUT  DISPLAY

-------------------

If the modem is operated in asynchronous operation, we can tell the line

quality from the Throughput Display. If the speed displayed is slower than

usual or what you expected, check the S/N reading, if it is good then check

the configuration of the modem and the PC serial port setting or even the

configuration of the remote side.

RE-TRANSMISSION  INDICATOR

--------------------------

In error control mode, error occurring on the link will cause the data to be

re-transmitted, in the same time, the AA LED will flash. This is another way

to know the quality of the line. On the U-1496E, the EC LED will flash to

indicate retransmission.

DIALING  INDICATOR

------------------

On the U-1496E, the HS LED will flash 1 second on and 1 second off to

indicate that modem is in dialing.

HANDSHAKING  AND  RETRAIN  INDICATOR

------------------------------------

On the U-1496E, the HS LED will flash 1/2 second on and 1/2 second off to

indicate that modem is in handshaking or retrain.

EYE  PATTERN  GENERATOR                                             (15 - 7)

-----------------------

The Eye Pattern Generator (EPG) is a diagnostic tool used with an

oscilloscope to evaluate telephone line or modem receiver performance. The

EPG converts the signal received over the telephone line to voltage signals

that the oscilloscope displays as an `eye' pattern or `constellation'

pattern.

Note:

EPG is available only on U-1496 upon special request,

commercial  model U-1496 product does not include EPG.

You can diagnose telephone line problems by noting if the eye pattern on

your oscilloscope is distorted. The quality of the line depends on the area

(country), weather, the route of each call, and equipment, etc... If the

modem cannot resist the distortion, it will retrain or fall back to a lower

rate if the modem is configured this way.

To see the eye pattern, connect one oscilloscope probe to the X-pin and one

probe to the Y-pin on the rear panel of the modem. Connect the ground of

each probe to the G-pin. Put the oscilloscope in XY mode. The modem speed

standard determines the number of points in the eye pattern.

RESETTING  THE  MODEM

---------------------

If you change modem's setting and can not get it back or you just want to

get back to factory default state, the following reset procedure will reset

the modem to factory state. On the U-1496E and U-1496B, this procedure also

makes modem run self test. It is a good way to test the modem hardware.

On the U-1496, holding the ENTER key down for a few seconds while powering

up will reset the modem.

On the U-1496E, holding the VOICE/DATA key down for a few seconds while

powering up the modem will reset the modem. The modem will also run

continuous analog loopback self-test. Printable characters will be shown on

the terminal connected to the modem's RS232 port. The HS, CD, DSR, CTS, RXD,

EC, SQ, and TST LEDs should be ON. If not, there is a hardware problem,

contact the nearest service center for help. If SQ flashes, the number of

flashes indicates the error number corresponding to the error message listed

in the beginning of this chapter.

On the U-1496B, short the TS2 jumper with a plug then power up the computer.

The modem will be reset and will continuously run self-test. Printable

characters will be shown on the computer screen. The TS1 LED should be

continuously ON. If the LED flashes, the number of flashes indicates the

error number corresponding to the error message listed in the beginning of

this chapter.

FIRMWARE  UPDATE

----------------

Most of the U-1496 series modems' functions and features are controlled by

the system firmware contained in two EPROMs. These two EPROMs are labeled

in the PCB (Printed Circuit Board) as U24 and U25. You can update/upgrade

your modem with new functions and features just by changing the EPROMs.

(15 - 8)

ZyXEL has a policy of free distribution of new firmware (but not EPROMs)

when it becomes available. New firmware is distributed in two binary files,

dealers/users can download them and program EPROMs of their own using an

EPROM programmer. Each U-1496 series model has a set of different firmware

code, the EPROMs are not exchangeable between different models.

Firmware files are named with version number, model suffix character and

chip label extension. An example is

R409E.U24

It means it is Revision 4.09 for U-1496E model and for EPROM U24. The other

suffix character meanings are:

S for U-1496 (stand alone), B for U-1496B, and R for U-1496R.

If there is one more character suffix P then it is for the plus model.

Each firmware release is a set of two firmware files with extension U24 and

U25.

For U-1496 and U-1496E models, you need to open the case in order to change

the EPROM. Following are the case opening procedures:

U-1496:  Remove the rear two rubber pads, unscrew the two screws under

the pads and the case can be opened.

U-1496E: Press on the side of the upper case and use finger nail or

screw driver to bend the locking flap a little to open. Do the

same to each locking flap.

Use a small screw driver to lever the EPROM chip out of its socket. Lever up

each end a little to avoid bending the IC pin. When you put the new firmware

EPROM in, make sure the IC orientation is right. Plug the EPROM IC in

reverse orientation will damage the IC.

ZyXEL is constantly doing firmware enhancements to its modem. The

enhancements include new features, performance improvements, bug fixes,

and/or higher speeds. ZyXEL will provide free distribution of the new

firmware but ZyXEL does not guarantee what the new firmware will be. ZyXEL

only guarantees features listed in the original product description.

CHAPTER 16                                                          (16 - 1)

U-1496B PC-INTERNAL CARD INSTALLATION

=====================================

THE  U-1496B  MODEM

-------------------

The U-1496B PC-internal card universal modem is functionally equivalent to

the U-1496E standalone model, and as such will be operationally similar to

the U-1496E. The distinction between the two is that the U-1496B does not

include switches and LED indicators. U-1496B has a built-in PC serial port

and it only works in asynchronous mode.

U-1496B has a 16550A type of UART chip built-in as the serial port. It

enables the PC host to handle high-speed serial data up to 57600bps without

losing data.

The U-1496B has the following speed and modulation modes:

Mode        Speed(bps)                   Modulation  Connection

V.32bis     14400/12000/9600/7200/4800   TCM/QAM     2-wire full duplex

V.32        9600/7200/4800               TCM/QAM     2-wire full duplex

V.22bis/22  2400/1200                    QAM/PSK     2-wire full duplex

Bell 212A   1200                         PSK         2-wire full duplex

V.23        1200/75                      FSK         2-wire full duplex

V.21        300                          FSK         2-wire full duplex

Bell 103    300                          FSK         2-wire full duplex

G3 Fax      According to V.17, V.29 and V.27ter.

COMMUNICATION  PORT  SETUP

--------------------------

The U-1496B Modem card has a serial communication port on the card that is

directly connected to the modem. The PC can access this serial port through

the PC bus. This serial port's I/O address and the interrupt number that it

is using can be set by three jumpers.

Locate the jumper block near the lower right part of the card above the

finger connector; see Fig. 16-1.

The left four jumper positions are used to select the COM port address.

Refer to Fig. 16-2 to set to any one of COM1 to COM4.

The right four positions of the jumper block is to choose the interrupt

number being used with the serial port. Refer to Fig. 16-3 to choose an

interrupt. COM1 and COM2 normally uses interrupt 4 and 3, respectively. The

interrupt you chose may interfere with other add-on card. In this case,

choose another interrupt number and try again. If all the interrupts have

been used, you must disable some other device in order to use an interrupt.

CHAPTER 17                                                          (17 - 1)

HINTS,  TIPS,  QUESTIONS, AND  ANSWERS

======================================

LOW  THROUGHPUT  OR  LOSE  DATA  IN  FILE  RECEIVING

----------------------------------------------------

Your PC may be slow in serving the serial port that it loses data in

receiving. Use a faster PC and/or a more efficient communication software

will help. Use a NS16550A type of UART serial port chip and software

supporting this chip will definitely solve this problem. Set S42 bit1 = 1 to

enable throughput averaging may solve this problem, see Chapter 9 page 9-5

about this topic.

USE  V.25bis  COMMAND  SET

--------------------------

You may use asynchronous or HDLC synchronous V.25bis commands.

See Chapter 12 page 12-8 and Appendix C for this topic.

WANT  TO  DISABLE  DATA  COMPRESSION

------------------------------------

MNP5  will slow down throughput for compressed file (.ZIP file) transfer.

You can disable MNP5 by setting S38 bit5 = 1. V.42bis will not slow down

throughput for any kind of file transfer. Refer to page 9-4 for this topic.

USE  WITH  WINFAX  OR  WINFAX  PRO  SOFTWARE

--------------------------------------------

When install the fax software, be sure to select class 2 command set.

USE  WITH  BITFAX  SOFTWARE

---------------------------

For the DOS version, set bit3 and 4 of S39 to 1 (ATS39 = 24), and save it

to profile 0 (AT&W0). For BitFAX for Windows, please add the string

"ATE0" into the dialing prefix in the transmit section of the setup.

USE  WITH  VIDEOTEXT  SERVICE  BTX, MINITEL, ETC.

-------------------------------------------------

Please invoke V.23 mode explicitly using AT&N12.

USE  WITH  PROCOM  PLUS  SOFTWARE

---------------------------------

For version 2.0 and 2.01, use the MODEMS.DAT file in the software diskette

to replace the original file.

When you run PCINSTALL, select "ZyXEL U-1496".

USE  WITH  TELIX SOFTWARE

-------------------------

Use the TELIX.MDM file in the software diskette to replace the original

file.

When you run MODEMCFG, select ZyXEL U-1496 MODELS.

USE  WITH  PCANYWHERE SOFTWARE

------------------------------

For each version, use the corresponding configuration files in the software

diskette to replace the original files. When you run the software program,

select "ZyXEL U-1496".

(17 - 2)

Software Version    Configuration Files

V3.0                AWMODEM.CNF

V4.0                AWH.MSG and AWR.MSG

LAN version         AW.MSG

USE  WITH  CARBON  COPY  SOFTWARE

---------------------------------

For each version, use the corresponding file listed below in the software

diskette, and rename it to replace the original file. When you run the

software program, select "ZyXEL U-1496".

Software Version        File in Diskette        Rename to

V5.2                    CC52.DSC                MODEM.DSC

V6.0                    CC60.DSC                MODEM.DSC

V6.0 with &D3 Setting   CC60D3.DSC              MODEM.DSC

Windows version 1.0     CCW1.DSC                CCMODEM.DSC

USE  WITH  CLOSE-UP  SOFTWARE

-----------------------------

Use the CLOSE-UP.MDM file in the software diskette to replace the original

file.

When you run the software, select "ZyXEL U-1496".

USE  WITH  QMODEM SOFTWARE

--------------------------

For each version, use the corresponding file listed below in the software

diskette and rename it to replace the original file.

When you run the software program, select "ZyXEL U-1496".

Software Version         File in Diskette         Rename to

V4.03                    QMODEM40.MDF             QMODEM.MDF

V5.01                    QMODEM50.MDF             QMODEM.MDF

NEED  FAX  SOFTWARE  FOR  UNIX?

-------------------------------

There is a public domain Unix fax software driver that may be useful to you.

We have put a patch on it. It is in the file NETFAX.ZIP in the software

diskette. This software is offered for user convenience, not for sale. It is

NOT sold to you.

TIPS  FOR  USING  MODEM  IN  UNIX  SYSTEM

-----------------------------------------

Disable throughput averaging (S42 bit1 = 0), this is the default setting and

it reduces data delay and increases throughput.

Set ATQ2 to remove result code in answering. The result code may confuse the

computer.

TIPS  FOR  LEASED  LINE  USAGE

------------------------------

For situations that data may flow from DTE to DCE even when modem is not

on-line, these unexpected data will abort modem's handshaking attempt. This

happens most likely in leased line situation. Set S35 bit1 = 1 to disable

this aborting.

APPENDIX A

DTE/DCE EIA-232D INTERFACE SIGNALS

==================================

DB-25   CCITT   EIA     Description                   Signal Flow

Pin     Signal  Signal

Number  Name    Name                                  DTE - DCE

1       101     AA      Protective Ground

2       103     BA      Transmitted Data              --------->

3       104     BB      Received Data                 <---------

4       105     CA      Request to Send               --------->

5       106     CB      Clear to Send                 <---------

6       107     CC      Data Set Ready                <---------

7       102     AB      Signal Ground

8       109     CF      Carrier Detect or Received

Line Signal Detector          <---------

15       114     DB      Transmitter signal element

timing (DCE source)           <---------

17       115     DD      Synchronous Receive

signal timing                 <---------

18       141             Local Analog Loopback

test                          --------->

20       108/2   CD      DataTerminal Ready            --------->

108/1           Connect data set to line

21       140             Remote Digital Loopback

test                          --------->

22       125     CE      Ring/Calling Indicator        <---------

24       113     DA      Transmitter signal element

timing (DTE source)           --------->

25       142             Test Indicator                <---------

CONNECTION  FROM  PC-AT'S  9-PIN  COM  PORT  TO  MODEM

------------------------------------------------------

AT COM port               Modem DB25

pin number                RS232C pin number

1          CD               8

2          RXD              3

3          TXD              2

4          DTR             20

5          Signal Ground    7

6          DSR              6

7          RTS              4

8          CTS              5

9          RI              22

APPENDIX B

PHONE JACK PIN ASSIGNMENTS

==========================

All ZyXEL modem models have two RJ11 phone jacks, one for the dial-up line

connection and one for an optional connection to a telephone set.

The signals on the pins are:

PIN 1:               A

PIN 2:               RING

PIN 3:               TIP

PIN 4:               A1

A and A1 signals are used with KTS (Key Telephone System).

The U-1496 has a leased line jack also. This jack is an 8-pin JM8 type jack,

the signals on the pins are:

PIN 1:               Transmit Signal (R1)

PIN 2:               Transmit Signal (T1)

PIN 3:               NC

PIN 4:               NC

PIN 5:               NC

PIN 6:               NC

PIN 7:               Receive Signal (T)

PIN 8:               Receive Signal (R)

4-wire leased line connection uses Pin 1 and 2 for transmission and Pin 7

and 8 for receiving. 2-wire leased line connection uses Pin 1 and 2 for both

transmission and receiving. On the circuit board, around this leased line

jack, there are for jumper positions. When jumped with wires, pin 1, 2 will

connect to pin 4, 5 and pin 7, 8 will connect to pin 3, 6. This will lead

to a RJ45 type of signal connection on the central four pins. Note that the

other four pins still have the same signals connected which do not conform

to RJ45.

Some early units have an 8-position 6-pin jack, the signals on this type of

jack are:

PIN 1:               NC

PIN 2:               Receive Signal (T)

PIN 3:               Transmit Signal (R1)

PIN 4:               Transmit Signal (T1)

PIN 5:               Receive Signal (R)

PIN 6:               NC

APPENDIX C

V.25bis COMMAND  SET

=====================

V.25bis  ENCODING  OF  COMMANDS  AND  INDICATIONS

-------------------------------------------------

Command/Indication           Syntax          Command/Parameter Format

Call request with:

number provided           CRN  CRN        <Number to be dialed>

memory address provided      CRS  CRS n,     n=0-9 is the memory address

Program memory               PRN  PRN n;     <Number to be stored>

with number                               n=0-9 is the memory address

List request of

all stored numbers        RLN  RLN

Disregard incoming call      DIC   DIC

(Do not answer)

Connect incoming call        CIC   CIC

Call failure indication      CFI    CFI XX, XX is the failure type

Failure Type:

Parameter          IA5

coding

Engaged Tone       ET

Number not stored  NS

RING Tone          RT

Abort Call         AB

Answer Tone not

detected         NT

Incoming call                INC  INC

Valid                        VAL  VAL

Invalid                      INV  INV

List of stored numbers       LSN  LSN n;     <stored number>

(Response to RLN)                          n=0-9 is the memory address

Change to asynchronous       RST  RST

AT command mode

Notes:

(1) There can be space or no space between command syntax and parameter.

(2) Number to be dialed or stroed is a string of dialing digits or

modifiers. See AT dial command for dial modifiers.

(3) The modem responses to RLN with a sequence of LSN indications, each LSN

message is a memory address with its stored number.