FEBRUARY 1993

NetBIOS Emulator Installation

=================================

The ETHER NetBIOS Emulator is a standardized network interface

program that allows this Adapter to run NetBIOS-based network

operating systems like the IBM PC LAN Program and others.  No

software modification or additional installation is necessary.

All operations as well as installation procedures for the network

operating system will remain unaltered.

The following files will be used in this installation:

Files           Contents

-----------     ------------------------------------------------

METH16.EXE      ETHER-16 series Driver Program

NBIOS.EXE       NetBIOS Emulator Program

To install the NetBIOS Emulator, do as follows:

1. Load in the driver program METH16.EXE.

>METH16 /IOP:n /IRQ:n

/IOP:n  Define the base I/O address for your network interface card.

n is a 3-digit hexadecimal number representing the base I/O

address.

If you don't enter this parameter, a default I/O address

300H will be used.

/IRQ:n  Define the IRQ line used by your network interface card.

n is 2-digit number representing the IRQ line.

If you don't enter this parameter, a default IRQ line 3 will

used.

2. Load in the NetBIOS Emulator program NBIOS.EXE.  This program

is memory-resident and will stay in your computer's memory

until its power is turned off or until you re-boot.

>NBIOS [/Sm] [/Ck]

Typing the parameter after NBIOS is optional.  The parameter is used

as follows:

/Sm   specifies the maximum number of sessions allowed on your

computer, where m can be a value from 1 to 254.  If this

parameter is not specified, the default number of sessions

will be set to 32.

/Ck   specifies the maximum number of NetBIOS command available for

your computer where k can be a value from 1 to 255.  If this

parameter is not specified, the default number of NetBIOS

commands will be set to 32.

Your computer now is ready to run a NetBIOS-based network operating system.