NDIS Driver Program Installation             FEBRUARY 1993

================================

The NDIS driver programs in this disk support all ETHER-16 series Ethernet

cards.

This diskette is labeled as LAN_DRV_ETH and the files are organized as

follows:

Directory       Contents

---------       -----------------------------

\NDIS\DOS       NDIS driver for DOS

\NDIS\OS2       NDIS driver for OS2

ETHER NDIS driver program for DOS follows the Microsoft's NDIS spec v2.0.1.

ETHER NDIS driver program support the following network operating systems:

MS LAN Manager v2.1

DECnet PathWork v4.0

WIN/TCP PathWay Access for DOS v1.1

3COM 3+ Open v1.1

Banyan VINES v4.00(2)

IBM LAN Support Program v1.2.1

Windows for Workgroups v3.1

************************************************

*  How to install METH16.DOS on MS LAN MANAGER *

************************************************

1)  Prepare a diskette with volume name "DOS_DRV_ETH" and create directories

in the following manner:

>label a: DOS_DRV_ETH     --insert a blank disk in drive 'A'

>md MSLANMAN.DOS          --make a directory called MSLANMAN.DOS

>cd MSLANMAN.DOS

>md DRIVERS               --make a directory called DRIVERS

>cd DRIVERS

>md ETHERNET              --make a directory called ETHERNET

>md NIF                   --make a directory called NIF

>cd ETHERNET

>md METH16                --make a directory called METH16

MSLANMAN.DOS  (root)

|

DRIVERS

________|_________

|                  |

ETHERNET            NIF

|

-------------

|           |

METH16

2) Insert the ETHER-16 driver program diskette in drive 'A' and insert the

prepared diskette (DOS_DRV_ETH) in drive 'B'.  Afterwards, copy the

appropriate NDIS driver files from drive 'A' to 'B'.

>a:                     --go to drive 'A'

>cd \ndis\dos

>copy METH16.DOS     B:\MSLANMAN.DOS\DRIVERS\ETHERNET\METH16

>copy PROTOCOL.INI  B:\MSLANMAN.DOS\DRIVERS\ETHERNET\METH16

>copy METH16.NIF     B:\MSLANMAN.DOS\DRIVERS\NIF

3) MS LAN Manager provides a SETUP.EXE program on their Setup disk.

Execute this SETUP program to specify the proper driver for ETHER-16

card.

-If you are installing LAN Manager for the first time, do the following:

Upon completion of retrieval of the last workstation's disk contents,

you will be asked if you want to import additional network drivers.

Answer "YES".

-If you have already installed LAN Manager in your harddisk, do the

following:

Select the Configuration menu, and then select "Add New Config" option.

Afterwards, choose the "Other Driver" option.

4) Insert the prepared diskette (DOS_DRV_ETH) in drive 'A' and follow the

instructions on the screen.  The SETUP program will copy the necessary

NDIS driver programs from the DOS_DRV_ETH diskette to your harddisk (into

the LAN Manager directory).

5) After completion of the SETUP, your CONFIG.SYS file will be modified and

contain statements referring to the ETHER NDIS driver programs.

Reboot your computer and follow the NDIS binding procedure to use the ETHER

NDIS driver programs.

********************************************************

*  How to install METH16.DOS on WIN/TCP PathWay Access *

********************************************************

1) Copy METH16.DOS to directory C:\LANMAN.

2) Go to the directory C:\LANMAN, and create a text file called PROTOCOL.INI:

[METH16]

drivername = METH16$

3) Add the following statements to your CONFIG.SYS file:

device=c:\lanman\METH16.DOS

4) Add the following statements to your AUTOEXEC.BAT file:

ndis

winbind

wintcp

****************************************************

* How to install METH16.DOS on DECnet PathWork v4.0 *

****************************************************

You can use the DECnet PathWork's  NETSET.EXE program to install ETHER NDIS

driver program. Please refer to DECnet installation manual for detailed

information. The following instructions assume that you have already installed

the DECnet PathWork with 3Com's Etherlink II card.

1) Modify your CONFIG.SYS file as follows:

device = c:\decnet\ELNKII.SYS

to

device = c:\decnet\METH16.DOS

2) Modify the PROTOCOL.INI file as follows:

from:

---------------------------------

[ELNKMACII]

drivername = ELNKII$

ioaddress = 0x300

interrupt = 3

xmitbufs = 2

[IPX4MAC]

:

bindings = ELNKMACII

:

[DATALINK]

:

bindings = ELNKMACII

:

to:

-----------------------------------

[METH16]

drivername = METH16$

[IPX4MAC]

:

bindings = METH16

:

[DATALINK]

:

bindings = METH16

:

********************************************

* How to install METH16.DOS on 3Com 3+ Open *

*********************************************

Supposing that you have installed the 3Com 3+ Open with 3Com's 3C501 card:

1) Copy METH16.DOS to  C:\3OPEN\DOSWKSTA\LANMAN\DRIVERS.

2) Modify the device driver statement for 3C501 card in your CONFIG.SYS to:

device = c:\3open\doswksta\lanman\drivers\METH16.DOS

3) Modify the PROTOCOL.INI file to:

[ETHERLINK]

:

drivername = METH16$

:

*****************************************************

* How to install METH16.DOS on Banyan VINES v4.00(2) *

*****************************************************

NOTE : To install ETHER NDIS driver program, you need Banyan VINES v4.00(2)

or above.

1. Create a bootable diskette (booted with DOS 3.1 or above) which will be

used as boot diskette of Banyan VINES workstation or create a sub-directory

on your hard disk.

There are two methods to copy the required files for Banyan VINES workstation

to the bootable diskette or sub-directory of hard disk.

Method 1.

1) Execute 'INSTALL' command which was saved in the diskette labeled

"VINES MASTER DISK".

2) Follow the instructions to copy the files to the bootable diskette

or sub-directory of hard disk.

Method 2.

1) Execute 'BAN' command.

2) Login VINES system.

3) Execute 'PCCOPY' command.

4) Select 'NDIS Ethernet' item and follow instructions as menu driven

for copying the files to the bootable diskette or sub-directory of

hard disk.

2. Insert the ETHER-16 driver program diskette in drive 'A' and insert

the

prepared bootable diskette in drive 'B'. Afterwards, copy the appropriate

NDIS driver files METH16.DOS from drive 'A' to 'B' or sub-directory of hard

disk.

3. Create a CONFIG.SYS file which contains the following statements:

DEVICE=PROTMAN.DOS /I:A:\

DEVICE=METH16.DOS

NOTE:" A:\" is the path which may be changed depending on the location

of the files for Banyan VINES workstation.

4. Create a METH16.NIF file and include the following statements in this file:

Model = ETHER ETHER-16 Ethernet Card

Path = A:\

DeviceDriver = METH16.DOS

DeviceName = METH16$

Type = NDIS

NOTE:" A:\" is the path which may be changed depending on the location

of the files for Banyan VINES workstation.

5. Create a PROTOCOL.INI file as follows:

[PROTOCOL MANAGER]

drivername = PROTMAN$

[VINES_XIF]

drivername = NDISBAN$

bindings = "METH16"

[METH16]

drivername = METH16$

6. Execute 'PCCONFIG' command

1) The "CONFIGURING A PC FOR THE NETWORK" is displayed on the screen

then select item '1 - Network Card Settings'.

|

The "Network Card Setting" is displayed on the screen

then select item 'NDIS Ethernet'.

|

The "NDIS Ethernet" is displayed on the screen

then set your choice for ETHER NIC

Ex. Interrupt Level (2-15) : 3

PROTOCOL.INI bindings = METH16

|

2) Return to Screen "CONFIGURING A PC FOR THE NETWORK"

then select item '2 - Login Environment Settings'.

|

The "Network Card Setting" is displayed on the screen

then select item 'NDIS Ethernet'.

|

then 'PCCONFIG.DB' file will be generated.

7. Reboot your computer.

8. Login into VINES by executing the 'BAN' command.

**************************************************************

* How to install METH16.DOS on IBM LAN Support Program v1.2.1 *

**************************************************************

Supposing that you have installed the IBM LAN Support Program in your computer,

you can use METH16.DOS to replace the installed driver program. Follow the

instructions below to change the contents of CONFIG.SYS and PROTOCOL.INI files.

1) Copy METH16.DOS to the directory where your IBM LAN Support program resides.

2) Modify the original NDIS device driver statement in CONFIG.SYS as follows:

DEVICE = [driver]:\path\METH16.DOS

3) Modify the PROTOCOL.INI file as follows:

BINDINGS = METH16

:

:

[METH16]

DRIVERNAME = METH16$

***********************************************************

* How to install METH16.DOS on Windows for Workgroups v3.1 *

***********************************************************

There are two methods for installing METH16 using Windows for

Workgroups v3.1

Method I:

Manual editing of pre-selected configuration

Involves manual editing of installed Windows for Workgroups

configuration files using pre-selected NE2000 driver.

Please refer to WFW1.DOC for detailed installation procedures.

Method II:

Menu-driven installation

Using a ETHER supplied OEMSETUP.INF file, the user can choose to

install the appropriate ETHER adapters through a menu.

Please refer to WFW2.DOC for detailed installation procedures.

==============================================================================

ETHER NDIS PROTOCOL.INI User Guide

The keywords used to create the PROTOCOL.INI file for METH16 is as follow:

DRIVERNAME -This specifies the network card's driver name.

**Following is an example of a PROTOCOL.INI file that can be used with ETHER

ETHER-16 series Ethernet card:

Drivername = METH16$