Notes for the Original Ethernet NetPort Maintenance Update Version 2.57

Sept. 23, 1992

OVERVIEW

This is a maintenance update that primarily fixes a print server mode

connection problem with version 2.53.  This does not add any functionality

to NetPort beyond the 2.53 release.

CONNECTION PROBLEM FIX

NetPort upgraded v2.53 (BackPort) is in Print Server mode.

If a printer attached to the NetPort is off-line for about 15 minutes,

the Novell Watch Dog timer will break the connection.  NPLIST reports that

the NetPort is in print server mode.  NetPort never goes into the attaching

mode as it should.  Pconsole's Currently Attached Servers is blank.  A

reset is required to reconnect NetPort.  This has been fixed.

OTHER PROBLEM FIXES

In print server mode the banner will print correctly if the queue

name and file server name is longer than 27 characters.

NP2UPDAT.EXE has been improved to help prevent update failures.

A problem with tab expansion has been corrected.

NP2CON and NP2UPDAT replace NPCON and NPUPDATE

Because this update is based on the NetPort II code, the same utilities

work for NetPort II and this update. Therefore, NP2CON.EXE and NP2UPDAT.EXE

replace NPCON.EXE and NPUPDATE.EXE. This is convenient if you have both

the original NetPort and NetPort II on your network.  NPCON.EXE and

NPUPDATE.EXE should not be used with NetPort II.

SERVERNAME=  (PREFERRERD SERVER)  ***THIS IS A REQUIRED PARAMETER***

This new REQUIRED parameter is used to tell NetPort what server

(file server or Novell Pserver) it should associate with.  This means

NetPort no longer searches all the servers on the network to look for

its configuration.  Instead, it "attaches" to the server defined by the

SERVERNAME= option.  Now, when no servername is specified, your NetPort

remains "AVAILABLE", and does not search through the servers on the network.

This ensures that NetPort will NOT disappear from your network while

it searches for its configuration.

NOTE: Do NOT give your Novell Pserver and file server the same name.

If you do have a Novell Pserver and a file server with the same name,

NetPort will not be configured as a print server even though you have

defined it as a print server and used the SERVERNAME= option to assign

NetPort to a specific file server.

Because NetPort checks for a Pserver match first, then tries

to attach as a remote printer to the Pserver, it finds a match

with the Pserver and does not try the file server with the same name.

For information on how to use this option, see the "CONFIGURING NETPORT..."

sections below.

ATTACHING MODE

This functionality allows NetPort to dynamically reattach to the file

server or Pserver after the file server or Pserver is brought down and

back up.  You do not have to reset the NetPort.  NetPort also goes into

ATTACHING mode as it is attaching to the server as set by the

SERVERNAME= option.

If the NetPort cannot successfully attach to the server, it goes into an

attaching error condition.  Information about this error condition is

displayed with the NPLIST and in more detail with the NPSTAT utilities.

You can also get complete information by calling FaxBACK at 800-525-3019

(US & Canada) or 503-629-7576 (International) and requesting document

number 6015, NetPort and NetPort II Attaching mode Information.

ERROR CONDITION NOTIFICATION

NetPort notifies all users defined as NetPort's Queue Operators in

PCONSOLE any time the printer is off-line or halted for forms.

Users are notified on 25th line of their monitor.

To avoid getting this message, go into Pconsole and delete the user's

name from Queue Operators under Print Queue Info.  If you want to have

users as Queue Operators, but do not want them to get the off-line

notification message, you can create a group in SYSCON and add any user

you want to be a queue operator to this group.  Add this group name to the

PCONSOLE Queue Operators field for the NetPort queues.  NetPort does NOT

notify groups defined as Queue Operators, only users.

QSCAN=

This new option allows you to set the amount of time that a NetPort

(configured as a print server) waits before checking to see if there is a

print job in the queue.  To set the scan time, type this at the DOS prompt:

NP2CON NetPortName QSCAN=n

where n is the time in tenths of seconds, from 10 to 6000, that NetPort

waits before checking the queue.  The default is 40, which means NetPort

checks the queue every 4 seconds.

SPLITTING PORTS

This new functionality allows you to configure one port as a remote printer

or print server, and the other port mapped as a virtual circuit.  You can

not configure one port as a print server and the other as a remote printer.

NAMING

You can no longer name an individual port.  You can only give the NetPort

box a name.  This means when you rename NetPort

(NP2CON NPxxxxxx NAME=new_name), that same name shows up under NPLIST

as new_name-P and new_name-S.

UPDATING YOUR NETPORT TO THE NEW VERSION OF SOFTWARE

To complete the NetPort upgrade, you will update the NetPort's

Flash memory (the non-volatile, read-only memory that stores the

NetPort's internal operating instructions).  You can either update

over the network or over a serial cable.

The following instructions are for updating over the network.

If you want to do a serial port update, see the Installation

Guide for complete instructions.  Or, call one of the following

FaxBACK phone numbers to request document number 6012. You will

need a special serial cable to do a serial port update.

o  From U.S. and Canada    800-525-3019

o  From Europe             +44-793-432509

o  From anywhere           +503-629-7576

1)  Copy the new update software into a public directory on your network.

You may use the DOS copy command.

2)  Determine which Flash ROM version your NetPort currently uses.

This information is necessary to complete step 5 below.

To do this, type:

NPLIST VER

The NetPort must show up on the network under NPLIST before you can

proceed with the update.

3)  Log into the network as a Supervisor.

4)  Perform A, B, and C listed below ONLY if NetPort is currently

set up as a remote printer.  If NetPort is set up as a remote

printer, you must complete these steps to set it to AVAILABLE:

A)  Assign NetPort a new name.  Type:

NP2CON NPxxxxxx NAME=temp_name

B)  Reset the NetPort.  Type:

NP2CON NPxxxxxx RESET

C)  After you complete the update (as described in update steps

1 through 6), your NetPort will be set to the default NPxxxxxx

name. If your NetPort had a new_name before the update, you

must set it back to that new_name, before configuring NetPort.

At the DOS prompt, type:

NP2CON NPxxxxxx NAME=original_name

Where NPxxxxxx is the default NetPort name.

Where temp_name is the new name you're temporarily

assigning to NetPort.

Where original_name is the name NetPort had before you started

the update process.

5)  Complete one of the following update procedures from the directory

to which you copied the new software.

***NOTE: The following MUST be followed to assure a successful update:

o The NetPort and workstation must be local.  Do NOT

update across a wide area link.

o Do NOT update your NetPort while running under Windows.

You should only update from DOS.

o Update when network traffic is at its lowest.

o Do not update a NetPort that is in the process of printing.

o If you are running on Ethernet 802.3 and want to update your

NetPort from version 2.10 or lower to the new version, type this

at the DOS prompt:

CONVERT NPxxxxxx ECONVERT.UPD NETWORK

o If you are running on Type II, and want to update your

NetPort from version 2.10 or lower to the new version, type this

at the DOS prompt:

CONVERT NPxxxxxx ECNVRTEN.UPD NETWORK

o If you are running on Ethernet 802.3 and want to update your

NetPort from version 2.11 or 2.13 (unreleased patches for specific

remoter printer problems) to the new version, then type

this at the DOS prompt:

NP2UPDAT NPxxxxxx ECONVERT.UPD NETWORK

o If you are running on Type II, and want to update your

NetPort from version 2.11 or 2.13 (unreleased patches for

specific remote printer problems) to the new version, type

this at the DOS prompt:

NP2UPDAT NPxxxxxx ECNVRTEN.UPD NETWORK

o If you are running ethernet 802.3 and want to update your NetPort

from 2.53 or above, to the new version, you can either update your

NetPort using NPAdmin (see the section below entitled 'Updating

NetPort from version 2.53 to 2.57 using NPAdmin') or type this at

the DOS prompt:

NP2UPDAT NPxxxxx NP257.UPD NETWORK

or (for Enet Type II only)

NP2UPDAT NPxxxxx NP257EN2.UPD NETWORK

where NP257.UPD is the latest release of the NetPort software and

NP257EN.UPD is the ENET Type II version of the latest release.

If you ever want to restore your NetPort to use version 2.10 (over

the network), do one of the following:

***NOTE: You can not restore your NetPort to type II using NP210EN2.UPD

across a serial cable.  You must restore over the network.

o  For Ethernet 802.3, type:

NP2UPDAT NPxxxxxx NP210.UPD NETWORK

o  For Type II, type:

NP2UPDAT NPxxxxxx NP210EN2.UPD NETWORK

6)  Reset the NetPort by typing the following at the DOS prompt:

NP2CON NetPort_Name RESET

where NetPort_Name is the default name shown on the bottom of

the NetPort box (NPxxxxxx).

***NOTE: Your NetPort will now be at the default settings.

Remember, if your NetPort had a new name before

the update, it no longer does.  You'll need to reassign

the new name to NetPort (NP2CON NPxxxxxx NAME=new_name).

Also, any other non-default parameters will need to be

reassigned, such as serial ports setting, form checking

flags etc.

UPDATING NETPORT FROM 2.53 to 2.57 USING NPADMIN

NPAdmin can be used to update your NetPort from version 2.53 or above to

this release.  You must have NPadmin version 1.55 or above. If you have

an earlier version, see the end of this document which explains how

to get the current NPAdmin from the Intel BBS.

NPadmin Upgrade:

1) Copy the new update software into a public directory on

your network.

2) Log into the network as a Supervisor or equivalent.

3) Change to the directory where the new update software resides.

4) At the DOS prompt type: NPADMIN  (or NPADMIN /MONO)

5) Select the Update menu. Then select the Update option.

6) From the NetPort list, select the NetPort you want to update.

7) In the new NetPort software field, press <Insert> to

display a list of update files.  Highlight NP257.UPD (IEEE 802.3)

or NP257EN2.UPD (enet type II) and press <Enter> twice.

8) NetPort displays an update verification statement.  If

you want to stop the update, press <Escape>; otherwise, press any

key.

NPAdmin will tell you when the update is complete.  You must

refresh the NetPort list window to see the new NetPort version

number. NetPort II should be back in the configuration that it was

set to prior to the update.

CONFIGURING NETPORT AS A PRINT SERVER

***NOTE: You can use NPAdmin, NetPort II's menu-driven interface,

to configure an original NetPort with this update version

2.57, as long as the version of NPAdmin you are using is

1.55 or above.  If you have an earlier version of NPAdmin,

see the end of this document which explains how to get the

current NPAdmin from the Intel BBS.

NetPort configuration has not changed with the exception of the

following:

o You must assign NetPort to a specific file server.  If

you do not assign NetPort to a file server, NetPort goes into

attaching mode and does NOT connect as a print server.

NP2CON NPxxxxxx SERVERNAME="file_server_name"

where "file_server_name" is the name of the file server you want

NetPort to use.

NOTE: To remove a specified server name, type:

SERVERNAME=DEFAULT

o Use the MODE= option to set up one port as a print server,

and keep the other AVAILABLE (this is useful if you want

to use the available port as a virtual circuit).  You cannot set

up one port as a print server and the other as a remote printer.

Prior to this update, you could not split a NetPort's ports

in this way.  If you don't want to split the ports, you can

ignore this option.

NP2CON NPxxxxxx-X MODE=SERVER

where -X is -P for parallel port, or -S for serial

NOTE: to take NetPort out of server mode, type:

MODE=AVAILABLE

CONFIGURING NETPORT AS A REMOTE PRINTER

***NOTE: You can use NPAdmin, NetPort II's menu-driven interface,

to configure an original NetPort with this update version

2.53, as long as the version of NPAdmin you are using is

1.55 or above.  If you have an earlier version of NPAdmin,

see the end of this document which explains how to get the

current NPAdmin from the Intel BBS.

o You must assign NetPort to a specific Novell Pserver.  If you do not

assign NetPort to a Pserver, NetPort goes into attaching mode and does

not connect as a remote printer.

NP2CON NPxxxxxx SERVERNAME="Novell_Pserver_name"

where "Novell_Psever_name" is the name of the Pserver you

want NetPort to use.

NOTE: To remove a specified server name, type:

SERVERNAME=DEFAULT

o Use the MODE= option to set up one port as a remote

printer and keep the other as AVAILABLE (this is useful if you

want to use the available port as a virtual circuit).  You cannot

set up one port as a remote printer and the other as a print server.

Prior to this update you could not split the ports in this way.

If you don't want to split the ports, you can ignore this command.

NP2CON NPxxxxxx-X MODE=SERVER

where -X is -P for parallel port, or -S for serial

To take NetPort out of SERVER mode, type:

NP2CON NPxxxxxx-X MODE=AVAILABLE

NPADMIN

To get the latest version of NPAdmin, call Intel's BBS.

The BBS numbers are:

o From the USA             (503) 645-6275

o From everywhere else     +44-793-432955

The file will be called NPADUP.EXE and it will reside on the BBS under:

File Downloads/Network Products/Print and Fax Servers/NetPort Products/

Current Release Software.