*****************************************************************************

*                Pocket Ethernet Adapter LANtastic driver                   *

*****************************************************************************

A. Install Artisoft's LANtastic driver 4.x & 5.x

================================================

To install the Pocket Ethernet Adapter Artisoft's LANtastic driver,

please follow the instruction given below:

1. Insert the driver disk into the system's floppy drive.

2. On the network card selection menu, select any of the available network

card on the available list.

3. Follow LANTASTIC's software setup prodedure to complete software

installation.

4. When installation of LANTASTIC software is completed, then copy the

Pocket Adapter's LANtastic driver files onto the subdirectory where

the software located (default is C:\LANTASI ). In addtion, there are

two different driver files for different versions of LANTASTIC software

(4.x & 5.x) under two sub-directory of LANTASTIC directory in the

Pocket LAN Adapter's driver diskette.

for example, the LANTASTIC software is V4.0, then copy the driver

files under 4.x subdirectory in its driver diskette:

>COPY A:\LANTASTI\4.x\*.* C:\LANTASTI

5. Return to the DOS prompt and use a text editor to modify the

STARTNET.BAT file. The following is an example of a STARTNET.BAT

file.

@ECHO OFF

PATH C:\LANTASTI;%PATH%

ADDK                         <-------- Modify this line as "ADDK"

AILANDIO/MPX=C7

REDIR <MACHINE NAME> LOGINS=2

NET LPT TIMEOUT 10

6. If the driver is installed successfully, the ADDK.EXE driver will

produce the message below when STARTNET.BAT is executed:

" ADDK AI-LANBIOS Compatible Driver installed "

B. Install Artisoft's LANtastic driver 6.x

==========================================

1. Insert LANTASTIC diskette labeled "DISK 1 of 4" and run INSTALL.EXE.

2. Select prefer installation options as required.

3. When asked to select your network adapter to run with your LANTASTIC

software, select "NDIS Support for Network Adapter".

4. Insert the driver diskette in which contains the NDIS driver

directory(MSLANMAN.DOS) then select "OK".

5. Make sure your network adapter information is correct before you go on.

6. After the whole Installation is completed, reboot your PC.