******************************************************************************

*          Ethernet Pocket LAN Adapter Diagnostic program            100R00  *

******************************************************************************

Date: 08/15/1994

A. OVERVIEW

===========

The Diagnostic Program is used to verify the correct Ethernet

configuration and operation of an Pocket Ethernet LAN Adapter

before installing the network software.

B. BEFORE STARTING THE PROGRAM

==============================

1. Be sure you know the configuration of any adapters that use

jumpers to set the configuration.

2. After turning off the PC's power, insert the adapter into the PC.

** Ethernet adapters must be on a properly terminated cable

(use a BNC " T " connector with two 50 Ohm terminators or

an AUI loopback connector).

3. Boot the PC to a DOS prompt.

** Memory resident (TSR) programs and device drivers should

not be loaded while the Diagnostic Program is active, so

you may need to temporarily rename your CONFIG.SYS and

AUTOEXEC.BAT files.  The Diagnostic Program is always run

without loading any network software.

C. STARTING THE PROGRAM

=======================

1. To run the Diagnostic Program, change to directory with the

Diagnostic Program, type DIAG and press Enter.

2. Read the opening screen and press Enter.

3. The adapter configuration is displayed.

Base I/O        - found by searching all base I/O's

IRQ             - found by searching the hardware setting.

Network address - found by reading universal ID.

D. Ethernet Card Test

=====================

1. The Ethernet Card Test must be run to verify correct

operation.

2. Move the highlight bar using the Up/Down arrows

over the Ethernet Card Test option in the menu and press Enter.

3. This selection tests the basic adapter functions.  The test

takes less than a minute and displays OK upon successful

completion.

4. The test has failed if any of the tested item failed.

5. If the adapter fails, recheck the adapter settings, try a

different BNC "T" connector and terminator, or remove other

adapters which may be causing a conflict from the PC.

E. Transmit Receive Test

========================

1. The best test environment is two PCs directly cabled and

terminated, but this test can be run with any PCs on

a network.

2. These two tests must be run together on two different

machines, both of which must have passed the basic adapter

test.

3. In this example we will refer to COMPUTER A and COMPUTER B.

Start with COMPUTER A:

Move the highlight bar using the Up/Down arrows

to Transmit/Receive Test, and press Enter.

COMPUTER A is now ready to respond to test messages and is

now sending packets.

Go to COMPUTER B:

Move the highlight bar using the Up/Down arrows to

Transmit/Receive Test, and press Enter.

COMPUTER B is now sending and receiving packets from COMPUTER

A, the screen will display COMPUTER A's Ethernet address.

COMPUTER A should be receiving packets from COMPUTER B now.

To stop the continuous test press ESC key.

This concludes the Transmit/Receive test.

F. SUPPORT

==========

If you have any additional questions regarding your Diagnostic

Program, pleasecontact your dealer or distributor technical support

department.

======================================================================