****************************************************************************

*   Pocket Ethernet Adapter Series  ---  Driver information         V1.22  *

****************************************************************************

Date: 02/24/1995

Compatibility Report

====================

Pocket Ethernet LAN Adapters have been tested and found fully

compatible with the following NOS:

Novell NetWare V2.2

Novell NetWare V3.11

Novell NetWare V3.12

Novell NetWare V4.01

Novell Personal NetWare V1.0

Microsoft LAN Manager V2.0C & V2.1

Microsoft Windows For Workgroups V3.11

FTP software PC/TCP V2.05

ARTISOFT LANtastic V4.x, 5.x & V6.x.

EPP (Enhanced Parallel Port) Mode Installation

==============================================

To install the EPP.EXE, include a command line of the following

form in your CONFIG.SYS file:

device=[drive][path]EPP.EXE

for example:

device=C:\PE\epp.exe

Or install this driver under DOS prompt

for example:

A:\EPP  <Enter>

MUX (Multiplexer Printer Port) Driver Installation

==================================================

This driver is provided for users to print from local printer port, while

the pocket LAN adapter is attached but the network driver is not loaded,

To install the MUX.COM driver, type "MUX" under DOS prompt

for example:

A:\MUX  <Enter>

Note: This MUX driver has to be loaded before loading network driver.  Or