Date: April 22, 1992

Compatibility Report

--------------------

Our NE1000/NE2000 compatible Ethernet boards have been tested and found

fully compatible with following NOS:

Novell NetWare 286 V2.2

Novell NetWare 386 V3.11

Novell NetWare Lite V1.0

Artisoft Lantastic V4.0 & V4.1

Microsoft LAN Manager V2.0C & V2.1

FTP software PC/TCP V2.05

Driver Copyright Registered

----------------------------

We provide NDIS drivers for Microsoft LAN Manager and other NDIS compliant NOS.

Please read "README.1ST" file in this diskette for how to install our driver in LAN Manager.

Drivers Provided by NOS Vendors

-------------------------------

Use following drivers when you configure the server or station:

NetWare 286 V2.2

Server: Select "NetWare NE1000" for NE1000 compatible board

Select "NetWare NE2000" for NE2000 compatible board

Shell:  IPX/SPX Select "NetWare NE1000" for NE1000 compatible board

Select "NetWare NE2000" for NE2000 compatible board

DOS ODI Use "NE1000.COM" for NE1000 compatible board

Use "NE2000.COM" for NE2000 compatible board

* Pre-configured files from NOS vendor

NetWare 386 V3.11

Server: Load "NE1000" module for NE1000 compatible board

Load "NE2000" module for NE2000 compatible board

Shell:  IPX/SPX Select "NetWare NE1000" for NE1000 compatible board

Select "NetWare NE2000" for NE2000 compatible board

DOS ODI Use "NE1000.COM" for NE1000 compatible board

Use "NE2000.COM" for NE2000 compatible board

* Pre-configured files from NOS vendor

NetWare Lite V1.0

Select "Novell/Eagle NE1000"   for NE1000 compatible board

Select "Novell/Eagle NE2000"   for Ne2000 compatible board

Artisoft Lantastic V4.0 & V4.1

Select "Novell NE1000 8-bit ISA bus or compatible"

for NE1000 compatible board

Select "Novell NE2000 16-bit ISA bus or compatible"

for NE2000 compatible board

Microsoft LAN Manager V2.0C & V2.1

Select "Novell NE1000 adapter" for NE1000 compatible board

Select "Novell NE2000 adapter" for NE2000 compatible board

FTP PC/TCP V2.05

Use "NE1000.COM" for NE1000 compatible board

Use "NE2000.COM" for NE2000 compatible board

* Pre-configured files from NOS vendor