*************

* HPGL2.DOC *

*************

The HP-GL/2 Plotter Driver for the

Autodesk Device Interface(TM) Specification

For use with AutoCAD(R) Release 10 and Release 11

HPGL2.EXE Ver 1.11e for AutoCAD

Document Revision 1.1 (Nov 4, 1991)

Supported plotters include:

--------------------------

HP DesignJet Plotter

HP 7600 Series Electrostatic Plotters

Models 240D,240E,250,255,and 355

HP DraftMaster SX,RX,MX Series Plotters

HP 7550 Plus Plotter

HP PaintJet XL Printer with HP-GL/2 Language Cartridge

HP LaserJet III Printer

*********************

* TABLE OF CONTENTS *

*********************

I.    Introduction

II.   Driver Software Installation And Configuration Overview

(A) Copying The Driver Software

(B) Installing The HP-GL/2 Driver

(C) Configuring AutoCAD For The HP-GL/2 Plotter Driver

III.  Plotting On Standard Size Paper (Sizes A-E)

IV.   Plotting A Long Axis Plot (Up To 50 Ft)

V.    Line Weights

VI.   Multiple Copies Of The Plot

VII.  Stopping The Plot

VIII. Tips On Operation

IX.   Plotter Specific Information

Hardware Requirements

Serial Mode

HP DesignJet

HP 7600 Series of Monochrome Electrostatic Plotters

HP Draftmaster SX,RX,and MX Plotters

HP Color Electrostatic and HP PaintJet XL

HP LaserJet III In HP-GL/2 Mode

Default Pen Width Settings On HP LaserJet III

Multiple Plots On HP LaserJet III

X.    Using The -S Option With The HP-GL/2 Driver

Plotting To File With AutoSketch

XI.   Limitations of the HP-GL/2 Driver

*******************

* I. INTRODUCTION *

*******************

This document explains the installation and use of HPGL2.EXE, an Autodesk

Device Interface -- ADI(R) -- driver designed to run HP-GL/2 language plotters

(listed above) with AutoCAD and AutoSketch(R) .  You may want to print out a

hard copy of this document for reference.  Please read the section of the

AutoCAD Installation and Performance Guide concerning plotting to Hewlett-

Packard Plotters.  Also see the AutoCAD Reference Manual and your plotter's

manual.

This driver is a real mode ADI plotter driver.  ADI is an acronym for the

Autodesk Device Interface specification that was used to create this driver.

You can use this driver (HPGL2.EXE) with any version of AutoCAD that supports

ADI plotter drivers.  Please see the AutoCAD Installation and Performance

Guide, which shipped with your version of AutoCAD, to see if you have such a

driver option.  This driver was tested using AutoCAD Release 10 and Release

11.

***************************************************************

* II. DRIVER SOFTWARE INSTALLATION AND CONFIGURATION OVERVIEW *

***************************************************************

Your HP-GL/2 Driver Software disk is shipped with the following files:

HPGL2.EXE     The HP-GL/2 Driver program to be used with

AutoCAD.

HPGL2.DOC     This document.

HPGL2.TXT     Message file to be displayed on the screen when

the HPGL2.EXE is invoked by the user.

HPGL2.HLP     Help message file that is displayed by HPGL2.EXE

when invoked with the -? option.  This help file

shows the switch options that can be specified to

the driver.

The following steps should be taken to enable output of AutoCAD drawings to

your HP-GL/2 plotter:

(A)     Copy the HP-GL/2 driver software files from the diskette

to the AutoCAD directory on your hard disk.

(B)     Add a command to start up the HP-GL/2 driver either in

your autoexec.bat file or a custom start up batch file

you may be using to start up AutoCAD.

(C)     Configure AutoCAD to use the ADI plotter driver.

Each of these steps are discussed in detail below.

(A)  COPYING THE DRIVER SOFTWARE

================================

If you received your HP-GL/2 driver disc in the box with the plotter,

type INSTALL at the A: prompt.  The INSTALL batch file will prompt you

for a target directory and copy the driver files to this directory.

The remaining discussion in this portion (A) applies only if you

received your driver disc with your AutoCAD package.  Go to step (B).

You probably will want to install the driver in the same directory where you

store AutoCAD.  Here's how to copy the driver software onto your hard disk.

The example assumes an AutoCAD directory is on drive C and named \ACAD and

the distribution disk for the driver is in drive A:

1.  Change to the AutoCAD directory:

C:> CD ACAD [Return]

2.  Insert the ADI Driver diskette in drive A.

3.  Copy the driver files to your hard disk:

C:> COPY A:HPGL2.*  [Return]

(B)   INSTALLING THE HP-GL/2 DRIVER

===================================

The HP-GL/2 plotter driver for ADI must be installed (loaded into memory)

prior to entering AutoCAD.  It is a "terminate and stay resident" (TSR)

software program.  Once loaded, it remains in memory until you turn off or

reboot your computer.

THE IMPORTANCE OF COMMAND LINE SWITCHES

---------------------------------------

Your ADI driver does not have the ability to communicate with AutoCAD to

modify plot specifications. Thus, these can only be set at the time of driver

installation by using the command line switches (options) listed below. Think

through your plotting requirements prior to installing the HP-GL/2 driver as

it will remain in memory until you turn off or reboot your computer. Remember

that you must configure AutoCAD with the same options you select when

installing this driver.

If you plan to use the same command line options each time you use this

driver, you may want to create a small batch file to invoke the driver with

your selected options, or you may want to add this command line to your

autoexec.bat file.

To install the driver:

----------------------

C:> HPGL2 [options] [Return]

Invoking HPGL2 without options will start up the driver with the following

defaults:

- Direct output to LPT1

- Plot size in inches

- No additional copies per plot

- Page Advance is enabled

- Page Cutter is enabled (if your plotter supports one)

- Pen speed will be interpreted as a line weight index

- Install driver to respond to Interrupt vector 78 HEX

- Driver will prompt for LongPlot and X-axis length

To change this you can include one or more of the following options on

the command line:

HPGL2  -Lx [Return]  Where x is 2 or 3 for LPT2 or LPT3.

Specify a parallel port other than the default (LPT1).

HPGL2  -Cx [Return]  Where x is 1 or 2 (COM1 or COM2).

Specify a serial port instead of the default parallel port.

HPGL2  -M [Return]

Specify the plot sizes in millimeters.

Default plot sizes are in inches.

***  NOTE:  You must be sure to configure AutoCAD with the

same option you select here.

HPGL2  -Z [Return]

For an HP LaserJet III only.  Issues HP LaserJet III specific

commands that reset the printer.

HPGL2  -J [Return]

Specifies a PaintJet XL with the HP-GL/2 cartridge.

HPGL2  -A [Return]

For operation with a printer/plotter RAM buffer or an external

automatic data switch on the serial port.  It removes

checking for a timeout value when outputting to a serial port.

It should be used cautiously because the HP-GL/2 driver

program will loop constantly waiting for a ready signal

from the device.  It will appear as if AutoCAD is hung up if

the serial cable is disconnected or the device goes off line.

There will be no error messages or chance for the user to quit.

HPGL2  -R [Return]

Specify the number of additional copies you want of the plot.

(You will be asked how many additional copies you want at

plot time).  This option is not available if you use the -S,

AutoSketch option.

***  NOTE:  This feature is not supported on the HP LaserJet III.

Please see the Plotter Specific Information section for a full

discussion on generating multiple plots on an HP LaserJet III.

HPGL2  -P [Return]

Disable the page advance after the plot is completed. (See

the section of this document entitled "VIII. TIPS ON OPERATION"

for additional information.)

HPGL2  -E [Return]

Disable the automatic paper cutter (if your plotter has one).

We recommend this option not be used for those HP7600 electrostatic

models that provide a take up reel.

HPGL2  -V [Return]

Change from line weights to pen speed

(otherwise the pen speed value will be interpreted as a line

weight index).

*** IMPORTANT: If you use a supported pen plotter, be sure to

enable  this option. Otherwise unpredictable results may occur.

AutoCAD defaults to setting all pen speeds to 1, so you will

need to change AutoCAD's pen speed configuration if you pick

this option.

HPGL2  -Ixx [Return]  Where xx is the new interrupt number.

Install the driver for a different interrupt vector

(default is 78 HEX).

HPGL2  -Ffname [Return]

Specify the plot file name.  This will create a file named

'fname' on your default drive.

You will have the chance to rename the file before

plotting (unless you also choose the -S AutoSketch option).

If you decide to stop the plot by issuing a Control-C (^C)

at this prompt, you must hit RETURN to complete the action.

The file will then be closed and erased.

If you choose the -S option together with the -F option,

you will not be able to change the plot file name, nor will

you be able to use ^C to cancel the plot. You can rename

each plot file after it has been produced (to avoid writing

over a previous file with the same name) or you may reboot

and reload the driver, giving a different plot file name.

HPGL2  -S [Return]

Install the driver to work with AutoSketch or AutoCAD

scripts.  With this option, all prompts to the screen

will be removed and all plots will have an X axis of 45

inches.  You will not be asked longplot or page size

questions.  Longplots are disabled.  Script files can

run without any user input with this option.

*** CAUTION: Failure to do this when using AutoSketch will

cause your computer to hang up because AutoSketch does not

allow screen messages and keyboard input.

*** NOTE: For more information on the -S option, refer to

the section of this document entitled "X. USING -S WITH

THE HPGL2 DRIVER".

To install the driver using a combination of the above options:

HPGL2 -Fplotdata.plt -R -I7b -L2   This will do the following:

1.  Create a plot file 'plotdata.plt' of your drawing on

your default drive for plotting at a later time.

2.  Allows you to make additional copies of the plot.

3.  The interrupt vector will be 7b (in hex).

4.  AutoCAD expects the plotter to be connected to parallel

port 2 (LPT2).

*** CAUTION: Each time you type in the command "HPGL2" another copy of

the driver is loaded into memory, using up more RAM. You will need

to "reboot" the system (to clear out any copies of HPGL2.EXE). If

you want to change any of the installation options for the driver,

we suggest that you reset your system prior to configuring the

driver to save valuable memory for AutoCAD.

Reloading the driver without resetting the computer will use more

memory but will activate any new command line options (if issued).

Reloading the driver without resetting the computer is NOT

recommended.  You should exit all active programs and reset your

computer prior to changing any of the command line options for

this driver.

*** CAUTION: Do not reset your computer while AutoCAD is running! Exit

AutoCAD prior to resetting the driver.

After resetting your computer, call HPGL2 with any new command line options,

configure AutoCAD again if necessary and then plot as you would normally.

(C)  CONFIGURING AUTOCAD FOR THE HP-GL/2 PLOTTER DRIVER

=======================================================

It is necessary to configure AutoCAD for an ADI DRIVER.  You will need to do

this before you use the HP-GL/2 driver for the first time.  AutoCAD will

save these settings until you change them. If you change the configuration of

the HP-GL/2 driver, you may also need to change your AutoCAD

configuration. Refer to your AutoCAD Installation and Performance Guide for a

more in-depth explanation of the steps below.

To configure AutoCAD for an ADI plotter driver:

Select option 5 - Configure AutoCAD from the Main Menu.

Your current configuration will be displayed.  Press RETURN.

Select option 5 - Configure plotter from the Configuration Menu.

Select ADI driver from the list of displayed plotters.

DO NOT select ADI P386 Plotter.

The AutoCAD menu displays and prompts you to Select the driver's output

format.

Select output format 3, installed ADI driver.

AutoCAD then displays the following prompt:

Hexadecimal interrupt code (INT 0XXh) <78>:

Press RETURN

*** NOTE: If you have used the -I command line option, be sure that

you supply the same hex address you used when installing the driver.

AutoCAD next asks questions about the number of pens, line types and speed.

Use the table below to answer the questions pertaining to your printer or

plotter:

(A) HP DesignJet plotter

(B) HP 7600 electrostatic plotters (all models)

(C) HP Draftmaster SX, RX, and MX plotter (specify the -V option)

(D) HP 7550 Plus plotter (specify the -V option)

(E) HP PaintJet printer with HP-GL/2 cartridge

(F) HP LaserJet III printer

(A)   (B)   (C)   (D)   (E)   (F)

-----------------------------------------------------------

Multiple pens?             Y     Y     Y     Y     Y     Y

How many pens?            16    16     8     8    16    16

Multiple line styles?      Y     Y     Y     Y     Y     Y

How many line styles?      9     9     9     9     9     9

Variable speed?            Y     Y     Y     Y     Y     Y

Fastest speed?            20    20    60    60    20    20

-----------------------------------------------------------

Next a series of questions are asked about media sizes.  You must specify the

physical plotting area for AutoCAD - the driver will ask you again at plot

time. If you are doing a LongPlot, see the section of this document entitled

"IV. PLOTTING A LONG AXIS PLOT" for more information.

*** NOTE: 25.4 millimeters = 1 inch

Maximum Plot Sizes

---------------------------------------------------------------------

paper sizes:         A        legal        B        roll

MODEL                     X    Y      X    Y    X     Y    X    Y

---------------------------------------------------------------------

HP DesignJet                                            42.66" 33.60"

HP DesignJet                                              1083 853mm

HP7600 240D and 250                                        64" 23.10"

HP7600 240D and 250                                       1626 587mm

HP7600 240E, 255 and 355                                   64" 35.10"

HP7600 240E, 255 and 355                                  1626 892mm

HP Draftmaster SX,RX,MX                                    64" 33.60"

HP Draftmaster SX,RX,MX                                   1626 853mm

HP 7550 Plus                                  16.12"  9.97"

HP 7550 Plus                                   409mm  253mm

HP PaintJet XL w/ HPGL2  10.72 8.17"          16.60" 10.60"

HP PaintJet XL w/ HPGL2  272mm 207mm           421mm  269mm

HP LaserJet III          10.5" 8.00"  13.5" 8.00"

HP LaserJet III          266mm 203mm  343mm 203mm

----------------------------------------------------------------------

Specify plot size in millimeters? <?>  Answer Y or N as you wish.

Remember you have to configure the driver to agree with the answer

you give here!

Maximum horizontal (X) plot size in inches <#>:

Refer to the table above.  If you want to do LongPlots, you must

specify 64" (1626mm).

Plotter steps per inch in the horizontal (X) direction <#>:

If you are using inches, use 1016.

If you are using millimeters, use 40.

*** NOTE: You must configure AutoCAD so it agrees with this driver

regarding units of inches or millimeters!

Maximum vertical (Y) plot size in inches <#>:

Refer to the table above.

Plotter steps per inch in the vertical (Y) direction <#>:

If you are using inches, use 1016.

If you are using millimeters, use 40.

*** NOTE: You may enter larger page sizes, but due to the operation of

the paper handling mechanism, you may clip the plot in the X or the

Y dimension.  Experimentation may allow you a modest increase in both

directions.

Would you like to calibrate your plotter? <N>

Answer as you wish.

You will now go through the standard plotter configuration dialog. Be sure to

specify MAX paper size if you want to do LongPlots.

If you have any questions see the Configuration section of the Installation &

Performance Guide to AutoCAD.  Also see the AutoCAD Reference Manual for

AutoCAD details, and your plotter user's guide for plotter details.

AutoCAD will now be configured to plot using the installed ADI driver.  It can

do Long-Axis plots of up to fifty feet in length in addition to the normal

paper sizes.

****************************************************

* III. PLOTTING ON STANDARD SIZE PAPER (Sizes A-E) *

****************************************************

To plot on standard size paper, either issue the Plot command from the

drawing editor or select option 3 from the AutoCAD main menu.  You will be asked

two additional questions before the plot is sent off to the plotter.  These

questions can only be answered from the keyboard.  Make sure your plotter is

cabled correctly, is turned on, and is on-line.

*** NOTE:  If you have enabled the -S option for AutoSketch, you are limited

to a page size of 45 inches and will not be asked the following

configuration questions.

After AutoCAD's plot questions, you will be prompted by the HP-GL/2 driver to

answer the following questions:

LongPlot(L) or Standard sheet(S) <S>?

Enter "S".

X-Axis Length for Plot?

Answer with the length in inches appropriate for the plot size you want

AutoCAD to draw.  If you specified the -M command line option for metric

units, be sure to convert the length of the X-axis from inches to

millimeters.  To convert inches into millimeters, multiply the inches by

25.4.

*** NOTE: This step determines how far the paper on electrostatic or

HP DraftMasters with roll paper will advance. This will save a

considerable amount of paper if you do small-size plots. The answer you

enter has no effect unless your plotter has rollfeed paper loaded, but you

still must answer with a valid integer that is more than 0 and less than

600.

*************************************************

* IV. PLOTTING A LONG AXIS PLOT (UP TO 50 feet) *

*************************************************

*** NOTE:  Only roll-feed plotters can do LongPlots. Do not use these

instructions for creating plots less than 64".  You may LongPlot to a file

by using the -F command line option.

AutoCAD doesn't directly support long plots. AutoCAD can plot only 65,535

plotter steps in each direction, which is about 64.5 inches or 1638

millimeters.  Long plots are done by rescaling the plot data after it is sent

to the driver to magnify the plot at a small cost in plot resolution. Because

there is no two-way communication between this ADI plotter driver and

AutoCAD, the scaling process is a bit complicated.

From AutoCAD's drawing editor, issue the Plot command or select option 3 from

AutoCAD's main menu.  Specify the part of the drawing you wish to plot by

answering the prompt:

What to plot -- Display, Extents, Limits, View, or Window <D>:

AutoCAD displays the basic plot specifications and asks if you want to change

any of them:

Do you want to change anything? <N>

Answer "Y"

When you come to the question:

Enter the Size or Width, Height (in units) <default>:

Answer "MAX"

*** NOTE:  MAX width should have been set to 64 inches in the AutoCAD plotter

configuration dialogue.

Then, when you receive this prompt:

Specify scale by entering:

Plotted units=Drawing units or Fit or ? <default>:

*** IMPORTANT:  There are special plot scaling requirements for long plots!!

In order to create long axis plots, HPGL2.EXE has to magnify the plot sent

from AutoCAD.  This question (asked by AutoCAD) must be answered with a

scale factor which will keep the total number of plotter steps in each

direction below the 65,535 AutoCAD addressable limit (64.5 in x 1016 steps

per inch).

The formula to do this is as follows:

DX - is the size of the long axis of your drawing in AutoCAD drawing editor

units - for example 300 feet (3,600 inches).

DY - is the height of your drawing in AutoCAD drawing editor units - for

example 30 feet (360 inches).  This is not used to compute the actual

formula, instead it is a check for you to make sure that your desired

plotted scale will not clip the top of the drawing.

US - is your scale factor for the final drawing - for example 1"=1'.  In

this example it will produce a desired drawing 25 feet in length.

But, you will need to convert feet to inches - so in this example,

think of your scale as 1"=12".  The total plotted length of your

drawing cannot exceed 600 inches (50 feet).

M - is the additional amount of magnification to multiply your scale (US)

to get your drawing plotted to your scale and the desired plot length.

To determine the actual scale you will enter, refer to the chart below.

If your final drawing length is:        Your magnification (M) will be:

-----------------------------------------------------------------------

<=  64 inches                                       1

>  64 but <= 128 inches                            2

> 128 but <= 192 inches                            3

> 192 but <= 256 inches                            4

> 256 but <= 320 inches                            5

> 320 but <= 384 inches                            6

> 384 but <= 448 inches                            7

> 448 but <= 512 inches                            8

> 512 but <= 576 inches                            9

> 576 but <= 600 inches (max plot length)         10

As an example, the magnification value for a 300 inch plot is 5.  Applying the

formula 1 inch = US * M, when AutoCAD asks you:

Specify scale by entering:

Plotted units=Drawing units or Fit or ? <default>:

Answer with 1"=60"  (12 inches * 5)

AutoCAD will calculate and display a maximum plotting area based on the scale

factor you supplied.  This will be smaller in each direction than your actual

plot when your magnification is greater than one.  The plot will be larger by

the magnification M applied by the ADI driver.  In the example above (300 ft

x

30 ft), AutoCAD will display a maximum plot size of 60"x6".  Ignore this

message.  The true plot size will be controlled by the X-Axis length

selection.

If you correctly answer that prompt, your plot will really be 300 inches x 30

inches.

*** NOTE:  To calculate your Y-axis height, divide the total original inches

(360 inches in this example) by the scale factor you tell to AutoCAD

(60). Multiply by "M" (5) to get the thirty inches in the above example.

If the height exceeds the plotting area limits of your plotter you will

need to adjust your X-axis scale or accept a plot clipped at the top.

AutoCAD will prompt you to position paper.  Just press return.  The ADI

driver will prompt:

LongPlot(L) or Standard sheet(S) <S>?

Press the 'L' key (except when your magnification is one, then answer 'S').

It will then prompt:

X-Axis Length for Plot?

Enter the number of inches that you want the plotted length of the X-axis

to be.  In our example you would enter 300 (inches).  The ADI driver will

automatically scale your drawing to fit within the desired length. This

length will also control how far the plotter advances the paper at the end

of the plot.

*** NOTE:  You must enter a legal integer value here, otherwise the prompt

will be repeated. A legal value is numeric, greater than 0 and less than

600 inches or 15,240 millimeters (depending on your selection of inches

or millimeters).

If your plot is less than 64 inches, enter the value for the length of your

drawing - for example, if your plot is 17 inches long, enter 17, this will

save paper as your plot will only advance 17 inches.

The driver will figure the magnification factor, M, by which to scale plot

data by (assuming inches) figuring M = INT(X-Axis/64) + 1. In other words, it

figures M just as we did earlier. Then the driver scales up all plot data by

multiplying by M. The plot is magnified by this factor in both X and Y

directions. You may have noticed that any answer between 64 and 127 would

produce the same magnification (but would advance the paper a different

amount at the end of the plot).

*** NOTE:  Because the magnification that takes place is along the X-axis,

you may encounter two Y-axis situations.  The first, as in our example

above, will result in a Y-axis not completely filling the space available.

The second is that you will clip the top of your drawing if your Y-axis

exceeds the maximum plotting height available by your plotter. Refer to

the "Maximum Drawing Sizes" chart for your plotter to find your maximum

Y-axis height.

If you specified the -M command line option for metric units, be sure to

convert the length of the X-axis from inches into millimeters.  To convert

inches into millimeters, multiply the inches by 25.4.  Remember that AutoCAD

must be configured for metric units if the driver is configured for metric

units, and the plotter steps per millimeter (40 per mm) must be properly set

in AutoCAD.

If you installed the driver with the -R (replot) option you will be asked for

the number of additional copies you want.

The HP-GL/2 driver will send all the vectors to the plotter's buffer or

hard disk.  The plotter will automatically advance the page to plot each

frame.

*******************

* V. LINE WEIGHTS *

*******************

All of the devices supported by this driver are capable of plotting various

width lines. These widths are sometimes called "line weights".  AutoCAD has

no provision for assigning different line weight indexes to different pens, but

you can use the pen speed field instead since this driver defaults to mapping

the value in the pen speed field to the line weight index.  After you have

selected what you are going to plot, you will be asked:

Do you want to change any of the above parameters? <N>

Answer "Yes"

You will be shown a table of Entity Colors from 1 to 15 and the associated

Pen Numbers, Line Types and Pen Speeds.

Substitute for the Pen Speed value, the line weight numbers from the

following table:

Line Weight   Line Weight Widths

No.          (mm.)

---       -----------

1        .350 mm. (default)

2        .075

3        .100

4        .120

5        .187

6        .250

7        .300

8        .400

9        .500

10        .600

11        .700

12        .800

13        1.00

14        1.25

15        1.50

16        1.75

17        2.00

18        2.50

19        3.00

20        4.00

For example if you assign to Entity Color #4 (cyan) Pen Speed 9 from the

above table, all the color 4 entities drawn in AutoCAD will be plotted with

lines .5 millimeters wide.  At run time configuration when you make the pen

table assignments you would enter a pen speed of 9 to Layer/Color 4.  For

example:

Layer          Pen     Line     Pen

Color          No.     Type     Speed

4(cyan)         1        0       0         Pen Speed <0>:

Enter 9 and all entities of layer 4 will be plotter with a .5mm. line width.

Do not use line weights greater than .8 mm to draw text.  The results are not

satisfactory.

For faster throughput on pen plotters, use different width pens in the

carousel to get pen widths.  Otherwise, the plotter will multi-stroke each

vector.

***********************************

* VI. MULTIPLE COPIES OF THE PLOT *

***********************************

If you included -R on the command line when you installed the HP-GL/2 driver

you will see the prompt:

How many additional copies do you want? <0>

Enter the number of additional copies you want.

*** Note: Assure that you have sufficient paper to produce the plots

requested! Check the roll if you choose this option.  This option is

not supported on HP LaserJet III printers. See the detailed discussion in

the section of this document entitled "IX. PLOTTER SPECIFIC INFORMATION."

**************************

* VII. STOPPING THE PLOT *

**************************

If you want to stop the plot while AutoCAD is still sending vectors to the

plotter, abort it in the normal manner (Control C).  To cancel a plot at the

plotter, follow the instructions pertaining to your printer or plotter in the

table below.  In most cases, you can send a file containing ';BPPG;' to the

plotter.  On raster plotters, no print out will actually occur.

PRINTER or PLOTTER             INSTRUCTIONS

-------------------------------------------------------------------

HP DesignJet plotter           Cycle power on the plotter.

HP 7600 240D/E                 Press the Reset button on the

plotter's front panel.

HP 7600 (All other models)     a. Take plotter off line.

b. Press the Plot Management button.

c. Select Queuing Operations.

d. Select the plot.

e. Select Delete from Queue.

HP Draftmaster Series          a. Press the Cancel button on the

plotter's front panel.

b. Select "Yes" to confirm.

HP 7550 Plus plotter           a. Press the Cancel button on the

plotter's front panel.

b. Select "Yes" to confirm.

HP PaintJet XL printer         Hold down the On Line button until

all lights stop flashing.

HP LaserJet III printer        Press and hold the Reset button until

the Form Feed light goes out.

-------------------------------------------------------------------

If you wish to stop the plotter after AutoCAD has finished sending all the

vectors and while it is still plotting, follow the procedures outlined above

for your specific device.  Consult the plotters' User's manual for more

information.

***************************

* VIII. TIPS ON OPERATION *

***************************

Here are some tips for using the HP-GL/2 driver with AutoCAD:

1. You may wish to place a call to HPGL2.EXE in your AUTOEXEC.BAT

file.  This insures that you have loaded the driver before

entering your application.

2. The plotter must be connected, turned on, and on line before

you plot using the HP-GL/2 driver.

3. Do not turn the plotter on and off in the middle of a plotting

session.  An error message will be displayed if you try to use

the ADI driver while the plotter is turned off.

4. If you turn off the automatic page advance with the command

line option -P, you must manually activate the "PLOT" button

on the electrostatic plotters and the HP PaintJet XL printer.

Otherwise, the next plot from AutoCAD will cause the previous

plot to be erased.

5.  To send a plot file to the plotter, type the following at the

DOS command line:

copy 'filename' lpt(n):  - OR -  copy 'filename' com(n):

where filename is the name of the file you wish to plot and lpt(n)

or com(n) represents the port you have connected to your plotter.

If using the serial (com) port, you will first have to issue a MODE

statement to match the serial configuration of your plotter.

6. Disk Space Considerations: If you wish to plot to file (-f option)

please insure that you have sufficient disk space to do so. File

sizes are not dependent on the size of AutoCAD's DWG file, rather

they depend on the contents of that file.  CHROMA.DWG, while only

12,359 bytes as distributed with AutoCAD, expands to almost 240k

bytes when plotted to file.  This is due to the amount of fill the

plotter must do to plot this file.  Each solid filled object in the

drawing requires that the driver send pen movement information

sufficient to fill the object.  Simple line drawings will require

much less disk space.

7. Serial port may time out if your plotter is turned off, off line

or not connected.  You will get a message asking if you want to

quit, retry once, or retry constantly.  If you select constant

retries, error checking to the serial port will be disabled.

Please be cautious.

************************************

* IX. PLOTTER SPECIFIC INFORMATION *

************************************

HARDWARE REQUIREMENTS

=====================

The supported HP device must be connected to the computer with a standard

Centronics-type parallel cable or a serial cable.  The AutoCAD Installation &

Performance Guide section for Hewlett-Packard Plotters contains information

regarding pin connections, switch settings, serial and parallel cable

information for the HP7600 and HP Draftmaster plotters.

o A parallel card and cable as described in the  AutoCAD Installation

& Performance Guide is your best choice for good performance with raster

devices such as the HP DesignJet, electrostatic plotters, printers.

o A serial card and cable is required for plotting to the HP DraftMaster

series plotters. Serial mode  can be used with the raster devices but

plots will take significantly longer to send to the plotter. We do not

recommend serial communications for raster devices.

SERIAL MODE

===========

It is VERY IMPORTANT to use the correct RS-232 cabling for connecting

your PC to the plotter.  Contrary to the well-defined cabling for the

Centronics parallel connection, the RS-232 specification is often times

very loosely followed.  This HP-GL/2 driver MUST see RS-232 pins 2,3,7

4,5,6,20 active in order for serial communication to successfully occur.

Hewlett-Packard recommends the following RS-232 plotter cables for easy

connection of your HP plotter to your PC:

(part number 17255D)           (part number 24542G)

25-pin to 25-pin               25-pin  to  9-pin

PLOTTER     COMPUTER           PLOTTER       COMPUTER

1------------1                 4--------------1

2------------3                 2--------------2

3------------2                 3--------------3

7------------7                 5--+-----------4

20---------+--5                    |

5--+      |                    6--+

|      +--6                 7--------------5

6--+---------20               20-----------+--6

|

+--8

8--------------7

If you plan on making a minimum RS-232 cable, the AutoCAD Installation

and Performance Guide suggests a possible wiring diagram that can be

used to minimize the number of conductors necessary in your cable.

The diagram is displayed here for your reference.  Be aware this wiring

configuration will not provide true status information to the driver as

to whether the plotter is off-line, disconnected, or even powered off!

Printer end                      Computer end

2  ----------------------------- 3

3  ----------------------------- 2

4  --+                       +-- 4

|                       |

5  --+                       +-- 5

7  ----------------------------- 7

6  --+                       +-- 6

|                       |

20  --+                       +-- 20

The driver will configure the COM port to the following settings:

For HP DesignJet, HP 7600 Electrostatics, HP Draftmaster:

9600 baud, even parity, 7 data bits, and 1 stop bit.

For HP PaintJet XL and HP LaserJet III:

9600 baud, no parity, 8 data bits, and 1 stop bit.

HP DESIGNJET PLOTTER

====================

As a general rule, as you're looking at the paper in the plotter, the side

margins (Y axis) are .2 inches each (5mm).  The top and bottom margins are .67

inches each (17mm).  The following table shows the imageable area for

standard paper sizes.

Size      Inches            Millimeters

---------------------------------------

A      9.66 X 8.1           245 X 205

B     15.66 X 10.6          397 X 269

C     20.66 X 16.6          524 X 421

D     32.66 X 21.6          829 X 548

E     42.66 X 33.6         1083 X 853

The DesignJet will plot the AutoCAD drawing in greyscale if you had replied

Y to the multiple pens question during AutoCAD plotter configuration.  If

you answered N, all the AutoCAD colors default to pen #1.  Because the

AutoCAD color #1 is RED, the plotted resultant color is a dark grey.  If

you do not intend to plot greyscale, make sure you map all the AutoCAD

colors to pen number 7, solid black.

The HP-GL/2 language provides MERGE CONTROL.  MERGE CONTROL determines

whether vectors and polygons are to be plotted either in MERGE MODE

or in OVERWRITE MODE.  MERGE MODE tells the plotter to merge vectors and

polygons with previously plotted elements. The pen plotters, such as the

HP DraftMaster, can only plot in MERGE MODE due to its inherent pen

plotting technology. OVERWRITE MODE tells the plotter to plot vectors

and polygons to overlap previously plotted elements where they intersect.

This is the typical mode used with raster display devices such as VGA.

All the HP raster plotters support OVERWRITE MODE since it provides the

closest match to the drawing as displayed on the screen. Some of the HP

rastor plotters, such as the HP 7600 series, allow the user to select

either mode via the front panel.

The driver currently does not specify the MERGE CONTROL plotting mode

when plotting to the plotter.  After plotting to file, you can edit the plot

commands in the plot file to turn MERGE MODE ON by inserting the command,

MC1, immediately after the BPIN command in the first part of the plot file.

HP 7600 SERIES OF MONOCHROME ELECTROSTATIC PLOTTERS

===================================================

This includes the models 240D, 240E, 250 and 255.

If you want all solid black lines, assign all pen colors to black (pen 7).

Otherwise, the 250 and 255 models will print the non-black colors in different

shades of gray.  The 240D and 240E will print all colors as solid black.

HP DRAFTMASTER SX, RX, AND MX PLOTTERS

======================================

The following table details the actual physical plotting areas for the cut-

sheet media sizes supported by the HP DraftMaster plotters:

Media     Plotting Width     Plotting Height        Media Orientation

Size     Normal  Expanded    Normal  Expanded    (as loaded into plotter)

-------------------------------------------------------------------------

A       8.83"    9.66"      7.3"     8.1"             Portrait

B      14.83"   15.66"      9.8"    10.6"             Portrait

C      14.83"   15.66"     20.8"    21.6"             Landscape

D      31.83"   32.66"     20.8"    21.6"             Portrait

E      41.83"   42.66"     32.8"    33.6"             Portrait

-------------------------------------------------------------------------

Drawings created to plot on an HP DraftMaster should keep these physical

limits in mind when they are initially created.  If plotting in expanded mode,

lines drawn at the edge of the plot will be under the pinch rollers and could

be subject to smearing.

HP COLOR ELECTROSTATIC AND HP PAINTJET XL

=========================================

Assign pen numbers to their equivalent entity numbers to get a close match to

the colors on the screen.

HP LASERJET III IN HP-GL/2 MODE

===============================

Please use the -Z command line option so the HP LaserJet III will reset after

each plot.  In addition, make sure that page protection is turned on.

HP LaserJet III's must have at least two megabytes of memory to work with the

HP-GL/2 driver.

You can also set up the HP LaserJet III to plot legal size pages (7.50 x 13.5)

by setting the page size from the printer's front panel.  To do so follow

these steps:

1) Turn the printer on if needed.

2) Take the printer off line.

3) Push the MENU button on the front panel until you see PAPER=LETTER.

4) Press the + button. The panel should say PAPER=LEGAL.

5) Press the ENTER button to confirm the change.

6) Cycle through the menu items until you see MANUAL FEED=OFF.

7) Press the + button to change OFF to ON.

8) Press the ENTER button to confirm the change.

9) Press the MENU button until you see READY.

10) Put the printer back on line.

You must manually feed the legal size sheets to the printer.

Default Pen Width Settings On HP LaserJet III

---------------------------------------------

Due to the finer resolution of the HP LaserJet III, you must configure the pen

width settings or you will end up with "streaks" in filled areas and/or broken

lines.  For testing purposes, any large filled area should show some streaking

at the default pen width of .010 .  Start with a pen width of .009 and reduce

the width by .001 until the plots are acceptable.

Multiple Plots On HP LaserJet III

---------------------------------

Because the HP LaserJet III does not support the RP (replot) HP- GL/2 command,

this command line option is ignored by the HP LaserJet III.  To generate

multiple plots of the same plot, you must do the following:

1. Press the ON-LINE button to take the printer off-line.

2. Press the menu button until COPIES appears in the menu.

3. Press the "+" key until the number of copies you want appears in the menu.

4. Press the ENTER key to store the configuration.

5. Press the ON-LINE button to put the printer back ON-LINE.

Plot as you would normally. The printer will duplicate the plot the number of

times you requested.

You must reset the number of copies to 0 by following the above procedure but

using the "-" key instead of the "+" key to set the number of copies to 0.

**************************************************

* X. USING THE -S OPTION WITH THE HP-GL/2 DRIVER *

**************************************************

This driver normally asks you questions and accepts answers from the

keyboard. It is impossible for a driver to do this when working with AutoCAD

scripts. The -S option suppresses all of this driver's questions and answers.

Just remember to use the -S flag on the control line.  This option was only

tested for the HP LaserJet III and HP PaintJet XL.  If used with a roll feed

device it will always advance the paper 45".  You cannot do long axis plots with

this command line option. With this option all printing to the screen is

suppressed, therefore no error messages will be sent.  Make sure your device

is connected, on-line, and filled with paper.

This driver normally asks you questions and accepts answers from the keyboard.

It is impossible for a driver to do this when working with AutoSketch. The -S

option suppresses all of this driver's questions and answers.

The ADI driver may be used with AutoSketch.  Just remember to use the

-S flag on the control line.  Failure to do so will hang up your system,

requiring a re-boot.

Before every plot from AutoSketch using this ADI driver, you must make sure

that the plotter is turned on, connected and on line.  Failure to do this

will return you to the drawing screen without any error messages.  If the

plotter goes off-line, runs out of paper or becomes disconnected at any time

during the plot, no more data will be sent to the plotter and you will be

returned to the drawing screen again without any error messages.  Therefore if

your drawing is not being plotted at all or stops plotting and you are returned

to the drawing editor, make sure the plotter has paper, is turned on, connected

and on line.

AutoSketch must be configured again so that it knows you will be using an ADI

driver to do plots.  To initiate AutoSketch's configuration type:

C:>SKETCH /R [Return]

or if you're installing AutoSketch for the first time type:

C:>SKETCH [Return]

Configure AutoSketch's plotter as a Autodesk Device Interface Plotter <4>.

AutoSketch will then prompt you to give it more information regarding this

ADI driver.  See the previous section titled 'CONFIGURING AUTOCAD FOR THE

HP-GL/2 PLOTTER DRIVER and Appendix B: the ADI Plotter section in your

AutoSketch User's Guide.

AutoSketch prompts you to indicate how your plotter is connected to

your computer, choose:

2. Parallel port, system printer, or hardware handshake serial port.

Then follow the instructions in Configuring AutoSketch in Chapter 2 of the

AutoSketch Users Guide.

*** Note:  When you open the plot area dialogue box, the values for x and y

may be incorrect.  Set them to the size of the paper you have in your

printer.  Rotating the plot may also work.

Plotting to file with AutoSketch

================================

Due to the way AutoSketch handles its drawing screen, the HP-GL/2 driver will

not output any questions to the user if it is configured with the -S (Sketch)

option.  If you plan to plot to file, you'll have to follow the instructions

in this document for the -f option and ignore the plot to file documentation

found in the AutoSketch manual.  This will allow you to plot to a single file

name given at the time you install the driver.  You must reset your computer

and reload the driver to invoke a different file name, but it may be easier to

exit AutoSketch as you would normally (by either saving or discarding the

changes to your drawing), and renaming the plot file.

*****************************************

* XI. LIMITATIONS OF THE HP-GL/2 DRIVER *

*****************************************

Real mode ADI plotter drivers, such as HPGL2.EXE have limited communication

with AutoCAD, compared to internal or protected mode drivers.  Due to this,

real mode ADI drivers lack some of the features of internal drivers or

protected mode ADI drivers.

For example, there is no provision for AutoCAD to tell a real mode ADI driver

about paper sizes or units of measurement (inches vs millimeters).  There is

no way for a real mode ADI driver to inform AutoCAD of its line types, changes

in units of measurement, scaling or paper size.  Thus it is sometimes

necessary for you to answer similar questions, once for AutoCAD and once for

the real mode ADI driver.

Real mode ADI drivers have no way of storing your configuration choices in

AutoCAD's configuration or drawing files. This driver is primarily configured

by command line options. Your choices for these options persist until you

reboot the computer or reload the driver with new options.  Reloading a real

mode ADI driver without rebooting your computer will waste memory because the

driver is loaded twice. Reloading the driver with different command line

options will, in some cases, require you to also configure AutoCAD so that

AutoCAD and the driver agree on the options you have selected.

*** NOTE:  Because of the way ADI drivers function, error trapping and user

prompts will not function in a familiar way.

In this driver, SPACE BAR acts as a space bar, not as a command line

terminator, all user input prompts are terminated by a CARRIAGE RETURN, and

CONTROL-C may have to be repeated more than once. ESC may have to be repeated

more than once.  Please be patient, most requests for termination will be

accepted and acted upon.

Protected mode ADI drivers do not have these limitations. Please contact your

HP representative if you wish to obtain more information on a protected mode

ADI HP-GL/2 driver.

AutoCAD, AutoSketch and ADI are registered trademarks of Autodesk,Inc.

Autodesk Device Interface is a trademark of Autodesk,Inc.

!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!

!                                                                          !

!                  Copyright 1991, Hewlett-Packard Co.                     !

!               Copyright 1986,87,88,89,90, Autodesk, Inc.                 !

!                                                                          !

!  License is granted for the use of this software and documentation on    !

!  a no-cost, sharing basis.  You may distribute and copy this software    !

!  freely as long as it remains a completely unaltered package and is      !

!  not sold for profit.                                                    !

!                                                                          !

!  These materials are provided with no warranty, either expressed or      !

!  implied, including but not limited to any implied warranties of         !

!  merchantability or fitness for a particular purpose, regarding these    !

!  materials and are made available on an "as-is" basis.                   !

!                                                                          !

!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!