**************

PLPHPGL2.DOC

**************

Protected mode ADI LongPlot driver for

Hewlett-Packard HP-GL/2 Devices

PLPHPGL2.EXP for AutoCAD 386 (R)

Copyright 1990-91, Autodesk, Inc.

Document Revision 1.00

Supported plotters include:

Hewlett-Packard DesignJet

Hewlett-Packard 7600 Series Electrostatic Models

Hewlett-Packard DraftMaster SX/RX/MX Series

Hewlett-Packard PaintJet XL with HP-GL/2 Language Cartridge

Hewlett-Packard LaserJet III

(Other Hewlett-Packard HP-GL/2 devices should plot using this driver)

*******************

TABLE OF CONTENTS

*******************

I.	INTRODUCTION

II.	DRIVER SOFTWARE INSTALLATION AND CONFIGURATION

(A) Copying The Driver Software

(B) Making the Driver Known to AutoCAD 386

(C) Configuring AutoCAD 386 For the Plotter Driver

III.	PLOTTING ON STANDARD SIZE PAPER (Sizes A-E)

IV.	PLOTTING A LONG AXIS PLOT

V.	LINE WEIGHTS

VI.     STOPPING THE PLOT

VII.    PLOTTING TO FILE

VIII.   PLOTTING ON NOVELL NETWORK

IX.     PEN OPTIMIZATION FOR RASTER PLOTTERS

X.      COLOR PALETTE AND MERGE MODE

XI.     PLOTTER SPECIFIC INFORMATION

Hardware Requirements

Hardware Installation

Serial Mode

HP DesignJet Plotter

HP 7600 Electrostatic Plotters

HP DraftMaster Plotters

HP PaintJet XL In HP-GL/2 Mode

HP LaserJet III In HP-GL/2 Mode

Page Protection

Fills

Legal Paper Size

Serial Configuration

Wait For Plot to Complete

*****************

I. INTRODUCTION

*****************

This document explains the installation and use of PLPHPGL2.EXP, a

protected mode Autodesk Device Interface (ADI) driver designed to run

HP-GL/2 language devices (listed above) with AutoCAD 386.  You may want

to print a hardcopy of this document file for reference.

Please read the AutoCAD 386 Installation and Performance Guide concerning

plotting and printer plotting with Hewlett-Packard plotters and printers,

the AutoCAD Reference Manual section on plotting, and your plotter manual.

This driver is a protected mode ADI (P386 ADI) plotter driver.  ADI is an

acronym for the Autodesk Device Interface specification that was used to

create this driver.  This driver was tested using AutoCAD 386 Release 11

and AutoCAD 386 Release 10.  If you are using AutoCAD Release 10 or

AutoCAD 286 Release 11, you should use the ADI 4.0 real mode driver

HPGL2.EXE and its accompanying document file HPGL2.DOC.

****************************************************

II. DRIVER SOFTWARE INSTALLATION AND CONFIGURATION

****************************************************

Your HP-GL/2 Driver Software Disk is shipped with both real mode and

protected mode drivers.  The two files that are important to you are:

PLPHPGL2.EXP		The protected-mode driver routine to be used

with AutoCAD 386.

PLPHPGL2.DOC		This installation and usage document.

The following steps should be taken to enable plot output of AutoCAD

drawings to your HP-GL/2 plotter:

(A)	Copy the HP-GL/2 driver software files from the diskette to

the AutoCAD directory on your hard disk.

(B)	Setting the environment variable PLPADI to the pathname

of your driver; either in your AUTOEXEC.BAT file or a

custom start-up batch file you may already be using to

start up AutoCAD 386.

(C)	Configure AutoCAD 386 to use the P386 ADI driver.

Each of these steps are discussed in detail below:

(A) COPYING THE DRIVER SOFTWARE

===============================

Here's how to copy the driver software onto your hard disk.  The flexible

disc drive is assumed to be A:.

1.  Insert the HP-GL/2 Driver Disc into your flexible disc drive .

2.  Type A: to make your active drive become the flexible disc drive.

3.  If you received your HP-GL/2 Driver Disc with your HP Plotter,

Type INSTALL at the A:> prompt.  Now refer to step 4.

If you received your driver disc from your purchase of AutoCAD 386,

copy the two files PLPHPGL2.EXP and PLPHPGL2.DOC from your driver

disc onto your hard disc directory.  Now refer to the next section

"(B) MAKING THE DRIVER KNOWN TO AUTOCAD 386".

4.  An itemized menu will be displayed to you as follows:

1. English

2. Francais

3. Deutsch

4. Espanol

5. Italiano

Entering one of the above numbers will cause INSTALL to copy

the selected language version of the driver files to your hard

disc. Enter the number of your desired language.

5.  You will be prompted (in your chosen language) to enter the

target disc drive and directory to store your driver files.

Example:   D:\ACAD386\DRIVERS

(B) MAKING THE DRIVER KNOWN TO AUTOCAD 386

==========================================

The preferred means for AutoCAD 386 to use this ADI driver are:

1) Use the SET command with the environment variable PLPADI and a complete

path name, for example if plphpgl2.exp is in the \acad\device directory

you would type:

SET PLPADI=\ACAD\DEVICE\PLPHPGL2.EXP

You may want to add a similar line to your autoexec.bat file.

** OR **

2)  Copy plphpgl2.exp to the file name adiplot.exp.  This file name is

unique and will be used when AutoCAD 386 is loaded if you have selected a

P386 ADI driver during configuration.

COPY PLPHPGL2.EXP ADIPLOT.EXP

Both of these methods require the .exp file to be located in a directory

accessible to AutoCAD 386.

In AutoCAD 386 Release 10 and Release 11, only a single ADI driver will

be recognized at configuration time.  The configuration menu for the

plotter devices will always display a P386 ADI driver as a choice.

The driver that is selected is determined through the use of the

PLPADI environment variable and/or the ADIPLOT.EXP magic file name.

If the user picks the P386 ADI driver choice, AutoCAD 386 will search,

first for the PLPADI environment variable which names the driver (with

either a fully qualified path name or just a file name).  Should this

method fail,  AutoCAD 386 will then attempt to access the ADIPLOT.EXP

magic file name.  Both searches start in the current default directory.

If the driver is not found, AutoCAD then searches the following

directories for the filename in the order given:  ACADCFG configuration,

ACAD execution, and AutoCAD command line execution prefix.  If the driver

is still not found, AutoCAD 386 will ask you which disk drive and directory

it should search to find the driver.

(C) CONFIGURING AUTOCAD 386 FOR THE PLOTTER DRIVER

==================================================

It is necessary to configure AutoCAD 386 for a P386 ADI DRIVER

(protected mode) before you use the PLPHPGL2 ADI driver for the first time.

AutoCAD will save these settings until you change them.  Refer to your

AutoCAD Installation and Performance Guide for a more in-depth

explanation of the steps below.

----------------------------

To configure AutoCAD for an P386 ADI plotter driver, select from the

AutoCAD Main Menu:

"Option 5 - Configure AutoCAD"

Your current display, plotter, digitizer configurations will be displayed.

Press <ENTER>.

The Configuration Menu will be displayed.

----------------------------

Select from the Configuration Menu:

"Option 5 - Configure plotter"

You will see the following prompt:

"Your current plotter is: xxx

Do you want to select a different one? <N>"

If xxx is already the 'ADI P386 plotter', press <ENTER>.

Otherwise, type     Y <ENTER>.

Select 'ADI P386 plotter' from the list of displayed plotters.

You will be asked to select the type of Hewlett-Packard plotter you have.

1. DesignJet

2. LaserJet III

3. PaintJet XL w/ HPGL2 Cartridge

4. DraftMaster SX/RX/MX Series

5. DraftMaster Series Roll Feed

6. 7600 Monochrome

7. 7600 Color

Here is a table of the various capabilities of the Hewlett-Packard

HP-GL/2 devices:

|          |  LINE    |          |  LONG    |

|  COLOR   |  WIDTHS  |  COPIES  |  PLOTS   |

------------+----------+----------+----------+----------+

DesignJet   |  Grey(1) |  Y       |  Y       |  Y       |

LaserJet    |  N       |  Y       |  Y       |  N       |

PaintJet    |  Y       |  Y       |  Y       |  N       |

DraftMaster |  Y(2)    |  N(3)    |  Y       |  Y       |

7600 Mono   |  Grey(4) |  Y       |  Y       |  Y       |

7600 Color  |  Y       |  Y       |  Y       |  Y       |

7550 Plus(5)|  Y       |  Y       |  Y       |  N       |

(1) The DesignJet will plot the AutoCAD drawing in greyscale if you select

(7. 7600 Color) as the plotter.  If you select item (1. DesignJet),

The DesignJet will plot using only solid black.

(2) The DraftMaster pen carousel can be loaded with colored pens.

(3) The DraftMaster supports PEN WIDTHS by stroking parallel lines.

The driver has chosen to support the ADI NEWSPEED packet for

changing the pen speed for the DraftMaster rather than

reinterpreting the packet as a PENWIDTH packet.

(4) The HP7600 Model 350 can plot in greyscale if you select item

(7. 7600 Color)

(5) The 7550 Plus can be configured as (7. 7600 Color).  Make sure

you do not select plot sizes greater than size B or size A3.

Doing so will cause severe hardware plot clipping.

----------------------------

All these devices can make extra copies of your plot. You will be asked:

" After the first plot, this device can print up to 99 additional copies.

Select the number of copies of each plot. <1>"

(You should enter the TOTAL number of copies you want)

"If you like, you can override the number you entered before each plot.

Would you like to do this? <N>"

If you answer Y(es), you can change the number of plots at plotting time.

You will then be asked the number of plots you want each time you plot.

Enter the new number you need and HP-GL/2 device will make that many

additional copies.

For roll feed plotters, insure that you have sufficient paper to produce

the plots requested! Check the roll if you choose this option.

For sheet feed plotters, you will be prompted at the plotter front panel

to load the new sheet before each additional plot.

----------------------------

Next a series of questions are asked about media sizes.  You must specify

the physical plotting area for AutoCAD. If you are doing a LongPlot, see

the section of this document entitled "IV. PLOTTING A LONG AXIS PLOT" for

more information.

( 25.4 millimeters = 1 inch )

Maximum Standard Plot Sizes

---------------------------------------------------------------------

paper sizes:         A        legal        B        roll

MODEL                     X    Y      X    Y    X     Y    X    Y

---------------------------------------------------------------------

HP DesignJet                                               64" 33.60"

HP DesignJet                                              1083 853mm

HP7600 240D and 250                                        64" 23.10"

HP7600 240D and 250                                       1626 587mm

HP7600 240E, 255 and 355(*)                                64" 35.10"

HP7600 240E, 255 and 355                                  1626 892mm

HP Draftmaster SX,RX,MX                                    64" 33.60"

HP Draftmaster SX,RX,MX                                   1626 853mm

HP 7550 Plus                                  16.12"  9.97"

HP 7550 Plus                                   409mm  253mm

HP PaintJet XL w/ HPGL2  10.72 8.17"          16.60" 10.60"

HP PaintJet XL w/ HPGL2  272mm 207mm           421mm  269mm

HP LaserJet III          10.5" 8.00"  13.5" 8.00"

HP LaserJet III          266mm 203mm  343mm 203mm

----------------------------------------------------------------------

(*) The HP7600/355 max Y is 34" (864mm) when plotting with color.

----------------------------

If you have specified a device that has a roll feed capability,

the driver can do long axis plotting (plots greater than 65", 1651 mm).

You will be asked the following question:

(For HP DraftMaster RX/MX series:)

"This device can do long axis plots from 65" to 1800" long."

(For other roll fed series:)

"This device can do long axis plots from 65" to 600" long."

If you wish to produce 64" or longer plots, an additional question

will be asked at run time configuration:

"Do you wish to do long axis plotting? <N>."  Answer Y or N.

If you answer Y(es), you can do plots of up to 50' (150' on the HP

DraftMaster Series).  Otherwise you will only be able to do plots up to

64" long.  You may reconfigure at any time and select long axis

plotting.  When you are ready to make a long axis plot, please refer to

the section "PLOTTING A LONG AXIS PLOT" later on in this document.

----------------------------

For all the devices except the DraftMaster series, the following question

will appear.

"Would you like to re-set line weight #20 to your own line weight? <N>"

Here you will be given the chance to change line weight no. 20 to

any value, but first you need to answer yes to the above question.

Hit the Y key.  AutoCAD 386 will then show the message below.

"Enter the line weight at the end of this configuration."

You will now go through the standard plotter configuration dialog.

Finally, if you answered Yes to the earlier question,

"Would you like to reset line weight #20 to your own line weight.",

AutoCAD 386 prompts:

"What thickness should line weight #20 be? <4.00>"

The following table shows the default line weights.

Line Weight  Line Weight            Line Weight  Line Weight

Index     Widths(mm.)               Index     Widths(mm.)

-----------  -----------            -----------  -----------

1          .350   (default)         11         .700

2          .075                     12         .800

3          .100                     13        1.00

4          .120                     14        1.25

5          .187                     15        1.50

6          .250                     16        1.75

7          .300                     17        2.00

8          .400                     18        2.50

9          .500                     19        3.00

10          .600                     20        4.00

Enter a line weight less than 25.0 mm.  The weight will be the thickness

of the line assigned to pen speed 20.  Don't forget the decimal point!

The default thickness for line weight #20 is 4.0mm.  When you do plot time

configuration, any AutoCAD color/layer that is given a pen speed of 20

will produce lines the thickness you entered for that color/layer.

See the LINE WEIGHT section (section V) for a complete discussion of how to

produce different line widths on your raster device.

----------------------------

AutoCAD 386 then asks:

"Is your plotter connected to a <S>erial, or <P>arallel port?<P>"

Type in S or P depending on where you have connected your HP-GL/2 device

onto your computer.  If you are using a plotter on a Novell network, type P.

Please refer to the section "PLOTTING ON NOVELL NETWORK" for more details.

----------------------------

AutoCAD 386 will display:

"If you have previously measured the lengths of a horizontal

and a vertical line that were plotted to a specific scale,

you may use these measurements to calibrate your plotter.

Would you like to calibrate your plotter?"

Hewlett-Packard plotting devices are designed to perform self calibration.

You should not have to perform the software-only calibration that is

offered here by AutoCAD 386.  Answer N.

----------------------------

AutoCAD 386 then asks:

"Write the plot to a file? <N>"

Responding N will cause AutoCAD 386 to send all plots to the port you

have identified earlier.

Responding Y will cause AutoCAD 386 at plot time to prompt for a

plotfile name.  Please refer to the section "PLOTTING TO FILE" for more

details.

----------------------------

AutoCAD 386 then prompts:

"Size Units (Inches or Millimeters) <I>:"

Respond with your desired size unit.  All plot specifications will

subsequently be in the size units you have selected.  However, plot

lengths for long axis plots must be entered in inches.

----------------------------

AutoCAD 386 then prompts:

"Rotate plot 0/90/180/270 <0>:"

Always respond with 0.  The three other choices will cause AutoCAD 386

to rotate the (x,y) coordinates the specified angle.  However, the final

plot will appear clipped since AutoCAD 386 does not inform the driver of

the picture rotation.

----------------------------

AutoCAD 386 then prompts:

"Pen width in inches <0.010>:"

Do not confuse this question with the line weights previously discussed.

This value is used by AutoCAD 386 to determine the distance between parallel

hatching lines used for polygon fills.  It has no effect on the line weight

you assign to a given AutoCAD color.

AutoCAD 386 then prompts:

"Adjust area fill boundaries for pen width? <N>"

Responding Y will enable AutoCAD 386 to take into account the pen width

you provided in the previous question when AutoCAD 386 does polygon fills.

----------------------------

AutoCAD 386 then prompts:

"Remove hidden lines? <N>"

Respond as you desire.  This is strictly an AutoCAD 386 capability not

affected by the driver.

----------------------------

AutoCAD 386 then prompts:

"Specify scale by entering:

Plotted (Inches/Millimeters)=Drawing Units or Fit or ? <F>:"

Respond as you desire.  This is an AutoCAD 386 option not affected by

the driver.  Please refer to your AutoCAD 386 Reference Manual for

more information.

----------------------------

You have completed the AutoCAD 386 plotter configuration dialog for your

HP-GL/2 ADI P386 plotter driver. If you have any configuration questions

see the Configuration section of the Installation & Performance Guide

in your AutoCAD 386 package.  Also see the AutoCAD 386 Reference Manual

for AutoCAD usage details, and your plotter's User guide for plotter details.

The remaining sections of this file discusses various plotting issues

when you are using this driver with AutoCAD 386.

****************************************************

III. PLOTTING ON A STANDARD SIZE PAPER (Sizes A-E)

****************************************************

To plot on standard paper, either issue the Plot command from the

drawing editor or select the "plot file option, item 3" from AutoCAD's

main menu. Before you press ENTER to AutoCAD 386's prompt,

"Do hardware setup now."  "Press RETURN to continue:"

Make sure: Your plotter is cabled correctly,

is turned on,

is On-Line,

and AutoCAD 386 is configured to do standard size plotting.

*******************************

IV. PLOTTING A LONG AXIS PLOT

*******************************

To make a long axis plot, you must have answered yes to the long-axis

plotting question asked during the plotter configuration dialogue.

AutoCAD does not directly support Longplots. AutoCAD can plot only

65,535 plotter steps in each direction, which is about 64" for Hewlett

Packard plotters which use 1016 steps per inch.  Longplots are done by

rescaling the plot data sent to the driver, magnifying the data sent to

the plotter at a small cost in plot resolution.

If you are using the HP Draftmaster for a LongPlot, Please refer to the

Plotter Specific Information - HP Draftmaster section for specific media

information for LongPlots.

From AutoCAD's drawing editor, issue the plot command or select the

plot option (#3) from the AutoCAD 386 main menu.

Specify the part of the drawing you wish to plot by answering the prompt:

"What to plot -- Display, Extents, limits, View, or Window <D>:"

AutoCAD 386 displays the basic plot specifications and asks if you want

to change any of them:

"Do you want to change anything? <N>"  -- Answer <Y>es

The following prompt will appear with the currently selected X-axis

length.

"X-axis paper length for this long plot? (Inches only) <XX>  "

If you want a different value than XX", enter it at this time.  Only

integer inches can be used here.

You must enter a legal value here, otherwise the prompt will be repeated.

A legal value is numeric, greater than 0 and less than 600" (1800" on

the HP DraftMaster series).  Do this here because it would be too late

if you waited for the run time configuration to be completed.

When you come to the question below you MUST enter the x-axis paper

length for long plotting you entered  earlier as the first value.

"Enter the Size or Width, Height (in Inches) <default>:"

You must change the width here to what you entered above, because

the width displayed is used for calculation by AutoCAD.

The Height you enter here should match the size of the paper roll

mounted in your plotter.  It will be 35" or less.

For example, you have a 500' roll of 35" wide paper in your plotter.  You

want a long plot 300 inches long.  You will enter the plot length of 300"

twice:  once at the question of x-axis page length and secondly at the size

question -- Enter the Size or Width, Height (in Inches) < >.  Here

300",35" would be correct.

Then, when you come to this question:

"Specify scale by entering:

Plotted units=Drawing units or Fit or ? <default>:"

IMPORTANT!! - SPECIAL PLOT SCALING REQUIREMENTS FOR LONG PLOTS

In order to create long-plots, the driver has to magnify the plot

sent from AutoCAD. This question (asked by AutoCAD) must be answered

with a scale factor to get accurate long plots, otherwise the resulting

plot is may be incorrect.

Consider a relatively simple situation where you want the plotted

size to be the same as the AutoCAD drawing editor size you selected.

Assume you would want to plot a scale of 1:1 if AutoCAD were able to

create long plots directly.

You have a drawing that is 100" long and you want a 100" plot.  Your

scale factor would be 1"=1".  What if drawing the is 300 yards long and

you want the plot to be 300".  Your scale would be 1"=3'.  For every

inch of Plotted units you need 3' of Drawing units.

The ADI driver will automatically scale your drawing to

fit within the desired length. This length will also control how

far the plotter advances the paper at the end of the plot.

The driver will send all the vectors to the plotter's buffer

or hard disk.  The pen plotters will automatically advance the page to

plot each frame.  Raster devices will wait until the entire drawing is

received before plotting.

For those who are curious, the driver actually will figure the

magnification factor, M, by which to scale plot data by (assuming

inches) using the equation:

M = (X-Axis length * plotter steps / AutoCAD's plot-space) + 1.

The AutoCAD plot-space is 65,535 steps and the plotter steps is 1016

steps/inch for all HP-GL/2 devices.

If you want a 300 inch long plot the factor M would be:

M = (300 * 1016 / 65,535) + 1

M = 5.65 or

M = 5 after truncation.

The driver modifies the plotter units (in steps/inch) by dividing the

default units (1016) by M (5) in this case.  Then the plotter scales

up all plot data by multiplying by 5. The plot is magnified by this

factor of 5 in both X and Y directions.

Make sure: Your plotter is cabled correctly,

is turned on, and

is On-Line before you press ENTER to plot.

*****************

V. LINE WEIGHTS

*****************

Line Weights are not presently supported by AutoCAD at this time.  We

are offering the feature here because users have requested it. All of

the devices supported by this driver are capable of plotting various

width lines. These widths are sometimes called "line weights".

This driver defaults to mapping the value in the pen speed field to

line weights.

The HP 7600 series of plotters will not produce a line of dots if

the line weight is greater than 0.8 mm.  This will happen on

the AutoCAD linetypes of dot2 and dotx2.  If you assign a line

weight 13 or greater to either of those AutoCAD linetypes, the

line will not be plotted.  This is built in by the HP 7600's

firmware.

The HP LaserJet and HP PaintJet will take a dot and produce a

dash the length of the line weight thickness.  This dash will

always be along or in the y-axis direction, regardless of the

orientation of the line.

After you have selected what you are going to plot, you will be asked:

"Do you want to change any of the above parameters? <N>" -- Answer Yes

You will be shown a table of Entity Colors from 1 to 15 and the

associated Pen Numbers, Line Types and Pen Speeds.

Substitute for the Pen Speed value, the line weight numbers from the

following table:

Line Weight  Line Weight            Line Weight  Line Weight

Index     Widths(mm.)               Index     Widths(mm.)

-----------  -----------            -----------  -----------

1          .350   (default)         11         .700

2          .075                     12         .800

3          .100                     13        1.00

4          .120                     14        1.25

5          .187                     15        1.50

6          .250                     16        1.75

7          .300                     17        2.00

8          .400                     18        2.50

9          .500                     19        3.00

10          .600                     20        4.00

For example if you assign to Entity Color #4 (cyan) Pen Speed 9 from

the above table, all the color 4 entities drawn in AutoCAD will be

plotted with lines .5 millimeters wide.

Do not use line weights greater than .8 mm to draw text.  The results

are not satisfactory.

For faster throughput on pen plotters, use different width pens in the

carousel to get pen widths.  Otherwise, the plotter will multi-stroke

each vector.

***********************

VI. STOPPING THE PLOT

***********************

If you want to stop the plot while AutoCAD is still sending vectors to the

plotter, abort it in the normal manner (Control C).  To cancel a plot at the

plotter, follow the instructions pertaining to your printer or plotter in the

table below.

PRINTER or PLOTTER             INSTRUCTIONS

-------------------------------------------------------------------

HP DesignJet plotter           Cycle power.

HP 7600 240D/E                 Press the RESET button on the

plotter's front panel.

HP 7600 (All other models)     a. Take plotter off line.

b. Press the RESET button.

HP Draftmaster SX,RX,MX        a. Press the Cancel button on the

plotter's front panel.

b. Select "Yes" to confirm.

HP 7550 Plus plotter           a. Press the Cancel button on the

plotter's front panel.

b. Select "Yes" to confirm.

HP PaintJet XL printer         Hold down the On Line button until

all lights stop flashing.

HP LaserJet III printer        Press and hold the Reset button until

the Form Feed light goes out.

-------------------------------------------------------------------

Do not turn the plotter on and off in the middle of a plotting session.

An error message will be displayed if you try to use the HP-GL/2 P386

ADI driver while the plotter is turned off.

************************

VII. PLOTTING TO FILE

************************

Plotting directly in AutoCAD to devices is often a time consuming

task.  Plotting to file allows you to minimize the lockout time

associated with converting an AutoCAD drawing (DWG) file to a plot.

Once the plot files are generated, they can be copied to the plotter

from your PC, or even to another computer that does not have AutoCAD

running, for eventual output to a remote plotter.  Because this driver,

PLPHPGL2.EXP, is a P386 ADI driver, it can be used with AutoCAD in an

AUTOSPOOL mode.  Please refer to the AutoCAD 386 Installation and

Perfiormance Guide, Chapter 5, for more details of Plot Spooling.

If you wish to plot to file, ensure that you have sufficient disk space

to store the HP-GL/2 plot commands.  File sizes do not necessarily depend

on the size of AutoCAD's DWG file.  For example, CHROMA.DWG contains

12,359 bytes as distributed with AutoCAD, expands to almost 240k bytes

when plotted to file.  This is due to the amount of fill lines AutoCAD

requests the driver to emit in order to fill the polygons in the drawing.

Although all the supported HP-GL/2 devices support polygon fills, the

current ADI 4.1 specification does not provide a polygon fill command

for such that the driver could take advantage of this HP-GL/2 capability.

Simple line drawings will require much less disk space.

Once you have plotted a drawing to a file, you can send a plot file to the

plotter by typing the following command at the DOS command line:

C> COPY 'filename' LPT(n):     OR  copy 'filename' com(n):

where filename is the name of the file you wish to plot and lpt(n)

or com(n) represents the port you have connected to your plotter.

If using the serial (com) port, you will first have to issue a

DOS MODE command to match the serial configuration of your plotter.

For PaintJet XL and LaserJet III, type:

C> MODE COM1:9600,N,8,1,P

for HP DesignJet, HP7600, and HP DraftMaster, type:

C> MODE COM1:9600,E,7,1,P

**********************************

VIII. PLOTTING ON NOVELL NETWORK

**********************************

AutoCAD users on a Novell network benefit in sharing a common set of

AutoCAD drawing files and other related utilities.  In addition, AutoCAD

users gain improved plotting throughput by sharing the plotting devices.

Novell Netware provides queueing capability of multiple plots from

multiple users destined to the same device for orderly plotting.

To take advantage of sharing a plotter on a Novell network, you should

take the following steps:

1. Configure AutoCAD 386 to Plot to File.

2. Specify the file name to be AUTOSPOOL.

3. Create a \spfiles\ directory on your hard disc.

4. Create a SPOOL.BAT file that contains the following command:

NPRINT %1 J=DESIGNJET

(where DESIGNJET is the print job name your network administrator

may have set up on the Novell server for the plotter)

5. Set AutoCAD's ACADPLCMD environment variable to invoke SPOOL.BAT

C:> SET ACADPLCMD=spool %s

This would allow AutoCAD 386 to create a temporary plot file in the

\spfiles\ directory.  Upon plot termination, AutoCAD 386 will execute the

command specified in the environment variable ACADPLCMD.  Because the

command is the Novell nprint command, the file contents are queued into

Novell Netware's print server queue DESIGNJET.  AutoCAD 386 then

automatically deletes the temporary file.  Make sure the SPOOL.BAT file

is in a directory accessible in your %PATH% variable.

!IMPORTANT!  DO NOT USE THE NOVELL CAPTURE COMMAND TO MAP AN LPT TO A

PRINT SERVER.  AUTOCAD 386 DOES NOT USE THE BIOS FOR PLOT OUTPUT.

Please refer to the AutoCAD 386 Release 11 Installation and Performance

Guide page 51 and page 80.

******************************************

IX. PEN OPTIMIZATION FOR RASTER PLOTTERS

******************************************

There is no need to do any Pen Motion Optimization for the raster plotting

devices.  This includes the HP DesignJet, HP 7600 Series, HP PaintJet XL,

and the HP LaserJet III.  The driver sets the pen optimization to level 1,

End Point Swap, to produce the quickest output with no loss in quality at

plot time.

*********************************

X. COLOR PALETTE AND MERGE MODE

*********************************

The raster plotters such as the HP DesignJet, HP 7600 Electrostatic,

and PaintJet XL, are used with a color palette that approximates the

AutoCAD colors.  When you select either of these devices, your pen tables

are filled with pen numbers that match the AutoCAD color/layer tables.

At run-time configuration, your pen table will already be filled out with

pen number's 1 to 15 assigned to AutoCAD color/layer's 1 to 15.  When you

draw any objects in red (AutoCAD color 1), your object will be plotted in

red. Any objects created in blue (AutoCAD color 5) will plot in blue.

You may change this, but each time you reconfigure the driver from the

reconfiguration menu the pen table will be set to these defaults.

The HP-GL/2 language provides MERGE CONTROL.  MERGE CONTROL determines

whether vectors and polygons are to be plotted either in MERGE MODE

or in OVERWRITE MODE.  MERGE MODE tells the plotter to merge vectors and

polygons with previously plotted elements. The pen plotters, such as the

HP DraftMaster, can only plot in MERGE MODE due to its inherent pen

plotting technology. OVERWRITE MODE tells the plotter to plot vectors

and polygons to overlap previously plotted elements where they intersect.

This is the typical mode used with raster display devices such as VGA.

All the HP raster plotters support OVERWRITE MODE since it provides the

closest match to the drawing as displayed on the screen. Some of the HP

rastor plotters, such as the HP 7600 series, allow the user to select

either mode via the front panel.

The driver currently does not specify the MERGE CONTROL plotting mode

when plotting to the plotter.

**********************************

XI. PLOTTER SPECIFIC INFORMATION

**********************************

HARDWARE REQUIREMENTS

=====================

These hardware requirements are specific to using the PLPHPGL2 plotter

P386 ADI driver with AutoCAD.

o A parallel card and cable as described in the  AutoCAD Installation

& Performance Guide is your best choice for good performance with

raster devices such as the HP DesignJet, HP 7600 series of

electrostatic plotters, the HP LaserJet III, the HP PaintJet

XL printer, and the HP 7550 Plus.

o A serial card and cable is required for plotting to the HP

DraftMaster series plotters and can be used with the HP DesignJet

and HP 7600 series electrostatic plotters. Plots will take

significantly longer to send to the plotter.  We do not recommend

serial communications for raster devices.

HARDWARE INSTALLATION

=====================

The supported Hewlett-Packard device must be connected to the computer

with a standard Centronics-type parallel cable or a serial cable.

Please refer to the HP User's Guide or Computer Information Manual for

your HP plotter for the best connection to your PC.  Additionally, the

AutoCAD Installation & Performance Guide (AutoCAD IG) section for

Hewlett-Packard Plotters contains information regarding pin connections,

switch settings and serial and parallel cable information for the HP 7600

Series and HP Draftmaster plotters.

SERIAL MODE

===========

It is VERY IMPORTANT to use the correct RS-232 cabling for connecting

your PC to the plotter.  Contrary to the well-defined cabling for the

Centronics parallel connection, the RS-232 specification is often times

very loosely followed.  This HP-GL/2 driver MUST see RS-232 pins 2,3,7

4,5,6,20 active in order for serial communication to successfully occur.

Hewlett-Packard recommends the following RS-232 plotter cables for easy

connection of your HP plotter to your PC:

(part number 17255D)           (part number 24542G)

25-pin to 25-pin               25-pin  to  9-pin

PLOTTER     COMPUTER           PLOTTER       COMPUTER

1------------1                 4--------------1

2------------3                 2--------------2

3------------2                 3--------------3

7------------7                 5--+-----------4

20---------+--5                    |

5--+      |                    6--+

|      +--6                 7--------------5

6--+---------20               20-----------+--6

|

+--8

8--------------7

If you plan on making a minimum RS-232 cable, the AutoCAD Installation

and Performance Guide suggests a possible wiring diagram that can be

used to minimize the number of conductors necessary in your cable.

The diagram is displayed here for your reference.  Be aware this wiring

configuration will not provide true status information to the driver as

to whether the plotter is off-line, disconnected, or even powered off!

Printer end                      Computer end

2  ----------------------------- 3

3  ----------------------------- 2

4  --+                       +-- 4

|                       |

5  --+                       +-- 5

7  ----------------------------- 7

6  --+                       +-- 6

|                       |

20  --+                       +-- 20

The driver will configure the COM port to the following settings:

For HP DesignJet, HP 7600 Electrostatics, HP Draftmaster:

9600 baud, even parity, 7 data bits, and 1 stop bit.

For HP PaintJet XL and HP LaserJet III:

9600 baud, no parity, 8 data bits, and 1 stop bit.

HP DESIGNJET PLOTTER

====================

As a general rule, when you're looking at the paper in the plotter, the side

margins (Y axis) are .2 inches each (5mm).  The top and bottom margins are .67

inches each (17mm).  The following table shows the imageable area for

standard paper sizes.

Size      Inches            Millimeters

---------------------------------------

A      9.66 X 8.1           245 X 205

B     15.66 X 10.6          397 X 269

C     20.66 X 16.6          524 X 421

D     32.66 X 21.6          829 X 548

E     42.66 X 33.6         1083 X 853

If you selected "1. DesignJet" from the plotter models menu, the

HP DesignJet plotter will print all colors in solid black.

If you selected "HP 7600 Color" from the plotter models menu, the driver

will send the AutoCAD color palette to the HP DesignJet plotter.  The

HP DesignJet will print levels of grey scale to match the relative

intensities of the AutoCAD colors.  The driver defaults the MERGE CONTROL

to OVERWRITE MODE.  Similar to the AutoCAD drawing that is displayed on

your screen, the plotted result may show lighter intensity lines to

overlap darker lines.

HP 7600 ELECTROSTATIC PLOTTERS

==============================

A plot on a smaller sheet than the default size for the 7600 will be

centered in the middle of the page.  For example, an A size plot on a

240E will be centered about 17 inches from the edge of the paper.

If you need to force a A, B, C... size drawing to the edge of the page,

simply make use of a user page size that is set to the full height of the

paper.  For example, if you want an A-size plot along the X-axis,

"Enter the Size or Width,Height (in Inches) :"

enter the paper size as 10.5,35.18 for an E size

plotter.  Not the 10.5,8.0 as given for the A size sheet.

The HP7600 Series Models 250,255,355 provide a front panel control to

nest smaller size plots to save paper.  Please refer to the HP 7600

Series User Guide documentation for details on this paper saving and

time saving feature.

HP DRAFTMASTER PLOTTERS

=======================

The following table details the actual physical plotting areas for each

page size supported by the Hewlett-Packard DraftMaster plotters:

Media     Plotting Width     Plotting Height        Media Orientation

Size     Normal  Expanded    Normal  Expanded    (as loaded into plotter)

A       8.83"    9.66"      7.3"     8.1"             Portrait

B      14.83"   15.66"      9.8"    10.6"             Portrait

C      20.83"   21.66"     14.8"    15.6"             Portrait

D      31.83"   32.66"     20.8"    21.6"             Portrait

E      41.83"   42.66"     32.8"    33.6"             Portrait

Drawings created to plot on a Hewlett-Packard DraftMaster should keep

these physical limits in mind when they are initially created.  If

plotting in expanded mode, lines drawn at the edge of the plot will be

under the pinch rollers and could be subject to smearing.

If you are using the HP Draftmaster for a LongPlot, you must follow the

instructions in the Hewlett-Packard User's Guide.  You must have

a black 0.3mm fiber-tip pen (for plotter paper) or a 0.35mm drafting pen

(for vellum or polyester film) in pen stall #8.  Only these pens enable

the plotter to detect the registration marks used for frame-to-frame

alignment.

Because HP Draftmaster uses multiple frames for LongPlots, the double-

matte polyester film is the only media type that will guarantee precise

registration between the multiple frames used to produce long axis

plots.  Non-polyester media expands and contracts with changes in

temperature and humidity, causing portions of the media exposed to the

air to roll up.  As a result, slight gaps may appear between frames

if you use paper, vellum or other non-polyester films.  Hewlett-Packard

recommends polyester media to be used when plots are to be made for

archival purposes.

HP PAINTJET XL IN HP-GL/2 MODE

==============================

The HP PaintJet XL must be allowed to finish a plot before starting

another plot with a parallel cable.  If you try to plot a second drawing

before the plotter finishes a plot, the HP PaintJet XL will reset itself,

clear its memory and terminate the original plot.

To configure the HP PaintJet XL as a serial device, the switches should

all be set to the down position as follows:

SWITCH          ON/UP   OFF/DOWN

Baud0                      X

Baud1                      X

Par0                       X

Par1                       X

DTR/XON-XOFF               X

MET/ENG                    X

ROM 8/PC 8                 X

HP LASERJET III IN HP-GL/2 MODE

===============================

Page Protection:

----------------

If you have a HP LaserJet III, the factory default setting for Page

Protection is OFF.  This sets aside only 191 Kbytes for Image Memory

which will cause the HP LaserJet III to display the error message:

"Printer Overrun".  Only a portion of your drawing will be plotted.

Setting Page Protection to "LTR/A4" should eliminate this problem.

See page 4-25 of the HP LaserJet III user's guide for an explanation

of Page Protection. Chapter 4 of the user's guide also provides

instructions on how to reconfigure your HP LaserJet III.  To get full-

size plots on legal size paper will also need more memory.

Hewlett Packard LaserJet III's must have two megabyte memory total to

work with the HP-GL/2 driver.

Fills:

------

Due to the finer resolution of the HP LaserJet III, you must

reconfigure the pen width settings or you will end up with "streaks" in

filled areas and/or broken lines.  Any large filled area should show

some streaking at the default pen width of .010 (Inches).

There is a workaround by varying the Pen Width selected at plot time

configuration and opting for the adjusted filled boundaries.  When

the plot time configuration asks:

"Pen width in Inches <0.010>"

start with a pen width of .009 and do a test plot.  Be sure to answer yes

to the following question:

"Adjust area fill boundaries for pen width? <N> "

If there still are streaks, reduce the pen width by .001 until the plot

output is acceptable.  Reducing the pen width will lengthen the plotting

time.  Please don't confuse this use of pen width with the use of line

weights referred to in other parts of this document.

Legal Paper Size:

-----------------

If you wish to plot a drawing longer than the 10.5" of letter size paper,

you will have to install your legal size paper tray.  If you don't

have a legal size tray, you can manually feed the legal size sheets

to the printer.  You must set your printer for manual feed before you

send the plot.  A good time to do this is when AutoCAD tells you to

"Position paper in plotter."

"Press RETURN to continue....".

If you do use manual feed, the HP LaserJet will display what size paper

to feed it.  Please see your Hewlett-Packard LaserJet III user's guide

for details of how to set it up for manual feed.

Serial Configuration:

---------------------

To configure the HP LaserJet III as a serial device, follow these steps:

1) Take the printer off line.

2) Hold the MENU button down for 5 seconds until the READY message

changes to AUTO CONT....

3) Press the MENU button once more.  It will display I/O=PARALLEL *

If it shows I/O=SERIAL *, put the printer back on line.

4) Press the '+' button once.  It should display I/O=SERIAL.

5) Press ENTER to save the setting.  A * should appear in the display.

6) Put the HP LaserJet back on line by pressing the ONLINE button.

AutoCAD, AutoSketch and ADI are registered trademarks of Autodesk, Inc.

IBM, PC, XT, AT are trademarks of International Business Machines Corporation.

HP is a trademark of Hewlett-Packard Corporation.

Novell, Netware are registered trademarks of Novell, Inc.

!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!

!                                                                          !

!                  Copyright 1991, Hewlett-Packard Co.                     !

!               Copyright 1986,87,88,89,90, Autodesk, Inc.                 !

!                                                                          !

!  License is granted for the use of this software and documentation on    !

!  a no-cost, sharing basis.  You may distribute and copy this software    !

!  freely as long as it remains a completely unaltered package and is      !

!  not sold for profit.                                                    !

!                                                                          !

!  These materials are provided with no warranty, either expressed or      !

!  implied, including but not limited to any implied warranties of         !

!  merchantability or fitness for a particular purpose, regarding these    !

!  materials and are made available on an "as-is" basis.                   !

!                                                                          !

!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!