README.DOC

Txplus Software Version 5.19

Note to users of Hard Disk compresssion software (i.e. Stacker, SuperStore)

If any difficulties are encountered during a backup using a

compression software, it may be necessary to utilize the

the special executable file found on the floppy disks that

provide the TapeXchange software. Use the following procedure.

1. Copy the file TXPLUSTA.EXE from the TapeXchange floppy to

the TXPLUS directory on your hard drive.

2. Change to the TXPLUS directory.

3. Invoke TapeXchange by entering TXPLUSTA

4. Use the TapeXchange in a normal manner.

Users are cautioned to not use this procedure unless required to do

so by incompatibility problems. This executable uses a slightly

different and somewhat slower methodology to access hard disk

information thereby increasing backup time.

This version of software also includes support for additional tape

drives including the Sony DAT and the TX2000.

NOTE: The Sony DAT drive is being introduced with this software

release. This drive is identifable by the presence of

3 LED indicators on the front. These are: BUSY, TAPE, STATUS.

The drive also features an automatic notification that the

tape drive needs to be cleaned. This notification is presented

to the user by a flashing "STATUS" LED when the drive is in

a normal condition and no drive read or write activity is

taking place. The LED will flash ON for 3.5 seconds and OFF

for 0.5 seconds. This indication will continue until the heads

have been cleaned using an authorized cleaning kit. This kit

is available from your Dealer and is identified as a SONY

CLEANING CARTRIDGE Model 5CL.

The following items are not contained in the manual and should

be noted by the user.

1. If the user starts a backup while logged into a Novell

network and that backup spans midnight according to

Novell time, Novell will not update the user's PC date

after crossing midnight. The PC date can be corrected

manually or by re-logging into the Novell network after

completing the backup.

2. The TapeXchange printer pass-through port is designed for

use with printers only and should not be used to connect

other devices.

3. The use of "background" printing processes (i.e. spooling)

is not allowed while the TapeXchange unit is in use for

backup, restore, or list functions.