NOTE: In Flight Simulator 2002, the Bell 206B Jet Ranger III engine is always running.

# BEFORE TAKEOFF

- Fuel Quantity -- CHECK

- Engine and Transmission Instruments -- CHECK WITHIN LIMITS

- Lights -- AS REQUIRED

- Radios and Avionics -- SET

- Throttle -- FULL OPEN

- Power and Flight Instruments -- SET and CHECK

- Power Turbine (N2) -- SET FOR 100% in flat pitch

# TAKEOFF

- Collective Pitch -- INCREASE TO HOVER

- Engine Instruments -- CHECK

- Directional Control -- AS REQUIRED to maintain desired heading

- Cyclic Control -- AS REQUIRED to accelerate smoothly

- Collective -- AS REQUIRED for desired climb rate and airspeed

# IN-FLIGHT OPERATIONS

- Airspeed -- AS DESIRED (below 130 KIAS)

- Engine Instruments -- CHECK

- Fuel Quantity -- CHECK

- Altimeter -- SET and CHECK

# DESCENT AND LANDING

- Throttle -- FULLY OPEN

- Power Turbine RPM (N2) -- 97-100%

- Flight Path -- AS REQUIRED

- LDG LTS Switch -- ON AS REQUIRED

## REFERENCE INFORMATION

Total Flight Simulator aircraft weight with full fuel

2,700 lbs

VNE - Never Exceed Speed

130 KIAS (<3,000 lbs, sea level-3000' density altitude,

decrease 3.5 KIAS per 1000' above 3000')

122 KIAS (>3,000 lbs,  sea level-3000' density altitude,

decrease 7.0 KIAS per 1000' above 3000')

Autorotation Maximum Speed

100 KIAS

Maximum Rate of Climb

52 KIAS (N2 engine RPM 100%)

Autorotation Minimum Descent Speed

52 KIAS

Autorotation Maximum Glide Speed

69 KIAS

Specifications

Cruise Speed			115 kts, 132 mph, 213 kph

Engine				Allison 250-C20J 420 shp

Maximum Range			435 nm, 500 m, 805 km

Service Ceiling			20,000 ft, 6,096 m

Hovering Ceiling			19,600 ft, 5,974 m

Fuel Capacity			91 U.S. gal, 344 L

Empty Weight			1,640 Ibs, 744 kg

Maximum Gross Weight		3,200 lbs, 1,451 kg

Maximum Gross Weight		3,350 lbs, 1,519 kg

(External Loading)

Length				31.2 ft, 9.51 m

Rotor Span			33.3 ft, 10.15 m

Height				11.7 ft, 3.51 m

Seating				Up to 5

Useful Load			1,498 lbs, 679 kg