# BEFORE STARTING

- Parking Brake -- SET

- All Avionics -- OFF

- Landing Gear Handle -- DOWN

- Cowl Flaps -- CHECK OPEN

- Fuel Selectors -- CHECK ON

- All Switches and Equipment Controls -- CHECK

- Battery and Alternator Switches -- ON

- Fuel Quantity Indicators -- CHECK QUANTITY

- Landing Gear Position Lights -- CHECK

# STARTING

LEFT ENGINE:

- Mixture Control -- FULL RICH

- Propeller Control -- HIGH RPM

- Throttle -- FULL OPEN

- Fuel Boost Pump Switch -- HI (until fuel flow peaks, then OFF)

- Throttle -- CLOSE; THEN OPEN APPROX. 1/2 INCH

- Magneto/Start Switch -- START position; release to BOTH position when engine starts

- Throttle -- 900 to 1000 RPM after start

- Oil Pressure -- 10 PSI WITHIN 30 SECONDS

- Warm-up -- 900 to 1000 RPM

- Alternator Switch -- ON

- Loadmeters and Voltmeter -- CHECK FOR BATTERY CHARGE

- Red "START" Annunciator Light – CHECK (should be illuminated during start and extinguished after start)

RIGHT ENGINE:

- Mixture Control -- FULL RICH

- Propeller Control -- HIGH RPM

- Throttle -- FULL OPEN

- Fuel Boost Pump Switch -- HI (until fuel flow peaks, then OFF)

- Throttle -- CLOSE; THEN OPEN APPROX. 1/2 INCH

- Magneto/Start Switch -- START position; release to BOTH position when engine starts

- Throttle -- 900 to 1000 RPM after start

- Oil Pressure -- 10 PSI WITHIN 30 SECONDS

- Warm-up -- 900 to 1000 RPM

- Alternator Switch -- ON

- Loadmeters and Voltmeter -- CHECK FOR BATTERY CHARGE

- Red "START" Annunciator Light – CHECK (should be illuminated during start and extinguished after start)

- Avionics -- ON

# BEFORE TAKEOFF

- Parking Brake -- SET

- Fuel Boost Pumps – OFF (if ambient temperature is 32 degrees C or above, use LOW pressure boost)

- Fuel Indicators -- CHECK QUANTITY INDICATED

- Mixture -- ADJUST AS REQ BY FIELD ELEVATION WHEN SETTING FULL POWER FOR TAKEOFF

- Fuel Selectors -- CHECK ON

- Red "START" Annunciator Light – CHECK (should be illuminated during start and extinguished after start)

- Throttles -- 2200 RPM

- Propellers – EXERCISE (to obtain 200-300 RPM drop, then return to high RPM)

- Throttles -- 1700 RPM

- Magnetos – CHECK <150 RPM drop on each; <50 RPM drop between (click each to L, R, L, Both)

- Throttles -- 1500 RPM

- Propellers -- FEATHERING CHECK (Do not allow an RPM drop of more than 300 RPM)

- Throttles -- IDLE (note RPM)

- Throttles -- 900 to 1000 RPM

- Trim -- AS REQUIRED FOR TAKEOFF

- Flaps - CHECK AND SET FOR TAKEOFF

- Ice Protection Systems -- AS REQUIRED

- Parking Brake -- OFF

TAKEOFF

- Minimum Takeoff Oil Temperature -- 24 degrees C

- Throttles -- FULL

- Propellers -- MAX RPM

- Mixture -- FULL RICH

- Landing Gear – RETRACT (when positive rate of climb is established)

- Airspeed -- ESTABLISH DESIRED CLIMB SPEED (when clear of obstacles)

# CLIMB

- Throttles -- FULL

- Propellers -- 2700 RPM

- Mixture -- LEAN AS REQUIRED

- Cowl Flaps -- OPEN

- Airspeed -- 105 KIAS

# CRUISE CLIMB

- Throttles -- FULL

- Propellers -- 2500 RPM

- Mixture -- LEAN AS REQUIRED

- Cowl Flaps -- AS REQUIRED

- Airspeed -- 136 KIAS

- Engine Temperatures -- MONITOR

- Fuel Boost Pumps -- AS REQUIRED

# CRUISE

- Cowl Flaps -- CLOSED

- Power --SET

- Fuel Boost Pumps -- AS REQUIRED

- Mixtures -- SET USING EGT

# DESCENT

- Altimeter – SET (press B to autoset to current setting)

- Mixture -- ENRICH AS REQUIRED

- Cowl Flaps -- CLOSED

- Flaps -- AS REQUIRED

- Power -- AS REQUIRED (avoid prolonged idle settings)

# BEFORE LANDING

- Fuel Selector Valves -- CHECK ON

- Fuel Boost Pumps -- OFF OR LOW AS PER AMBIENT TEMPERATURE

- Cowl Flaps -- AS REQUIRED

- Mixture Controls -- AS REQUIRED BY FIELD ELEVATION

- Flaps -- APPROACH (15 degrees; when below 152 KIAS)

- Landing Gear -- DOWN (below 152 KIAS)

- Flaps -- FULL (30 degrees; when below 122 KIAS)

- Airspeed -- ESTABLISH NORMAL APPROACH SPEED (approx. 95 KIAS)

- Propellers -- HIGH RPM

AFTER LANDING (CLEAR OF RUNWAY)

- Landing and Taxi Lights -- AS REQUIRED

- Flaps -- UP

- Trim Tabs -- SET TO ZERO

- Cowl Flaps -- OPEN

- Fuel Boost Pumps -- AS REQUIRED

# SHUTDOWN

- Parking Brake -- SET

- Propellers -- HIGH RPM

- Throttles -- 1000 RPM

- Fuel Boost Pumps -- OFF

- Electrical and Avionics Equipment -- OFF

- Mixture Controls -- IDLE CUT-OFF

- Magneto/Start Switches -- OFF AFTER ENGINES STOP

- Battery and Alternator Switches -- OFF AFTER ENGINES STOP

## REFERENCE INFORMATION

Total Flight Simulator aircraft weight with full fuel

5,500 lbs

VA - Maneuvering Speed

156 KIAS

VFE - Maximum Flaps Extended Speed

152 KIAS (Approach--flaps 15 degrees)

122 KIAS (Full down--flaps 30 degrees)

VLE - Maximum Landing Gear Extension Speed

152 KIAS

VLO - Maximum Gear Operating Speed

152 KIAS

Landing Approach Speed (5,400 lbs)

95 KIAS (full down--flaps 30 degrees)

VNO - Maximum Structural Cruising Speed

195 KIAS

Turbulent Air Penetration Speed

156 KIAS

VNE - Never Exceed Speed

223 KIAS

VS  -  Stalling Speed (maximum weight)

84 KIAS

VSO  -  Stalling Speed in the Landing Configuration

(maximum weight)

75 KIAS

VX  -  Two-Engine Best Angle-of-Climb Speed

92 KIAS

VY  -  Two-Engine Best Rate-of-Climb Speed

105 KIAS

VXSE  -  One-Engine Inoperative Best Angle-of-Climb Speed

100 KIAS

VYSE  -  One-Engine Inoperative Best Rate-of-Climb Speed

101 KIAS

Minimum Icing Conditions Speed

130 KIAS

VMCA - Air Minimum Control Speed (single-engine)

84 KIAS

Maximum Glide Range Speed

115 KIAS

Specifications

Cruise Speed				200 kts

Engine					Teledyne Continental Motors IO-550-C 300 hp

Propeller					Three McCauley constant-speed, variable pitch

Maximum Range				1,569 nm

Service Ceiling				20,688 ft

Fuel Capacity				142 gal

Maximum Gross Weight			5,524 lbs

Length					29 ft, 10 in

Wingspan					37 ft, 10 in

Height					9 ft, 9 in

Seating					Up to 6

Useful Load				1,634 lb