# BEFORE START

- Parking Brake -- SET

# ENGINE START

- Fuel Control Switches – ON

- Throttles – IDLE

- Engine Start Switch – START (hold until engine starts)

# AFTER START

- De-ice -- AS REQUIRED

- Flight Controls -- CHECK

- Autopilot -- SET AND OFF

- Instruments -- CHECKED

- Autobrake -- RTO (REJECTED TAKEOFF)

- Avionics Switch -- ON

- Avionics -- SET

- Trim -- SET

- Beacon Light Switch -- ON

# BEFORE TAKEOFF

- Flaps -- SET FOR TAKEOFF

- Flight Director -- ON

- Autothrottle – ARM (if using TO/GA mode for takeoff)

# TAKEOFF

- Brakes -- RELEASE

- Strobe Lights -- ON

- Transponder -- ALT

- Thrust Levers -- ADVANCE TO 1.05 EPR

- Thrust Levers -- ADVANCE SMOOTHLY TO 100% N1 --or-- TO/GA Mode -- Engage

- Thrust -- VERIFY CORRECT FOR TAKEOFF

- Airspeed 80 KIAS-- CALLOUT "80 KNOTS"

- Airspeed V1 -- CALLOUT "V1"

- Airspeed VR -- CALLOUT "ROTATE" -- ROTATE TO APPROX. 10 DEGREES PITCH UP

- Airspeed V2 -- CALLOUT "V2"

- Landing Gear -- UP (WHEN POSITIVE CLIMB ESTABLISHED)

- Autopilot Heading Select switch -- ON IF DESIRED

- Airspeed --MAINTAIN V2+15 KIAS

- Autopilot -- ENGAGE

- Flaps-- START RETRACT ON SCHEDULE AT 1,000' AGL

# CLIMB

- Autothrottle -- OFF

- Landing Lights -- OFF ABOVE 10,000' MSL

- Altimeter -- SET TO 29.92 CROSSING 18,000' MSL

# CRUISE

- Thrust Levers-- AS DESIRED

- Trim -- AS NECESSARY

# DESCENT

- Airspeeds (VREF, VAPP) -- COMPUTED AND SET

- Autobrake -- AS DESIRED

- De-ice -- AS REQUIRED

- Autopilot -- AS DESIRED

- Thrust Levers -- AS DESIRED

- Altimeter -- SET TO LOCAL SETTING CROSSING 18,000' MSL

- Avionics -- SET

- Airspeed -- <250 KIAS BELOW 10,000' MSL

- Landing Lights -- ON BELOW 10,000' MSL

- Approach Procedure -- REVIEW

# APPROACH

- Airspeed -- AS DESIRED

- Thrust Levers -- AS DESIRED

- Flaps -- AS DESIRED

- Autopilot -- AS DESIRED

# LANDING

- Airspeed -- AS DESIRED

- Thrust Levers -- AS DESIRED

- Landing Gear -- DOWN and CONFIRMED

- Flaps -- AS DESIRED

- Speedbrake -- ARMED

- Autopilot -- AS DESIRED

- Autothrottle -- ARM

# LANDING ROLL

- Thrust Levers - CLOSED

- Autothrottle -- CHECK OFF

- Speedbrake Lever -- CHECK FULL UP

- Thrust Levers-- REVERSE

- Thrust Levers -- IDLE AT 60 KIAS

- Autobrake -- OFF

- Brake -- AS NECESSARY

- Autopilot -- CHECK DISENGAGED

# TAXI-IN

- Speedbrake Lever -- DOWN

- Lights -- AS DESIRED

- Flap Lever -- UP

- Transponder -- STBY

# PARKING

- Parking Brake -- SET

- Fuel Control Switches -- CUTOFF

- De-ice -- OFF

- Lights -- AS REQUIRED

- Flight Director – OFF

## REFERENCE INFORMATION

Total Flight Simulator aircraft weight with full fuel

580,069 lbs

VMO - Maximum Operating Speed (airspeed)

330 KIAS

MMO - Maximum Operating Speed (Mach)

.84 Mach

Turbulent Air Penetration Speed

280 KIAS/.84 Mach

VLO - Maximum Gear Operating Speed

270 KIAS/.82 Mach

VLE - Maximum Landing Gear Extension Speed

270 KIAS/.82 Mach

Maximum Flap (Placard) Speeds

Flaps      KIAS

1	255

5	235

15	215

20	200

25	190

30	180

V1 - Takeoff Decision Speed

Standard temperature, sea-level pressure altitude

153 KIAS (580,000 lbs)

Standard temperature, 3,000' pressure altitude

154 KIAS (580,000 lbs)

VR - Rotation Speed (dry runway; flaps 5)

Standard temperature, sea level pressure altitude

156 KIAS (580,000 lbs)

Standard temperature, 3,000' pressure altitude

157 KIAS (580,000 lbs)

V2 - Takeoff Safety Speed (dry runway; flaps 5)

Standard temperature, sea level pressure altitude

162 KIAS (580,000 lbs)

Standard temperature, 3,000' pressure altitude

162 KIAS (580,000 lbs)

VREF - Landing Approach Speed (flaps 30; gear down)

157 KIAS (580,000 lbs)

Specifications

Cruise Speed			Mach 0.84, 555 mph, 893 kmh

Engines				(three options) P&W 4000 | RR Trent 800 | GE 90 series

Maximum Range			5,960 nm, 6,859 sm, 11,038 km

Maximum Operating Altitude		43,100 ft, 13,137 m

Typical Cruising Altitude		35,000 ft

Fuel Capacity			45,200 U.S. gal, 171,160 L

Maximum Takeoff Weight-Basic	660,000 lb, 299,370 kg

Length				242 ft, 4 in, 73.9 m

Wingspan				199 ft, 11 in, 60.9 m

Height				60 ft, 8 in, 18.5 m

Seating Seats			386 to 550

Configurations Seating ranges		from 6- to 10-abreast with two aisles

Cargo Capacity			7,552 cu ft, 213.8 cu m