# BEFORE STARTING ENGINE

- Brakes -- TEST and SET

- Electrical Equipment -- OFF

- Landing Gear Lever -- DOWN

- Cowl Flaps -- OPEN

- Fuel Selector Valve -- MAIN

# ENGINE START

- Propeller -- HIGH rpm

- Mixture -- RICH

- Throttle -- OPEN 1 inch

- Propeller Area -- CLEAR

- Master Battery Switch -- ON

- Ignition -- BOTH

- Engine Start Switch -- ON

Then:

- Throttle -- 1,000 rpm

- Oil Pressure -- CHECK

# BEFORE TAKEOFF

- Brakes -- SET

- Canopy -- OPEN

- Wings -- SPREAD

Do not run up engine past 1,500 rpm with wings folded.

- Flight Controls -- FREE and CORRECT

- Flight Instruments -- CHECK and SET

- Primer -- LOCKED

- Mixture -- RICH

- Fuel Quantity -- CHECK

- Fuel Selector Valve -- RECHECK main

- Elevator Trim -- SET for takeoff

- Rudder Trim -- SET for takeoff

- Throttle -- 30 in. Hg.

- Magnetos -- CHECK

- Propeller -- CYCLE high rpm/ rpm/high rpm

- Engine Instruments and Ammeter -- CHECK

- Throttle -- 800-1,000 rpm

- Flaps -- SET for takeoff (0-20 degrees)

- Cowl Flaps -- OPEN

# TAKEOFF

- Flaps -- 0-20 degrees

- Arresting Hook -- UP

- Mixture -- RICH (may be leaned above 3,000')

- Propeller -- MAX rpm

- Throttle -- FULL

- Brakes -- RELEASE

- Takeoff Speed -- 75 knots

- Brakes -- APPLY momentarily when airborne

- Landing Gear -- UP

- Flaps -- UP

# NORMAL CLIMB

- Airspeed -- 150 knots

- Throttle --  34 in. Hg. or FULL (whichever is less)

- Propeller -- 2,400 rpm

- Fuel Selector Valve -- MAIN

- Mixture -- FULL RICH (may be leaned above 3,000')

- Cowl Flaps -- OPEN as required

- Canopy -- CLOSED

# MAXIMUM PERFORMANCE CLIMB

- Airspeed --  125 knots at sea level

- Throttle -- FULL

- Propeller -- 2,700 rpm

- Fuel Selector Valve -- MAIN

- Mixture -- FULL RICH (may lean above 3,000')

- Cowl Flaps -- FULL OPEN

# CRUISE

- Throttle -- Set for 65% power

- Propeller -- 2,000 rpm

- Elevator Trim -- ADJUST

- Rudder Trim -- ADJUST

- Mixture -- LEAN

- Cowl Flaps -- CLOSED

# COMBAT

- Weapons -- ARM

- External fuel tanks -- DROP

- Extinguish all smoking materials

- Use water injection as necessary

# DIVING

- Canopy -- CLOSED

- Landing Gear -- UP or may be used as dive brakes

- Dive Brake -- OFF or ON as desired Operate before dive is entered

- Wing Flaps -- UP (press SHIFT+V)

- Propeller -- 2,400 rpm or less

- Mixture -- RICH

- Throttle -- SLIGHTLY OPEN

- Cowl Flaps -- CLOSED

Maximum limits 3,100 rpm (not over 30 seconds)

# DESCENT

- Fuel Selector Valve -- MAIN

- Throttle -- AS DESIRED

- Propeller -- AS DESIRED

- Mixture -- ENRICH as required

- Cowl Flaps -- CLOSED

- Wing Flaps -- AS DESIRED

# BEFORE LANDING-ON FIELD

- Fuel Selector Valve -- BEST

- Landing Gear -- DOWN

- Arresting Hook -- UP

- Mixture -- RICH

- Propeller -- HIGH rpm

# LANDING -- ON FIELD

- Airspeed --  90-95 KIAS (flaps DOWN)

- Flaps -- AS DESIRED

- Trim -- ADJUST

- Touchdown -- MAIN WHEELS FIRST

- Landing Roll -- LOWER TAIL WHEEL GENTLY

- Braking -- MINIMUM REQUIRED

# AFTER LANDING

- Wing Flaps -- UP

- Cowl Flaps -- OPEN

# BEFORE LANDING -- CARRIER

- Fuel Selector Valve -- BEST

- Arresting Hook -- DOWN

- Mixture -- RICH

- Propeller -- HIGH rpm

- Canopy -- OPEN

- Enter pattern starboard side

- Altitude -- 300 feet

- Turn to downwind appx 1,000-1,200 yds abeam deck

- Landing Gear -- DOWN

- Wing Flaps -- DOWN

- Landing checklist complete on downwind, appx 150-200 ft

- Abeam threshold, turn to base

- 90-degree position, 90 kts, turn final, pick up LSO

# LANDING-ON CARRIER

- Airspeed -- 80-85 knots

- Wing Flaps -- FULL DOWN

- Trim -- ADJUST

- Approach in 3-point attitude

- Watch for LSO cut signal

- At touchdown, go to full throttle until arrest confirmed

- Reduce throttle to idle

- Apply brakes

- Wing Flaps -- UP

- Arresting Hook -- UP

- Wings -- FOLD (clear of the landing area)

- Follow directions to parking

# ABORTED LANDING -- ON CARRIER

- Advance throttle to FULL

- Arresting Hook -- UP

- Landing Gear -- UP

- Wing Flaps -- UP as soon as practical

BELLY LANDINGS and WATER LANDINGS  (DITCHING)

- Landing Gear -- UP

- Landing Flaps -- DOWN

- Shoulder Harness -- LOCKED

- Canopy -- OPEN

# PRIOR TO CONTACT WITH GROUND

- Switches -- OFF

- Fuel Selector -- OFF

# BAILING OUT

- Airspeed -- REDUCE as much as possible

- Canopy -- OPEN

- Abandon airplane

CAUTION: Before bailing out at altitude, inhale as much oxygen as possible.

# SECURING AIRPLANE

- Brakes -- SET

- Throttle -- IDLE

- Propeller -- HIGH rpm

- Mixture -- IDLE CUTOFF

- Ignition Switch -- OFF

- Master Battery Switch -- OFF

- Cowl Flaps -- CLOSE