PUSHBACK (if parked at a gate)

- Request Pushback -- SET

# BEFORE START

- Parking Brake -- SET

# ENGINE START

- Engines 1,2,3,4 Control Switches – ON

- Throttles 1,2,3,4 – IDLE

- Engine 1 Start Switch – DOWN

- Engine 2 Start Switch – DOWN (when engine 1 has started)

- Engine 3 Start Switch – DOWN (when engine 2 has started)

- Engine 4 Start Switch – DOWN (when engine 3 has started)

# AFTER START

- De-ice -- AS REQUIRED

- Flight Controls -- CHECK

- Autopilot -- SET AND OFF

- Instruments -- CHECKED

- Autobrake -- RTO (REJECTED TAKEOFF)

- Avionics Switch -- ON

- Avionics -- SET

- Trim -- SET

- Beacon Light Switch -- ON

# BEFORE TAKEOFF

- Flaps -- SET FOR TAKEOFF

- Flight Director -- ON

- Autothrottle -- ARM

# TAKEOFF

- Brakes -- RELEASE

- Strobe Lights -- ON

- Transponder -- ALT

- Thrust Levers -- ADVANCE TO 1.05 EPR

- Thrust Levers -- ADVANCE SMOOTHLY TO 100% N1 --or-- TO/GA Mode -- Engage

- Thrust -- VERIFY CORRECT FOR TAKEOFF

- Airspeed 80 KIAS-- CALLOUT "80 KNOTS"

- Airspeed V1 -- CALLOUT "V1"

- Airspeed VR -- CALLOUT "ROTATE" -- ROTATE TO APPROX. 10 DEGREES PITCH UP

- Airspeed V2 -- CALLOUT "V2"

- Landing Gear -- UP (WHEN POSITIVE CLIMB ESTABLISHED)

- Autopilot Heading Select switch -- ON IF DESIRED

- Airspeed --MAINTAIN V2+15 KIAS

- Autopilot -- ENGAGE

- Flaps-- START RETRACT ON SCHEDULE AT 1,000' AGL

# CLIMB

- Autothrottle -- OFF

- Landing Lights -- OFF ABOVE 10,000' MSL

- Altimeter -- SET TO 29.92 CROSSING 18,000' MSL

# CRUISE

- Thrust Levers-- AS DESIRED

- Trim -- AS NECESSARY

# DESCENT

- Airspeeds (VREF, VAPP) -- COMPUTED AND SET

- Autobrake -- AS DESIRED

- De-ice -- AS REQUIRED

- Autopilot -- AS DESIRED

- Thrust Levers -- AS DESIRED

- Altimeter -- SET TO LOCAL SETTING CROSSING 18,000' MSL

- Avionics -- SET

- Airspeed -- <250 KIAS BELOW 10,000' MSL

- Landing Lights -- ON BELOW 10,000' MSL

- Approach Procedure -- REVIEW

# APPROACH

- Airspeed -- AS DESIRED

- Thrust Levers -- AS DESIRED

- Flaps -- AS DESIRED

- Autopilot -- AS DESIRED

# LANDING

- Airspeed -- AS DESIRED

- Thrust Levers -- AS DESIRED

- Landing Gear -- DOWN and CONFIRMED

- Flaps -- AS DESIRED

- Speedbrake -- ARMED

- Autopilot -- AS DESIRED

- Autothrottle – ARM (if using TO/GA mode for go-around)

# LANDING ROLL

- Thrust Levers - CLOSED

- Autothrottle -- CHECK OFF

- Speedbrake Lever -- CHECK FULL UP

- Thrust Levers-- REVERSE

- Thrust Levers -- IDLE AT 60 KIAS

- Autobrake -- OFF

- Brake -- AS NECESSARY

- Autopilot -- CHECK DISENGAGED

# TAXI-IN

- Speedbrake Lever -- DOWN

- Lights -- AS DESIRED

- Flap Lever -- UP

- Transponder -- STBY

# PARKING

- Parking Brake -- SET

- Fuel Control Switches -- CUTOFF

- De-ice -- OFF

- Lights -- AS REQUIRED

- Flight Director – OFF

## REFERENCE INFORMATION

Total Flight Simulator aircraft weight with full fuel

824,861 lbs

VMO - Maximum Operating Speed (airspeed)

355 KIAS

MMO - Maximum Operating Speed (Mach)

.88 Mach

Maximum Landing Gear Retraction Speed

270 KIAS/.82 Mach

Maximum Landing Gear Extension Speed

270 KIAS/.82 Mach

Maximum Landing Gear Extended Speed

320 KIAS/.82 Mach

Maximum Flap (Placard) Speeds

Flaps     KIAS

1	280

5	260

10	240

20	230

25	205

30	180

V1 - Takeoff Commit Speed

Standard temperature, sea-level pressure altitude

160 KIAS (880,000 lbs, flaps 10)

Standard temperature, sea-level pressure altitude

155 KIAS (880,000 lbs, flaps 20)

Standard temperature, 5,000' pressure altitude

163 KIAS (880,000 lbs, flaps 10)

Standard temperature, 5,000' pressure altitude

158 KIAS (880,000 lbs, flaps 20)

VR - Rotation Speed (dry runway)

Standard temperature, sea level pressure altitude

177 KIAS (880,000 lbs, flaps 10)

Standard temperature, sea level pressure altitude

171 KIAS (880,000 lbs, flaps 20)

Standard temperature, 5,000' pressure altitude

179 KIAS (880,000 lbs, flaps 10)

Standard temperature, 5,000' pressure altitude

173 KIAS (880,000 lbs, flaps 20)

V2 - Minimum Climb Speed

Standard temperature, sea level pressure altitude

188 KIAS (880,000 lbs, flaps 10)

Standard temperature, sea level pressure altitude

181 KIAS (880,000 lbs, flaps 20)

Standard temperature, 5,000' pressure altitude

188 KIAS (880,000 lbs, flaps 10)

Standard temperature, 5,000' pressure altitude

181 KIAS (880,000 lbs, flaps 20)

VREF - Landing Approach Speed

188 KIAS (850,000 lbs, flaps 25; gear down)

181 KIAS (850,000 lbs, flaps 30; gear down)

134 KIAS (450,000 lbs, flaps 25; gear down)

129 KIAS (450,000 lbs, flaps 30; gear down)

## Specifications

Cruise Speed				0.85 Mach, 565 mph, 910 km/h

Engines					Pratt & Whitney PW4062 63,300 lb 28,710 kg

Rolls Royce RB211-524H 59,500 lb 26,990 kg

General Electric CF6-80C2B5F 62,100 lb 27,945 kg

Maximum Range				7,325 nm (13,570 km) Maximum Certified

Operating Altitude				45,100 ft, 13747 m

Fuel Capacity				57,285 gal, 216,840 L Basic Operating

Weight					403,486 lb, 182,020 kg

Length					231 ft, 10 in, 70.6 m

Wingspan					211 ft, 5 in, 64.4 m

Height					63 ft, 8 in, 19.4 m

Seating					Typical 3-class configuration – Up to 416

Typical 2-class configuration – Up to 524

Typical 1-class configuration – N/A