## BEFORE STARTING ENGINE

- Landing Gear Handle – DOWN

- Water Rudders -- DOWN for taxiing on water or UP for taxiing on land

- Parking Brake – SET

- Switches -- OFF

- Fuel Tank Selector -- BOTH

- Emergency Power Lever -- NORMAL

- Power Lever – IDLE

- Propeller Control Lever

- Fuel Condition Lever – CUTOFF

- Fuel Shutoff -- ON (push in)

- Battery Switch -- ON

- Wing Flaps – UP

## ENGINE START

- Battery Switch -- ON

- Emergency Power Lever -- NORMAL (full aft) position press xx as necessary)

- Fuel Boost Switch -- ON

- "AUX FUEL PUMP ON" Annunciator – ON

- "FUEL PRESS LOW" Annunciator -- OFF

- Confirm -- NO FUEL FLOW

- Starter Switch -- START

- "IGNITION ON" Annunciator -- CHECK ON

- Engine Oil Pressure -- CHECK for indication

- Ng -- STABLE (12% minimum)

- Fuel Condition Lever -- LOW IDLE

- Fuel Flow -- CHECK for 80 to 110 pph

- ITT -- MONITOR (1,090 degrees C max)

- Ng -- 52% MINIMUM

- Starter Switch -- OFF (check "STARTER ENERGIZED"

- Generator -- "CHECK GENERATOR OFF" annunciator OFF

- Fuel Boost Switch -- NORM (check "AUX FUEL PUMP ON" annunciator OFF)

- Avionics Master Switch -- ON

- Navigation Lights and Flashing Beacon -- ON as required

- Suction Gauge -- CHECK

### BEFORE TAKEOFF

- Parking Brake -- SET

- Fuel Boost Switch -- RECHECK NORM

- Fuel Tank Selector -- RECHECK BOTH

- Fuel Shutoff -- RECHECK FULLY ON

- Elevator Trim Controls -- SET for takeoff

- Aileron Trim Controls -- SET for takeoff

- Rudder Trim Controls -- SET for takeoff

- Power Lever -- 400 FT-LBS

- Suction Gauge -- CHECK

- "VOLTAGE LOW" annunciator -- CHECK OFF

- Inertial Separator -- CHECK

- Engine Instruments -- CHECK

- Overspeed Governor – CHECK (stabilized at 1750 +/- 60 RPM)

- Power Lever -- IDLE

- Pitot/Static Heat -- ON when OAT is below 4 degrees C (40 degrees F)

- Ice Protection -- AS REQUIRED

- Avionics -- CHECK and SET

- GPS/NAV Switch -- SET

- Strobe Lights -- AS REQUIRED

- Annunciators -- EXTINGUISHED or considered

- Wing Flaps -- SET at 20 degrees

- Brakes -- RELEASE

- Fuel Condition Lever -- HIGH IDLE

### TAKEOFF ON WATER

- Landing Gear -- UP

- Landing Gear Blue Indicator Lights -- CHECK ILLUMINATED

- Wing Flaps -- 0-20 degrees (20 degrees recommended)

- Rudder Trim -- SET

- Control Wheel -- HOLD AFT

- Power -- SET FOR TAKEOFF (1900 RPM)

- Annunciators -- CHECK

- Water Rudders -- UP

- Control Wheel -- APPLY LIGHT BACK-PRESSURE to lift off.

- Climb Speed -- 85-95 KIAS (with obstacles ahead, climb at 80 KIAS)

- Wing Flaps -- RETRACT after reaching 90 KIAS

### TAKEOFF ON LAND

- Water Rudders -- UP

- Wing Flaps -- 10-20 degrees (20 degrees for short field)

- Power -- SET FOR TAKEOFF (1900 RPM)

- Annunciators -- CHECK

- Rotate -- 65-70 KIAS

- Climb Speed -- 80-95 KIAS (with obstacles ahead, climb at 80 KIAS)

- Wing Flaps -- RETRACT after reaching 90 KIAS

- Landing Gear -- RETRACT

### CRUISE CLIMB

- Ice Protection -- AS REQUIRED

- Pitot/Static Heat -- ON when OAT is below 4 degrees C (40 degrees F)

- Airspeed -- 110-120 KIAS

- Propeller -- 1600-1900 RPM

- Torque -- AS DESIRED

### MAXIMUM PERFORMANCE CLIMB

- Ice Protection -- AS REQUIRED

- Pitot/Static Heat -- ON when OAT is below 4 degrees C (40 degrees F)

- Airspeed -- 104 KIAS (sea level to 10,000 feet MSL) 87 KIAS (20,000 feet MSL)

- Propeller -- 1900 RPM

- Torque -- AS DESIRED (1865 FT-LBS MAXIMUM)

### CRUISE

- Ice Protection -- AS REQUIRED

- Pitot/Static Heat -- ON when OAT is below 4 degrees C (40 degrees F)

- Propeller -- 1600-1900 RPM

- Power -- AS DESIRED

### BEFORE LANDING ON WATER

- Landing Gear -- UP

- Landing Gear Blue Indicator Lights -- CHECK ILLUMINATED

- Water Rudders -- UP

- Fuel Selector -- BOTH

- Fuel Condition Lever -- HIGH IDLE

- Propeller Control Lever -- MAX (fully forward)

- Autopilot -- OFF

- Wing Flaps -- AS DESIRED (flaps down preferred)

0-10 degrees <175 KIAS

10-20 degrees <150 KIAS

20-30 degrees <125 KIAS

Airspeed -- 75-85 KIAS (flaps FULL DOWN)

BEFORE LANDING ON LAND

- Landing Gear -- UP

- Landing Gear Amber Indicator Lights -- CHECK ILLUMINATED

- Water Rudders -- UP

- Fuel Selector -- BOTH

- Fuel Condition Lever -- HIGH IDLE

- Propeller Control Lever -- MAX (full forward)

- Autopilot -- OFF

- Wing Flaps -- AS DESIRED (flaps down preferred)

0-10 degrees <175 KIAS

10-20 degrees <150 KIAS

20-30 degrees <125 KIAS

Airspeed -- 75-85 KIAS (flaps FULL DOWN)

### LANDING ON WATER

- Wing Flaps -- FULL DOWN

- Airspeed -- 75-85 KIAS

- Touchdown -- SLIGHTLY TAIL LOW

- Control Wheel -- HOLD AFT as amphibian decelerates to taxi speed

- Power Lever -- BETA range after TOUCHDOWN, if desired, for faster deceleration

### LANDING ON LAND

- Wing Flaps -- FULL DOWN

- Airspeed -- 75-85 KIAS

- Touchdown -- SLIGHTLY TAIL LOW

- Control Wheel -- EASE FORWARD to lower bow wheels gently to runway.

- Power Lever -- BETA range after TOUCHDOWN

- Braking -- MINIMUM REQUIRED

BALKED LANDING (GO-AROUND)

- Power Lever -- ADVANCE for takeoff power (1900 RPM)

- Wing Flaps -- RETRACT to 20 degrees

- Climb Speed -- 80 KIAS MINIMUM until obstacles are cleared

- Wing Flaps -- RETRACT after reaching safe altitude and airspeed

### AFTER LANDING

- Water Rudders -- DOWN (except on land)

- Fuel Condition Lever -- LOW IDLE

- Wing Flaps -- UP

- Ice Protection Equipment -- OFF

- Strobe Lights -- OFF

- Landing and Taxi Lights -- AS REQUIRED

- Fuel Condition Lever -- LOW IDLE when clear of the runway

### SHUTDOWN AND SECURING AIRPLANE

- Parking Brake -- SET

- Avionics Switches -- OFF

- Fuel Boost Switch -- OFF

- Power Lever -- IDLE

- ITT -- STABILIZED at minimum temperature for one minute

- Propeller Control Lever -- FEATHER

- Fuel Condition Lever -- CUTOFF

- Lighting Switches -- OFF

- Battery Switch -- OFF

- Fuel Tank Selector -- LEFT OFF or RIGHT OFF

REFERENCE INFORMATION

Total Flight Simulator aircraft weight with full fuel

8,000 lbs

VMO - Maximum Operating Speed

175 KIAS

VA - Manoeuvring Speed

143 KIAS (8,000 lbs)

130 KIAS (6,400 lbs)

117 KIAS (5,200 lbs)

VS - Stalling Speed

65 KIAS (flaps up)

VSO - Stalling Speed in Landing Configuration

51 KIAS (flaps 30 degrees)

VFE - Maximum Flaps-Extended Speed

175 KIAS (to 10 degrees flaps)

150 KIAS (10-20 degrees flaps)

125 KIAS (20-30 degrees flaps)

VR - Rotation Speed

50-60 KIAS (on water)

65-70 KIAS (on land)

VX - Best Angle of Climb

81 KIAS

VY - Best Rate of Climb

97 KIAS (sea level)

94 KIAS (10,000 feet MSL)

85 KIAS (20,000 feet MSL)

Cruise Climb

110-120 KIAS

Landing Approach Speed

Flaps Up                  95-105 KIAS

Flaps 30 degrees      75-85 KIAS

Maximum Glide Speed (with cargo pod)

87 KIAS (8,000 lbs)

78 KIAS (6,400 lbs)

70 KIAS (5,200 lbs)

208 Caravan Amphibian Specifications

Maximum Speed					175 KIAS

Cruise Speed					143 KIAS (8,000 lbs)

130 KIAS (6,400 lbs)

117 KIAS (5,200 lbs)

Engine						Pratt & Whitney Canada, Inc., Free Turbine

Flat Rated at 675 Shaft hp PT6A-114A

Propeller Three-Bladed, Constant Speed

Full Feathering, Reversible McCauley

106-inch diameter

Maximum Range					4.6 hours with maximum cruise at 10,000 feet

5.7 hours with maximum cruise at 20,000 feet

6.8 hours with maximum range at 10,000 feet

8.0 hours with maximum range at 20,000 feet

Service Ceiling					13,500 ft

Fuel Capacity					335 gal

Empty Weight					4,895 lb

Maximum Gross Weight				8,035 lb

Length						41 ft, 7 in

Wingspan						52 ft, 1 in

Height						15 ft, 5-½ in

Seating						Up to 14