## BEFORE STARTING ENGINE

- Parking Brake -- SET

- Switches -- OFF

- Fuel Tank Selector -- BOTH

- Emergency Power Lever -- NORMAL

- Power Lever -- IDLE

- Propeller Control Lever -- MAX (fully forward)

- Fuel Condition Lever -- CUTOFF

- Fuel Shutoff -- ON (push in)

- Battery Switch -- ON

- Wing Flaps -- UP

## ENGINE START

- Battery Switch -- ON

- Emergency Power Lever -- NORMAL (full aft) position

- Propeller Area -- CLEAR

- Fuel Boost Switch -- ON

- "AUX FUEL PUMP ON" Annunciator -- ON

- "FUEL PRESS LOW" Annunciator -- OFF

- No Fuel Flow -- CONFIRM

- Starter Switch -- START

- "IGNITION ON" Annunciator -- CHECK ON

- Engine Oil Pressure -- CHECK for indication

- Ng -- STABLE (12% minimum)

- Fuel Condition Lever -- LOW IDLE

- Fuel Flow -- CHECK for 80 to 110 pph

- ITT -- MONITOR (1,090 degrees C max)

- Ng -- 52% MINIMUM

- Starter Switch -- OFF (check "STARTER ENERGIZED" annunciator OFF)

- Engine Instruments -- CHECK

- Generator -- CHECK "GENERATOR OFF" annunciator OFF

- Fuel Boost Switch -- NORM (check "AUX FUEL PUMP ON" annunciator OFF)

- Avionics Master Switch -- ON

- Navigation Lights and Flashing Beacon -- ON as required

- Suction Gauge -- CHECK

- Radios -- AS REQUIRED

## BEFORE TAKEOFF

- Parking Brake -- SET

- Flight Controls -- FREE AND CORRECT

- Flight Instruments -- CHECK and SET

- Fuel Boost Switch -- RECHECK NORM

- Fuel Tank Selector -- RECHECK BOTH

- Fuel Quantity -- RECHECK

- Fuel Shutoff -- RECHECK FULLY ON

- Elevator Trim Controls -- SET for takeoff

- Aileron Trim Controls -- SET for takeoff

- Rudder Trim Controls -- SET for takeoff

- Power Lever -- 400 FT-LBS

- Suction Gauge -- CHECK

- "VOLTAGE LOW" annunciator -- CHECK OFF

- Inertial Separator -- CHECK

- Engine Instruments -- CHECK

- Overspeed Governor – CHECK (stabilized at 1750 +/- 60 RPM)

- Power Lever -- IDLE

- Pitot/Static Heat -- ON when OAT is below 4 degrees C (40 degrees F)

- Ice Protection -- AS REQUIRED

- Avionics -- CHECK and SET

- GPS/NAV Switch -- SET

- Strobe Lights -- AS REQUIRED

- Annunciators -- EXTINGUISHED or considered

- Wing Flaps -- SET at 20 degrees

- Brakes -- RELEASE

- Fuel Condition Lever -- HIGH IDLE

## TAKEOFF

- Wing Flaps -- 20 degrees

- Power -- SET FOR TAKEOFF (1900 RPM)

- Annunciators -- CHECK

- Rotate -- 70-75 KIAS

- Climb Speed -- 85-95 KIAS

- Wing Flaps -- RETRACT to 10 degrees after reaching 85 KIAS and 0 degrees after reaching 95 KIAS)

## CRUISE CLIMB

- Ice Protection -- AS REQUIRED

- Pitot/Static Heat -- ON when OAT is below 4 degrees C (40 degrees F)

- Airspeed -- 110-120 KIAS

- Propeller -- 1600-1900 RPM

- Torque -- AS DESIRED

## MAXIMUM PERFORMANCE CLIMB

- Ice Protection -- AS REQUIRED

- Pitot/Static Heat -- ON when OAT is below 4 degrees C (40 degrees F)

- Airspeed -- 104 KIAS (sea level to 10,000 feet MSL 87 KIAS (20,000 feet MSL)

- Propeller -- 1900 RPM

- Torque -- AS DESIRED (1865 FT-LBS MAXIMUM)

## CRUISE

- Ice Protection -- AS REQUIRED

- Pitot/Static Heat -- ON when OAT is below 4 degrees C (40 degrees F)

- Propeller -- 1600-1900 RPM

- Power -- AS DESIRED

## BEFORE LANDING

- Fuel Selector -- BOTH

- Fuel Condition Lever -- HIGH IDLE

- Propeller Control Lever -- MAX (full forward)

- Autopilot -- OFF

- Wing Flaps -- AS DESIRED

0-10 degrees <175 KIAS

10-20 degrees <150 KIAS

20-30 degrees <125 KIAS

## NORMAL LANDING

- Wing Flaps -- FULL DOWN

- Airspeed -- 75-85 KIAS

- Touchdown -- MAIN WHEELS FIRST

- Power Lever -- BETA range after TOUCHDOWN

- Brakes -- AS REQUIRED

## SHORT FIELD LANDING

- Wing Flaps -- FULL DOWN

- Airspeed -- 78 KIAS

- Power Lever -- REDUCE to IDLE after clearing obstacles

- Touchdown -- MAIN WHEELS FIRST

- Power Lever -- BETA range after TOUCHDOWN

- Brakes -- APPLY HEAVILY while holding elevator control full aft

- Wing Flaps -- RETRACT for maximum brake effectiveness at light weights

BALKED LANDING (GO-AROUND)

- Power Lever -- ADVANCE for takeoff power (1900 RPM)

- Wing Flaps -- RETRACT to 20 degrees

- Climb Speed -- 80 KIAS MINIMUM until obstacles are cleared

- Wing Flaps -- RETRACT after reaching safe altitude and airspeed

## AFTER LANDING

- Wing Flaps -- UP

- Ice Protection Equipment -- OFF

- Strobe Lights -- OFF

- Landing and Taxi Lights -- AS REQUIRED

- Fuel Condition Lever -- LOW IDLE when clear of the runway

## SHUTDOWN AND SECURING AIRPLANE

- Parking Brake -- SET

- Avionics Switches -- OFF

- Fuel Boost Switch -- OFF

- Power Lever -- IDLE

- ITT -- STABILIZED at minimum temperature for one minute

- Propeller Control Lever -- FEATHER

- Fuel Condition Lever -- CUTOFF

- Lighting Switches -- OFF

- Battery Switch -- OFF

- Fuel Tank Selector -- LEFT OFF or RIGHT OFF

# REFERENCE INFORMATION

Total Flight Simulator aircraft weight with full fuel

8,750 lbs

VMO - Maximum Operating Speed

175 KIAS

VA - Maneuvering Speed

148 KIAS (8,750 lbs)

137 KIAS (7,500 lbs)

125 KIAS (6,250 lbs)

112 KIAS (5,000 lbs)

VS - Stalling Speed

63 KIAS (flaps up)

VSO - Stalling Speed in Landing Configuration

50 KIAS (flaps 30 degrees)

VFE - Maximum Flaps-Extended Speed

175 KIAS (to 10 degrees flaps)

150 KIAS (10-20 degrees flaps)

125 KIAS (20-30 degrees flaps)

VR - Rotation Speed

70-75 KIAS

VX - Best Angle of Climb

72 KIAS

VY - Best Rate of Climb

104 KIAS (sea level to 10,000 feet MSL)

87 KIAS (20,000 feet MSL)

Cruise Climb

110-120 KIAS

Landing Approach Speed

Flaps Up                 100-115 KIAS

Flaps 30 degrees      75-85 KIAS

Maximum Glide Speed (with cargo pod)

95 KIAS (8,750 lbs)

87 KIAS (7,500 lbs)

79 KIAS (6,250 lbs)

71 KIAS (5,000 lbs)

208B Caravan Specifications

Maximum Speed 			175 kts

Cruise Speed			164 kts

Engine				Pratt & Whitney Canada, Inc., Free Turbine.

Flat Rated at 675 Shaft hp PT6A-114A

Propeller				Three-Bladed, Constant Speed, Full Feathering,

Reversible McCauley, 106-inch diameter

Maximum Range			5.1 hours with maximum cruise at 10,000 feet

6.6 hours with maximum cruise at 18,000 feet

6.4 hours with maximum range at 10,000 feet

7.2 hours with maximum range at 20,000 feet

Service Ceiling			22,800 ft

Fuel Capacity			335 gal

Empty Weight			4,040 lb

Maximum Gross Weight		8,785 lb

Length				41 ft, 7 in

Wingspan				52 ft, 1 in

Height				15 ft, 5-½ in

Seating				Up to 14

Useful Load			4,745 lb