# BEFORE ENGINE STARTING

- Parking Brake -- SET

- Landing Gear Control -- CHECK DOWN

- Power Levers -- IDLE

- Propeller Controls -- FULL FORWARD

- Condition Levers -- FUEL CUTOFF

- Fuel Quantity -- CHECK

- Anti-ice -- ON

- Beacon -- ON

- "Anti-ice" Annunciators -- ILLUMINATED

ENGINE STARTING (BATTERY)

- Battery Switch -- ON

- Beacon -- ON

- Propeller Area -- CLEAR

- Fuel Flow -- ON

- Right Ignition and Engine Start Switch -- ON

- Right Condition Lever -- LOW IDLE (after the N1 Turbine RPM has stabilized, 12% minimum)

- Right ITT and N1 Turbine RPM -- MONITOR

- "R FUEL PRESS LO" Annunciator -- EXTINGUISHED

- "R OIL PRESS LO" Annunciator -- EXTINGUISHED

- Right Oil Pressure -- CHECK

- Right Condition Lever -- HIGH IDLE

- Right Generator -- ON

- Left Ignition and Engine Start Switch -- ON

- Left Condition Lever -- LOW IDLE (after the N1 Turbine RPM has stabilized, 12% minimum)

- Left ITT and N1 Turbine RPM -- MONITOR

- L FUEL PRESS LO Annunciator -- EXTINGUISHED

- L OIL PRESS LO Annunciator -- EXTINGUISHED

- Left Oil Pressure -- CHECK

- Left Condition Lever -- HIGH IDLE

- Right Condition Lever -- REDUCE TO LOW IDLE

- Left Generator -- ON

- Avionics Master Switch -- ON

BEFORE TAKEOFF (RUNUP)

- Avionics -- CHECK

- Autopilot -- CHECK

- Electronic Pitch Trim Control -- CHECK

- Trim Tabs -- SET

- Primary Governors -- EXERCISE AT 1500 RPM

- Autofeather -- ARM

- Manual Propeller Feathering -- CHECK

- Anti-ice -- AS REQUIRED

- Fuel Quantity, Flight, and Engine Instruments -- CHECK

BEFORE TAKEOFF (FINAL ITEMS)

- Annunciator Lights -- EXTINGUISHED OR CONSIDERED

- Transponder -- ON ALT

- Lights -- AS REQUIRED

- Anti-ice -- AS REQUIRED

- Takeoff speeds (V1, VR, and V2) -- CONFIRM

# TAKEOFF

- Brakes -- HOLD

- Power Levers -- 100% N1 TURBINE RPM (OBSERVE ITT LIMITS)

- Brakes -- RELEASE

- VR -- ROTATE TO APPROX. 10 DEGREES PITCH UP

- Landing Gear -- UP (WHEN POSITIVE CLIMB ESTABLISHED)

- Airspeed -- MAINTAIN V2 UNTIL CLEAR OF OBSTACLES

- Flaps -- UP (BLUE LINE, 125 KIAS MINIMUM)

# CLIMB

- Climb Power -- 90% N1 TURBINE RPM (OR AS DESIRED)

- Propellers -- 1600 RPM (OR AS DESIRED)

- Propeller Synchrophaser -- ON

- Engine Instruments -- MONITOR

- Lights -- AS REQUIRED

# CRUISE

- Cruise Power -- AS DESIRED

- Engine Instruments -- MONITOR

- Autofeather -- OFF

# DESCENT

- Altimeter -- SET

- Autofeather -- ARM

- Fuel Balance -- CHECK

- Power -- AS REQUIRED

# BEFORE LANDING

- Autofeather -- VERIFY ARMED

- VREF -- CONFIRM

- Altimeter -- SET

- Flaps -- APPROACH

- Landing Gear -- DOWN

- Lights -- AS REQUIRED

- Anti-ice -- AS REQUIRED

# NORMAL LANDING

- Flaps -- DOWN

- Airspeed -- VREF

- Condition Levers -- HIGH IDLE

- Propeller Levers -- FULL FORWARD

- Power Levers -- IDLE

After touchdown:

- Power Levers -- GROUND FINE

- Bakes -- AS REQUIRED

# AFTER LANDING

- Engine Auto-Ignition -- OFF

- Anti-ice -- OFF

- Lights -- AS REQUIRED

- Transponder -- STANDBY

- Trim Tabs -- SET

- Flaps -- UP

# SHUTDOWN AND SECURING

- Parking Brake -- SET

- Avionics Master -- OFF

- Autofeather -- OFF

- Lights -- OFF

- ITT -- STABILZED AT MINIMUM TEMPERATURE FOR 1 MINUTE

- Condition Levers -- FUEL CUTOFF

- Propellers -- FEATHER

- Battery and Generator Switches -- OFF (below 15% N1)

## REFERENCE INFORMATION

Total Flight Simulator aircraft weight with full fuel

14,992 lbs

VMO - Maximum Operating Speed (airspeed)

263 KIAS

MMO - Maximum Operating Speed (Mach)

.58 Mach

VA  -  Maneuvering Speed

184 KIAS

Turbulent Air Penetration Speed

170 KIAS

VMC - Air Minimum Control Speed

94 KIAS (flaps up)

93 KIAS (flaps approach)

VLO - Maximum Gear Operating Speed

184 KIAS (extension)

166 KIAS (retraction)

VLE - Maximum Landing Gear Extension Speed

184 KIAS

VFE - Maximum Flaps Extended Speed

202 KIAS (approach)

158 KIAS (fully down)

V1 - Takeoff Decision Speed

(flaps up; 5,000' pressure altitude; standard temperature)

108 KIAS (15,000 lbs)

104 KIAS (<10,500 lbs)

VR - Rotation Speed

(flaps up; 5,000' pressure altitude; standard temperature)

111 KIAS (15,000 lbs)

104 KIAS (<10,500 lbs)

V2 - Takeoff Safety Speed

(flaps up; 5,000' pressure altitude; standard temperature)

117 KIAS (15,000 lbs)

111 KIAS (<10,500 lbs)

VX - Best Angle-of-Climb (2 engines)

125 KIAS

VY - Best Rate-of-Climb (2 engines)

140 KIAS

Cruise Climb

Sea level to 10,000       170 KIAS

10,000 to 15,000          160 KIAS

15,000 to 20,000         150 KIAS

20,000 to 25,000        140 KIAS

25,000 to 30,000        130 KIAS

35,000 to 40,000        120 KIAS

VREF - Landing Approach Speed (flaps down; gear down)

109 KIAS (15,000 lbs)

105 KIAS (14,000 lbs)

102 KIAS (13,000 lbs)

100 KIAS (12,000 lbs)

100 KIAS (11,000 lbs)

100 KIAS (10,000 lbs)

Best Glide Speed

135 KIAS (both engines inoperative)

Specifications

Cruise Speed					315 kts, 363 mph, 583 kph

Engines						Pratt & Whitney 1,050 bhp PT6A-60A

Maximum Range					1,894 nm 2,180 mi 3,508 km

Service Ceiling					35,000 ft 10,668 m

Fuel Capacity					539 U.S. gal 2,040 L

Maximum Takeoff Weight				15,000 lbs 6818 kg

Length						46.7 ft, 14.23 m

Wingspan						57.9 ft, 17.65 m

Height						14.3 ft, 4.36 m

Seating						Up to 11

Useful Load					5,810 lb, 2,635 kg