# BEFORE STARTING ENGINE

- Brakes -- TEST and SET

- Master Switch -- ON

- Avionics Master Switch -- OFF

- Electrical Equipment -- OFF

- Alternator -- ON

- Nav/Strobe Lights -- ON

# ENGINE START

- Propeller -- HIGH RPM

- Mixture -- RICH

- Throttle -- OPEN 1/4 travel

- Boost Pump -- ON

- Ignition Switch -- START (release when engine starts)

- Boost Pump -- OFF

Then:

- Oil Pressure -- CHECK

- Avionics Master Switch -- ON

# BEFORE TAKEOFF

- Flight Controls -- FREE and CORRECT

- Flight Instruments -- CHECK and SET

- Oil Pressure -- CHECK

- Mixture -- RICH

- Fuel Quantity -- CHECK

- Throttle -- 1800 RPM

- Magnetos – CHECK <175 RPM drop on each; <50 RPM drop between

- Propeller -- CYCLE high RPM/low RPM/high RPM

- Boost Pump -- ON (check fuel flow)

- Engine Instruments and Ammeter -- CHECK

- Throttle -- 800-1000 RPM

- Strobe Lights -- AS DESIRED

- Avionics Master Switch -- ON

- Radios and Avionics -- SET

- Brakes -- RELEASE

# TAKEOFF

- Propeller -- HIGH RPM

- Mixture -- RICH (may be leaned above 3000')

- Throttle -- FULL

- Elevator Control -- GENTLY PUSH FORWARD ON STICK (until tail comes up)

- At 65 KIAS, pull back and  fly off the runway

- Propeller -- 2400 RPM

- Climb Speed -- 100 KIAS (normal climb)

- Boost Pump -- OFF

# CRUISE

- Throttle -- AS DESIRED

- Propeller -- AS DESIRED

- Mixture -- LEAN

- Elevator Trim -- ADJUST

# DESCENT

- Throttle -- REDUCE

- Mixture -- FULL RICH

- Propeller -- 2400 RPM

- Elevator Trim -- ADJUST

# BEFORE LANDING

- Boost Pump -- ON

- Mixture -- RICH

- Propeller -- HIGH RPM

- Airspeed -- 80 KIAS

# LANDING

- Airspeed -- 80 KIAS on approach

- 70 KIAS on final

- Trim -- ADJUST

- Touchdown -- ALL THREE WHEELS SIMULTANEOUSLY

- Throttle -- IDLE

- Braking -- MINIMUM REQUIRED

# SECURING AIRPLANE

- Boost Pump -- OFF

- Engine -- RUN for 1 minute at 1000 RPM

- Throttle -- IDLE

- Avionics Switch , Electrical Equipment -- OFF

- Mixture -- IDLE CUT-OFF (pulled fully out)

- Ignition Switch -- OFF

- Master Switch -- OFF

## REFERENCE INFORMATION

Total Flight Simulator aircraft weight with full fuel

1,908 lbs

Speeds are given for Normal (2,028 lbs) and Acrobatic (1,808 lbs) weights.

# AIRSPEEDS

VA  -  Maneuvering Speed

140 KIAS (Normal)

158 KIAS (Acrobatic)

VNO - Maximum Structural Cruising Speed

158 KIAS

VNE - Never Exceed Speed

220 KIAS

VS  -  Stalling Speed

59 KIAS (Normal; forward CG)

55 KIAS (Acrobatic; forward CG)

VX  -  Best Angle-of-Climb Speed

93 KIAS (Normal)

87 KIAS (Acrobatic)

VY  -  Best Rate-of-Climb Speed

104 KIAS (Normal)

96 KIAS (Acrobatic)

Best Glide Speed

90 KIAS (Normal)

80 KIAS (Acrobatic)

# RECOMMENDED ACROBATIC MANEUVER ENTRY SPEEDS

## MANEUVER		MIN		MAX

Horizontal line		VS		VNO

45-degree climbing          	80 KIAS        	VNO

90-degree up                    	158 KIAS       	VNO

45-degree diving              	VS                	VNO

90-degree diving              	VS                 	VNO

1/4 loop climb                  	100 KIAS      	190 KIAS

Looping                             	100 KIAS      	190 KIAS

Stall turn                             	100 KIAS      	190 KIAS

Aileron roll                          	80 KIAS       	158 KIAS

Snap roll                             	80 KIAS       	140 KIAS

Tail slide                             	100 KIAS    	190 KIAS

Spin                                     	VS

Knife edge                          	>150 KIAS

Inverted flight                     	>VS              	190 KIAS

# LOAD FACTOR LIMITATIONS

+6 G/-3 G (Normal)

+10 G/-10 G (Acrobatic)

NOTE:  For explanations of speeds used on this tab, see "V-speeds" in the Glossary section of Help.

## Specifications

Maximum Speed				200 kts, 230 mph, 370 kmh

Cruise Speed				178 kts, 205 mph, 330 kmh

Engine					Textron Lycoming AEIO-540 L1B5 300 hp

Propeller					Three-bladed constant speed

Maximum Range				415 nm, 478 sm, 769 km

Service Ceiling				16,000 ft, 4,877 m

Fuel Capacity				42.3 U.S. gal, 160 L

Empty Weight				1,470 lb, 667.8 kg

Maximum Gross Weight			2,095 lb, 950 kg

Length					23.36 ft, 7.12 m

Wingspan					26.25 ft, 8 m

Height					8.6 ft, 2.62 m

FAA Certified Load Factor			+/- 10 G

Seating					1

Useful Load				625 lb, 283.5 kg