Aeroperlas

Shorts SD3-60

By Robert Versluys

&

Michael Robison

FSRepaints

HTTP://FLY.TO/DFP

##### Index

###### 1	Introduction								3

  - Aircraft data							3

  - Flight model settings					3

  - Installation							3

  - System requirements					3

###### 2	Operating manual							4

2.1	Limitations							4

2.1.1 Weight							4

2.1.2 Speed							4

2.1.3 Operating limitations					4

2.2	Speeds and Fuel						5

2.2.1 Landing, Takeoff and Climb Speeds			5

2.2.2 Long range cruise Speed/ Fuel tables			5

  - Checklist							6

###### 3	Copyrights								9

###### 4	Credits									9

###### 5	Contact information							9

1 About the Shorts 360

The Shorts 360/ SD3-60 is a larger version of the Shorts 330. It first flew at June 1 1981 and was certified on September 3 1982. The first 360 entered service in late 1982 with Suburban Airlines. The aircraft is powered by two Pratt & Withney Canada PR6A65R turboprops (990kW), which power five bladed constant speed Hartzell propellers. The main difference with the 330 is the "normal" tail.

# 1.1 Aircraft data:

Crew:			2 Pilots and 1 Stewardess

Passengers:		36 Passengers

MTOW:		26.000lb 			11792kg

Op. Empty weight:	16.600lb 			7870kg

Length:		70ft10in			21.58m

Height:			23ft10in			7.27m

Wing span:		47ft10in			22.80m

Wing area:		454.0sq ft			42.2m2

1.2 Flight model settings:

Zero fuel weight:	20.224lb			9172kg

Max fuel load:		3776lb				1713kg

MTOW:		24000lb			10885kg

1.3 System requirements:

The aircraft is designed to work with Microsoft Flight simulator Pro 2000. It may work with the standard version, but we don't guarantee this. To make shure the landing lights of this, and other FSDS add-on planes work, you should use the fsuipc add-on, which can be downloaded from a number of internet sites. (for example www.flightsim.com) The visual model is pretty detailed and contains many moving parts. This means you will need a fast computer to use this plane. I tested it on a PIII 550, with 128MB and a 32MB video card. This is the very least you will need to fly this plane smoothly I think.

2 Operating manual:

2.1 Limitations

2.1.1 Weight:

Maximum Ramp				26100lbs

Maximum Takeoff				26000lbs

Maximum Landing				25700lbs

2.1.2 Speed:

Maximum operating speed (VMO)		196kts

Landing gear speeds:

Operating (VLO)				150kts

Extended (VLE)				162kts

Flap speeds:

5						150kts

15						145kts

30						135kts

Rough Air Speed (VRA)

16000lb					148kts

26000lb					155kts

Manoeuvre Speed (VA)

16000lb					129kts

26000lb					159kts

    - Operating limitations:

Air temperature:

Minimum ground				-40C

Minimum Flight				-45C

Maximum					+50C

Wing Speed:

Passenger Doors				50kts

Crosswind					30kts

Maximum Certified Altitude			20000ft

  - Speeds & Fuel

    - Landing, Takeoff and Climb speeds.

Weights in pounds

| Weight x 1000lbs | 19; 20; 21; 22; 23; 24; 25; 26; 26.5. |
| --- | --- |
|  | Takeoff Speeds: |
| VR. V2. 15 Flap; VSEC. | 90; 91; 93; 95; 97; 99; 101; 104; 105. 92; 93; 95; 97; 99; 101; 103; 105; 106. 93; 95; 97; 99; 101; 103; 105; 107; 106. |
|  | Climb Speeds: |
| 2 Engine Climb 0 Flap | 123; 125; 127; 129; 131; 133; 135; 137; 138. |
|  | ## Landing Speeds |
| 30. Flaps; 15. 0. | 93; 93; 96; 98; 100; 102; 104; 106; -. 96; 98; 100; 103; 105; 107; 109; 111; -. 111; 111; 114; 116; 118; 120; 122; 129; -. |

2.2.2 	Long range cruise Speed/ Fuel tables:

Flaps - 0, Air Conditioning – Off, Engine Anti-icing – Off.

Range reduction:

Air Conditioning - on 					– 1.7% decrease in NAM/LB

Air Conditioning & Engine Anti-icing – on		- 5.7% decrease in NAM/LB

# ISA  - 20C :

| Wt. (lb) | 20000 | 22000 | 24000 |
| --- | --- | --- | --- |
| Alt (ft) | IAS Fuel Knots lb/hr | IAS Fuel Knots lb/hr | IAS Fuel Knots lb/hr |
| 2000 4000 6000 8000 10000 12000 | 157; 735. 154; 705. 151; 675. 148; 650. 145; 630. 143; 610. | 161; 775. 157; 745. 155; 720. 151; 685. 159; 670. 147; 655. | 165; 815. 161; 780. 158; 735. 154; 730. 153; 715. 150; 700. |

ISA :

| Wt. (lb) | 20000 | 22000 | 24000 |
| --- | --- | --- | --- |
| Alt (ft) | IAS Fuel Knots lb/hr | IAS Fuel Knots lb/hr | IAS Fuel Knots lb/hr |
| 2000 4000 6000 8000 10000 12000 | 157; 765. 154; 735. 150; 685. 148; 680. 145; 650. 142; 630. | 162; 805. 158; 775. 153; 735. 151; 715. 149; 695. 146; 675. | 165; 845. 161; 815. 157; 780. 155; 760. 153; 740. 150; 720. |

# ISA   + 20C :

| Wt. (lb) | 20000 | 22000 | 24000 |
| --- | --- | --- | --- |
| Alt (ft) | IAS Fuel Knots lb/hr | IAS Fuel Knots lb/hr | IAS Fuel Knots lb/hr |
| 2000 4000 6000 8000 10000 12000 | 157; 790. 154; 755. 150; 725. 147; 700. 145; 675. 143; 655. | 160; 830. 157; 795. 154; 770. 151; 740. 148; 715. 145; 695. | 164; 870. 161; 840. 158; 810. 155; 785. 152; 765. 149; 740. |

2.3 Checklist

# BEFORE START

### CALL F/O						RESPONSE PIC

EXTERNAL/INTERNAL CHECK				COMPLETED

*CIRCUIT BRAKES					CHECKED

INTERPHONE						CHECKED

*ESCAPE HATCH					CLOSED

*BATT L & R						ON

*ELECTRICAL MASTER				INTERNET/EXTERNAL/VOLTS

*EMERGENCY LIGHTS					ARMED

*PASSENGER SIGNS					ON

*LEVELING VALVE					CLOSE

*BOOST PUMPS					ON

*AIR CON/BLEEDS					OFF

FIRE & SMOKE DETECTION				TESTED

*RECIRC S GASPER					OFF

*AVIONICS MASTER					OFF

*FUEL							TOTALIZER SET

*PARK BRAKE						SET – PSI

*POWER LEVERS					FLT IDLE

*PROP LEVERS						FEATHER

*FUEL LEVERS						FUEL OFF

#### CLEARED TO START

DOOR WARNING LIGHTS				OUT

AIR CON BLEED					OFF

ANTI COLLISION					ON

START MASTER					ARMED

*=Station stop

#### AFTER START

START MASTER					NORMAL

IGN. L & R						OFF

ELECT MSTR						INTERNAL

GEN. L & R						ON

VOLTS AMP						CHECK

SCHEDD BUS						NORMAL

ELECT EI's						VERTICAL

AVIONICS						ON SET

RADAR							STBY

EMERGE BRAKE					NORMAL

HYD BRAKE						NORMAL

NOSE STEERING					ON

PROPS							TAXI

### ENGINE TEST

#### DO INTO WIND 45 AUTO FEATHER TEST FIRST FLIGHT OF THE DAY L.BA. SECONDARY STOP & REVERSE PITSCH INDICATION TEST RESERVE POWER SYSTEM TEST

PROPELLER OVERSPEED GOVERNOR TEST BEFORE TEST AND TRAINING FLIGHTS

### BEFORE TAKE OFF/ TAXI

ICE PROT						AS REQUIRED

RESERVE POWER					ARM

FLAPS							15 SELECTED

CONTROL TRIMS					3 SET

FLIGHT DATA RECORD				SET

T.O. DATA & BRIEF					REVIEW

CLEARED FOR TAKEOFF

PILOT HEAT/STALL					ON

IGNITION						AS REQUIRED

BLEED AIR						OFF

TRANSPONS.TCAS					ALT/AUTO

CONTOL LOCKS					OFF

FUEL LEAVER						FLT

PROPS							MAX

CAUTION WARNING LIGHTS				CHECK

LANDING LIGHS/STROBES				ON WHITE

### AFTER TAKEOFF

LANDING GEAR					UP/NO LIGHTS

FLAPS							UP

CLIMB POWER						SET

BLEED AIR						ON

RESERVE POWER					OFF

SYN PHASE						ON

IGNITION						AS REQUIRED

ICE PROT						AS REQUIRED

### CRUISE

CRUISE POWER					SET

CABIN/SIGNS						AS NECESSARY

TREND MONITORING					AS NECESSARY

### DESCENT

AIRCON						CHECKED

ICE PROT						AS REQUIRED

ALTIMETER						SET

APP/LANDING BRIEF					REVIEW

### APPROACH

# CALL F/O						RESPONSE PIC

PASSENGER SIGNS					ON

IGNITION						AS REQUIRED

SYN. PHASE						OFF

X FEED							OPEN/CLOSE

LANDING LIGHT					ON/OFF

# LANDING

LANDING GEAR					3 GREEN

FLAPS							INDICATED

BRAKE							CHECK NO RESIDUE

CAUTION WARNING					CHECK

PROPS							MAX

BLEED AIR						OFF

# AFTER LANDING

CONTROL LOCK					ON

FLAPS							UP

FUEL LEVER						GROUND

PROP							TAXI

RADAR/XPDR						STBY

ICE PROT						OFF

LANDING LIGHT					OFF

AIRCON/BLEED					AS REQUIRED

# SHUTDOWN

EMERGENVY BRAKE					EMERGENCY

PARK BRAKE						SET

POWER LEVERS					FLT.IDLE

PROP LEVERS						FEATHER

PROP LEVERS						OFF

AIRCON BLEED					OFF

GENERATOR						OFF

SHEDD BUS						EMERGENCY

FUEL LEVER						OFF

PAX SIGNS						OFF

EMERG. LIGHT						OFF

BEACON						OFF

BOOST PUMP						OFF (10 % NG)

BATT MASTER						OFF (HYD MAIN SYS 0psi)

BATT L & R						OFF

# 3 Copyrights:

- 	This Shorts 360 is a freeware model, which MAY NOT be uploaded to or included in any commercial website or package.

-	All the files included in this package are copyrighted to Robert Versluys and may not be used without my permission.

-	Repainting of the model is not allowed, unless you have my permission.

-	Should there be any damage done to your system because of using these files, I will not be held responsible.

# 4 Credits:

Design credits:

Visual and Flight model:		Robert Versluys

Aeroperlas livery:			Michael Robison

Other credits:

Michael Robison

For getting my attention to the Shorts, providing me with great and detailed pictures and videos.

Steven J Mathis (SkyWay Enterprises - Shorts Captain)

For providing me with data about the way the shorts handles and thus helping me create the flight dynamics model.

Christopher B. de Barros (Bahamasair - Shorts Pilot)

For providing me with data about the way the shorts handles and thus helping me create the flight dynamics model and for providing the Shorts Checklist.

# 5 Contact information:

For any more information or for repaint requests:

Robert Versluys - versluys@tref.nl

FSRepaints: http://fly.to/dfp

ICQ: 23841530