- Emerald Air  –  A true Gem among VA's

- Boeing 747 - 400  Jumbo

- Aircraft operation and flight manual

- Aircraft Category

- Pilot Qualification Category:	4	(heavy jets)

- Route Utilization Category:	4	(long-haul)

- Approach Speed Category:	D	(up to 165kts)

- ICAO Landing Capability:		II

- Aircraft Description

The legendary Boeing 747 “Jumbo” has been the world’s biggest airliner and the flagship of the world’s premier airlines for more than three decades. More than 1,200 747s have flown a total over 33billion miles. The only version still in production, the 747-400 has got a stretched upper deck, new, aerodinamically enhanced wings, winglets, high-bypass turbofan engines, and a two-pilot glass cockpit with state-of-the-art avionics. The aircraft is capable of carrying 416 - 524 passengers to more than 7,000 nautical miles with the lowest operating coast per seat among all jetliners. After 33 years it is still bigger, flies farther, and carries more people then any other commercial airplane.

- Aircraft Specifications

- Dimensions:

- Length:	231 ft 9 in	70.6 m

- Wingspan:	211 ft 5 in	64.4 m

- Height:	63 ft 8 in	19.4 m

- Capacity:

- Cabin:	416 passengers + 16 crew

- Cargo bay:	6,025 ft3	170,5 m3

- Fuel tanks:	57,285 US Gal	216,840 L

- Weights:

- Empty Weight:	390,980 lb	177,315 kg

- Zero Fuel Weight:	486,090 lb	220,450 kg	(current flight model set to  75% payload)

- Max TakeOff Weight:	875,000 lb	396,825 kg	(max fuel quantity 100% at 75% payload)

- Max Landing Weight:	630,000 lb	285,715 kg	(max fuel quantity   39% at 75% payload)

- Engines:

- Number & Type:	 4 × Rolls-Royce RB211-524H turbofans located under the main wings

- Thrust:	4 × 59,500 lb	279,5 kN	APR

- Avionics:

4-Channel Autopilot with Heading, VOR Radial and GPS Course tracking + Cat II automatic approach capability + automatic IAS and Mach hold through integrated Autothrottle control.

Structural Limitations:

- Max. Operating Mach Number (MMO):	M 0.92

- Max. Operating Airspeed Limit (VMO):	380 KIAS

- Max. Flaps Extended Speed (VFE):

01° (1 notch)	280 KIAS

05° (2 notch)	260 KIAS

10° (3 notch)	240 KIAS

20° (4 notch)	230 KIAS

25° (5 notch)	205 KIAS

30° (6 notch)	180 KIAS

- Max. Gear Extended Speed (VLE):	270 KIAS

- Max. Gear Operating Speed (VLO):	270 KIAS

- Stall  Speed with Full Flaps (VSO):	125 KIAS

- Performance Data:

- Max. Cruise Speed:	500 kts	930 km/h

- Max. Certified Altitude:	44,985 ft	13,715 m

- Max. Range:	7,320 nm	13,570 km	(without reserve fuel)

- Runway Requirements:

- Min. TakeOff Runway Length:	11,000 ft	3,353 m	(at Sea Level, MTOW)

- Min. Landing Runway Length:	11,000 ft	3,353 m	(at Sea Level, MLW)

- Max. T/O & Land Crosswind Component:	40 kts

- Max. T/O & Land Tailwind Component:	12 kts

- Flight Characteristics

- Takeoff:

T/O Weight		Fuel	Flaps	Power	V1	VR	V2

610,886 lb	277,046 kg	33%	10°	94% N1	130	145	160	KIAS

735,682 lb	333,642 kg	66%	10°	96% N1	145	160	174	KIAS

860,478 lb 	390,240 kg	99%	10°	98% N1	160	175	188	KIAS

- Note: Never exceed 100% N1 in Takeoff Phase.

- Climb (100% fuel):

Altitude	Power	Airspeed	Mach	Climb Rate

< 10,000 ft	92% N1	300 KIAS		3,600 fpm

< 15,000 ft	92% N1	300 KIAS		3,200 fpm

< 20,000 ft	92% N1	300 KIAS		2,700 fpm

< 25,000 ft	92% N1	300 KIAS		2,200 fpm

< 30,000 ft	91% N1		M 0.69	1,800 fpm

< 35,000 ft	91% N1		M 0.69	1,200 fpm

< 40,000 ft	91% N1		M 0.69	   600 fpm

Note: Never exceed 95% N1 in Climb Phase. Comply with speed restrictions under FL 100.

Typical Cruise (FL 350):

Cruise	Power	Mach	KIAS	KTAS	Fuel Consumption

Normal	81% N1	M 0.85	299	490	29,000 PPH	4,392 GPH

Fastest	92% N1	M 0.87	307	500	33,600 PPH	5,090 GPH

- High Altitude Cruise (FL 400):

Cruise	Power	Mach	KIAS	KTAS	Fuel Consumption

Normal	79% N1	M 0.83	260	480	22,000 PPH	3,332 GPH

Fastest	88% N1	M 0.85	265	490	24,400 PPH	3,696 GPH

- Note: Never exceed 93% N1 in Cruise Phase.

- Descent:

Altitude	Power	Airspeed	Mach	Descent Rate

> 25,000 ft	45% N1		M 0.69	2,500 fpm

< 25,000 ft	40% N1	300 KIAS		2,500 fpm

Note: Let airspeed build up then apply speed brakes when necessary. Comply with speed restrictions under FL 100.

- Approach:

Distance	Flaps	Power	Airspeed	Descent Rate	(3° G/S)

LOC active	10°	58% N1	215 KIAS

G/S  active	20°	58% N1	185 KIAS	970 fpm

Outer Marker 	30°	58% N1	160 KIAS	840 fpm

- Note: Reduce your airspeed and descent rate by deploying flaps gradually.

- Landing:

Landing Weight	Fuel	Flaps	Power	VTT

535,252 lb	242,744 kg	13%	30°	52% N1	141	KIAS

584,414 lb	265,040 kg	26%	30°	54% N1	148	KIAS

633,576 lb 	287,336 kg	39%	30°	56% N1	154	KIAS

Note: At 50 feet AGL throttle back & flare. On touchdown gently push forward yoke to bring the nose wheel down. When the nose gear is down, apply reversers and spoilers. At 60 kts disengage reversers and apply full wheel brakes. Do not attempt to land over certified MLW!

- Fuel Management

- average fuel consumption during climb from SL to FL 350:	54,450 PPH	8,248 GPH

- average fuel consumption during normal cruise at FL 350:	29,000 PPH	4,392 GPH

- average fuel consumption during descent from FL 350 to SL:	13,000 PPH	1,970 GPH

- Fuel required to cover a given distance (at FL 350, no headwind, D = distance in nm):

15125 + 59.2×(D-180) + 3780 lbs	2290 + 8.97×(D-180) + 575 Gal

IMPORTANT NOTICE: the above manual describes the flight characteristics of the aircraft as modeled in Microsoft Flight Simulator 2000™. Under no circumstances may any part of this manual be used in real life aviation!