# Project Airbus Tutorial Flight

# Airbus A300-600

Contents

-History                page 3

-Introduction       page 4

-Flight Plan         page 5

-Tutorial Flight   page 5

-Checklists           page 10

-Glossary              page 19

-Copyrights and Disclaimer		page 22

###### History of the A300-600 Series

With the success of the A300-B2 and A300-B4 series, Airbus’s A300 design was revised significantly in 1980 for a ‘Second Generation’ introducing new features. The ‘go-ahead’ was given on the 16th of December 1980 and the A300-600 series was born. The A300-600’s fuselage is longer than the earlier variants, greater passenger seating of up to 360 in a single class layout and freight capacity, composite materials was introduced, resulting in significant aircraft weight reduction. Other modifications include wing-tip fences, all-speed inboard aileron and deletion of the outer aileron, single slotted flaps and introduction to a mix of standard gauges and CRT screens, a first in an Airbus aircraft..

The launch customer of the A300-600 was Saudia Arabia, powered by Pratt & Whitney JT9D-7R4H engines. This aircraft first flew on the 8th of July 1983 and entered service nine months later. The certification of General Electric’s CF6-80C2 engine was given and this engine choice proved to be the most popular. The A300-600 series project continued with the extended range A300-600R. The inaugural flight of the A300-600R was on then 9th of December 1987 and entered service with American Airlines.

Airbus offers a range of options to its customers including the -C4 and F4. The -C4 is a convertible whilst the -F4 is a freighter including a side loading door and strengthened floor. There have been poor sales with the C4’s; the sole customer being Amiri Flight of the Abu Dhabi government. Despite the poor C4 orders, Federal Express has orders for over 60 Airbus A300-600F’s and United Parcel Service (UPS) has order for over 30 and options for 30 more A300-600F4’s these boost of orders will keep the A300 line open for a few more years. Much credit has to go to the A300 series because without the success of the A300, Airbus would not be one of the leading aircraft manufacturers it is today.

Introduction

Many of us would like to fly as commercial pilots or just handling the controls of a general aviation aircraft. Obtaining a Private Pilots License is a relatively easy goal however the goal to becoming a commercial pilot is a difficult one. Many of us can only dream taking the controls of an airliner, and what Project Airbus has done, is brought that dream to you.

Introducing the all-new Project Airbus Version 3.0 Airbus A300-600 series. We have spent 8 months using aircraft specification books, aircraft manuals, pictures and live spotting, trying to make this one of the most visually correct and realistic handling Airbus’s available for FS2000.

## Flight Simulator Start-up

The flight will begin at Hong Kong International Airport at Chek Lap Kok (VHHH), select ‘Airports’ under the ‘World’ heading and type in Hong Kong in the search section, you can taxi the aircraft to a gate for a more realistic approach to it or if you want to get on your way select Runway 25R.

(If you would some good Hong Kong Int. scenery run a search for Thomas Kwong at flightsim.com, Avsim or simflight)

This is the Flight Plan you will be flying. If you are flying an aircraft with an FMC you can log in all the waypoints into it, make sure to remember to set your altitudes when flying and know when to ascend and descend, if you are using a panel with out an FMC, just fly normally with the corresponding VOR frequency with the waypoint.

| ID | Name | Route | Heading | Distance | Altitude |
| --- | --- | --- | --- | --- | --- |
| EGLL | Heathrow |  |  |  | 80 |
| OCK | OCKHAM | UR123 | 182 | 10.4 | 9000 |
| STEPC | Step Climb Point | UB3 | 30 | 19 | 23000 |
| BMK | Brookmans Park | UB3 | 30 | 10.6 | 25000 |
| LAM | LAMBOURNE | UG39 | 128 | 11.4 | 29000 |
| STEPC | Step Climb Point | UG39 | 104 | 3 | 33000 |
| GILDA | GILDA- Intersection | UG39 | 104 | 13.3 | 33000 |
| REPLO | REPLO- Intersection | UG39 | 104 | 21 | 37000 |
| TEBRA | TEBRA-Intersection | UG39 | 104 | 18 | 41000 |
| *TOC* | Top of Climb | UG39 | 105 | 5 | 41000 |
| RAPIX | RAPIX- Intersection | UG39 | 104 | 9.7 | 41000 |
| DENUT | DENUT- Intersection | UL608 | 104 | 63.4 | 41000 |
| BUB | BRUSSELS | UL608 | 125 | 38.8 | 41000 |
| TERLA | TERLA- Intersection | UL608 | 109 | 44.7 | 41000 |
| BATTY | BATTY- Intersection | UB69 | 108 | 7.3 | 41000 |
| RINTA | RINTA- Intersection | UL610 | 107 | 15.2 | 41000 |
| *TOD* | Top of Decent |  | 111 | 16 | 41000 |
| STEPD | Step Decent Point |  | 111 | 15 | 35000 |
| STEPD | Step Decent Point |  | 111 | 18 | 24000 |
| EDDF | Frankfurt/Main |  | 112 | 46.5 | 364 |
|  | Total Distance |  |  | 386.1 |  |
|  | Alternate Airport |  |  |  |  |
| EDDL | Dusseldorf |  |  | 101.8 | 147 |

Tutorial Flight Project Airbus A300-600 London Heathrow (EGLL) to Frankfurt/Main (EDDF)

This tutorial is for those who will like to fly the Airbus A300-600 in a more realistic approach. It will help with your navigation skills using GPS and autopilot input or get more familiar with the Flight Management Computer (FMC) if one is provided in the panel you are using.

The flight from London Heathrow (EGLL) to Frankfurt/Main (EDDF) Lufthansa flight LH4567, departing at 13:05 will take approximately 2hours 35 minutes, due to arrive at 15:40 depending on weather conditions.

I hope this will help you to fly our Airbus A300-600 and discover the realistic features and advanced flight dynamics this Project Airbus aircraft has to offer to you.

### Pre-flight

When doing the Exterior Inspection (provided on the checklist page) you might want to check out the features and textures.

Startup

When you’re in the cockpit and ready for startup, make sure you have accomplished the before start checklist. Initialize pushback and then proceed with the engine start. Start procedures may vary so we start engine no. 1 first, wait for it to stabilize, and start engine no. 2. Once both of the engines are stabilized we then go through the after start checklist. After accomplishing the after start checklist, we are now then ready for taxi.

Taxi

With a gross weight of around 140 tons, we should be able to taxi at idle power. We just need to spool the engines up to around 40% N1 to make the aircraft start moving. A nice procedure is to set the takeoff flaps once you release the parking brake to start taxiing. (Normal takeoff flaps setting is slats full and flaps 15 for the A300-600) Also increase thrust during sharp turns. With idle power, the aircraft should be able to accelerate to around 20kts so tap on the brakes to maintain a safe taxi speed of around 15-18kts. Accomplish the before takeoff checklist during the taxi period when nearing the holding point for the runway.

Takeoff/Climb

When cleared for takeoff, turn on the strobes and runway turn-off lights and taxi to the centerline of the runway and turn on your landing lights. To start the takeoff roll, spool the engines up to around 60% N1 and wait for both of the engines to stabilize to give an even acceleration for takeoff. Once they’re stabilized, advance the thrust levers to takeoff power. (Normally around 96-104% N1) PNF calls out power set at 60kts and PF confirms it by saying, “check”. PNF calls out “100 knots” when reaching 100 knots and PF crosschecks the instruments. At V1 (around 139kts), the PNF calls out “V1” and the PF removes his hand from the thrust levers and places it on the control column. At Vr, PNF calls out “Rotate” and the PF pulls back on the yoke to raise the aircraft to a pitch attitude of 8 degrees, then to a pitch attitude of 15 degrees maintaining a V2+10 or V2+15 speed of around 160kts. Call for gear up once a positive rate of climb is established. Maintain this 15-degree, V2+10/15 attitude and speed until thrust reduction, which usually occurs at around 1500ft. AGL. During thrust reduction, reduce thrust to climb power and pitch down to around 12.5 degrees. Follow the flap retraction procedures as scheduled.

When flaps are up, call and accomplish the after takeoff checklist. Don’t forget to turn off the non-required lights. (Landing lights, runway turnoff, taxi)

Normal procedure is to not go faster than 250 knots IAS below 10,000 ft. At 10,000’, reduce your climb rate to around 1200-1400fpm to accelerate to a climb speed of around 300kts. Once nearing 300kts, increase climb rate to around 2400fpm. The climb rates at a GW of around 140-160T are as follows:

Typical FL330 cruise @ Mach .76-78

Below 10,000’:

250kts, 2800-3400fpm

10,000-18,000’:

290-300kts, 2300-2600fpm

18,000-FL250

290-300kts, 1700-2200fpm

FL250-FL290

290/300/Mach .76, 1300-1700fpm

FL290-FL330

Mach .76, 800-1200fpm

Cruise

Typical FL330 cruise is at Mach .76-78 with fuel flows reading around 2500 kg/h for each engine. N1 numbers should be at the 89-93% range.

Descent:

Formula for a normal descent is altitude (without the zeros) multiplied by three.

Ex.

FL330:

33 x 3=  99 (99 NM will be your top of descent)

Some pilots prefer to add or subtract a few nautical miles from the formula. My personal preference is to subtract 5-10nm for this particular aircraft. So if I would subtract 10 from the computed TOD, my TOD would therefore be at 89nm from the station.

How will I know the descent rate for my indicated airspeed?

Simple- groundspeed divide by two.

Ex.

Groundspeed= 460kts

460 divide by 2= 230

Add a zero, our decent rate will therefore be at around 2,300fpm.

How do I compute for groundspeed if I only have an IAS indicator?

Let’s say you want to plan your decent at 300kts indicated at flight level 330 or 33,000 ft:

How do I compute for groundspeed if I only have an IAS indicator?

-Altitude multiplied by 20 percent. (330 x .20=66)

-Get the answer and multiply it with your IAS. (300kts x 66=198)

-Get the answer and add it to your IAS. (300+198=498) Answer will be your groundspeed.

300kts @ FL330

330 x .20=66

300 x 66=198

300+198=498 (this is our groundspeed)

498 divide by 2= 249 or 2,500fpm rounded off.

These are just some simple rule of thumbs for aviation that you may want to learn.

### Decent

Maintain your descent speed and descent rate until approaching 10,000’. Be sure you’re at or below 250kts when passing 10,000’. Descent rate should be around 1,400-1,500fpm at 250kts. Approach checklist should be done when approaching transition altitude.

At 220kts, you may start extending the slats. Flaps 15 should be set at around 190kts. You may maintain 160-170kts with Flaps 15.

Landing

On the ILS, wait for the GS dot to become active and call for gear down and arm the spoilers when the GS dot nears the center. Also start slowing down to Vref +5/10 when gear is extended. Follow flap extension procedures as scheduled and accomplish the before landing checklist. At a landing weight of 140-150T, Vref should be around 138-142kts so try maintaining around 142-145kts for the approach with full flaps.

Flare happens at around 5 degrees pitch up and should be initiated at around 30ft AGL. (Personal preference really so this will depend on you)

As soon your mains touch down, confirm speed brakes are up, and engage reverse thrust. Full reverse should only be used at or above 70kts. Idle reverse may be used until a safe taxi speed.

After landing

When runway is vacated. PNF selects flaps up, puts down the speed brakes, and turns off the landing lights and strobes. Also start the APU. Accomplish the after landing checklist.

### Parking

Call for “Cabin crew doors to manual” or  “Cabin crew disarm slides”. When parked, set parking brakes and shut down the engines. Don’t forget to turn off the seatbelt signs. Accomplish the shutdown checklist.

By following these procedures, your flying experience will be much closer to the real thing. We hope you enjoy flying our A300-600.

Check-lists

EXTERIOR INSPECTION

1. Left Forward Fuselage

- Static Ports: CLEAR

- Exterior Lights: CONDITION

- Angle of Attack Sensor: CONDITION

- Toilet Service Access Door: CLOSED

____________________________________

2. Nose Section

- Pitot Tubes and Covers: CHECK/REMOVED

- TAT Probes: CONDITION

- Randome and Latches: CHECK

- Oxygen Compt Verturi: CHECK

- Oxygen Over-pressure Disk: GREEN

____________________________________

3. Nose Gear

- Nose Wheel Chocks: IN PLACE

- Wheels and Tires: CONDITION

- Nose Gear Structure: CONDTION

- Taxi Lights: CONDITION

- External Power Access Door: CLOSED (if EXT PWR not necessary)

- Nose Wheel Steering Selector and By-pass Pin: CHECK

- Hydraulic Lines and Cables: CONDITION

- Wheel well: CHECK

- Safety Pin: REMOVED

____________________________________

4. Right Forward Fuselage

- Cargo Doors: CHECK

- Cargo Loading Control Door: CHECK

- Cargo Door selector Access Door: CHECK

- Exterior Lights:

- Static Ports: CLEAR

- Angle of Attack Sensors: CONDTION

- Lower Fuselage Access Doors: CLOSED

____________________________________

5. Lower Center Fuselage

- Extract Valve/Fan: OPEN and RUNNING

- Antennas: CONDITION

- Ground Air-conditioning Door: CLOSED

- Fresh Water Service Door: CLOSED

- Ram Air Inlet Flap: CLOSED

- Ground Air-conditioning Engine Starting Door: CLOSED

- Pack Air Intakes: CLEAR

- Landing Gear Access Door: CLOSED

- Refueling Electric Control Panel: CLOSED

- Packs Air Outlets and Flaps: CLEAR

____________________________________

6. Right Center Wing

- Wing Ventilation Air Intake: CLEAR

- Right-hand Inner Fuel Tank Water Drain Valves: CHECK

- Drip Sticks: SECURED

- Landing Light: CONDITION

- Yellow Hydraulic Access Door: CLOSED

- Ram Air Turbine Doors: CLOSED

- Slat 1 and Krueger: CONDITION

____________________________________

7. Engine #2 Left Side

- Cowlings: CLOSED/LATCHED

- Drain Mast: CHECK

- Hydraulic Filter Access Door: CLOSED

- Starter Valve Manual Override Access Door: CLOSED

- Engine Inlet and Fan Blades: CHECK

____________________________________

8. Engine #2 Right Side

- Anti-Ice Exhaust: CLEAR

- Engine Oil Access Door: CLOSED

- Turbine Exhaust: CLEAR

- Pylon Overpressure Door: CLOSED

- Fire Agents Bottles Access Door: CLOSED

- Pylon Drains: CHECK

____________________________________

9. Right Wing Leading Edge

- Slats 2 and 3: CONDITION

- Refuel Plugs: CHECK

- Drip Sticks: SECURED

- Right-hand Outer Fuel Tank Water Drain Valves: CHECK

- Surge Tank Air Inlet: CLEAR

- Fuel Vent 0/ Pressure Disk: INTACT

- Wing Tip: CHECK

- Navigation and Strobe Lights: CHECK

____________________________________

10. Right Wing Trailing Edge

- Static Dischargers: CHECK

- Control Surfaces: CHECK

- Flap Track Fairings: CHECK

- Flaps: CONDITION

____________________________________

11. Main Right Landing Gear and Fuselage

- Safety Pin: EMOVED

- Wheel Cocks: REMOVED

- Wheels and Tires: CONDITION

- Brake and Brake wear Indicators: CONDITION

- Proximity Sensors: CLEAN

- Hydraulic Lines: CHECK

- Pitch Damper Pressure Drop Indicator: CHECK

- Landing Gear Structure: CHECK

- Gear Uplock Hook: CONDITION

- Downlock Spring: CHECK

- Wheel well: CHECK

- Landing Gear Door: CLOSED

- Water Drain Mast: CHECK

- Antennas and Beacon: CHECK

____________________________________

12. Right Aft Fuselage

- Cargo Loading Control Access Door: CHECK

- Cargo Compartment Door Hydraulic Selector Access Door: CHECK

- Cargo Door Lock-Unlock Control Lever: CHECK

- Cargo Doors: CHECK

- Outflow Valve: CHECK

- Toilet Service Access Door: CHECK

- Tailcone Access Door: CLOSED

- Right-hand Landing Gear Downlock Indicator Pin: EXTENDED

- Tail Skid: CHECK

____________________________________

13. Tail Section

- Stabilizer and Elevator: CHECK

- Fin and Rudder: CHECK

- Static Dischargers: CHECK

- Drip Stick: SECURED

____________________________________

14. APU

- APU Doors: CLOSED

- APU Air Intake: OPEN

- APU Drain and Vents: CHECK

- Oil Cooler Exhaust: CHECK

- APU Exhaust: CLEAR

- APU Fire Agent Discharge Indicator: CHECK RED DIAPHRAGM IN PLACE

____________________________________

15. Left Aft Fuselage

- Left-hand Landing Gear Downlock Indicator Pin: EXTENDED

- Stabilizer and Elevator: CHECK

- Fresh Water Drain: CHECK

- Green Hydraulics Access Door: CLOSED

____________________________________

16. Main Left Landing Gear and Fuselage

- Safety Pin: REMOVED

- Wheelwell: CHECK

- Landing Gear Door: CLOSED

- Wheel Cocks: REMOVED

- Wheels and Tires: CONDITION

- Brakes and Brakeswear Indicator: CONDITION

- Proximity Sensors: CLEAN

- Hydraulic Lines: CHECK

- Pitch Damper Pressure Drop Indicator: CHECK

- Landing Gear Structure: CHECK

- Gear Uplock Hook: CONDITION

- Downlock Spring: CHECK

____________________________________

17. Left Wing Trailing Edge

- Flaps: CONDITION

- Flaps Track Fairings: CHECK

- Control Surfaces: CHECK

- Static Discharger: CHECK

____________________________________

18. Left Wing Leading Edge

- Navigation and Strobe Lights: CHECK

- Wing Tip: CHECK

- Surge Tank Air Inlet: CLEAR

- Fuel Vent 0/Pressure Disk: INTACT

- Left-hand Outer Fuel Water Drain Valves: CHECK

- Drip Sticks: SECURED

- Slats 2 and 3: CONDITON

____________________________________

19. Engine #1 Left Side

- Pylon Drains: CHECK

- Fire Agent Bottle Access Door: CLOSED

- Pylon Overpressure Door: CLOSED

- Cowlings: CLOSED/LATCHED

- Drain Mast: CHECK

- Hydraulic Filter Access Door: CLOSED

- Starter Valve Manual Override Access Door: CLOSED

- Engine Inlet and Fan Blades: CHECK

____________________________________

20. Engine #1 Right Side

- Anti-Ice Exhaust: CHECK

- Engine Oil Access Door: CLOSED

- Turbine Exhaust: CLEAR

____________________________________

21. Left Center Wing

- Slat 1 and Krueger: CONDITION

- Blue Hydraulic Access Door: CLOSED

- Landing Lights: CONDITION

- Drip Sticks: SECURED

- Left-hand Inner Fuel Tank Water Drain Valve: CHECK

- Wing Ventilation Air Intake: CLEAR

____________________________________

22. Entering Cockpit

- Gear Pins: ON BOARD

- Exterior Lights: AS REQUIRED

____________________________________

____________________________________

COCKPIT PREPARATION

____________________________________

1. Overhead Panel

- No Smoking Sign: AUTO

- Seatbelts Sign: ON

- Hydraulic Power: CHECK

- Flight Recorder Ground Control: ON

- ISDU: OFF

- Strobe Lights: Auto

- Beacon: OFF

- Navigation Lights: NAV/LOGO ON

- Anti-Ice Panel: CHECK

- Pitch Trim 1 and 2: ON

- Yaw Damper: 1 and 2: ON

- ATS: ARM

- Electrical Power Panel: CHECK

- Engine #1 Fire Handle: IN/GUARDED

- Engine #1 Fire Protection: TEST

- Electrical Indication Panel: SET AS DESIRED

- Cockpit Voice Recorder Test: PRESS

- Engine Start Selector: OFF

- Fuel Quantity Panel: CHECK

- Fuel Panel: SET (refueling complete)

- APU Fire Handle: IN/GUARDED

- APU Fire Protection: TEST

- HF Radios and SELCAL: AS REQUIRED

- Cabin Pressure Panel: CHECK

- Cargo Compartment Smoke Detector: CHECK

- Oxygen Panel: CHECK

- Engine #2 Fire Handle: IN/GUARDED

- Engine #2 Fire Protection: TEST

- Emergency Exit Lights: ARM

- Air Bleed Panel: AS REQUIRED

- Compartment Temperature Panel: CHECK

- Landing Gear Annunciator: CHECK

- Flight Management System (FMS): CHECK/SET

____________________________________

2. Captain/First Officer Side Console

- Hydraulic Ram Air Turbine Handle: STOWED

- SVCE Interphone Jacks: OFF

- Sniffer Fan: CHECK

- Oxygen Mask: TEST

____________________________________

3. Captain/ First Officer Instrument Panel

- GPWS Selector: SAFETIED

- Captain Switching: CHECK

- Primary Flight Display: CHECK

- Navigational Display: CHECK

- Compass Indications: X-CHECKED

- Standby Airspeed Indicator: CHECK

- Vertical Speed indicator: CHECK

- Clock: CHECK

- Altimeters: SET and CHECK

- GPWS: TEST

- First Officer Switching: CHECK

____________________________________

4. Glareshield

- NAV Selector: NAV

- FD/FPV Switch: ON

____________________________________

5. Center Instrument Panel

- Standby Altimeter: CHECK

- QNH: SET

- Brake Selector: NORMAL

- Anti Skid Selector: ON

- SFPI: CHECK

- Engine Instruments: CHECK

- Oil Quantity Bugs: SET

- Thrust Rating Panel: CHECK

- Landing Elevation: SET

- Landing Gear Warning Test: PERFORM

____________________________________

6. Pedestal

- ADF: AS REQUIRED

- Radios: CHECK

- VHF COM: SET

- Speedbrake: ARM

- ATC Transponder: AS REQUIRED

- Fuel Levers: OFF

- ACARS: SET

- Emergency Audio Cancel Button: GUARDED

- ILS: SET

- FMS 1 and 2: CHECK/SET

- FCU: SET

____________________________________

7. Final Cockpit Preparation

- Takeoff Data: PREPARE

- FMS: SET

- Speed Selector: V2 Set

- Thrust Rating Panel: SET

- Fuel Quantity: CHECK

- Fuel Receipt: CHECK and SIGN

- Departure Briefing: PERFORM

- Seats: ADJUST

____________________________________

NORMAL PROCEDURES

Before Engine Start Checklist

- Doors: CHECK CLOSED

- Sliding Windows: CHECK CLOSED

- Beacon: ON

- Parking Brake: AS REQUIRED

- Flight Recorder: ON

- No Smoking Sign: AUTO

- Seatbelt Sign: ON

- Oxygen Systems: CHECK

- Oxygen Masks: CHECK

- Take Off Speeds: CALLOUT V1/Vr/V2

- Altimeters: SET and X-CHECKED

- Fuel: ANNOUNCE QUANTITY

- TRP: ANNOUNCE

- Speedbrake: ARMED

____________________________________

Taxiing Checklist

- Ground Crew Clearance: RECEIVED

- Parking Brake: RELEASE

- Taxiing: PERFORM

- Brakes: CHECK

- Slats Extension: Take Off Setting

- Flaps Extension: Take Off Setting

____________________________________

Cruise Checklist

- Seat Belts Sign: AS REQUIRED

- Cruise N1: CHECK AND SET

- Oil Quantity: CHECK

- Aircraft Trimming: PERFORM

- ECAM System Display: CHECK

- Fuel Consumption: MONITOR

- Flight plan Program: MONITOR

____________________________________

Descent Checklist

- Altitude Select: SET

- Target Speed: SET

- Autopilot: AS REQUIRED

____________________________________

Final Checklist

- Gear Lights: 3 GREEN

- Speedbrake: ARMED

- Slats Extension: Landing Setting

- Flaps Extension: Landing Setting

____________________________________

Landing Checklist

- Touchdown: PEFORM

- Reverse Levers: PULL

- Brakes: AS REQUIRED

- Deceleration: MONITOR

- Eighty Knots: Announce

- Reverse: IDLE/STOW

- Taxiing: PEFORM

____________________________________

Leaving Aircraft Checklist

- ECAM: DIM/OFF

- Speedbrake: DISARM

- EFIS: DIM/OFF

- Carpet Heat: OFF

- ISDU: OFF

- MSUs: OFF

- Fuel Pumps: OFF

- Crew Oxygen: OFF

- Emergency Exit Lights: DISARM

- APU Bleed: OFF

____________________________________

Parking Procedure Checklist

- Gear Pins: INSTALLED

- Wheel Cocks: IN PLACE

- Water Drain System: SERVICE AS REQUIRED

- Engine Inlets and Pitot: COVER IN PLACE

- Service Panels: CLOSED

- Doors and Windows: CLOSED

- Warning Lights: POSITION AROUND THE AIRCRAFT

- Ground Equipment: REMOVED

### Glossary

##### ADF (Automatic Direction Finder)

A radio that can determine the direction to an NDB or commercial AM radio station automatically.

ALS (Approach Light System)

An ALS provides a visual mean of determining the landing threshold. There are different types employed including ALSF, MALS, MALSF, MALSR, ODALS, REIL, SSALF, SSALR, SSALS and VASI. These all incorporate different colour lights, pulsating lights or sequenced lights.

ATIS (Automatic Terminal Information)

This is a radio frequency associated with an airport and its operation. ATIS provides a loop message providing information about local weather and information about the airport operations. ATIS frequencies can be found on aeronautical charts.

DH (Decision Height)

This is a point on an ILS glide slop in which a pilot must decide whether to continue the approach for landing or execute a missed-approach. In order to continue the approach, the minimum requirements to continue the approach must be met. DH is given as an altitude and/or height above ground. There are three categories of ILS (Instrument Landing System) approaches, with its’ own minimum decision height.

##### Displacement Threshold

This is a location on the runway, identified with markings on the runway that indicates the point for an aircraft to land safely.

DME (Distance Measuring Equipment)

A radio transmitter that enables a NAV radio to determine the distance in nautical miles from the VOR. If you are given the distance and bearing from the VOR, a position fix can be established.

FAF (Final Approach Fix)

This is a point on an instrument on approach where final decent for landing starts. It is usually located at the outer marker beacon.

GPS (Global Positioning System)

The Global Positioning System, known as GPS consists of 24 satellites that transmit accurate time signals in which a GPS receiver can resolve the signal from 3 or more satellites to a latitude/longitude position and it can measure altitude from five or more satellites.

ILS (instrument Landing System)

The ILS is a system of navigational aids and approach lights that provide both horizontal and vertical guidance to an aircraft, providing it with a 3 dimensional representation.

##### Latitude

Latitude is the angular distance from the equator north or south to the geographic poles. The unit it is measure in is Degrees 0-90 north or south. The lines of latitude are parallel to each other and are called parallels of latitude. 1 degree of latitude is equal to 60 nautical miles and a minute of latitude is equal to 1 nautical mile. (Also see Longitude)

LOM (Locator Outer Marker)

The LOM is a navigation aid and is similar to a NDB. It transmits a non-directional signal used to locate the outer marker of an airport runway. You tune the ADF to the frequency of the LOM and it displays the bearing from your position to the transmitter.

Longitude

Longitude is the angular distance east or west of the prime meridian. This is measured in degrees east or west from (0-180). The “prime meridian” is the circle that runs through Greenwich in England. A line of longitude is called a meridian and runs through each geographic pole. Meridians of longitude converge at the poles. (Also see Latitude)

MAP (Missed Approach Point)

This is the point on an instrument approach where the Decision Height (DH) is reached and a decision must be made to continue with the approach or execute a missed approach.

##### NM (Nautical Mile)

A Nautical Mile is equal to 1852 metres or 6076.1 feet.

NDB (Non-Directional Radio Beacon)

This is a navigation aid that transmits non-directional radio signals

PAPI (Precision Approach Path Indicator)

These lights are similar to VASI (see VASI Visual Approach Slop Indicator) but are laid out in one row of usually four lights. They can be placed on just one side of the runway or both. Like the VASI lights, if all lights are white, you are too high, if all lights are red, you are too low, if the outer lights are white and the inner lights are red, the aircraft is on the correct glide slope.

##### REIL (Runway End Identifier Lights)

This system consists of synchronized flashing lights on either side of a runway threshold. They help pilots distinguish runways surrounded by other lights.

##### Runway Identification

The Runway ID is a 2 number approximate runway heading in degrees. If two or more runways have the same orientation, letters are used to help distinguish the runways.

VASI (Visual Approach Slop Indicator)

The VASI system uses a pair or three sets of two-coloured bars usually placed on the left hand side of the runway. These lights show the glide slope and touchdown point.  These lights are hooded red and white filters and set so that only one of the two colours can be seen at a time. Like the PAPI system, if you are too high, the lights from both bars show white, if you are too low, they show red. When you are on the correct glide slope the closer light shows white and the father shows red.

VOR (Very High Frequency Omni-directional Range)

A VOR is a navigation aid. It transmits a high frequency radio signal. You tune your NAV radio to the frequency and adjust the indicator needle to the centre of the dial. The numbers on the dial is the bearing u are on from or going away from the VOR, this is called a radial and you are on it.

Legal notes/Disclaimer

Project Airbus International Copyright Statement

All files contained in this package belong to Project Airbus and are freeware/non-commercial products; repaints of Project Airbus aircraft must be made available freeware. Project Airbus holds the copyrights to these files. These files may not be distributed or uploaded onto any other website without written permission from Project Airbus. Project Airbus’s work may not be used in any commercial package of any kind without written permission.

Graphics, content, or any other material that came with this file may not be used or included in any payware/freeware packages or on any other website without permission from Project Airbus.

The respective owners copyright aircraft manufacturer logos and airline logos presented. They are reproduced here as an artist presentation for entertainment and/or illustrative purposes only.

Project Airbus is protected by the Intellectual Property Law (Copyright Law). If you use Project Airbus files or services in violation of the above statement(s), you are violating the Copyright Law and maybe subjected to prosecution by Project Airbus.

There is NO warranty or guarantee of any kind, expressed or implied, for any problems arising from the use of Project Airbus’s products. This includes but is not limited to any hardware and/or software problems. All use of Project Airbus’s products and its related files is at the user’s own risk.

If there are any problems, questions or comments, please register and post them in the Project Airbus Forum

##### Credits

| Project Airbus Member | Position |
| --- | --- |
| Thomas Ruth | Aircraft & Animator Designer, Texture Artist |
| Nicholas Wu | Texture & Graphics Artist, Webmaster |
| Trevor Slack | Technical Advisor, Aircraft Designer, Team Organiser |
| Peter Binamira | Flight Dynamics Creator, Sound Engineer |
| Adam Reynolds | Texture Artist, Aircraft Designer |
| Hirata Takamitsu | Texture Artist |

##### Contacts

ProjectAirbus@the-wu.org