Vulcanair (formerly Partenavia) P68 C

Model for fs2002 and 2004

Model and flight dynamics by Massimo Taccoli,

textures by Massimo & Gerardo Taccoli

Model with the Principality of Monaco registration 3A-MOI

The model

The aircraft
The P68C is the result of the experience-driven improvement of the original P68 design. The aircraft has maintained its legacy with the past with regards to balancing performance, safety, comfort and versatility. Relative to the previous versions, the P68C presents numerous improvements such as a longer nose for easier Avionic and Weather Radar installation, a single high capacity fuel tank in each wing, improved soundproofing, main gear spring fairings for increased performance, internal opening luggage compartment door providing an additional emergency exit and a modified rear bench seat-back for easier access to the luggage compartment.

Powerplant
The P68C aircraft is available in two powerplant settings. Two 200-hp Lycoming IO 360 A1B6 normally aspirated engines compose the first. For customers requiring increased performance at high altitudes the aircraft can be equipped with two 210-hp turbocharged Lycoming TIO 360 C1A6D engines (P68C-TC).
Both powerplants are equipped with 130-Ampere alternators.

Performance
The exceptionally clean lines of the P68C are the result of many hours of wind tunnel testing and give the aircraft a maximum speed of 200 m.p.h. (322 k.p.h.) and a rate of climb of 1500 ft/min (7.6 m/sec) figures so far unattained by a fixed gear twin. Maximum cruising range (with 45' reserve) at 55% power and 12000 ft is 1525 nautical miles (2825 Km) with the 184 USG standard tanks.

Safety
Painstaking work on the part of an ideal designer/test pilot combination, have produced truly remarkable handling characteristics particularly at low speeds where crisp aileron responsiveness and the raised wingtips make easy work of approaches and landings in turbulent weather conditions.
The forward position of the pilots in relation to the wing, provides excellent rearward visibility in turns, making for greater safety in crowded airspace.
With complete de-icing equipment installed, the P68C has been operated night and day, with safety and regularity, in the extremely severe winter conditions over the North Sea and throughout Northern Europe.
The P68C is approved for flight in known icing conditions in Scandinavia.

Flexibility
The easy and extremely safe handling characteristics of the P68C coupled with its excellent performance and low operating costs, make the aircraft an equally attractive proposition for a private owner graduating to his first twin, as well as for a commercial operator for whom it is the ideal light transport.
The quickly removable seats make the large cabin volume available for the carriage of goods of all kinds, with bulky items loaded through the generous luggage compartment door, large enough to allow the entry of stretchers for aerial ambulance duties.
The optional executive interiors respond to the discriminating customer seeking a higher level of style and comfort both for business transport or simply for leisure. An additional standard feature of the P68C is a 2.1X1.5 ft. hatch (63X46cm) which further increases the aircraft's spectrum of applications.

Economy
The simplicity of the P68C construction, with its rugged maintenance free, leaf spring fixed undercarriage, lack of complicated systems and unsurpassed accessibility of servicing points, reduce maintenance costs to the absolute minimum. The high wing configuration enables the engine intakes and propeller tips to be kept well clear of the ground, thereby reducing dust ingestion and stone denting to the minimum and extending engine and propeller service lives to the maximum.

Performances and specifications

| Engines (n°2 Lycoming) | IO 360 A1 B6 (200 HP) |
| --- | --- |
| Max. Ramp weight | 2100 kg 4630 lb |
| Max. Take-off weight | 2084 kg 4594 lb |
| Max. Landing weight | 1980 kg 4365 lb |
| Empty weight - standard | 1350 kg 2976 lb |
| Max. usefull load | 750 kg 1653 lb |
| Fuel | 696 lt 184 USG |
| Number of seats | 6 |
| Baggage capacity | 0,56 mc 20 cu.ft |

| Max speed at sea level. | 322 km/h 174 kts |
| --- | --- |
| Max. Range at cruise speed (@ 75% & 7500 ft) | 315 km/h 170 kts |
| Rate of Climb (2 Engines S.L. @ MTOW) | 6,3 m/s 1240 fpm |
| Rate of Climb (1 Engine S.L. @ MTOW) | 1,1 m/s 210 fpm |
| Stall Speed (full flaps) | 115 km/h 62 kts |
| Service Ceiling (2 engines) | 6096 mt 20000 ft |
| Service Ceiling (1 engine) | 1753 mt 5750 ft |
| Max Range (best economy @ 7000 ft) | 2965 km 1525 nm |
| T.O. Distance Over 50 ft (ISA, S.L, MLW, no wind) | 415 mt 1360 ft |
| T.O. Distance (ISA, S.L, MLW, no wind) | 230 mt 755 ft |
| Landing Distance Over 50 ft (ISA, S.L, MLW, no wind) | 485 mt 1590 ft |
| Landing Distance (ISA, S.L, MLW, no wind) | 200 mt 656 ft |
| Load Factor | + 3,74 G - 1,50 G |

Dimensions

| Wing span | 12.00 m |
| --- | --- |
| Lenght | 9.55 m |
| Height | 3.40 m |
| Wing surface | 18.60 m² |

The model

The model has been made with FSDS v.2.24 from Abacus, it has all moving parts, ailerons, flaps, all moving tailplane as in the real thing , rolling wheels, suspensions and steering nose wheel.

Interior of cockpit which  shows two pilot figures has been made in a very detailed way with realistic rendered seats.

The panel contained in the model is default Cessna C182, but I deeply recommend to download the panels for fs2002 and fs2004 expressly made by Jean-Pierre Langer with Custom P-68 gauges by Arne Bartels and Jean-Pierre Langer. This 2D panel has photorealistic panoramic cockpit views accessible with the hat button of your joystick.

Names of the zipped files containing these panels which you’ll find in the web are respectively :

P68_panel_2k2.zip

P68_panel_2k4.zip

Installation

The zipped file contains the model  this documents and textures for the 3A-MOI example.

Panel and sound are aliased to that of default Cessna C182.

Extract all files contained in the zipped archive straight in fs2002/fs2004 folder . Each file will go in the right place.

Massimo Taccoli

Web site : www.maxtac.it

e.mail maxtaccoli@libero.it

Roma, 27th  October 2003