Test report Saab 340B (ver. 3, Dove Wings , by Jens Borgstroem, Duckling Air, 13/5-98)

Testing performed using International Standard Atmosphere (ISA) conditions (15°C = 59°F at sea level, temperature gradient -6.5 °C/1000 m, 1013 mb air pressure at mean sea level).

Panel used during tests: Saab 340B captain’s panel, version 3, by Eric Ernst.

Take-off:

Flaps 7, trim a few notches up (+2-3). Full throttle. Slowly rotate at 115 kias (knots indicated airspeed). The plane should lift off at ~120 kias. At positive climb rate retract gear, at 140 kias retract flaps.

Take-off in 90 degrees cross winds at 20 kts is possible using rudder control only. At such a strong cross wind be prepared for some rotation into the wind at speed above 100 kias. Asymmetric thrust helps in that case.

Climb:

Keeping indicated airspeed at 170 kias, while having 95% TRQ, resulted in the following climb rates (approximately) under ISA conditions:

0000 -   5000 ft: 2700 ft/min

5000 -   7500 ft: 2500 ft/min

7500 -   9000 ft: 2200 ft/min

9000 - 11000 ft: 2000 ft/min

11000 - 15000 ft: f1800 t/min

15000 - 17000 ft: 1600 ft/min

17000 - 19000 ft: 1400 ft/min

19000 - 20000 ft: 1300 ft/min

20000 - 21000 ft: 1200 ft/min

21000 - 22000 ft: 1100 ft/min

> 22000 ft:          < 1000 ft/min

To reach FL 140 it takes 7.0 minutes and 60 gallons or 386 lbs of fuel Þ average fuel consumption = 514 gallons/hr ~ 3309 lbs/hr.

To reach FL 220 it takes 12 min. 40 s and 86 gallons or 568 lbs of fuel Þ average fuel consumption = 407 gallons/hr ~ 2690 lbs/hr.

Average climb rate to reach FL 220 is ~1740 ft/min, average speed over ground in zero wind is 195 kts.

Cruise:

At 78% TRQ (again ISA conditions for temperatures and pressures):

FL 140: 236 kias, 283 ktas

FL 220: 206 kias, 280 ktas, 134 gallon/hr = 888 lbs/hr

FL 250: 187 kias, 273 ktas, 113 gallon/hr = 747 lbs/hr

At cruise level and cruise speed the trim should be negative.

Fuel consumption decreases with increasing altitude.

The stability of the plane under auto pilot is excellent. At FL 220 no oscillations in altitude occur.

S340dwi.doc (c) 1/2

- 2 -

Stall speeds:

Stall speed clean: ~118 kias.

Stall speed full flaps + gear down: ~100 kias.

Maneuverability:

Roll rate (tested at FL 220 cruise speed): ~ 5s from -30 degrees to + 30 degrees.

Descent:

Reduce throttle to initiate the descent. Gradually trim the plane for slower speeds. (Remember that trim is used to control the air speed and thrust to control climb/descent rate.)

Descending using the autopilot works flawlessly as well. In that case, since the AP determines the descent rate, use thrust to adjust speed.

Some thinking ahead is required when it comes to slowing down, since the plane tends to bleed off speed slowly, exactly like the real plane. Therefore, trying to reduce speed from 200 kias to 125 kias in the very last stage of final is not such a good idea.

Final approach:

Both manual and autopilot control may be used to track ILS GS and localizer. In case the autopilot is used, remember to switch it off 100 feet above the ground. Landing should be performed manually.

Normal procedure: Flaps 20, gear down, trim +1, 46% TRQ Þ hands-off approach at 130 kias and ~650 ft/min down.

For short runways: Full flaps (35), gear down, trim neutral, 50% TRQ Þ hands-off approach at 125 kias and ~650 ft/min down.

In all these cases landing is quite easy, since the plane does not have a nose high attitude during final approach. As a result one has a clear view of the runway.

Landing:

Start the flare at about 20 feet above the ground. Reduce throttle. After touch down of the main gear lower the nose and apply reverse thrust. At speeds below 60 kts bring throttle to idle position and apply brakes. Retract flaps.

In high cross winds (> 20 kts) some asymmetric reverse thrust and differential braking might be needed.

Performed on behalf and

by request of Duckling Air

May 13, 1998.

Henri Werij, Pilot.

S340dwi.doc (c) 2/2