AFG HEADS UP GUIDANCE SYSTEM

For FS2002 and FS2000

By Jon Gibbs

Copyright November 2001

www.alliedfsgroup.com

Introduction:

The AFG HGS (Heads up Guidance System) is designed for the pilot to be able maintain a scan over the important instruments while maintaining increased visual awareness with the outside world without having to change views or sacrifice screen space for instruments. It is a concept borrowed from combat fighters but it has made flying commercial and business jet aircraft much easier. Increased visual awareness is accomplished by making the HGS display transparent so one can see the plane’s critical information and keep a sharp eye on the outside world at the same time with out changing view or sacrificing much window space. The AFG HGS has different modes designed for IMC (instrument metrological conditions) and VMC (visual metrological conditions) approaches along with cruise and taxi. To make the approach phase of the flight easier, all of the important information such as altitude radio altitude, and glide slope will be displayed in a special format that allows the pilot to conduct a quicker scan and detect changes in the flight path far quicker then when using any conventional heads down instrument panel. Once the wheels are down, the HGS automatically switches to the necessary ground mode and only information essential to the taxi will be displayed so the pilot can have more visibility.  There is also a flare cue to guide the pilot all the way down to touchdown.

It should be noted that although the AFG HGS is intended for use in EVERY phase of the flight, it does not completely replace the main panel.  I feel that for most situations, it is better to reconfigure the airplane while looking at the main panel. This way when the plane is reconfigured, you can scan all the gauges to make sure the aircraft is operating within its limits.   However, going heads down is always a risk because you become less aware of what is going on outside. You will have to use air traffic control and your own discretion to determine how and when to reconfigure the airplane safely.  Keep in mind that in the real world, there are two pilots monitoring the aircraft while in flight simulator you are the only one in control.  I suggest flying with the HGS during big online fly-ins because that is when you really need to be totally aware of the traffic outside! The AFG HGS is based on the HGS for the 737 that is made by Flight Dynamics: www.flightdynamics.com.  If you enjoy flying with the HGS and you are satisfied with all the hard work that went into it, please vote for it in the flightsim.com monthly contest.

Installation:

1. Unzip the file named jg_hgsa.gau into your gauges folder.  There are some modified panel.cfg files included in this zip for use with popular panels.  To install those panel.cfg’s simply backup the originals and unzip the modified version into your panels folder. If you want to modify your own panel.cfg file, first backup your original panel.cfg file and then follow these instructions. If you do not want to attempt a modification or if you are having trouble, send the panel.cfg file to jon@alliedfsgroup.com and I will “install” the HGS for you and send the custom panel.cfg file back to you.  If the modified panel.cfg file has problems, please send me an email and I will correct the problem.

Included modified panel.cfg’s for the HGS (back up your originals. Just open them and extract):

Fs2k2_734_mod.zip: FS2002 737-400 HGS

FS2k_734_mod.zip: FS2000 737-400 HGS

Dreamfleet734mod.zip: DreamFleet 2000 737-400

ca400mod.zip: Chris Arrington’s 737-400

Fs2k2_777_mod.zip: FS2002 777

Fs2k2_747_mod.zip: FS2002 747-400

Fs2k2_Lear_45_mod.zip: FS2002 Lear 45

Rp733_mod: Richard Probst’s 737-300

737200almod.zip: 737200al.zip on flightsim.com

Dreamfleet 737-400 users: The HGS is on the landing panel. To access it, click the up arrow on the view control panel (VCP Shift + 2)

TO CREATE YOUR OWN CUSTOM PANEL.CFG FOR THE HGS: Unzip the bmp file to the panel folder you want to use the HGS with.

1. Open the panel.cfg file for the panel you want to use the HGS with. Here is an example for what you need to do from Richard Probst's panel.cfg file:

Change

[Window Titles]

window00=Main Panel

window01=Radio Stack

window02=GPS

window03=Throttle Quadrant

window04=Overhead Panel

window05=Pushback

to

[Window Titles]

window00=Main Panel

window01=Radio Stack

Window02=GPS

window03=Throttle Quadrant

window04=Overhead Panel

window05=Pushback

window06=HGS Panel

change

[Views]

VIEW_FORWARD_WINDOWS=MAIN_PANEL,RADIO_STACK_PANEL,20,COMPASS_PANEL

VIEW_FORWARD_DIR=10.0, 0.0, 0.0

to

[Views]

VIEW_FORWARD_WINDOWS=MAIN_PANEL,RADIO_STACK_PANEL,20,COMPASS_PANEL

VIEW_FORWARD_DIR=3.0, 0.0, 0.0

Note: If the views section isn’t in your original panel.cfg file you should add it.  Make sure your window number is 1 greater then the last window number. Press Shift + (HGS window # + 1) to view the HGS. So for example if the HGS is window 06, press shift + 7 view and hide it. You can also use the view menu. It is best to hide all other panel windows before opening the HGS window.

Add:

[Window06]

size_mm=1024, 768

position=1

visible=0

window_pos=0.0,0.0

background_color=0,0,0

render_3d_window=1

window_size=1.000,1.000

ident=HGS

gauge00=jg_hgsa!jg_hgs, 0, 0

Tip: If you want the fs2k horizon line to match up with the HGS's horizon try messing around with

VIEW_FORWARD_DIR=3.0 <--- this number. I get the best results with 9.0 to 9.2. However, you may want to experiment with lower values around 3.0 so you won't have to adjust your seat when looking at the main panel. So you don’t have to adjust your seat when using the HGS I have set 9.2 as the value in the panel.cfgs in this zip. Feel free to change and experiment with them. However, I use the HGS far more then I use the adjusting my seat when using the main panel is not an issue for me ;-)

Becoming familiar with the AFG HGS click areas:

There are 3 main modes displayed on the HGS: PRI, IMC, and VMC. To switch modes: left-click the knob in the upper left hand corner of the HGS housing.

I suggest cycling through all the modes on the ground before you start your flight just get familiar with what each one looks like.

To hide or show the HGS, RIGHT-CLICK the lever in the upper right hand corner.

Note: when the HGS is hidden the lever won’t show. To find it put your mouse in the upper right hand corner of your monitor and move it to the left until you see a hand. Once you see the hand, right click.

To show the guidance cue, left-click in the center of the HGS housing.

HGS Modes:

Primary Mode: When cycling through the modes, the first one that will be displayed is the Primary mode.  This is an all-purpose mode. It is designed for taxi, takeoff, go-around, visual and instrument approaches. When the plane is in flight, the primary mode will display all critical information.  Zoom in if you have trouble reading the magenta labels. A larger image is also included in this zip.  Note: The numbers on the HGS horizon correspond to the plane’s magnetic heading. They match up exactly with the FS2k world so you can treat the horizon not only as a horizon, but also as a magnet compass. A digital readout of your magnetic heading is right below the turn coordinator. This is true for all modes.  Note “GPS DME” refers to the DME from the Fs2k GPS.  A wind vector and wind speed indicator are visible in the upper right hand corner of the HUD display only when the wind speed is greater the 0 knots.

On the ground, the PRI mode will look like this:

PRI GROUND MODE:

The second mode you will see when cycle through the modes is the IMC mode. It is for instrument approaches.  When flying an instrument approach it is best to switch to IMC mode once the glide slope and the localizer have been intercepted.  The IMC mode offers a less cluttered layout designed especially for instrument approaches, a longer HGS horizon line and increased visibility. The words “AW” meaning approach warning will appear right above the boresight if the HGS detects that you are off course, off glide slope, or if the autopilot is engaged.  The localizer and the glide slope lines represent the horizontal and the vertical components of the guidance cue respectively. By default, they will command a glide slope angle that is 3.00 degrees which is marked by the glide slope reference line.  In this version of the HGS, there is no way to change the 3.00 degree glide slope value commanded by the guidance cue.  If this is not the intended glide slope angle, you will have to adjust your flight path accordingly. Most glide slope angles don’t differ from 3 degrees by a significant amount so I felt this value would serve all users.

IMC FLIGHT MODE:

The last mode you will see is the VMC mode. The mode is designed for visual approaches. It can be used for instrument approaches but I only recommend this to

save frame rates. The only difference it has from the IMC mode is that the localizer and glide slope lines are not visible. However, in VMC mode you can still use the guidance cue which commands both the glide slope and the localizer which enables you to fly an instrument approach.

VMC FLIGHT MODE:

Marker Beacons, the Flare Cue and the Approach Warning:

Approach Warning:

This appears in IMC and VMC modes when the HGS detects that the approach may not be safe. The criteria for this warning are: Greater than .25 dots of deflection in the localizer, greater than 1.2 dots of deflection on the glide slope, or if the autopilot is engaged. Any one of these criteria will trigger the approach warning.  The approach warning will override the marker beacon display.  If you are at or below your decision height and see the approach warning for more then 5 seconds, you should execute a missed approach.

Marker Beacons:

The letters “OM” (outer marker) “IM” (inner marker) and “MM” (middle marker), will display in the same place as the approach warning when the aircraft picks up that respective marker beacon. The approach warning will override the beacon display.

The Flare Cue:

The letters “FL” will display in the same place as the marker beacons and the approach warning when the HGS commands a flare. This happens when the plane is in flight, in IMC or VMC mode and is below 30 feet radio altitude.  Make sure the guidance cue is showing before you begin your flare.  When you see the flare cue above the boresight, begin your flare. The guidance cue will command a small, change in the flight path which if followed, should result in a smooth landing.  The flare cue along with the guidance cue is specially designed for the AFG 737ng series (but will work fine with most any other aircraft) and it is at your discretion whether or not to follow it. During an instrument approach though, I highly recommend following the flare cue to ensure a safe landing.

Flying with the Guidance cue and the Flight Path Vector:

The guidance cue and the flight path vector are very powerful tools that allow pilots to fly more precisely with the HGS.  The flight path vector draws itself by combining the vertical component (vertical speed) and horizontal component (ground speed) of the plane’s flight path into a single symbol showing the plane’s flight path in relation to the FS2k world.  If the plane’s flight path in this picture didn’t change, it would land right on the edge of the lake instead of 9L at Heathrow.   During approach and landing, you can figure out where you will touch down by looking where the center of the flight path vector circle falls. With the flight path vector on the AFG HGS, a pilot can detect an extremely small change in the plane’s flight path (about .04 degrees) before it becomes an extremely large change in flight path (say 1.00 or 2.00 degrees).  One reason for this is that it has a large range of motion (actually about 2/3 of your screen). The advantage is that the pilot can make quicker, smaller, barely noticeable, corrections to stay on course that will keep the virtual passengers very happy.

The guidance cue is helpful because it commands a precise flight path.  What the pilot needs to do is to keep the guidance cue circle inside the bigger circle of the flight path vector.

Note: To perfect your skills, try keeping the circle exactly in the center of the flight path vector.  Look closely at the picture and you will notice that although the guidance cue is inside the flight path vector, the plane is slightly to the left of the exact flight path that is being commanded. Keeping the circle exactly in the center of the flight path vector is difficult, but worth the challenge.  Keeping the circle inside the flight path vector, even in close vicinity will still ensure a safe landing.

Like the flight path vector, the guidance cue is far more sensitive then most instruments on any main panel. In fact, it is so sensitive that the FS2k autopilot can’t even follow it exactly.  By default, the guidance cue will command an intercept of the VOR or ILS tuned on NAV 1. It will act as a heading bug on the horizon if HDG is selected on the autopilot panel.  If a glide slope is active, the guidance cue will command it also.  The guidance cue will also command a flare at 30 feet Radio Altitude if the HGS is in IMC or VMC mode.  Switching from NAV to GPS mode will make the guidance cue command the GPS heading to the next waypoint. You can follow the flare cue all the way to the ground and this should result in a smooth landing.

Warnings and Known Issues:

-The HGS doesn’t redraw itself if there is no visibility.

This happens on most computers and it is a problem that I have not been able to solve. The flag that is supposed to prevent this from happening is in the source code but for some reason, it is being ignored when only clouds and/or fog are visible.  The problem corrects itself as soon as there is a break in the clouds or the fog and some visibility is restored.  If this happens, go to the main panel until you get a slight amount of visibility.

-The HGS is hard to read in the daytime.

Some users have reported that the HGS is hard to read and during the day when there is a nice light blue sky.  My suggestion is to adjust the contrast and brightness of your monitor. If your eyes start to hurt simply pause the FS2k and adjust the brightness in contrast. If your eyes still hurt, stop using the HGS until there is a darker background fly with (say some more clouds or night time).   AFG will not take responsibility or liability for any injuries that result from using the HGS. Use it at your own risk.

-No Failures:

Due to time constraints in writing the source code, the AFG HGS doesn’t respond to fs2k failures.

-Dream Fleet Users

You cannot hear flap callouts or cabin announcements while using landing panel with the HGS. This is possible but it will result in an illegal operation when exiting FS2k and when changing aircraft.  If you still want to hear the flap callouts, send me an email and I will send the panel.cfg that accomplishes this.

The Dream Fleet autopilot is independent from the FS2k autopilot so the MCP altitude on the Dream Fleet panel may not match up with the MCP altitude displayed on the HGS.

-Bad Frame Rates

After the beta stage, I recommend at least a 1ghz processor and a 32-64mb 3d card if you want to get 20 or more frames per second when using the HGS along with highly detailed scenery. One user with a 1 ghz computer, a 64mb geforce 3d card and 256 mb of ram got 20fps when using the AFG HGS and the Simflyers ATL scenery. Another option would be to put those sliders back.

Special Thanks To:

Gary Hayes: For doing some of the more tedious gauge and main bitmaps and for beta testing. Thanks Gary!

All the beta testers: Thank you for your time and patience.

Anonymous contributors: Thanks for all the info. It was nice being able to talk to people who work with the real world version of the HGS everyday.

Fast Facts about the AFG HGS:

-First true heads up guidance system for Microsoft Flight Simulator

-Over 3200 lines of source code were written from scratch

-1.5 months to complete

-60 megabytes of bitmaps made

-Photo realistic graphics

-Based on the real world heads up guidance System by Flight Dynamics: www.flightdynamics.com

-Tested by a real world 737 captain who uses the real world HGS

-Please vote for it in the Flightsim.com Monthly Contest

COPYRIGHT NOTE:

This addon for Microsoft Flight Simulator is copyrighted by AFG (Allied FS Group) and Jon Gibbs.

By using the contents of this file you agree to the terms of the copyright

statement:

COPYRIGHT STATEMENT:

This file is given to you free of charge. But keep in mind that we are the

copyright owners. This file may not be uploaded to any other site without our

written permission. Our work is non-commercial and our files and work may NOT be sold or repackaged in any way.

Any duplication, reproduction, partial or complete of any material made by AFG is

totally forbidden without prior authorization from the author(s). This file may

NOT be used or included in payware packages of any kind. This goes for freeware

thefts as well as payware thefts.

AFG does not give any warranties, either implied or specific. Any damage to your

computer, programs or files resulting from the use of these files is not our

responsibility. Any use is at your own risk.

If you use the files in this package in violation of the above statement, you may

be in violation of local and federal laws.

Any and all wrongful use of these files will open yourself to prosecution to the

full extent of the law.

If you have any questions regarding this Copyright Statement, please send your

enquiry to: legal@alliedfsgroup.com