SCASM 2.50

pseudo commands

and more

Pseudo commands are commands that do not produce any code. They are used to send aditional instructions or information to the assembler itself.

Set( flag value )

-

This is a SCASM configuration command to set some

internal flags and variables.

-

table of flag/variable names:

ppperr    Number of points per polygon to trigger the

error message. Default = 100.

RAW       If set to 1 the RAW output is enabled.

The -RAW command line switch is not longer

needed.

LOGFILE   1 enables error output to "SCAERROR.LOG"

Same as "-l" command line switch

MAPFILE   1 enables the mapfile (.MPF) output

-> -m

OBJID     This flag controls the object ID generator

when object libraries are compiled (2.36+).

1   (default) ID0 value is incremented

0   ID3 value is incremented. Only needed

for backward compatibility with FS98 style

library format.

BUF       wanted buffer size in KB. This option is

only needed for compiling large RAW BGL

sections for MDL files of more than 100kB

size or for very large numbers of

synth tiles or nav aids.(v 2.43)

Note, the buffer size has nothing to do

with the maximum code size of an Area()

command. This size is limited by the

binary format of the BGL command(s).

Use this option before increasing the

AREAMX threshold.

areamx    Maximum allowed size of an Area() block in

KB. Default = 16, maximum 64.

area15mx  Maximum allowed size of an Area15() block

in KB. Default = 16, maximum 64.

LABELS    sets the size of the internal symbol

table. The default size is 1500. You

only need to increase this number if you

are working on very large MDL files.

(version 2.45 and later)

PATCHES   As all one-pass assemblers SCASM needs

a patch table. The size of this table

should be about 1.3 to 1.5 of the

maximum label number.

Default value is 2000.

(version 2.45 and later)

LINBUF    This option sets the size of the internal

line buffer. You will only need to

increase the default size of 20 KB if

you get an "Line buffer overflow" error

during compile time. This sometimes

happens with point lists of very large

objects.

This option is mainly the same as the

-LB command line switch.

The only difference is that the wanted

size must be entered in KB.

MAXENTRY  Sets the maximum size of the object entry

index table. The default size is 3500.

A larger size is only needed for a large

number of objects which needs to be

sorted, such as navaids or classic

Synth Tiles.

MAXPTLST  Sets the maximum number of points in the

point list buffer (Points(), VecPoints()).

Default size is 1000.

For it's automatic vector calculation

feature, SCASM needs to have ALL point

coordinates for all point numbers

available all the time.

interr    Same as -i switch, 1 = ignore integer

errors. Default = 0

fmtcast   Same as -f switch, 0 = format casting

disabled. Default = 1

-

A change of some of these values may probably result

in display problems in older FS versions.

Set commands should be placed at the beginning of

the source code file.

Include( filname.ext )

-

Includes this text file. The file is simply copied

into your 'main' source file. In SCASM 2.10 and later

an include file can contain other Include()'s or

Macro()'s.

Any correct path/filename is accepted (up to 260

characters).

This makes it possible to make a 'main' file like

Header( ... )

LatRange( ... )

Include( ... )

...

Include( ... )

with all the other commands in the Include() files.

Note, Include()'s are handled now as Macro()'s

without parameters. You cannot use them to increate

the nesting level.

Macro( filename.scm p1 ... pn )

-

Similar to include files the text of an macro file

is simply copied into your source file. But the

advantage of an macro is that parameters can be

passed to it. These parameters are inserted into the

text during compile time. This makes it very easy to

'recycle' scenery objects. If you want to design

a house, it is usually a good idea to do this as a

macro file and use the strings %1 and %2 for the

latitude and longitude values of its position.

Now you can put this house into your scenery simply

by puting its actual Lat/Lon position as parameters

one and two into the Macro() instruction. SCASM

allows up to 30 parameters and each can have 32

characters.

Macros are allowed in visual scenery (section 9),

dynamic scenery (section 15) and section 16.

In SCASM 2.10 and later macros are allowed to contain

other macros. This nesting is limited to 8 levels,

and Include()'s are handled now as macros without

parameters.

If a macro (file) name or a macro parameter contain

spaces you have to use " to mark the begin and end of

it (SCASM 2.15 and later).

example:

macro call:     macro( example.scm N54:10 E10:05 )

macro file:     Area( 5 %1 %2 20 )

-> %1 is replaced by N54:10

-> %2 is replaced by E10:05

Note: In most cases macros will contain a whole

Area() block. But if you are using the macro feature

to generate some code within an area (for example

'hand made' aproach lights in an runway area), and

if you are using the same macro more than once in

the same area, and if the macro contains label

definitions you will get an "duplicated label error".

To avoid this you can include the special '@'

character in the label name. This character causes

SCASM to insert the internal macro counter in the

Label name. Since the internal macro counter is a

3 digit code the name is then limited to 12

characters.

example:

:Label@ is expanded to :Label095 during the

expansion of the

95th macro.

Mif( <expression> )

Melse

Mifend

-

These pseudo commands are used to enable and control

conditional compilations. In earlier SCASM versions

they are only used in macros for visual scenery.

That's why they have the "M" in the name. Now they

can be used anywhere in the source text and they

even can be nested (up to 8 levels).

<expression> can be a mathematical formula or a

single number.

-

mif( %1 )

mif( [%2 == 3] )

The macro parameter 1 is tested.

If it's value is NOT 0

(that is TRUE) the following commands

are compiled. Only a zerotest is

done.

In example 2 the expression

"[%2 == 3]" is evaluated and then

tested.

see -> annex 1 for expressions and

operators.

melse           Optional. If the 'mif( %# )'

condition is FALSE

this section is compiled by the

assembler.

mifend          Marks the end of the conditional

compilation.

If an error is detected and the compiling condition

is FALSE, then the error messages will have the line

number of the Mifend command.

GRP(  <Lat> <Lon> )

-

This command writes the given position to a

SCASM internal General Reference Point memory

so it can be used later to calculate new

positions.

-

Every time an Area() command is compiled, the position (Lat/Lon) is stored into a temporary memory. This position is valid until the EndA command is executed. This is not only true for the automatically stored position but also if you explicit define a new GRP(). So a GRP() defined in an Area()

is really a temporary one.

You can use the stored position to calculate a new referencepoint position in every command that requires a Lat/Lon pair input.

There are two options to calculate a new position:

d <delta_Lat/north-south> <delta_Lon/east-west>

r <heading> <distance>

If your new position is 150 meters east and 60 meters south of the stored position you can enter:

RefPoint( abs :Label 1.0  d -60 150 )

and SCASM will calculate the new reference point.

If your new position (i.e. for LandMe) is 400 meters away in

heading 273.4 degrees you can enter:

LandMe ( r 273.4 400    0  90

r  93.4 400    0  27

)

Do not use this feature for long distances.

CopyRight( any text with up to 80 characters )

-

Traditionaly many 3rd party scenery tools insert

a text signature in the output file. Typically

this is the name and version of the tool which

generated this file. SCASM does the same.

This instruction lets you add your own text to

this signature.

This text has no effect to FS. This command

should be used only once per source/BGL file.

If this command is found more than once, only

the last one is used (no error message). For

this reason it is not a good idea to use it in

a macro file.

No underscores '_' are needed for SPACE characters

in the text.

Uvar( $name value )

-

This is an internal command to set an SCASM internal

user variable. This command has no effect to the BGL

code. User variables can be used by experts for more

flexible parameter calculations.

Note: variables which are defined within an Area()

are only valid in this Area() and are deleted when

the EndA statement is compiled. Other variables are

always valid and are even visible from macros and

includes.

SCASM can handle up to 200 user variables.

SCASM internal variables / constants

-

Besides user variables there are some other

internal values which might be of interest.

-

$IC             internal instruction counter,

valid in Area()'s

$PI             The number PI = 3.414 (2.02)

$Section        This is SCASM internal section

bitmask.

There is only one bit set for

the currently scanned section

(i.e. 0x001 for section 0 = nav

or 0x200 for section 9 = visible

scenery).(2.03)

$Version        The SCASM version number * 100

Map( stat )

-

This is another internal command which can be used

to suppress the map file generation for some parts

of the source code file.

See also the "-m" command line option.

-

stat	1 = on,

0 = off

-

For more information please see the documentation

which comes with the BGLTST.EXE program which uses

this information to display the source code line(s)

related to an detected error.

Error( any error message )

-

This pseudo command produces an error message and

stops SCASM.

This instruction can be used as a reminder for

changes you want to do later or to ensure that a

specific source file (macro) is compiled with the

correct SCASM version.

For example, if internal functions (version 2.02

and later) are used for the parameter calculations,

you can use this instruction to notify the user

that he needs an compiler update:

...

mif( [$Version < 202] )

Error( You need at least SCASM 2.02 to

compile this code )

mifend

...

Points( 0

-15  0  int[ %6 / 2 ]

...

)

...

In this example the macro receives a parameter from

the caller. If the %6 macro parameter has the value

15 the result would be 7.5. Since FS does not accept

fractional numbers in point coordinates, an error

message is normally generated. The int[] or the

round[] functions can now be used to convert the

number format but these are not available in earlier

versions.

VTrace( mask "any text" <expression> )

-

This is an SCASM internal test command for debugging.

It does not produce any code. You can use this

command if you want to know the result of of some

SCASM calculations.

The output is done to the standard error device,

which is the screen or the "SCAERROR.LOG" file.

-

mask         This is a bitmask in HEX format to

indicate the section when this command

shal be executed. If you are unsure

about this value set it to -1

(or 200 for section 9, visual scenery).

"any text"   Any text you want to see on the output

device. (max 120 chraracters)

<expression>	Any valid expression in SCASM syntax.

-----------------------------------------------------------

Calculations

------------

When I started the SCASM project it never came to my mind that it could become necessary to do calculations and so there is no delimiter character to separate parameters. For this reason SCASM needs some help. This is why you have to use squared brackets "[]" in every arithmetic expression.

Expressions are always evaluated from the left to the right

regardless of the common mathematical rules. The only way to change this is to use nested brackets. Expressions can also contain macro parameter numbers like this:

[[90 - %3] / 45 ]

In expressions the following operators are allowed:

+

-

*

/

==      equal (comparison)

!=      not equal

<       less than

<=      less or equal

>       greater than

>=      greater or equal

|       OR (bitwise)

^       exclusive OR

&       AND (bitwise)

<<      shift left

>>      shift right

It is also possible to "cast" the number format.

Some examples:

0xAB23          hex format, with "0x" prefix

0b101010101     binary format, with "0b" prefix

0d12345         decimal format, with "0d" prefix

0b1010101       binary format,

To avoid problems with the current number format recognision routine please use only upper case charaters for hex numbers and use only lower case characters in the "number cast" sequences

"0x..." & "0b..." & "0d...".

The expression evaluator is called for every numeric parameter input. If the evaluator is called without the "[" at the beginning of the expression, it stops at the first space charater. Therefore I strongly recommend to use the brackets and separate the elements of the expression by spaces. This will also increase the readability of the source code.

Since SCASM is a one_pass assembler :Label's in arithmetic expressions cannot be used, if these :Label's are defined in a later source text line. This is because the :Label is still undefined when the expression is evaluated.

All calculations are done at compile time, NOT at runtime.

Functions (2.02)

----------------

To improve the parameter calculations there are now some functions available. Please remember, all calculations are done at compile time. Function names are case sensitive.

int[ <expression> ]

This function converts the result of

<expression> into an integer number by

truncating the fractional part.

round[ <expression> ]

This function converts <expression> into an

integer number by doing a normal round

up/down.

sqrt[ <expression> ]

This calculates the square root of

<expression>. The result is still a

floating point value. If needed you

can use:

round[ sqrt[ 100 / 2 ] ]

abs[ <expression> ]

This function makes sure that the result

of <expression> is always a positive number.

sin[ <expression> ]

cos[ <expression> ]

tan[ <expression> ]

asin[ <expression> ]

acos[ <expression> ]

atan[ <expression> ]

atan2[ <expression> <expression> ]

This is the atan2( y / x ) function. Format

corrected in version 2.07

adrpat[ :Label ]

This address patch funktion is a workaround

for the address calculation problem in

Dwx() commands in the case the label is

defined in a later line of the source code.

This function simply adds the current

address to the internal patch table and

returns a 0 value which will be patched

later. (v.2.04)

ipt[ index flag ]

-

version 2.07

index   the wanted point's index number

flag    selects the coordinate element

(x, y, or z) of that point.

-

This function imports point coordinates from

an existing SCASM internal point list.

For example, you have defined an point list

somewhere in an Area(). SCASM holds an

internal copy of this list for the polygon

vector automatic function. This function

gives you access to this list, so you can

copy coordinates to instructions which do

not accept point numbers.

If you want to draw a dotted line with

7 dots from, lets say, point number

5 to 6 of your list, you can write:

DotLine( ipt[5 x] ipt[5 z] ipt[5 y]

ipt[6 x] ipt[6 z] ipt[6 y]   7 )

-----------------------------------------------------------

Information about aditional texture handling options

Sometimes it is pretty difficulty to find the correct X-Y-coordinates for textured polygons. One of the most common examples for this are mountains.

If you are such an lazy scenery designer like me <g> you can try the different features of the bitmap automatic. In this version bitmaps are always aligned to the lower edge or the lowest point of the polygon.

Texture automatic is limited to polygons with a maximum of 16 vertices (vers. 2.29 and later).

T       enables Texture Point calculation automatic

for any ..TexPoly's. SCASM tries to use the

whole texture map, but the aspect ratio is

maintained. Only valid with ..TexPoly's

B       The same as above, but texture aspect ratio

is not maintained. This may result in a

different scaling for the bitmap in x- and

y- direction. Only valid with ..TexPoly's

R       This flag reverses the bitmap in y direction

(up/down). Only valid with ..TexPoly's. This

works only with the T or B flag active.

aditional flags in version 2.04

S scale

maybe changed in next version !!!

With the above flags allone you cannot

control the size of the pixels (scaling).

If this flag is used SCASM expects a

scaling factor for the next parameter. The

scaling factor can be a fractional number

or even an expression.

This scale factor can be calculated as:

(last_usable_pixel

- first_usable_pixel) / longest_dist

If you want to use the whole bitmap and your

distance is 25 meters for your largest

polygon edge, this is simply:

255 / 25 = 10.2

This scale factor is used for X and Y

scaling.

L x1 y1 x2 y2

This flag indicates that 4 aditioanl

parameters follow.

These parameters are bitmap coordinates to

limit the usable bitmap area for this

polygon. SCASM tries to stretch this area

to fit the polygon. If this flag is not

used SCASM asumes

x1 = 0, y1 = 0, x2 = 255, y2 = 255

for the first and the last usable pixel.

This is the whole bitmap.

N       No reset of the S and L values.

Normally the scale factor and the limit

values are resetted everytime a polygon

is computed. If you want to use the

values from a previously defined polygon,

use this flag to tell SCASM not to reset

these values.

example:

...

TexPoly( ats [127/25] L 0 0 127 127   0 1 2 )

TexPoly( atn                          1 2 3 )

...

In this example only a small part of the

bitmap is used and the bitmap scale factor

is calculated to fit a polygon with up

to 25 meters width (x) and height (y).

The second polygon is calculated with the

same scaling and pixel limits since the

previous values are not resetted.

Some important BGL section numbers and their hex bitmask values internally used by SCASM. Used in section mask's in Mif() and SCLINK.

0       00001   Nav frequencies (ILS, VOR)

1       00002   Synth tiles (Seed) size 1 to 6

...

6       00002   Synth tiles size 6 (smallest, high

priority / all tiles are internally

handled with the same mask by SCASM)

9       00200   visual scenery

10      00400   object library

11      00800   Airport menu (FS6 and before)

13      02000   ATIS

14      04000   NDB

15      08000   dynamic scenery

16      10000   Markers, LandMe, TimeZone,

elevated surface

-------------------------------------------------------------