Util5k DEMO Version 2.1

November 30, 1996

-----------------------------------------------------------

****DISCLAIMER****

Util5k diskettes, programs and documentation are distributed AS IS, with no guarantee it will work correctly in any or all situations. In no event will Sigrid Co. be liable for any damages or losses of any type. YOU USE Util5k AT YOUR OWN RISK. IF YOU USE THIS SOFTWARE, YOU ARE ACCEPTING ALL CONDITIONS AS STATED ABOVE.

****DISCLAIMER****

------------------------------------------------------------

* * * Overview * * *

Util5k is a utility program which allows you to create one or several different command files to program your Scom repeater controller. Commands are stored in a standard ASCII text file. This file may be created using any text editor or word processor which can save the document in unformatted text. For example, you could use the editor that comes with DOS.  Comments may be included in the command line or separate line.

* * * Entering Commands in the command file * * *

Commands are placed one to a line. Basically, all you have to do is type the command line just as you would send it using the DTMF pad on an HT. As an example, to set the ID'er interval on a 5K controller, you would key your HT and send your password then the root command for the function, and the data or value.

Assume your master password is 456B and the control op password is A3. You would send:

456B5199 or 456B5199*

Using Util5K the command line would be entered like this:

51 99*		;Set ID interval to 9.9 min

Util5k would actually send the line to the modem as such:

456B5199*

The line is ALWAYS prefixed with the password entered into the program. Spaces will be deleted as well as comments. Comments are entered by inserting a semi-colon. ONLY ONE semi-colon per line!

To set the normal ID to WB8XYZ the command line would be:

52 32 11 08 33 34 35 38 27 *	;Set Normal ID to WB8XYZ

Util5k would actually send the line to the modem as such:

456B523211083334353827*

* * * Running the Program * * *

To run the program, log to the drive and directory it is stored in. Just type 'Utildemo' and hit enter. The program will display a title screen. Hit enter to continue. Util5k will then prompt you for some information.

1.	Command line delay seconds

This is used by the program to set the delay between command lines sent to the modem. It may be necessary to adjust the value for your particular computer and modem combination.

2.	Command File name

A pick list window will open from which you choose the command file to be sent to the repaeter controller. Highlight your choice and hit ENTER. Only files with an extension of ".CMD" will be shown.

3.	Master Password

Enter the Master password used on this controller. Using the master password insures the highest security level for command evaluation.

4.	COM Port

Enter the COM port of your modem. COM1 & COM2 are supported

After the last entry the screen will clear and be split. The command file name will be displayed at the top center. The upper half of the screen will be used to present the command file as it is stored on the disk with comments. The lower half will display the actual command line being sent to the modem.

To exit, just hit any key. The program will abort, close the files, hang-up the modem and exit to DOS.

* * * Interfacing Hints * * *

Interfacing varies greatly from rig to rig as well as from modem to modem. Using a standard telco cable from the modem, (with an RJ-11 plug on one end), the wires from pins 2 & 3 of the RJ-11 will have the audio. Some modems also implement the black wire as ground. Basically the output of the modem will be nominally 600 ohms. The majority of amateur mobile and portable rigs today have a low impedance Mic audio input, (about 600 ohms). The trick is getting the DTMF output from the modem reduced to a level that won't overdrive the Mic input stage of the rig and cause distortion. Modem output is moderately high level, (-9dBm to -3dBm). The modern transceiver Mic level is generally much lower. Therefore an attenuator pad is needed. You might try a 600 ohm in parallel with the green and red wires with a 100K ohm in series from either side of the 600 ohm to the Mic input. A more desirable attenuator is the balanced H configuration.:

---R1---R1---           Where:

|                       Attn  10dB 20dB 30dB @600ohm

R2                       R1 = 156  245  282

|                        R2 = 422  121  38

---R1---R1---                  all values in ohms

As far as keying the radio, most HT's key up with 10K or less between the Mic input and ground. As the output of the modem is 600 ohms, the rig will key as soon as connected. You could cap isolate the rig from the modem and pad. By cap coupling the rig to the modem and pad, you could used a transistor to key a relay with some sort of current supply to drive the transistor.