ERRORLEV.DOC

Preliminary description of returncodes after termination of

FREC, FSEND and ANSW (DLL and EXE).

Three values are significant after termination:

ErrorLevel (0 - 255)

---------------------

This value is mainly for a calling CMD to react with 'if errorlevel' and

maintain program flow.

HangUp (00 - FF)

-----------------

Could only be excerpted from 'close message' in LOG file and is nearby or

exactly what defined in ITU-T.32 standard. HangUp code FF is reported as

'undefined or still off-hook'.

Returncode of modul (0 - 255)

------------------------------------

If an EXE calls directly a modul (FREC, FSEND or ANSW), than the errorlevel

of the EXE is identical to the returncode of the modul.

If one modul (eg ANSW) calls an other one (eg FREC), returncodes are NOT

chained. Moduls returncode is only available from LOG of calling EXE.

-------------------------------------

Old error levels, which may be valid or not, depending on state of

implementation of the new design:

FREC

// ErrorLevel   0 ... no error, at least one page received

// ErrorLevel   1 ... error, no page received

// ErrorLevel   2 ... wait for RING aborted

// ErrorLevel  20 ... DATA received, finish with open port

// ErrorLevel  30 ... IMGPROC.DLL does not match

// ErrorLevel 100 ... deb_thread not started

// ErrorLevel 101 ... com_thread not started

// ErrorLevel 102 ... raw_thread not started

// ErrorLevel 103 ... decompr_thread not started

// ErrorLevel 104 ... time_thread not started

// ErrorLevel 249 ... '-i' and '-x' defined

// ErrorLevel 250 ... faxidx not found (obsolate)

// ErrorLevel 251 ... SIGNAL termination

// ErrorLevel 252 ... modem init error (timeout or ATV0 is set)

// ErrorLevel 253 ... COM does not exist or is already in use

// ErrorLevel 253 ... COM error get/set parameters

// ErrorLevel 253 ... can not initialize COM

// ErrorLevel 254 ... error in CFG

// ErrorLevel 255 ... no commandline parameters

// ErrorLevel 255 ... incorrect commandline parameter

// ErrorLevel 255 ... -? or -h in commendline

// ErrorLevel 255 ... -i<filename> not found

FSEND

// ErrorLevel 0 ..... all sendjobs done

// ErrorLevel 1 ..... wrong (combination of) commandline switch

// ErrorLevel 2 ..... irregular condition, see FSEND.DOC

// ErrorLevel 3 ..... No or empty configuration file

// ErrorLevel 4 ..... Nothing to do in FAX.LOG (no 'Spool' entries)

// ErrorLevel 5 ..... Too less information in or no FAX.LOG

// ErrorLevel 6 ..... Spool active, but delayed

// ErrorLevel 10 .... UserExit (exit/retry or ESC) or invalid comport

// ErrorLevel 11 .... No Connect after dial, retry later

// ErrorLevel 12 .... Dial Error

// ErrorLevel 20 .... DATA received in poll server

// ErrorLevel 30 .... IMGPROC.DLL does not match

// ErrorLevel 100 ... DebugThreadError

// ErrorLevel 101 ... TIFFThreadError

// ErrorLevel 102 ... BitMapThreadError

// ErrorLevel 103 ... ComInThreadError

// ErrorLevel 104 ... TimeThreadError

// ErrorLevel 200 ... send maybe correct, but nonstandard termination

// ErrorLevel 251 ... Error after connection

// ErrorLevel 253 ... ModemError

// ErrorLevel 254 ... ComPortError

// ErrorLevel 255 ... Speicher nicht ausreichend

ANSW

// ErrorLevel   0 ... state undefined

// ErrorLevel   1 ... on-hook, Message received and stored

// ErrorLevel   2 ... off-hook, FAX-CNG detected

// ErrorLevel   3 ... off-hook, DATA-CND detected

// ErrorLevel  20 ... DATA received, finish with open port

// ErrorLevel 100 ... deb_thread not started

// ErrorLevel 251 ... SIGNAL termination

// ErrorLevel 252 ... modem init error (timeout or ATV0 is set)

// ErrorLevel 253 ... COM does not exist or is already in use

// ErrorLevel 253 ... COM error get/set parameters

// ErrorLevel 253 ... can not initialize COM

// ErrorLevel 254 ... error in CFG

// ErrorLevel 255 ... no commandline parameters

// ErrorLevel 255 ... incorrect commandline parameter

See ERRORLEV.H for values in new implementation!