Preliminary documentation for

ANSW.EXE and ANSW.DLL

Created 1992 - 1997 by Dr. Harald Pollack & Bernhard Seidl

This application is for IBM OS/2 32 bit operation system only.

1) CONFIGURATION FILE

While the name of this file can be anywhat, ANSW defaults (see at

commandline options) to ANSW.CFG

USER

Name of user of this application may be defined here

(31 characters max.). See also KEY.

KEY

A hex digit string to verify users certification. Keyword USER must

also be defined BEFOR this keyword is used. 'USER' plus 'KEY' gives

a personal signature which will be used in further versions. A

valid key may be obtained from author.

[TASK <n>]

Starts a section within the configuration file which only effects

the selected task. To end a section you can either use another

[TASK <n>] keyword which would start another section or use the

keyword [COMMON], described below.

[COMMON]

All keywords after this statement belong to all tasks.

CONFIGEND

ANSW will stop working off the configuration file and everything

following this keyword will be ignored.

LOGBINKLEYSTYLE

Using this keyword ANSW will generate BinkleyTerm like log entries.

DEBUG

Enables a lot of stuff for debugging. Do not use, because LOG will

increase enormous.

LOGLEVEL <n>

Writing to DEBUGLOG depends on this value (0 to 5).

0 ... do NOT write to DEBUGLOG

1 ... ERROR only

2 ... ERROR + SEND + RECEIVE only

3 ... ERROR + SEND + RECEIVE + ACTION only

4 ... ERROR + SEND + RECEIVE + ACTION + WAIT FOR only

5 ... ERROR + SEND + RECEIVE + ACTION + WAIT FOR + INFO (default)

CLMSGOK <script or errorlevel or shellno>

If ANSW.DLL is called from CantaLoup/2, a simple mechanism will allow

further actions of the mailer if a message was recorded sucessful:

- A positiv value forces mailer to exit with this errorlevel.

- A negativ value forces mailer to 'shell' to abs(value).

- A name of a script forces the mailer to execute it.

BAUDRATE <DTE-DCE bitrate>

Bitrate for initialisation of modem if not called from a frontend.

REDIALWAIT <seconds>

Time to wait between two dial tries.

DIALRETRIES <count>

Number of dial tries. Default is '0'. That means after an unsuccessful

call, ANSW will terminate. If 'DIALRETRIES' is defined, ANSW will try

to establish a connection for the determined number of times until

terminating.

PRELISTEN <milliseconds>

If value greater than 0 (minimum is 1000, maximum is 4500 and default

is 2500), ANSW will 'listen' on the line to detect a calling tone

(from modem or fax) before 'GreetingMessage' is played. If a calling

tone is detected, ANSW immediatly terminates, so mailer or other

frontend can handle this call.

ATTENTION!

Though this function could be disabled (by value of 0) it is enabled

automatically (to 2500 ms), if FRECCFG is defined. This behaviour is

mandatory and defined in ITU-T.30 Annex D.

DIAL <dial command>

Default, 'ATD' is used. If an other dial command is required (e.g.

'ATD0W') 'DIAL' keyword can be used.

DEBUGLOG <[+]file name>

This is the name of the log and debug log file of ANSW. Nearly all

activities are stored in this file. If a '+' precedes the path,

append mode is used: one log is chained to the next. Select NUL as

filename to disable logging.

The logfile contains a lot of informations in case of errors or

situations where help is needed. Bugreports to the author are only

sensful if this logfile is part of the report.

PORT <serial port>

This keyword is mandatory. Even if ANSW is called using a port handle

(from frontend or mailer) this keyword must exist! The name of the

device is used to create a unique identifier for internal use!

INITFIFO

If a hardware fifo (UART 16550) is detected by COM-device driver, the

fifo is initiallized to Rx=8 and Tx=16.

DEBUGIDLEPRIORITY <value> or DEBUGREGULARPRIORITY <value>

These keywords, which are mutual exclusive, define the priority (0-31)

for the task (thread) which writes the data to DEBUGLOG file. Default

is IDLEPRIORITY 30. Change this setting only, if you are absolutly sure

knowing what you do! Lower priority may yield in a endless blocking

debug_thread! No data will be written to DEBUGLOG and ANSW may become

'uncloseable'. On very slow or high stressed machines, you can

try REGULARPRIORITY 0, if IDLEPRIORITY blocks writing to DEBUGLOG.

BUSYCOUNT <cnt>

In voicemode, busy tone is signaled from DCE to DTE by supplying

<DLE>+'b'. Modem sensitivity to detect this may differ depending on

firmware version. Human voice may simulate busy to modem in playback

of a message. By default, BusyCount is 1 (one detected busy will

stop all further activities) but can be set from 1 to 5.

VOICEMODE <modemtype>

Depending on modemtype (manufacturer) keywords ZYXEL, ROCKWELL or USR

may be selected. Default is ZYXEL.

ATTENTION !!!

VoiceMode USR is only experimental and for further studies.

VoiceMode ROCKWELL will also fit for other brands modems.

FLOWCONTROL <type>

Normally, ANSW uses software flowcontrol (XON/XOFF), but type CTS or

HW can be set to enable hardware flowcontrol.

IGNOREONDTMF <digit> | <string_of_digits>

One or more dialable digits, which are ignored if received. No default.

STOPONDTMF <digit> | <string_of_digits>

One or more dialable digits, which stop ANSW in any state,

default is '*'.

BUSYONDTMF <digit> | <string_of_digits>

One or more dialable digits, which simulates a received <DLE>b from

modem. Do NOT USE without authors instruction.

QUIETONDTMF <digit> | <string_of_digits>

One or more dialable digits, which simulates a received <DLE>q from

modem. Do NOT USE without authors instruction.

FAXONDTMF <digit> | <string_of_digits>

One or more dialable digits, if received, they are treated like a

fax calling tone. Default is 'c'.

DATAONDTMF <digit> | <string_of_digits>

One or more dialable digits, if received, they are treated like a

data calling tone. Default is 'e'.

EXTONDTMF <digit> | <string_of_digits> <modemcommand>

One or more dialable digits, if received, 'modemcommand' is performed.

Not yet supported, but should be a way to connect current call to

an extension.

RECONDTMF <digit> | <string_of_digits>

One or more dialable digits, if received, ANSW switches immediatly

to record mode. Not yet supported.

SPAWNONDTMF <digit> | <string_of_digits> <EXE_or_CMD>

One or more dialable digits, if received, ANSW spawns (branches) to

EXE or CMD. Two parameters will be supported:

%1 ... porthandle

%2 ... tasknumber

Not yet supported!

PLAYONDTMF <digit> | <string_of_digits> <path_to_voicefile>

One or more dialable digits, if received, ANSW will immediatly

play back voicefile. Not yet supported, but would be a nice

help feature.

RINGCNT <n>

Number of RING, ANSW will wait to go off-hook.

RINGTIMEOUT <sec>

If RingCnt is defined and greater than 1, AND RingTimeOut is defined,

a 'ring timeout' is set after each RING is received from ANSW. So,

if receiver is lift by user, ANSW will terminate itself correctly.

RECTIMELIMIT <seconds>

By default, ANSW only records 30 seconds of a voice message. With this

keyword, a range of 20 - 600 seconds is selectable.

VOICEPATH <path>

Path to where voice messages are stored (and sent from).

This entry in CFG is mandatory! There will be no default, ANSW will

not start if not defined. VOICEPATH is used to form a fully quallyfied

path to VOICELOG and VOICEIDX. If one of both keywords has OFF in CFG,

voice messages are store in <voicepath>\ANSWnnnn.???, else voice

messages are stored as VCnnnnnn.??? (see VOICEIDX).

VOICELOG <file name> | OFF

Filename to replace VOICE.LOG and only neccessary, if other than

'VOICE.LOG'.

Received voice messages from ANSW will be joined to VOICE.LOG.

Each stored voice message is flagged as 'Rcvd'. If received voice

message is too short to save, no entry is made.

In case of far inquiry or callback, the voice message file is flagged

with 'Read' (far inquiry) or 'Sent' (callback). If callback fails,

the entry is flagged with 'NoAnsw' and if dialretry count is above 0,

a new entry (with same voice file) is made and flagged with 'Spool'.

Also MakeSem is started (with DialWait) to force mailer to call ANSW

again to satisfy callback.

VOICEIDX <file name> | OFF

Filename to replace VOICE.IDX and only neccessary, if other than

'VOICE.IDX'. Used to select value 'nnnnnn' for VCnnnnnn.???.

CLEANUP <modem command>

If ANSW finishes in a on-hook state (no 'quiet', no calling tone

detected), <modem command> is sent to modem. Multiple entries are

possible, recommended is at least 'AT', but you can use any

modem command. Before CleanUp, an AT+FCLASS=0 is sent to modem and

after CleanUp a DTR-drop is performed! Do not use any 'metacommands'

like '|' for carriage return or similar.

POSTINIT <modem command>

If ANSW finishes in a off-hook state ('quiet' or calling tone

detected), ANSW sends AT+FCLASS=0 to modem followed by all defined

<modem command>. Multiple entries are possible. Be carefully in

selecting modem commands. Normally, the same commands should be used

as in mailers 'Answer' string, EXCEPT ATA command, which is sent IN

ANY CASE automatically after PostInit regardless if anything is defined

or not! Do not use any 'metacommands' like '|' for carriage return or

similar.

Example for enabling adaptive autoanswer (fax class 2 / data)

after ANSW:

PostInit AT+FCR=1

PostInit AT+FAA=1

PREINIT <modem command>

Just befor ANSW switches modem to voice mode, modem command is sent

to modem. Multiple entries are possible, recommended is ATE0 to

disable echo from modem. Do not use any command like ATA, ATD or ATH!

Do not use any 'metacommands' like '|' for carriage return or similar.

INIT <modem command>

After ANSW switches modem to voice mode, modem command is sent

to modem. Multiple entries are possible, recommended is ATE0 to

disable echo from modem. Do not use any command like ATA, ATD or ATH!

Do not use any 'metacommands' like '|' for carriage return or similar.

MINMSGLENGTH <bytes>

By default, voice message less than 6000 bytes is NOT stored. You can

select any other value here.

MENUFILE <file name>

If user selects far inquiry, this message is played. Should contain

anything like:

'Far inquiry! Hit ZERO to cancel, ONE for all new messages, TWO for

all messages and THREE to delete all messages',

or in german:

"Fernabfrage! NULL fuer Abbruch, EINS fuer alle neuen Anrufe, ZWEI

fuer alle bisherigen Anrufe, DREI zum Loeschen aller Anrufe".

ANSW is hardcoded to this four digits (0, 1, 2, and 3)!

This entry is mandatory!

GREETINGFILE <file name>

This voice file is played when ANSW becomes activ (after PreListen).

Should contain anything like:

'Hello, there is <your name>, please speak after the beep tone',

or in german:

'<Name>, hinterlassen sie mir bitte nach dem Signalton eine

Nachricht'.

This entry is mandatory!

ENDRECFILE <file name>

This voice file is played after successful storage of callers message.

Should contain anything like:

'Your call is recorded',

or in german:

'Ihr Anruf wurde aufgezeichnet'.

This entry is mandatory!

CALLBACKFILE <file name>

If callback feature is enabled, this voicefile is played in front of

further actions. Should contain anything like:

'Announcement of incoming call, please enter password',

or in german:

"Verstaendigung ueber einen eingegangenen Anruf, bitte

Passwort eingeben".

This entry is mandatory!

FARINQUIRYPWD <digits>

Up to five (dialable) digits can be defined as a password for the

far inquiry feature.

CODINGSCHEME <value> [<WAV_SampleRate>]

Defines, how voice data is retrieved from modem and in which kind

of file it is stored. Value can be.

NATIV ..... voice data is saved exactly as received from modem

WAV ....... voice data is saved in 8 bit PCM RIFF WAV

WAVIMA .... voice data is saved in 4 bit IMA ADPCM RIFF WAV

WAVGSM .... voice data is saved in MS GSM 6.10 RIFF WAV (see GSM)

GSM ....... voice data is saved in GSM 6.10 format (see below)

ZVD ....... voice data is saved in Zyxel format

RVD ....... voice data is saved in Rockwell format (see below)

numerical . any codingscheme, the modem is capable (AT+VSM or AT#VBS).

If NATIV (or 'numerical') is selected, than voicefile contains a

16 byte header, depending on voicemode:

ZYXEL:

ZVD zvd = { {'Z','y','X','E','L'},2,0,0,0,0,0};

|                   | | | | | |

|                   | | | | | +-- samplerate [14]

|                   | | | | +---- reserved2 [12]

|                   | | | +------ compressed [10]

|                   | | +-------- reserved1 [8]

|                   | +---------- version [6]

|                   +------------ number (?) [5]

+-------------------------------- title [0]

ROCKWELL:

RVD rvd = { {'R','o','c','k','w','e','l','l'},0,0,0,0};

|                               | | | |

|                               | | | +-- samplerate [14]

|                               | | +---- reserved2 [12]

|                               | +------ compressed [10]

|                               +-------- reserved1 [8]

+---------------------------------------- title [0]

GSM:

GSM gsm = { {'G','S','M','V','D'},2,0,0,0,0,0};

|                   | | | | | |

|                   | | | | | +-- samplerate [14]

|                   | | | | +---- reserved2 [12]

|                   | | | +------ compressed [10]

|                   | | +-------- reserved1 [8]

|                   | +---------- version [6]

|                   +------------ number (?) [5]

+-------------------------------- title [0]

The 'samplerate' should always be 0 to indicate that mode specific

rate is used (Zyxel .. 9600, Rockwell .. 7200, GSM .. 8000 1/sec).

The values for 'number', 'reserved1' and 'reserved2' are ignored.

The value of 'compressed' depends on voicemode (modem):

Zyxel:

0 .... (AT+VSM = 1)  CELP (not supported)

1 .... (AT+VSM = 2)  2-Bit ADPCM

2 .... (AT+VSM = 3)  3-Bit ADPCM

3 .... (AT+VSM = 4)  4-Bit ADPCM

8 .... (AT+VSM = 30) NEW 3-bit ADPCM (not supported)

9 .... (AT+VSM = 40) 4-Bit DVI ADPCM

10 ... (AT+VSM = 80) 8-Bit A-Law PCM

11 ... (AT+VSM = 81) 8-Bit Mu-Law PCM

Not all modems support each mode, check with 'AT+VSM=?' before.

If NATIV or ZVD is selected, the mode is depending of DCE-DTE bitrate.

If 'numerical' is selected, and bitrate does not fit, mode is changed

to fit. ANSW (and its related utilities) are capable to convert

each format to each other.

Rockwell:

1 .... (AT#VBS = 2) 2-Bit ADPCM

2 .... (AT#VBS = 3) 3-Bit ADPCM

3 .... (AT#VBS = 4) 4-Bit ADPCM

4 .... (AT#VBS = 8) 8-Bit WAV PCM

Not all modems support each mode, check with 'AT#VBS=?' before.

If NATIV or RVD is selected, the mode is depending of DCE-DTE bitrate.

If 'numerical' is selected, and bitrate does not fit, mode is changed

to fit.

ATTENTION!!!

Due to 'Intellectual Properties', Rockwells ADPCM format can not be

build from other voice formats, therefore use only NATIV or RVD.

GSM:

Due to uncertain 'Intellectual Properties', GSMDLL.DLL is not

delivered. Makefile (CSet/VAC and EMX-GCC) and sources are free

available (GSMDLL.ZIP). Without GSMDLL, voice data in GSM format

can not be altered to other formats.

CALLBACKEVENT <day_spec.> <starttime> <endtime> <phonenumber> [<password>]

This keyword enables ANSWs callback feature. Each well received message

is sent to <phonenumber> if current day and time is in the range

specified. Multiple declarations are possible to dial to more than

one phonenumbers at different day/time.

<day_specifier> may be 'Mon', 'Tue', 'Wed', 'Thu', 'Fri', 'Sat' and

'Sun'. Also possible is 'Weekday' or 'Weekend' or 'All'. Any

combinations (delimited by blank character) are allowed:

CallbackEvent Weekend Mon 08:00 22:00 034598712 4711

CallbackEvent Weekday 07:30 17:00 789123 0815

Overlapping of eventdefinitions for the same phonenumber should be

avoided, because ANSW will than call more times:

CallbackEvent Week 08:30 16:30 54321

CallbackEvent Mon  15:00 21:00 54321

will overlap on monday from 15:00 to 16:30 ! Each call at this time

will result in TWO callbacks to 54321 !!!

Up to five (dialable) digits can be defined as a password for the

callback feature.

Callback feature is only possible if VOICELOG and VOICEIDX are NOT

set to OFF !

ACTIVITYEVENT <day_specifier> <starttime> <endtime>

If defined, activity period of ANSW can be set. Multiple declarations

are possible.

<day_specifier> may be 'Mon', 'Tue', 'Wed', 'Thu', 'Fri', 'Sat' and

'Sun'. Also possible is 'Weekday' or 'Weekend' or 'All'. Any

combinations (delimited by blank character) are allowed:

ActivityEvent Mon Tue 08:00 20:00

ActivityEvent Weekday 09:30 17:15

INBOUND <path to inbound>

If DESTNAME and DESTADDR are also defined, a netmail (????????.PKT)

is created each time a voice message is received or transmitted.

DESTNAME <string>

If this, INBOUND and DESTADDR is defined in ANSW.CFG, a netmail

is created and addressed to.

DESTADDR <FidoNet style address> [<paket password>]

Must be defined to create an announcing netmail. Address MUST be in

strict 4D format (zone:net/node.point). To address a node, point 0

must be used.

SRCNAME <string>

Used to form the 'From:' part of the announcing netmail.

Default is: "ANSW".

SRCADDR <FidoNet style address>

Normally this is the address of local system. Defaults to DESTADDR.

SUBJECTSENDVOICE <string or %>

Used in creation of announcing netmail on transmitted voice message.

If '%' is the first (or only) character, the filename of the sent voice

message (the first only, if more than one is sent) will be placed in

subject part of netmail.

Default is: "Announcement of outgoing voice message".

SUBJECTRECVOICE <string or %>

Used in creation of announcing netmail on received voice message.

If '%' is the first (or only) character, the filename of the stored

voice message (the first only, if more than one is sent) will be

placed in subject part of netmail.

Default is: "Announcement of incoming voice message".

FLAGDIR <path>

Path, were an activity flag can by deposited. ANSW looks, if it finds

<path>\NOANSW.FLG and if it finds it, it will terminate immediatly.

This filename is hardcoded, but could be patched in EXE or DLL.

EXITSEM <semaphor name>

Semaphor file to force mailer (BinkleyTerm or Cantaloup) to exit with

error level. Should be either BTEXIT (for Binkley) or CLEXIT (for

Cantaloup).

In combination with TASK and ExitErrorLevel a semaphor file is created.

EXITERRORLEVEL <level>

Errorlevel for mailer when forced to exit by semaphor.

ENDGREETINGTONE <duration in ms> <frequence1> [<frequence2>]

Can be used instead of fix programmed 'EndGreetingTone' (1000 880 0).

Format in CFG is:

EndGreetingTone 1000 880 0

Multiple entries are supported (to play 'music'), use 0 for <frequence>

to simulate silence.

This option is preliminary and may be altered in future to

GENTONE <purpose> <duration> <frequence1> [<frequence2>]

FRECCFG <cfg for FREC> [<commandline parameters for FREC>]

If this is configured, ANSW calls FREC directly if it detects a

fax calling tone.

FAESPECIAL

Some modems on the market have a certain behaviour, if used in

adaptiv autoanswer and fax class 1 or 1.0 (AT+FAE=1). If modem

detects a fax call it responses with FAX and CONNECT to indicate the

successful session establishment. If modem detects a data call, it

responses with DATA and reenters the command mode. So software has to

send ATO to modem to establish the data session which is triggered by

this parameter.

USROBOTICS

See FAESPECIAL, if VOICEMODE USR is used (experimental), than this

keyword should also be set.

CIDSTRING

Obviously. there exists some modems on the market, which will report

callers phonenumber in different forms. from the fact, that there is

no real good documentation available, a free defineable string can

be used to filter out some data from a modemresponse.

Let's expect, that modem responses with "CALLER ID 98761234", than a

definition of

CIDSTRING CALLER ID

will yield " 98761234" in VOICE.LOG 'DstTSI' (use EDIT view whole

record).

Attention!!! Absolutely NOT TESTED !!!

2) COMMANDLINE OPTIONS

-rec

ANSW is set to recording mode. You can record any voice file by

calling your modem with a telephon set. Voice file is saved as

ANSWnnnn.??? in VoicePath (VOICE.IDX and VOICE.LOG is NOT used).

-NoRecord

ANSW will play only greeting message and listen for calling tones. No

message from caller is collected.

-Task<n>

Where <n> specifies the task number to be used. See also [TASK <n>]

and [COMMON] keywords in configuration file. If this parameter is

not used the common task will be used.

-c<config file>

Default ANSW assumes the configuration file is named ANSW.CFG and

located in the current directory. The '-c' command line parameter

can be used to define an other path and file name.

-o<voice file>

Input file for playback. More that one '-o' parameter is allowed in

a single command line. Wildcards (*, ?) also are allowed. Extension

".SQC" is reserved for testing tone sequences.

-p<com port handle>

If this parameter is used, ANSW assumes it is called by a mailer or

other front end. Note: The port handle is not the same as

the port number (e.g. COM1)!

-f

Internally used to force callback in a command script.

-Subclass

This 'switch' enables a tricky way to STOP action of feature DLL

immediatly. No warrenty for proper function!

In fact, Cantaloups 'Transfer Window', which is selected by the content

of "Feature DLL" in windows headerline, is 'subclassed' and if keystroke

of ESC is detected, a 'break_request' is sent to DLL.

If there are any 'unpredictable results' to watch, immediatly hit

Ctrl-Alt-Del (or press RESET button of machine), because if DLL or mailer

is killed by some mechanism, synchronisation of window procedures is LOST.

-WatchDog<seconds>

Change internal watchdog interval (default 300 sec) to other values

or even disable function (0 sec).

ATTENTION!

Watchdog function not fully tested. If any unexpected watchdog

timeout occurs, please inform author (with complete LOG) and disable

feature by '-WatchDog0'.

-------------------------------------------------------------------------------

HISTORY OF ANSW:

Version number is normally in the form X.YYZ, where

X ... major version number

YY .. minor version number

Z ... a single character to distinguish between minor updates

Major version number increments, if either minor number overflows or

product is completely redesigned.

Minor version number increments mainly, if conceptional corrections (new,

documented keywords or other behaviour) are done or heavy bugs are removed.

The single character (a - z) changes if any corrections in source are

done. This correction are normally tiny, but not always ...

-------------------------------------------------------------------------------

version 1.14

Keyword 'VOICEVOLUME <value>' in CFG is removed and no longer valid!

Will be replaced in future by RECORDGAIN and PLAYBACKGAIN which is now

under study.

Keyword 'VCONSENSITIVITY <val>' in CFG is removed and no longer valid!

Keywords 'RECACTSPK <val>' and 'PLAYACTSPK <val>' are removed and no longer

valid.

Will be replaced by INPUTDEV and OUTPUTDEV in conjunction with

RECORDGAIN and PLAYBACKGAIN.

Version 1.13:

Construction of commandline switches/options is complete new designed.

New form of supplying options is now:

-x argument, where '-' is mandatory, 'x' is the letter, which describes

the option and 'argument' is no, one or more arguments for the option.

Version 1.12:

Saving and retrieving of voice data is completly new designed, so

ANSW will store data in a chooseable format (see CODINGSCHEME above).

New PCM2WAV.DLL was created to fullfill this approach. Not all

desireable function are enabled (GSMDLL.DLL) and/or are forbidden by

'Intelectual Property' (ADPCMDLL.DLL). It is highly recommended, to

use 'CODINGSCHEME NATIV'.

Experimental implementation of USR Message Modem. Use only

carefully and with authors help.

New keyword QUIETONDTMF and BUSYONDTMF. Experimental, do not use

without author instructions.

Version 1.11:

First approach to implement modern Rockwell Chipset based functions.

Missing correct documentation makes behaviour unpredictable in some

cases. Obviously, Rockwell Chipset based voice modems change their

behaviour in dependance of firmware versions also!

New keyword 'CIDSTRING', which will not give any results as expected.

New keyword 'FEASPECIAL', which should be used if Fax Class 1

adaptiv autoanswer is used with some brand of Rockwell Chipset based

modems. Will not work as expected in some cases.

Version 1.08 - 1.10:

Not documented

Version 1.07:

ErrorLevel redesigned.

Version 1.06:

Change of meaning of CFG-keyword STOPONDTMF.

New keywords:

IGNOREONDTMF

FAXONDTMF

DATAONDTMF

EXTONDTMF

RECONDTMF

SPAWNONDTMF

PLAYONDTMF

only the first three are supported yet.

Version 1.02:

For some features (CallBack, Far Inquiry) a VOICE.LOG and VOICE.IDX

is invented. Limited support for userdefined 'tones'. As a test, only

EndGreetingTone is implemented. To test, generate a simple ASCII file

with extension .SQC, containing in each line a single tone command:

<duration> <frequence1> <frequence2>

PASSWORD_BAD

500 1000 0

200 0 0

1000 800 0

FAR_INQUIRY_PLAYBACK_INTRO

200 880 0

200 749 0

200 605 0

While it is really possible, to define 'dual tones', this should be

avoid because modem (or any line exchange) could interpret it as a

DTMF command!

New Keywords:

RINGCNT

VOICELOG

VOICEIDX

ENDGREETINGTONE

FRECCFG

Change in Keyword:

RECORDPATH --> VOICEPATH

CALLBACKEVENT ... now place for CallbackPassword

Keywords not longer supported:

RINGCNTMAIL

RINGCNTNOMAIL

CALLBACKPWD

Version 1.01:

Large parts of code (esp threadhandling, memoryallocation, exception-

handling and CFG/CMD-handling) was redesigned. All parts of code, which are

also in FSEND and FREC were put to FAXPROC, which is from now on a dynamic

link library (FAXPROC.DLL). Linking of C-runtime is never more statically,

instead, C-runtime is linked dynamically (at loadtime) for ANSW, FAXPROC and

IMGPROC. So, the overall amount of codesize was reduced. Some new keywords

in CFG are available:

INBOUND

SUBJECTRECVOICE

SUBJECTSENDVOICE

DESTADDR

DESTNAME

SRCADDR

SRCNAME

INITFIFO

Finally, the COM port open methode is now checked in FAXPROC. Only if COM

is opend in READWRITE and SHARE_DENYREADWRITE, work is continued! I hope

some problems (when called from a mailer, which uses a patched MAXCOMM to

open COM in SHARE_DENYNONE mode) are solved now. Also SIO.SYS in

'share mode' should never more work!

Maybe some users will disagree, but 'sharing' a communication port is NOT

usefull on a multitasking environment! Each application MUST have unique

access to its own resources.

Version 1.00:

Only internal test version and distributed only in a very early state.

Previous versions of ANSW and ANSWER (both by Bernhard Seidl and me)

It was decided, to split the work of both Bernhard and me. While

Bernhard distributes his work as ANSWER, me application is named from

now on ANSW. Differences between ANSWER and ANSW are therefore possible

but CFG should (almost) match for both products. Sourcecode of both

products is exchanged, so features of one will come soon in the other.

Bernhards ANSWER is primarly a PM application (for standalone use) and

ANSW is primarly designed as Feature-DLL for Cantaloup. There also

exists an ANSW.EXE, which may work as a voice and fax frontend and is

able to 'record' and 'playback' voice files.

Please feel free to contact the autor (see SUPPORT.DOC)