LANtastic (R) for TCP/IP v1.1 Information File (LTCP.DOC)

---------------------------------------------------------

(C) Copyright 1992 Artisoft, Inc. (Revised 10/1/93)

Password in TCPSTART.BAT

========================

If you are running TCPSTART.BAT from a TCP/IP workstation, in order

to avoid hardcoding your password in this batch file, you need to:

- log onto your TCP/IP management server first before running

TCPSTART.BAT

or

- use the LANtastic automatic log-in feature, for example:

NET USER $NAME ^"Enter password: "

NET USER is by default enabled with your redirector name as your

default user name.

Sharing information on the centralized TCP/IP management server

===============================================================

The centralized TCP/IP management server approach makes it easier for a

central network administrator to manage each user's configuration on the

network.  All users running LANtastic for TCP/IP from a centralized server

will be sharing the following files:

*  LANTCP.CFG - Stores configuration parameters for the TCP/IP kernel,

such as default terminal emulators, local time zone and IP addresses.

LANTCP.CFG can be modified using LTCP-MGR.EXE.

*  LTCP.CFG - Stores default execution path and command line parameters

for some of the TCP/IP applications like TELNET, FTP, etc.  LTCP.CFG can

be modified using LTCP.EXE.

*  HOSTS - The HOSTS file is a simple database that allows you to assign

user-friendly names (or aliases) to the various hosts on the network, even

if you do not have a domain name resolver on your network.  Use your

favorite text editor to create and modify the HOSTS file.  Refer to

Appendix C "Creating a Hosts File" in the manual for details.  A sample

HOSTS file looks like this:

89.0.0.101      ASFTUNX

89.0.0.105      ASFTVAX

*  USERS - The USERS file defines the log-in name, file read and write

permissions and passwords for each user that you intend to allow access to

your PC when you are running the FTP daemon (FTPD).  Use your favorite

text editor to create and modify the USERS file.  A sample USERS file

looks like this:

maria r hello8

fred w lognu

susan rw phoebe

If you do not intend to share any of the above files with other users, you

can set up your own TCP/IP management server.  Please refer to the Multiple

TCP/IP Server Installation section in the LANtastic for TCP/IP User's Manual

for details.

Setting the Write Size for Mount

==========================

If you experience disconnects or lockups at DOS when copying files back and

forth across a mounted drive using NFS, you may want to experiment with

lower read/write sizes on the MOUNT command line.  The default for read size

is 1k while the default for write size is 8k.  A lower write size, e.g. 1024,

2048 or 4096 is recommended in this case.

Removing the TCP/IP Kernel

================================

All TSRs involved in the LANtastic for TCP/IP product can be unloaded from

memory, if needed.  This will be useful when you want to reload the TCP/IP

kernel after making changes to the LANTCP.CFG file through LTCP-MGR, or just

to save memory when you are not connecting to a foreign host.  The TSRs and the

command to remove each is listed below:

ARTITCP (contains licensing info)                   ARTITCP/REM

AITCP (part of the TCP/IP kernel)                   ASTCPRM -d

(Caution: AITCP.EXE cannot be loaded high.)

ASTCP (the other part of the TCP/IP kernel)         ASTCPRM

NFS (the network file system redirector)            ASTCPRM

(ASTCPRM will remove both ASTCP and NFS at the same time).

(ASTCPRM -d removes ASTCP, NFS, and AITCP.)

Using Western Digital or SMC 8013-series adapters

===========================================================

There is a known problem with the 8013-series chipset: under heavy

load, the adapter may need to be re-initialized.  The WD8003 driver

provided by Arisoft does NOT compensate for this problem, and so users

with 8013-series adapter cards may experience disconnects.

To avoid this difficulty, you may use the NDIS driver provided by SMC

with the adapter.  This driver DOES compensate for the known problems of

the adapter card.  You will also need to download AINDIS.ZIP from Artisoft's

BBS (602 884 8648) and follow the instructions for installing NDIS on your

network.  The procedure for running LANtastic for TCP/IP will not change,

you may continue to run TCPSTART.BAT after REDIR is loaded exactly as before.

If you previously used WG-NDIS.EXE

=============================================================

Some NDIS users may have been using WG-NDIS.EXE to run LANtastic for TCP/IP

directly on top of the NDIS stack, without loading AITCP.EXE.  This is no

longer possible due to some load-order problems involving WG-NDIS.  You may

still run LANtastic for TCP/IP with NDIS, but the load order should be as

follows (For the precise syntax, see your NDIS documentation):

CONFIG.SYS:     device=protman.dos /i:c:\ai-ndis

device=smcmac.dos

STARTLAN.BAT:   AI-NDIS

AILANBIO

REDIR machine

ARTITCP

AITCP

ASTCP

This will allow AITCP to continue to make use of the netbios calls provided

by AILANBIO, which in turn will communicate through AI-NDIS to the adapter

(MAC) driver.

WINSOCK API support

============================================================

The WINSOCK.DLL provided with this release supports all standard

WINSOCK calls.  The documentation for this standard is provided by

Microsoft and will also be made available from Artisoft's BBS.

10/01/93   LFTCP/IP  README.TXT